| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20-28th Apr. 2021 | Document: JVET-V0084v3 |
| --- | --- |

| Title: | CE-related: On Rice parameter selection for regular residual coding (RRC) at high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution describes modifications to the method for the selection of Rice parameters for regular residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains when residual coefficient values become larger but do not affect version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. A further modification is described which alters the template function used to calculate a clipped sum of neighbouring coefficients used during Rice parameter evaluation. Results are provided both with and without this further modification.

Using the high bit depth, high bit rate CTC the following AI results were obtained for the low QP range against the CE anchor:

With template modification:

PQ:			-0.29%/-0.18%/-0.18%

HLG:		-0.56%/-0.34%/0.35%

SVT-12:	-1.67%/-1.26%/-1.27%

SVT-16:	-2.94%/-2.09%/-2.07%

Without template modification

PQ:			-0.26%/-0.16%/-0.16%

HLG:		-0.53%/-0.33%/-0.34%

SVT-12:	-1.65%/-1.26%/-1.26%

SVT-16:	-2.94%/-2.10%/-2.08%

# Introduction

In VVC version 1[1], the range of Rice parameters that may be used for coefficients during regular residual coding is limited to the range of 0 to 3 (inclusive). This range has been found to be suitable for both 8 and 10 bit data. However, as bit depths increase and QPs become lower, the size of each coded coefficient becomes larger. For high bit depth content at QPs below 0 it has previously been shown that VVC coding efficiency is below that of HEVC[2].

Previous work[3] has described an approach that increases coding efficiency for high bit depths and low QPs to match or exceed HEVC. This proposal represents a re-examination and refinement of this technique which aims to simplify the complexity of the original technique whilst attempting to maintain, or where possible, increase the observed gains. This proposal contains two possible formulations, the difference between which is a single additional modification to the template function used in the Rice parameter calculation.

The modification described here results in gains for 12 and 16-bit content over both the original technique and the technique already present in VTM and VVC version 1. The modification provides backwards compatibility with VVC version 1 as the existing Rice coding method can be considered as a subset of the modification and is controlled by a parameter sent in the SPS.

# Algorithm description

The algorithm in this proposal, and the one on which it is based, impact entropy coding and, in particular, the selection of Rice parameters for Golomb-Rice coding. The algorithm can be split into a series of components as follows each of which has been re-examined for this proposal:

- A coefficient size estimation function, which is backed by a series of counters, and an index function which selects the counter used for each coefficient.

- An update function for the selected counter.

- A modification to the current algorithm for computing the clipped sum of adjacent nearby coefficients

- A modification to the current algorithm for selecting Rice parameters.

## Coefficient size estimation

The technique requires an estimation of the log2 size of coefficients being coded. This is derived in a similar manner to Rice parameter estimation in HEVC (RExt) using a series of counters. The counters are mapped to a two dimensional array, with the first dimension being the component type. The second dimension of the array is an index function coded as shown in Table 1 giving a total of 4 counters per component type. This index function differs from the original algorithm in that the original proposal required a distance from the top left (DC) coefficient to be computed, whereas this updated algorithm uses the sub-block index. The counter selection for either can be implemented as a piece of logic (as described in the draft text changes) or by using a lookup table which requires 48 2 bit entries (as implemented in the VTM modification).

| Index | Description | 16x16 transform block |
| --- | --- | --- |
| 0 | Transform-skip (Required for lossless coding only) |  |
| 1 (Red) | DC coefficient in top left sub-block |  |
| 2 (Blue) | Remaining coefficients in top left sub-block |  |
| 3 (Yellow) | All other coefficients |  |

Table 1. Index function derivation

## Counter update function

Each counter is set to a default value computed from the slice QP at the beginning of every slice.  The counters are handled in the same way as CABAC contexts, thus allowing the values to be rewound during the encoder search. This also provides support for wave-front parallel coding as each wave-front copies the counters before starting.

In the previous proposal, a counter was updated each time a coefficient on the leading diagonal of the coefficient block was coded using Golomb-Rice. In the new proposal, this has been reduced to two components at fixed scan order positions per sub-block plus the DC coefficient in the top left sub-block, and is now done using the complete coefficient value rather the remainder coded with Golomb-Rice. This has allowed the update to be moved outside the coefficient coding passes. This removes the coupling between updates and Golomb-Rice coding and allows additional flexibility for implementations, as the results of the update process are not required until the start of the next transform unit.

The update process has been simplified in the new proposal as shown below. Previously three comparisons were used, one with zero and up to two with computed thresholds and in addition a clip to zero was used to stop the counter going negative. This technique with upper and lower thresholds was taken from HEVC, but was proven experimentally to be sub-optimal for VVC. This has been reduced to one comparison and the clip is now done once per TU rather than once per coded coefficient.

count += (abs(value) >= threshold) ? increment : -decrement;        // increment =4, decrement = 1

The threshold used in the update process is updated once per transform unit as below:

count = max(0, count);                                                                       // clip now once per TU

int rice = count  >> log2Divisor;				                            // log2Divisor = 4

threshold = (Clip3(12, 15, 16 - (rice >> 1)) << rice) >> 3;

The new threshold calculation includes a gradual relative reduction to model the effect of the reducing effect of the “greater than” flags used in coefficient coding.

## Modification to clipped sum of coefficients

For regular residual coding (RRC), VVC (section 9.3.3.2) defines locSumAbs as the sum of the absolute values of 5 previously coded coefficients below and right of the coefficient, less an offset which relates to the proportion of the coefficient that is coded with flags, and then clips this value to the range 0 to 31. This technique works well if clipping is only occasionally used, but as average coefficient values increase the number of times clipping is required increases.

In the modification described in the original proposal, a shift right is introduced as below (modification in green).  The purpose of this shift is to reduce the magnitude of the sum and thus reduce the amount of clipping. The size of the shift is described in the following section.

locSumAbs = Clip3( 0, 31, Max( locSumAbs − baseLevel * 5, 0  ) >> shift )

This modification is carried forward into the new proposal, but in addition a further change is introduced for the second formulation of this proposal. To understand the formulation consider the template function calculation applied to two 4*4 transform blocks.

When calculating the sum of five neighbouring coefficients, the yellow coefficient in the transform block on the left has 5 coefficients available. However, for the yellow coefficient on the right, only 3 coefficients are available. In VVC version 1 and the first formulation of this proposal the value of 0 is substituted. However for the second formulation the value used is an approximation of the expected value (1 << shift) which effectively cancels out the missing coefficient in the locSumAbs calculation.

## Rice parameter calculation

In VVC Version 1, RRC selects the Rice parameter by using the value returned by the template function described in section 2.3 as an index into a lookup table which gives the Rice parameter.

In the proposed modification one of two shift values can be used, one when a coefficient is coded using a parity flag and one when a coefficient is coded without a parity flag. Both of these values are based on the value of count derived as described in section 2.2.

int riceValueX2 = count >> log2DivisorMinus1

shiftWithParity  = min(12, max(0, riceValueX2 - 5) >> 1);

shiftNoParity  = min(12, max(0, riceValueX2 - 2) >> 1);

The shift is clipped to a maximum of 12 to support an extended table of Rice coding costs specified for dependent quantisation.

The computation of the two shift values per counter is done at the same time as the values of the thresholds described in section 2.2, i.e. once per transform block.

The selection of the Rice parameter can then be described using the pseudo code below:

int idx = cctx.templateAbsSum(scanPos, absCoeff, baseLevel, shift);

// compute template fn using sum shifted right by shift modifier

int rice = g_auiGoRiceParsCoeff[idx] + shift;		// add shift modifier onto result from lookup table

Thus the selection of the Rice parameter may be seen as a modified version of the existing VVC techniques rather than a new technique.  This modification ensure that when estimated coefficient values are larger a more appropriate range of Rice parameters is selectable. Unlike simply swapping to a technique based purely on size estimation, this modification introduces a hybrid technique which attempts to extend the operating point of the existing VVC technique.

## SPS control flag

A flag has been added to the SPS to allow the effect of the modification to be disabled. When this occurs operation returns to that specified in VVC version 1.

# Results

All results reported here are compared with anchors generated using the CE anchor (VTM12.0 with Rice coding modifications from CE-1.1 and CE-2.1) and the high bit depth, high bit rate and high frame rate CTC[4].

## Without template modification

### Low QP range

#### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.25% | -0.15% | -0.14% | -0.29% | -0.15% | -0.15% | 98% | 98% |
| PQ422 | -0.27% | -0.17% | -0.18% | -0.29% | -0.17% | -0.19% | 101% | 101% |
| Overall | -0.26% | -0.16% | -0.16% | -0.29% | -0.16% | -0.17% | 100% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.07% | -0.06% | -0.07% | -0.07% | -0.06% | 102% | 101% |
| PQ422 | -0.04% | -0.05% | -0.08% | -0.04% | -0.05% | -0.08% | 99% | 99% |
| Overall | -0.06% | -0.06% | -0.07% | -0.06% | -0.06% | -0.07% | 101% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.05% | -0.04% | -0.05% | -0.06% | -0.04% | -0.04% | 103% | 103% |
| PQ422 | -0.06% | -0.03% | -0.06% | -0.06% | -0.04% | -0.05% | 101% | 101% |
| Overall | -0.05% | -0.04% | -0.05% | -0.06% | -0.04% | -0.05% | 102% | 102% |
| Overall PQ | -0.12% | -0.09% | -0.10% | -0.13% | -0.09% | -0.09% | 101% | 100% |

#### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.54% | -0.32% | -0.34% | 102% | 100% |
| HLG422 | -0.53% | -0.34% | -0.35% | 100% | 100% |
| Overall | -0.53% | -0.33% | -0.34% | 101% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.06% | -0.07% | 104% | 105% |
| HLG422 | -0.09% | -0.08% | -0.07% | 102% | 103% |
| Overall | -0.08% | -0.07% | -0.07% | 103% | 104% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.06% | -0.06% | 100% | 101% |
| HLG422 | -0.09% | -0.06% | -0.09% | 100% | 101% |
| Overall | -0.08% | -0.06% | -0.07% | 100% | 101% |
| Overall HLG | -0.23% | -0.15% | -0.16% | 101% | 102% |

#### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.94% | -2.10% | -2.08% | 108% | 123% |
| SVT12 | -1.65% | -1.26% | -1.26% | 103% | 106% |
| Overall | -2.29% | -1.68% | -1.67% | 105% | 115% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.46% | -1.44% | -1.43% | 117% | 121% |
| SVT12 | -0.78% | -0.64% | -0.63% | 101% | 106% |
| Overall | -1.12% | -1.04% | -1.03% | 109% | 113% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.37% | -1.32% | -1.31% | 116% | 120% |
| SVT12 | -0.74% | -0.56% | -0.57% | 101% | 105% |
| Overall | -1.05% | -0.94% | -0.94% | 108% | 113% |

### Standard QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.07% | 0.01% | 0.00% | -0.09% | -0.04% | 0.00% | -0.19% | 0.00% | 101% | 102% |
| Class H2 |  |  |  |  |  | -0.02% | 0.05% | 0.02% | 102% | 102% |
| Overall | 0.07% | 0.01% | 0.00% | -0.09% | -0.04% | -0.01% | -0.07% | 0.01% | 101% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.02% | 0.00% | 0.00% | -0.04% | -0.10% | 0.00% | -0.03% | -0.08% | 100% | 100% |
| Class H2 |  |  |  |  |  | -0.01% | 0.00% | 0.05% | 100% | 100% |
| Overall | -0.02% | 0.00% | 0.00% | -0.04% | -0.10% | -0.01% | -0.02% | -0.01% | 100% | 100% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  |
| PQ444 | 2.5 | 2.6 | -1.85% | 3.0 | 3.1 | -2.35% | 3.0 | 3.1 | -2.27% |
| PQ422 | 2.3 | 2.4 | -1.96% | 2.8 | 2.8 | -2.38% | 2.8 | 2.9 | -2.29% |
| Overall | 2.4 | 2.5 | -1.90% | 2.9 | 3.0 | -2.36% | 2.9 | 3.0 | -2.28% |
| Enc Time[%] | 101% |  |  | 105% |  |  | 108% |  |  |
| Dec Time[%] | 117% |  |  | 106% |  |  | 106% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  |
| HLG444 | 1.7 | 1.8 | -3.30% | 1.9 | 1.9 | -3.69% | 1.9 | 1.9 | -3.62% |
| HLG422 | 1.6 | 1.7 | -2.95% | 1.9 | 1.9 | -3.48% | 1.9 | 1.9 | -3.40% |
| Overall | 1.7 | 1.7 | -3.13% | 1.9 | 1.9 | -3.58% | 1.9 | 1.9 | -3.51% |
| Enc Time[%] | 97% |  |  | 131% |  |  | 141% |  |  |
| Dec Time[%] | 133% |  |  | 115% |  |  | 118% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  |
| SVT16 | 1.2 | 1.2 | -1.70% | 1.2 | 1.2 | -0.57% | 1.2 | 1.2 | -0.57% |
| SVT12 | 1.2 | 1.3 | -3.21% | 1.3 | 1.3 | -1.82% | 1.3 | 1.3 | -1.75% |
| Overall | 1.2 | 1.2 | -2.46% | 1.2 | 1.3 | -1.20% | 1.2 | 1.3 | -1.16% |
| Enc Time[%] | 112% |  |  | 180% |  |  | 173% |  |  |
| Dec Time[%] | 150% |  |  | 124% |  |  | 124% |  |  |

## With template modification

### Low QP range

#### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.28% | -0.16% | -0.16% | -0.31% | -0.16% | -0.17% | 98% | 99% |
| PQ422 | -0.30% | -0.20% | -0.21% | -0.33% | -0.20% | -0.21% | 100% | 101% |
| Overall | -0.29% | -0.18% | -0.18% | -0.32% | -0.18% | -0.19% | 99% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.04% | -0.03% | -0.08% | -0.05% | -0.04% | 102% | 100% |
| PQ422 | -0.06% | -0.06% | -0.08% | -0.06% | -0.06% | -0.07% | 100% | 99% |
| Overall | -0.06% | -0.05% | -0.05% | -0.07% | -0.05% | -0.06% | 101% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.04% | -0.05% | -0.07% | -0.04% | -0.05% | 103% | 103% |
| PQ422 | -0.06% | -0.04% | -0.06% | -0.06% | -0.04% | -0.05% | 101% | 102% |
| Overall | -0.06% | -0.04% | -0.05% | -0.07% | -0.04% | -0.05% | 102% | 102% |

#### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.55% | -0.32% | -0.33% | 102% | 101% |
| HLG422 | -0.56% | -0.35% | -0.36% | 100% | 101% |
| Overall | -0.56% | -0.34% | -0.35% | 101% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.07% | -0.02% | -0.02% | 102% | 104% |
| HLG422 | -0.09% | -0.03% | -0.03% | 101% | 102% |
| Overall | -0.08% | -0.02% | -0.02% | 101% | 103% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.08% | -0.02% | -0.02% | 101% | 102% |
| HLG422 | -0.06% | -0.01% | -0.03% | 101% | 102% |
| Overall | -0.07% | -0.02% | -0.02% | 101% | 102% |
| Overall HLG | -0.24% | -0.13% | -0.13% | 101% | 102% |

#### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.94% | -2.09% | -2.07% | 107% | 124% |
| SVT12 | -1.67% | -1.26% | -1.27% | 102% | 106% |
| Overall | -2.31% | -1.67% | -1.67% | 105% | 115% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.43% | -1.40% | -1.39% | 117% | 122% |
| SVT12 | -0.73% | -0.59% | -0.59% | 103% | 108% |
| Overall | -1.08% | -0.99% | -0.99% | 110% | 115% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.34% | -1.28% | -1.27% | 115% | 121% |
| SVT12 | -0.69% | -0.51% | -0.52% | 101% | 107% |
| Overall | -1.02% | -0.90% | -0.90% | 108% | 114% |

### Standard QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.09% | -0.03% | -0.02% | -0.06% | -0.06% | -0.03% | -0.20% | -0.07% | 102% | 102% |
| Class H2 |  |  |  |  |  | 0.01% | -0.03% | 0.35% | 102% | 102% |
| Overall | 0.09% | -0.03% | -0.02% | -0.06% | -0.06% | -0.01% | -0.11% | 0.14% | 102% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.04% | 0.00% | 0.00% | 0.01% | -0.12% | 0.00% | -0.01% | -0.12% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0.00% | 0.03% | 0.06% | 100% | 100% |
| Overall | -0.04% | 0.00% | 0.00% | 0.01% | -0.12% | 0.00% | 0.01% | -0.03% | 100% | 100% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  |
| PQ444 | 2.5 | 2.6 | -1.91% | 3.0 | 3.1 | -2.47% | 3.0 | 3.1 | -2.39% |
| PQ422 | 2.3 | 2.4 | -2.01% | 2.8 | 2.9 | -2.48% | 2.8 | 2.9 | -2.39% |
| Overall | 2.4 | 2.5 | -1.96% | 2.9 | 3.0 | -2.48% | 2.9 | 3.0 | -2.39% |
| Enc Time[%] | 101% |  |  | 106% |  |  | 108% |  |  |
| Dec Time[%] | 116% |  |  | 109% |  |  | 106% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  |
| HLG444 | 1.7 | 1.8 | -3.33% | 1.9 | 1.9 | -3.75% | 1.9 | 1.9 | -3.67% |
| HLG422 | 1.6 | 1.7 | -2.97% | 1.9 | 1.9 | -3.53% | 1.9 | 1.9 | -3.45% |
| Overall | 1.7 | 1.7 | -3.15% | 1.9 | 1.9 | -3.64% | 1.9 | 1.9 | -3.56% |
| Enc Time[%] | 97% |  |  | 132% |  |  | 140% |  |  |
| Dec Time[%] | 134% |  |  | 117% |  |  | 119% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  | VTM_U_CE | RRC |  |
| SVT16 | 1.2 | 1.2 | -1.62% | 1.2 | 1.2 | -0.49% | 1.2 | 1.2 | -0.48% |
| SVT12 | 1.2 | 1.3 | -3.16% | 1.3 | 1.3 | -1.77% | 1.3 | 1.3 | -1.69% |
| Overall | 1.2 | 1.2 | -2.39% | 1.2 | 1.3 | -1.13% | 1.2 | 1.3 | -1.09% |
| Enc Time[%] | 111% |  |  | 180% |  |  | 173% |  |  |
| Dec Time[%] | 147% |  |  | 124% |  |  | 121% |  |  |

# Discussion of Results

The results presented above show that the BD-rate gains achieved are in excess of those published for CE-1.6[2] both with and without the additional template modification. The results also show that the additional gain achieved by the template modification may not justify the additional complexity required.

# Estimate of Complexity

This section attempts to give an estimate of the additional complexity of the proposal for both hardware and software. The additional complexity may be separated into five sections: memory requirements, coding a value, updating a counter, selecting a parameter set and updating a parameter set as follows.

### Memory requirements

8*10 bit memory for storage of counters

### Coding a value (every coefficient)

1 * shift (variable size)

1 * add

In addition if the template modification is used it is estimated this would use 12 adds and 1 variable size shift per 4x4 sub-block.

### Updating counters (maximum of 3 times in a 4x4 sub-block)

1 * conditional

Plus 2 times per sub-block on leading diagonal:

1 * conditional

3 * add

Plus if sub-block is DC sub-block:

1 * conditional

1 * add

### Updating LUT (every sub-block)

2 * conditional

16*2 bits of memory

### Updating parameter sets

1 * conditional

Plus up to 3 times per block, 2 if block is 4x4:

3 * adds

4 * clips (min+max in specification could be replaced by Clip3 by moving fixed shift but min+max kept for clarity)

6 * shift (fixed size)

1 * shift (variable size)

Please note: the updates in v3 of this document are made to match the updated specification text in JVET-V0084-JVET-T2001v3.docx.

Without the template modification, using these figures the total instructions are:

- for a 4x4 TU, 32 + 11 + 2 + 17 = 62

- for a 8x8 TU, 128 + 22 + 8 + 25 = 183

# Conclusions

This contribution has described modifications to the method for the selection of Rice parameters for regular residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains when residual coefficient values become larger but do not affect version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. A further modification is described which alters the template function used to calculate a clipped sum of neighbouring coefficients used during Rice parameter evaluation. Results are provided both with and without this further modification.

It is proposed that the modifications be included for consideration alongside proposals within the CE for adoption into VVC version 2 and VTM.

# Patent rights declaration(s)

Sony Group Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, B. Bross, J. Chen, S. Liu and Y-K. Wang

[2] “High bit depth coding”, JVET-R0351, A. Browne, S. Keating and K. Sharman

[3] “CE-1.5, CE-1.6, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, JVET-U0050, A. Browne, S. Keating and K. Sharman

[4] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu