| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0100-v2 |
| --- | --- |

| Title: | AHG11: Deep In-Loop Filter with Adaptive Model Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Li Zhang Kai Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yue.li@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network-based in-loop filtering method wherein adaptive model selection is introduced. The proposed Deep in-loop filter with Adaptive Model selection (DAM) method is developed from the prior contribution JVET-U0068. Several changes are further introduced: migration of the algorithm to VTM-11.0, improved network structure, refined model candidates, adaptive inference size. Compared with VTM-11.0, the proposed method reportedly shows on average {8.79%, 22.54%, 22.90%}, {11.79%, 27.62%, and 27.26%}, and {10.85%, 27.60%, and 27.79%} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively.

Introduction

In this contribution, convolutional neural networks (CNN) are utilized to construct in-loop filters for reducing the distortion incurred during compression. The network structure is revised from the one presented in our prior contribution JVET-U0068 [1]. Similar to JVET-U0068, residual blocks are utilized as the basic module and stacked several times to construct the final network. As a further development from JVET-U0068, the presented model is equipped with more residual bocks but less feature maps in each residual block, leading to an increased representation capability with a similar model size. In addition, to deal with different types of content, individual networks are trained for different types of slices and quality levels. Moreover, some new features are introduced to further improve the coding performance.

Proposed method

The architecture and basic concept of model selection presented in section 2.1 and 2.2, respectively, are the same as our prior contribution JVET-U0068. And the new changes introduced in this contribution are summarized in section 2.3.

Architecture

The backbone of the proposed CNN filtering method is shown in Fig. 1. To increase the receptive field and reduce the complexity, the proposed method contains a convolutional layer with a stride of 2 at the beginning. After passing through this layer, the spatial resolution of feature maps reduces to the half of the input size along both horizontal and vertical direction. The feature maps output from the first convolutional layer then go through several sequentially stacked residual blocks. The last convolutional layer takes the feature maps from the last residual block as an input and produces 4 feature maps of N×N. Finally, a shuffle layer is adopted to generate the filter image whose spatial resolution is the same as the input to the CNN, i.e. 2N×2N. Other details related to the network architecture are illustrated as below:

- For all of convolutional layers, kernel size of 3x3 is used. For internal convolutional layers, number of feature maps is M (M=96). For the activation function, PReLU is used.

- Different groups of models are trained for I slice and B slice, respectively.

- When training the CNN filters for intra slices, prediction and partition information are additionally fed into the network.

- When training the CNN filters for inter slices, prediction information is additionally fed into the network.

Fig. 1. (a) Architecture of the proposed CNN filtering method. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Residual Block in (a)

Model Selection

In the proposed technique, each slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, which model from a candidate model list including three models could be further decided. To this end, individual models are trained with QPs in {17, 22, 27, 32, 37, 42}. Given a test QP for coding a sequence, denoted as q, the candidate model list includes models trained with QPs equal to {q, q-5, q-10}. The selection process is based on the rate-distortion cost at the encoder side. Indication of on/off control as well as the model index, if needed, are signalled in the bitstream.

Other New Features

- To enable a higher representation capability and maintain a similar model size, the number of residual blocks increases from 16 to 32 while the number of feature maps in each residual block decreases from 128 to 96.

- The granularity of the filter determination and the model selection is dependent on resolution and QP. In the proposed scheme, given a higher resolution and a larger QP, the determination and selection will be performed for a larger region.

- The candidate model list is different across different temporal layers. For a low temporal layer, the third candidate is replaced by the intra-slice NN filter model if the ratio of intra-coded samples is no smaller than a threshold. For a high temporal layer, the third candidate, i.e. the model corresponding to q-10, is replaced by the model corresponding to q+5.

- For all-intra configuration, model selection is disabled while on/off control is still preserved.

Inference

PyTorch is used for performing the inference of the proposed CNN filters in VTM. The network information in the inference stage is provided in Table 1 as suggested by [2].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 5.51M/model |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | ~22.3M/model，24 models in total |
|  | MAC (Giga) | 11541 (Input: 3840×2160) |
| Optional |  |  |
|  | Total Conv. Layers | 69 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 32×32, 64×64, 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. The DIV2K [3] and BVI-DVC [4] datasets are adopted to train the CNN filters of I slices and B slices, respectively. Different CNN models are trained to adapt to different QP points. The network information in the training stage is provided in Table 2 as suggested by [2].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 90 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-9.0, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is tested on top of VTM-11.0 [5] according to the common test conditions defined in [2]. Deblocking filtering and SAO are disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. Test results are shown in Table 3 ~ Table 5. Under AI configurations, the proposed method can bring 8.79%, 22.54%, and 22.90% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 11.79%, 27.62%, and 27.26% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring 10.85%, 27.60%, and 27.79% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 3. Performance of the proposed method on top of VTM11.0 (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.75% | -21.93% | -22.62% | 285% | 34763% |
| Class A2 | -12.43% | -26.33% | -26.52% | 265% | 31237% |
| Class B | -11.19% | -31.30% | -28.89% | 277% | 32571% |
| Class C | -12.81% | -28.26% | -29.26% | 214% | 31552% |
| Class E |  |  |  |  |  |
| Overall | -11.79% | -27.62% | -27.26% | 258% | 32447% |
| Class D | -14.33% | -30.02% | -30.64% | 204% | 28984% |
| Class F | -6.01% | -17.34% | -17.56% | 381% | 12782% |

Table 4. Performance of the proposed method on top of VTM11.0 (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -10.04% | -28.93% | -29.14% | 264% | 24287% |
| Class C | -12.10% | -28.82% | -29.09% | 214% | 27845% |
| Class E | -10.54% | -23.74% | -23.81% | 525% | 17450% |
| Overall | -10.85% | -27.60% | -27.79% | 292% | 23403% |
| Class D | -13.49% | -31.42% | -30.43% | 203% | 28112% |
| Class F | -6.44% | -18.09% | -17.27% | 379% | 11483% |

Table 5. Performance of the proposed method on top of VTM11.0 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -7.57% | -18.35% | -18.54% | 234% | 19512% |
| Class A2 | -7.82% | -22.65% | -22.44% | 172% | 16331% |
| Class B | -7.89% | -23.52% | -22.77% | 156% | 14257% |
| Class C | -9.28% | -21.78% | -24.25% | 126% | 9553% |
| Class E | -11.82% | -26.01% | -26.12% | 164% | 15761% |
| Overall | -8.79% | -22.54% | -22.90% | 163% | 14295% |
| Class D | -9.03% | -22.91% | -24.43% | 120% | 8993% |
| Class F | -4.55% | -13.58% | -11.70% | 125% | 5637% |

Conclusions

This contribution presents a CNN-based in-loop filtering method. The proposed CNN-based FAMS method shows very promising coding gains. It is proposed to further study CNN-based in-loop filters in the Ad-Hoc group.

References

- AHG11: Convolutional neural network-based in-loop filter with adaptive model selection, JVET-U0068, Y. Li, L. Zhang, K. Zhang.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-U2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).