| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0101 |
| --- | --- |

| Title: | AHG11: Conditional In-Loop Filter with Parameter Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Li Zhang Kai Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yue.li@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents a conditional in-loop filtering method. It is developed from the prior contribution JVET-U0068. The proposed CNN-architecture is conditioned on an auxiliary parameter based on QP, leading to a unified model to handle different quality levels. To better capture local characteristics of an image, the proposed technique enables an adaptive selection of the auxiliary parameter at block level and slice level. Compared with VTM-11.0, the proposed method reportedly shows on average {xx%, xx%, xx%}, {xx%, xx%, xx%}, and {xx%, xx%, xx%} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively.

Introduction

In this contribution, convolutional neural networks (CNN) are utilized to construct in-loop filters for reducing the distortion incurred during compression. The network structure is revised from the one presented in our prior contribution JVET-U0068 [1]. Similar to JVET-U0068, residual blocks are utilized as the basic module and stacked several times to construct the final network. As a further development from JVET-U0068, the presented method introduces an auxiliary parameter based on QP as an input to the network, leading to a unified model that can handle various quality levels. Moreover, some new features are introduced to further improve the coding performance.

Proposed method

Architecture

The backbone of the proposed CNN filtering method is shown in Fig. 1. The convolutional parts are similar to the one presented in JVET-U0068. The process of feeding QP into the network is twofold. First, QP is tiled into a 2-dimensional arrays with the same size as the block to be filtered and then treated as an additional input plane. Second, QP is fed into fully connected layers to be expanded into a 1-dimensional vector, whose elements will serve as the channel-wise scaling factors to recalibrate the feature maps

Fig. 1. Architecture of the proposed CNN filtering method. M denotes the number of feature maps (M=96). N stands for the number of samples in one dimension.

Selection of Auxiliary Parameters

In the proposed technique, each slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, which auxiliary parameter from a candidate set with three possible auxiliary parameters could be further decided. Given a test QP for coding a sequence, denoted as q, the candidate set includes three auxiliary parameters. The candidate set is predefined to be to {q, q-5, q-10} and {q, q-5, q+5} for lower temporal layer and higher temporal layer, respectively. The selection process is based on the rate-distortion cost at the encoder side. The indication of on/off control as well as the auxiliary parameter index, if needed, are signalled in the bitstream. Other details regarding to the parameter selection are listed as below.

- The granularity of the filter determination and auxiliary parameter selection is dependent on the picture resolution and QP. In the proposed scheme, given a higher resolution and a larger QP, the determination and selection will be performed for a larger region.

- Intra-slice NN filter may be used for an inter-slice. For a low temporal layer, the inter-slice NN filter input with the third auxiliary parameter candidate is replaced by the intra-slice NN filter model if the ratio of intra-coded samples is no smaller than a threshold.

- For all-intra configuration, the parameter selection is disabled while on/off control is still preserved.

Inference

PyTorch is used for performing the inference of the proposed CNN filters in VTM. The network information in the inference stage is provided in Table 1 as suggested by [2].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 5.56M/model |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | ~22.6M/model，4 models in total |
|  | MAC (Giga) | 11624 (Input: 3840×2160) |
| Optional |  |  |
|  | Total Conv. Layers | 69 |
|  | Total FC Layers | 10 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 32×32, 64×64, 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. The DIV2K [3] and BVI-DVC [4] datasets are adopted to train the CNN filters of I slices and B slices, respectively. A unified CNN model is trained to tackle different QP points. The network information in the training stage is provided in Table 2 as suggested by [2].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 90 |
|  | Batch size: | 64 |
|  | Training time: | 120h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-9.0, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is tested on top of VTM-11.0 [5] according to the common test conditions defined in [2]. Deblocking filtering and SAO are disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. Test results are shown in Table 3 ~ Table 5. Under AI configurations, the proposed method can bring xx%, xx%, and xx% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring xx%, xx%, and xx% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring xx%, xx%, and xx% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 3. Performance of the proposed method on top of VTM11.0 (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -12.14% | -23.57% | -23.47% | 218% | 28362% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -13.79% | -24.15% | -23.65% | 208% | 25445% |
| Class F | -6.09% | -12.85% | -12.34% | 389% | 14745% |

Table 4. Performance of the proposed method on top of VTM11.0 (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 5. Performance of the proposed method on top of VT11.0 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Conclusions

This contribution presents a CNN-based in-loop filtering method. The proposed CNN-based in-loop filtering method shows very promising coding gains. It is proposed to further study CNN-based in-loop filters in the Ad-Hoc group.

References

- AHG11: Convolutional neural network-based in-loop filter with adaptive model selection, JVET-U0068, Y. Li, L. Zhang, K. Zhang.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-U2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).