| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0102 |
| --- | --- |

| Title: | AHG11 & AHG12: Deep In-Loop Filter with Adaptive Model Selection for Enhanced Compression Beyond VVC Capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Li Zhang Kai Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yue.li@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution evaluates the deep in-loop filter presented in JVET-V0100 on top of JVET-U0100. Compared with JVET-U0100, the proposed method reportedly shows on average {xx%, xx%, xx%}, {xx%, xx%, xx%}, and {xx%, xx%, xx%} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively.

Introduction

JVET-U0100 [1] was proposed to investigate non-NN-based compression efficiency methods beyond VVC and was further studied in the EE. In this contribution, Deep in-loop Filter with Adaptive Model selection (DAM) proposed in JVET-V0100 [2] is incorporated on the EE code, to demonstrate the coding efficiency of a combination of NN-based coding methods and non-NN based coding methods beyond VVC.

Experimental results

The proposed CNN-based in-loop filtering method is tested on top of the EE code, according to the common test conditions defined in [3]. Deblocking filtering and SAO are disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. Test results are shown in Table 1 ~ Table 3. The proposed method reportedly shows on average {xx%, xx%, xx%}, {xx%, xx%, xx%}, and {xx%, xx%, xx%} BD-rate reductions for {Y, Cb, Cr}, under AI, RA, and LDB configurations, respectively.

Table 1. Performance of the proposed method on top of JVET-U0100 (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.47% | -27.51% | -25.88% | 168% | 6911% |
| Class C | -9.30% | -23.07% | -24.12% | 137% | 6909% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -10.75% | -25.76% | -26.96% | 134% | 6057% |
| Class F | -4.20% | -13.58% | -13.14% | 232% | 3663% |

Table 2. Performance of the proposed method on top of JVET-U0100 (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -8.32% | -19.82% | -17.87% | 143% | 10775% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -9.01% | -22.37% | -20.01% | 135% | 9773% |
| Class F |  |  |  |  |  |

Table 3. Performance of the proposed method on top of JVET-U0100 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -5.90% | -19.71% | -19.59% | 128% | 7232% |
| Class C | -6.77% | -17.51% | -20.57% | 112% | 5540% |
| Class E | -9.32% | -23.27% | -22.99% | 128% | 8044% |
| Overall |  |  |  |  |  |
| Class D | -6.91% | -18.72% | -21.41% | 106% | 4843% |
| Class F | -3.34% | -10.18% | -8.89% | 115% | 2752% |

Conclusions

This contribution presents a CNN-based in-loop filtering method. The proposed CNN-based FAMS method shows very promising coding gains. It is proposed to further study CNN-based in-loop filters in the Ad-Hoc group.

References

- Compression efficiency methods beyond VVC, JVET-U0100, Y. -J. Chang, C. -C. Chen, J. Chen, J. Dong, etc.

- AHG11: Deep In-Loop Filter with Adaptive Model Selection, JVET-V0100, Y. Li, L. Zhang, K. Zhang.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-U2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).