| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0110 |
| --- | --- |

| Title: | AHG12: Two additional reference sample sets for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Yu Teng Yu-Chiao Yang | Email: | zachary.deng@fginnov.com yuciaoyang@fginnov.com |
| Source: | FG Innovation |  |  |

_____________________________

# Abstract

This contribution reports the test results of adding two additional linear model (LM) modes to the EE common software. The additional linear model modes are derived based on two reference sample position sets presented in JVET-P0500, and the reference sample position is different from the LM modes in the current VVC.

The simulation results are shown as follows:

Proposed method implemented on top of EE common software (Anchor: VTM11):

-5.13/-8.63/-8.75 BD performance on Y, Cb and Cr components for AI configuration.

xxx/xxx/xxx BD performance on Y, Cb and Cr components for RA configuration.

Proposed method implemented on top of EE common software (Anchor: EE common software):

-0.04/-0.61/-0.80 BD performance on Y, Cb and Cr components for AI configuration.

xxx/xxx/xxx BD performance on Y, Cb and Cr components for RA configuration.

Proposed method implemented on top of VTM11 (Anchor: VTM11):

xxx/xxx/xxx BD performance on Y, Cb and Cr components for AI configuration.

xxx/xxx/xxx BD performance on Y, Cb and Cr components for RA configuration.

# Introduction

Cross-component linear model (CCLM) predicts a chroma block based on a linear model and the downsampled reconstructed luma samples, as shown as follows:

predCi,j=α·recL'i,j+β

The linear model parameters α and β are derived by the reference samples neighboring the chroma block and the corresponding reference samples neighboring the luma block. In the current VVC [1], there are three LM modes denoted as LM_LT, LM_T, and LM_L, and four reference samples are used to derive the linear model parameters of each LM mode. The following shows the reference sample position of each LM mode, which are used to derive the LM parameters.

LM_LT: R[W/4, -1], R[3W/4, -1], R[-1, H/4] and R[-1, 3H/4]

LM_T:	R[W’/8, -1], R[3W’/8, -1] , R[5W’/8, -1] and R[7W’/8, -1]

LM_L: R[-1, H’/8], R[-1, 3H’/8], R[-1, 5H’/8] and R[-1, 7H’/8]

R[-1, y] is the left reference line, R[x, -1] is the above reference line and W’=H’=W+H is the number of the reference samples.

In additional, the Multi-model LM (MMLM) modes are introduced in JVET-U0100 [2] to improve the coding performance of the chroma components. In each MMLM mode, the reference samples are classified into two classes by a threshold which is the average of the luma reconstructed neighboring samples, and the two linear models are derived based on each classified luma reconstructed neighboring samples. And the Least mean squares (LMS) may be applied to derive the linear model parameters.

predCi,j=α1·recL'i,j+β1,     if recL'i,j ≤thresholdpredCi,j=α2·recL'i,j+β2,     if recL'i,j >threshold

# Proposed method

In JVET- P0500 [3], four new LM modes are introduced to improve the coding performance of the chroma components. This contribution selects two from the four new LM modes to implement on EE common software and VTM11. The two LM modes are denoted as LM_LA1 and LM_LA2. Each of the two proposed modes selects two reference samples from the above reference line of the current block and two reference samples from the left reference line of the current block, which is similar to the LM_LT mode in the current VVC. The number of the above reference samples and the number of the left reference samples are equal to W+H. Therefore, no additional neighboring region and reference sample are needed. And the derivation of the linear model parameters is the same as the LM modes in the current VVC. The following shows the selected reference sample positions of the two additional LM modes:

LM_LA1: R[W’/8, -1], R[7W’/8, -1], R[-1, H’/8] and R[-1, 7H’/8]

LM_LA2: R[3W’/8, -1], R[5W’/8, -1], R[-1, 3H’/8] and R[-1, 5H’/8]

Based on the EE common software, the binarization of the two additional LM modes and other LM modes are modified and shown in Table 1. The first two bins to encode the LM modes are context-coded and the latter bins are bypass-coded.

Table 1: Proposed binarization method of LM modes based on JVET-U0100

| LM_LT | 00 |
| --- | --- |
| LM_L | 0100 |
| LM_T | 0101 |
| LM_LA1 | 0110 |
| LM_LA2 | 0111 |
| MMLM_LT | 10 |
| MMLM_L | 110 |
| MMLM_T | 111 |

Based on the VTM11, the binarization of the proposed LM modes and other LM modes are modified and shown in Table 2. The first bin to encode the LM modes is context-coded and the latter bins are bypass-coded.

Table 2: Proposed binarization method of LM modes based on VTM11

| LM_LT | 0 |
| --- | --- |
| LM_L | 100 |
| LM_T | 101 |
| LM_LA1 | 110 |
| LM_LA2 | 111 |

# Simulation results

The proposed method has been implemented on top of EE common software (version 4f58aa9f) and VTM11. Tests are preformed under the default EE common software configuration (BASE_ENCODER=BASE_NORMATIVE=TOOLS=1) and CTC configuration [4]. Note that the proposed method is not affected by LMS which is proposed in JVET-U0100.

Based on EE common software, the proposed method has a BD-rate performance of -8.63%/-8.75% for chroma components for the AI configuration over VTM11, and BD-rate performance of xxx%/xxx% for chroma components for the RA configuration over VTM11 as Table 3. Compared to the EE common software, the proposed method has a BD-rate performance of -0.61%/-0.80% for chroma components for the AI configuration, and BD-rate performance of xxx%/xxx% for chroma components for the RA configuration as Table 4.

Table 3: Simulation results of the proposed method implemented on top of EE common software compared with VTM11

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.35% | -10.75% | -12.59% | 227% | 233% |
| Class A2 | -4.91% | -8.80% | -7.04% | 219% | 212% |
| Class B | -4.63% | -8.12% | -8.72% | 225% | 212% |
| Class C | -5.40% | -7.95% | -8.36% | 227% | 189% |
| Class E | -5.62% | -8.06% | -7.19% | 232% | 222% |
| Overall | -5.13% | -8.63% | -8.75% | 226% | 212% |
| Class D | -4.28% | -5.94% | -5.75% | 242% | 194% |
| Class F | -8.14% | -10.98% | -11.54% | 170% | 188% |

Table 4: Simulation results of the proposed method implemented on top of EE common software compared with EE common software

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over EE common software |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -1.29% | -1.67% | 101% | 99% |
| Class A2 | -0.02% | -0.64% | -0.41% | 101% | 100% |
| Class B | 0.00% | -0.72% | -0.99% | 101% | 100% |
| Class C | -0.01% | -0.44% | -0.53% | 100% | 100% |
| Class E | 0.01% | 0.11% | -0.33% | 101% | 100% |
| Overall | -0.04% | -0.61% | -0.80% | 101% | 100% |
| Class D | 0.02% | -0.20% | -0.19% | 101% | 100% |
| Class F | -0.06% | -0.55% | -0.72% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over EE common software |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Based on VTM11, the proposed method has a BD-rate performance of xxx%/xxx% for chroma components without encoding/decoding time increasing for the AI configuration, and BD-rate performance of xxx%/xxx% for chroma components without encoding/decoding time increasing for the RA configuration.

Table 5: Simulation results of the proposed method implemented on top of VTM11 compared with VTM11

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over EE common software |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over EE common software |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusions

This contribution presented the results of two additional LM modes implemented on top of EE common software and VTM11. The simulation results report xxx/xxx BD-rate improvement in chroma component and negligible encoding/decoding time increased in AI configuration compared with EE common software.

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, Oct. 2020.

- Y.-J. Chang, C.-C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, Z. Zhang (Qualcomm), “Compression efficiency methods beyond VVC”, JVET-U0100, Jan. 2021.

- P.-H. Lin, C.-Y. Teng (Foxconn), “Non-CE3: Multiple reference sample set for CCLM”, JVET-P0500, Oct. 2019.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing.”, JVET-U2017, Jan. 2021.

# Patent rights declaration(s)

FG Innovation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).