| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0121-v1 |
| --- | --- |

| Title: | AHG8: on coding of last significant coefficient position for high bit depth and high bit rate extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Luhang Xu Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes a modification to the derivation method of last significant coefficient position in regular residual coding (RRC), for consideration in VVC version 2. In the current VVC specification, the coordinate of last significant coefficient (LastSignificantCoeffX, LastSignificantCoeffY) is measured in reference to the top-left corner of a transform block (TB) and coded with syntax elements last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix, last_sig_coeff_y_suffix. It is observed that, when coding for high quality (or bit-rate) with high bit-depth video, the number of significant coefficients become larger and the last significant coefficient often occur in the bottom-right portion of a TB. To accommodate this new distribution aspect, this contribution proposes to code the relative coordinates of last significant coefficient in reference to the bottom-right corner of a zero-out transform block.

# Introduction

In the current VVC specification [1], the position of the last significant coefficient in a TB (LastSignificantCoeffX, LastSignificantCoeffY) is coded with syntax elements last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix, last_sig_coeff_y_suffix in RRC. Its coordinates are measured in reference to the top-left corner (0, 0), simply because of the fact that, with normal QPs and 8/10 bit-depth input video, non-zero coefficients mostly occur in the top-left portion of a TB.  However, when operating at high quality/bit-rate with high bit-depth video, the transform coefficients distribute much broadly in a transform block. As a result, the last significant coefficient position tends to be closer to the bottom-right corner rather than the top-left corner. Therefore, for VVC high bit-depth and high bit-rate extensions, it is rational and sensible to use the bottom-right corner as the reference when measuring the position of the last significant coefficient.

(a)                               (b)

Figure 1. position and coordinates of the last significant coefficient, (a). definition in VVC version 1; (b). proposed alternative definition for VVC high bit-depth and high bit-rate extensions.

# Proposed method

This contribution proposes that, for high bit-rate coding, a set of alternative coordinates of the last significant coefficient is measured and coded in reference to the bottom-right corner of zero-out transform block. The proposed coordinates are calculated by subtracting the relative distances from the width minus 1 or height minus 1. Figure 1 illustrates the proposed alternative coordinates for the last significant coefficient.

The usage of the proposed alternative coordinates is signaled by a flag sh_reverse_last_sig_coeff_flag in the slice header. When sh_reverse_last_sig_coeff_flag is equal to 1, the last coefficient position is coded by using the proposed alternative coordinates. Otherwise, the current coordinates for the last coefficient position are used.

The proposed method only adds a flag in the slice header as shown below, and the rest of the bitstream syntax remains unchanged.

### 7.3.7 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_high_bit_depth_flag ) |  |
| sh_ reverse_last_sig_coeff_flag | u(1) |
| … |  |

The proposed semantic modifications are highlighted in red below.

#### 7.4.12.11 Residual coding semantics

…

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2ZoTbWidth  <<  1 ) − 1, inclusive.

When last_sig_coeff_x_prefix is not present, it is inferred to be 0.

last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( log2ZoTbHeight  <<  1 ) − 1, inclusive.

When last_sig_coeff_y_prefix is not present, it is inferred to be 0.

last_sig_coeff_x_suffix specifies the suffix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_suffix shall be in the range of 0 to ( 1  <<  ( ( last_sig_coeff_x_prefix  >>  1 ) − 1 ) ) − 1, inclusive.

The column position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffX is derived as follows:

- If last_sig_coeff_x_suffix is not present, the following applies:

LastSignificantCoeffX = last_sig_coeff_x_prefix		(192)

- Otherwise (last_sig_coeff_x_suffix is present), the following applies:

LastSignificantCoeffX = ( 1  <<  ( (last_sig_coeff_x_prefix  >>  1 ) − 1 ) ) *	(193)
			( 2 + (last_sig_coeff_x_prefix & 1 ) ) + last_sig_coeff_x_suffix

If sh_reverse_last_sig_coeff_flag is equal to 1, the following applies:

LastSignificantCoeffX =  ( 1 << log2ZoTbWidth ) - 1- LastSignificantCoeffX

last_sig_coeff_y_suffix specifies the suffix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_suffix shall be in the range of 0 to ( 1  <<  ( ( last_sig_coeff_y_prefix  >>  1 ) − 1 ) ) − 1, inclusive.

The row position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffY is derived as follows:

- If last_sig_coeff_y_suffix is not present, the following applies:

LastSignificantCoeffY = last_sig_coeff_y_prefix		(194)

- Otherwise (last_sig_coeff_y_suffix is present), the following applies:

LastSignificantCoeffY = ( 1  <<  ( ( last_sig_coeff_y_prefix  >>  1 ) − 1 ) ) *	(195)
			( 2 + ( last_sig_coeff_y_prefix & 1 ) ) + last_sig_coeff_y_suffix

If sh_reverse_last_sig_coeff_flag is equal to 1, the following applies:

LastSignificantCoeffY =  ( 1 << log2ZoTbHeight ) - 1- LastSignificantCoeffY

# Simulation results

The CTC specified for high bit depth and high bit rate video coding in [2] were used in the simulations. In the tables below, the “anchor” is the CE base software [3], and the “test” is the proposed method implemented in the CE base software.

# Conclusion

This contribution proposes a modification on the derivation method of last significant coefficient position for regular residual coding (RRC), for consideration in VVC version 2. It is reported average coding efficiency gain is XX%. It is recommended to conduct core experiment for this method.

# References

[1] Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, Doc. JVET- T2001, 20th Meeting, by teleconference, 7 – 16 Oct. 2020

[2] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

[3] A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou “CE on entropy coding for high bit depth and high bit rate coding”, JVET-U2022

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).