| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0122-v1 |
| --- | --- |

| Title: | AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes a full-bypass mode in residual coding for VVC version 2. It is proposed to add this new mode in both regular residual coding (RRC) and transform skip residual coding (TSRC). When enabled, this new mode will code coefficient or residual levels only through the bypass mode of the arithmetic coding engine without using any context-coded bin. This mode is primarily designed to improve the throughput of arithmetic coding for high quality applications with high bit-depth video signals, such as content capture, creation, and editing. It is observed that the simulation results showed over 15% reduction in average decoding time, even with a slight coding efficiency gain, from the 16- bit SVT sequences. The tests were conducted by using the CE anchor software and running the latest CTC in JVET-U2018.

# Introduction

For high quality video applications, codecs running at high bit-rates demand high throughput.  When extending the VVC capabilities for high bit-depth video and high bit-rate applications, it is desirable and beneficial to enable a high throughput mode that can help reduce the implementation complexity and speed up the encoding/decoding process for software-based codec.

In this proposal, a full-bypass mode is added to the residual coding, in both RRC and TSRC modes, where a coefficient level or a residual level is coded through the bypass mode of the arithmetic coding engine without using context-coded bins at all. This mode is primarily designed for high quality applications with high bit-depth video signals, such as content capture, creation, and editing. An SPS signaling flag is proposed to enable this mode, and the proposed mode requires no change to the rest of the current VVC syntax.

## Entropy coding of coefficients in VVC Main 10 profile:

In the VVC residual coding [1], several syntax elements, such as sig_coeff_flag, abs_level_gtx_flag, par_level_flag, and coeff_sign_flag, are coded, as context coded bins, through context-adaptive arithmetic coding. To guarantee the throughput of arithmetic coding engine, the maximum number of context coded bins for each block is limited by the initial value of “remBinsPass1” or “RemCcbs”, which is set each time before coding the first coefficient.

When coding a TB in the RRC mode, the initial value of remBinsPass1 is derived as follows:

remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2,

and the coding of each coefficient level may yield up to 4 context-coded bins, which are sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag. When a coefficient level coded with context-coded bins is greater than 3, its remaining level above 3 is bypass-coded via a sysntax element abs_remainder. If there are still coefficinents left after the total of context-coded bins generated has reached the initial value of remBinsPass1, the coefficient levels are then bypass-coded via a syntax element dec_abs_level. For any non-zero coefficient level, a syntax element coeff_sign_flag is always coded in the bypass mode.

For the TSRC mode, the initial value of RemCcbs for each residual bloack is derived as follows:

RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2,

and the coding of a residual level with context-coded bins, which is carried out in two passes, may produce up to 8 context-coded bins each time, which are sig_coeff_flag, coeff_sign_flag, abs_level_gtx_flag[n], and par_level_flag. When the total number of context-coded bins has reached the intitial value of RemCcbs, the remaining coefficient levels are bypass-coded via a the syntax element abs_remainder.

# Proposed method

This contribution proposes to use full-bypass mode of residual coding for high bit depth, high bit rate coding, aiming at high quality video coding application. To implement this method, the initialization methods of remBinsPass1 in RRC and RemCcbs in TSRC are changed as follows highlighted with red color.

For RRC:

#### 7.3.11.11Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_level_ccbs_disabled_flag) { |  |
| remBinsPass1 = 0 |  |
| }else{ |  |
| remBinsPass1 = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| } |  |
| … |  |

sps_level_ccbs_disabled_flag is a SPS level flag which enable/disables the proposed method. If the sps_level_ccbs_disabled_flag is equal to 1, the value of remBinsPass1 is set to 0. As a result, the pass1 which contains sig_coeff_flag, abs_level_gtx_flag and par_level_flag with context modelling coded is always skipped. The coefficient levels at all positions are always coded by dec_abs_level and coeff_sign_flag in bypass mode. If the sps_level_ccbs_disabled_flag is equal to 0, the current VVC RRC keeps unchanged.

For TSRC:

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_level_ccbs_disabled_flag) { |  |
| RemCcbs = 0 |  |
| }else{ |  |
| RemCcbs = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| } |  |
| … |  |

If the sps_level_ccbs_disabled_flag is equal to 1, the value of RemCcbs is set to 0, all context modelling coded bins, including sig_coeff_flag, abs_level_gtx_flag par_level_flag and coeff_sign_flag are always skipped. The coefficient levels at all positions are coded by abs_remainder and coeff_sign_flag in bypass mode. If the sps_level_ccbs_disabled_flag is equal to 0, the current VVC TSRC keeps unchanged.

7.3.2.4 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_high_bit_depth_flag | u(1) |
| if( sps_high_bit_depth_flag ) |  |
| sps_level_ccbs_disabled_flag | u(1) |
| … |  |

# Simulation results

## TEST

The test conditions follow CTC for high bit depth and high bit rate video coding [2]. The anchor is CE base [3], the test is CE base with proposed method.

Low QP:

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.13% | -0.18% | -0.20% | 85% | 83% |
| SVT12 | 1.23% | 1.18% | 1.15% | 82% | 79% |
| Overall | 0.55% | 0.50% | 0.48% | 83% | 81% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.07% | -0.07% | -0.06% | 84% | 84% |
| SVT12 | 1.26% | 1.48% | 1.45% | 86% | 81% |
| Overall | 0.60% | 0.70% | 0.70% | 85% | 82% |
|  |  |  | RA |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 |  |  |  | #DIV/0! | #DIV/0! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall RGB | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 2.79% | 4.25% | 4.71% | 87% | 88% |
| HLG422 | 2.67% | 3.85% | 4.25% | 83% | 89% |
| Overall | 2.73% | 4.05% | 4.48% | 85% | 89% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 8.25% | 10.84% | 11.59% | 82% | 89% |
| HLG422 | 8.34% | 10.29% | 10.24% | 84% | 91% |
| Overall | 8.30% | 10.57% | 10.91% | 83% | 90% |
|  |  |  | RA |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 8.68% | 11.12% | 11.64% | 81% | 89% |
| HLG422 | 8.67% | 10.29% | 10.11% | 83% | 91% |
| Overall | 8.67% | 10.71% | 10.87% | 82% | 90% |
| Overall HLG | 6.57% | 8.44% | 8.75% | 83% | 90% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 6.66% | 9.56% | 9.82% | 6.51% | 9.32% | 9.66% | 90% | 91% |
| PQ422 | 5.12% | 7.02% | 6.93% | 4.97% | 6.86% | 6.82% | 86% | 90% |
| Overall | 5.89% | 8.29% | 8.37% | 5.74% | 8.09% | 8.24% | 88% | 91% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 10.25% | 13.89% | 13.98% | 9.75% | 13.41% | 13.82% | 88% | 92% |
| PQ422 | 8.93% | 10.15% | 10.03% | 8.44% | 9.77% | 9.93% | 88% | 93% |
| Overall | 9.59% | 12.02% | 12.01% | 9.10% | 11.59% | 11.87% | 88% | 92% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 10.22% | 13.57% | 14.22% | 9.76% | 13.17% | 14.15% | 86% | 89% |
| PQ422 | 8.85% | 10.12% | 9.95% | 8.40% | 9.84% | 9.94% | 87% | 92% |
| Overall | 9.53% | 11.85% | 12.08% | 9.08% | 11.50% | 12.05% | 87% | 91% |
| Overall PQ | 8.34% | 10.72% | 10.82% | 7.97% | 10.39% | 10.72% | 88% | 91% |

Lossless:

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| PQ444 | 2.5 | 2.5 | 2.20% | 3.0 | 2.9 | 3.76% | 3.0 | 2.9 | 3.70% |
| PQ422 | 2.3 | 2.3 | 1.80% | 2.8 | 2.7 | 3.41% | 2.8 | 2.7 | 3.34% |
| Overall | 2.4 | 2.4 | 2.00% | 2.9 | 2.8 | 3.58% | 2.9 | 2.8 | 3.52% |
| Enc Time[%] | 99% |  |  | 75% |  |  | 76% |  |  |
| Dec Time[%] | 69% |  |  | 78% |  |  | 77% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| HLG444 | 1.7 | 1.7 | -0.06% | 1.9 | 1.9 | -0.54% | 1.9 | 1.9 | -0.54% |
| HLG422 | 1.6 | 1.6 | 0.11% | 1.9 | 1.9 | -0.37% | 1.9 | 1.9 | -0.39% |
| Overall | 1.7 | 1.7 | 0.02% | 1.9 | 1.9 | -0.46% | 1.9 | 1.9 | -0.46% |
| Enc Time[%] | 98% |  |  | 56% |  |  | 54% |  |  |
| Dec Time[%] | 63% |  |  | 70% |  |  | 70% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| SVT16 | 1.2 | 1.2 | -0.63% | 1.2 | 1.2 | -0.66% | 1.2 | 1.2 | -0.62% |
| SVT12 | 1.2 | 1.2 | -0.42% | 1.3 | 1.3 | -0.17% | 1.3 | 1.3 | -0.14% |
| Overall | 1.2 | 1.2 | -0.53% | 1.2 | 1.2 | -0.41% | 1.2 | 1.3 | -0.38% |
| Enc Time[%] | 92% |  |  | 74% |  |  | 78% |  |  |
| Dec Time[%] | 80% |  |  | 85% |  |  | 82% |  |  |

## Analysis

From SVT 16-bit sequences, the proposed method can provide over 15% reduction in average decoding time with slightly coding gain for both lossy and lossless coding. For 12-bit SVT and PQ/HLG sequences, the proposed method also provides significant reduction in decoding time, but there are coding loss. The main reason for this loss is that the coding bitrate is not high enough.

# Conclusion

This contribution proposes a full-bypass mode in residual coding for high bit depth and high bit rate coding, aiming at high quality video coding applications. It is observed that the proposed method brings in both encoding and decoding time saving over 15%. In 16-bit SVT tests there is a slight coding gain of about xx%. It is recommended to conduct core experiment on and around this method.

# References

[1] Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, Doc. JVET- T2001, 20th Meeting, by teleconference, 7 – 16 Oct. 2020

[2] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

[3] A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou “CE on entropy coding for high bit depth and high bit rate coding”, JVET-U2022

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).