| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0122-v2 |
| --- | --- |

| Title: | AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a full-bypass mode in residual coding for VVC version 2. It is proposed to add this new mode in both regular residual coding (RRC) and transform skip residual coding (TSRC). When enabled, this new mode will code coefficient or residual levels only through the bypass mode of the arithmetic coding engine without using any context-coded bin. This mode is primarily designed to improve the throughput of arithmetic coding for high quality applications with high bit-depth video signals, such as content capture, creation, and editing. It is observed that the simulation results showed over 15% reduction in average decoding time, even with a slight coding efficiency gain, from the 16-bit SVT sequences. The tests were conducted by using the CE anchor software and running the latest CTC in JVET-U2018.

Introduction

For high quality video applications, codecs running at high bit-rates demand high throughput.  When extending the VVC capabilities for high bit-depth video and high bit-rate applications, it is desirable and beneficial to enable a high throughput mode that can help reduce the implementation complexity and speed up the encoding/decoding process for software-based codec.

In this proposal, a full-bypass mode is added to the residual coding, in both RRC and TSRC modes, where a coefficient level or a residual level is coded through the bypass mode of the arithmetic coding engine without using context-coded bins at all. This mode is primarily designed for high quality applications with high bit-depth video signals, such as content capture, creation, and editing. An SPS signaling flag is proposed to enable this mode, and the proposed mode only requires a small change, as shown below in Section 2, to the rest of the current VVC syntax.

Entropy coding of coefficients in VVC Main 10 profile:

In the VVC residual coding [1], several syntax elements, such as sig_coeff_flag, abs_level_gtx_flag, par_level_flag, and coeff_sign_flag, are coded, as context coded bins, through context-adaptive arithmetic coding. To guarantee the throughput of arithmetic coding engine, the maximum number of context coded bins for each block is limited by the initial value of “remBinsPass1” or “RemCcbs”, which is set each time before coding the first coefficient.

When coding a TB in the RRC mode, the initial value of remBinsPass1 is derived as follows:

remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2,

and the coding of each coefficient level may yield up to 4 context-coded bins, which are sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag. When a coefficient level coded with context-coded bins is greater than 3, its remaining level above 3 is bypass-coded via a sysntax element abs_remainder. If there are still coefficinents left after the total number of context-coded bins generated has reached the initial value of remBinsPass1, the coefficient levels are then bypass-coded via a syntax element dec_abs_level. For any non-zero coefficient level, a syntax element coeff_sign_flag is always coded in the bypass mode.

For the TSRC mode, the initial value of RemCcbs for each residual bloack is derived as follows:

RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2,

and the coding of a residual level with context-coded bins, which is carried out in two passes, may produce up to 8 context-coded bins each time, which are sig_coeff_flag, coeff_sign_flag, abs_level_gtx_flag[n], and par_level_flag. When the total number of context-coded bins has reached the intitial value of RemCcbs, the remaining coefficient levels are bypass-coded via the syntax element abs_remainder.

Proposed method

This contribution proposes to use a full-bypass mode in residual coding for high bit depth, high bit rate coding, aiming at high quality video coding application. To implement this method, the initialization methods of remBinsPass1 in RRC and RemCcbs in TSRC are modified as follows, with changes highlighted in red color.

For RRC:

- 7.3.11.11Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_level_ccbs_disabled_flag) { |  |
| remBinsPass1 = 0 |  |
| }else{ |  |
| remBinsPass1 = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| } |  |
| … |  |

The new syntax element sps_level_ccbs_disabled_flag is a SPS level flag which enables/disables the proposed method. If sps_level_ccbs_disabled_flag is equal to 1, the value of remBinsPass1 is set to 0. As a result, the context-adaptive coding of sig_coeff_flag, abs_level_gtx_flag and par_level_flag is always skipped, and the coefficient levels at all positions are always coded by dec_abs_level and coeff_sign_flag in the bypass mode. If sps_level_ccbs_disabled_flag is equal to 0, the current VVC RRC remains unchanged.

For TSRC:

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_level_ccbs_disabled_flag) { |  |
| RemCcbs = 0 |  |
| }else{ |  |
| RemCcbs = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| } |  |
| … |  |

If sps_level_ccbs_disabled_flag is equal to 1, the value of RemCcbs is set to 0, all the context coded bins, including sig_coeff_flag, abs_level_gtx_flag par_level_flag and coeff_sign_flag, are always skipped. The coefficient levels at all positions are coded by abs_remainder and coeff_sign_flag in the bypass mode. If sps_level_ccbs_disabled_flag is equal to 0, the current VVC TSRC remains unchanged.

7.3.2.4 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_high_bit_depth_flag | u(1) |
| if( sps_high_bit_depth_flag ) |  |
| sps_level_ccbs_disabled_flag | u(1) |
| … |  |

7.4.3.4 Sequence parameter set RBSP semantics

…

sps_level_ccbs_disabled flag equal to 1 specifies that parameters remBinsPass1 and RemCcbs in 7.3.11.11 Residual Coding syntax are set to 0, and the coefficient levels and residual levels, at all positions, are coded in the bypass mode through dec_abs_level, coeff_sign_flag, and abs_reminder accordingly. When sps_level_ccbs_disabled_flag is equal to 0, parameters remBinsPass1 and RemCcbs in 7.3.11.11 Residual Coding syntax define the maximum number of context-coded bins for each transform block, and the coefficient levels and residual levels are coded through context-coded bins together with “bypass-coded” bins. When sps_level_ccbs_disabled_flag is not present, its value is inferred to be equal to 0.

…

Simulation results

TEST 1

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [2]. The anchor is generated CE base [3], the test is produced by the CE base software with proposed method.

Low QP:

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.13% | -0.18% | -0.20% | 85% | 83% |
| SVT12 | 1.23% | 1.18% | 1.15% | 82% | 79% |
| Overall | 0.55% | 0.50% | 0.48% | 83% | 81% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.07% | -0.07% | -0.06% | 84% | 84% |
| SVT12 | 1.26% | 1.48% | 1.45% | 86% | 81% |
| Overall | 0.60% | 0.70% | 0.70% | 85% | 82% |
|  |  |  | RA |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.03% | -0.03% | -0.03% | 83% | 83% |
| SVT12 | 1.37% | 1.61% | 1.59% | 85% | 80% |
| Overall | 0.67% | 0.79% | 0.78% | 84% | 82% |
| Overall RGB | 0.61% | 0.67% | 0.65% | 84% | 82% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 2.79% | 4.25% | 4.71% | 87% | 88% |
| HLG422 | 2.67% | 3.85% | 4.25% | 83% | 89% |
| Overall | 2.73% | 4.05% | 4.48% | 85% | 89% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 8.25% | 10.84% | 11.59% | 82% | 89% |
| HLG422 | 8.34% | 10.29% | 10.24% | 84% | 91% |
| Overall | 8.30% | 10.57% | 10.91% | 83% | 90% |
|  |  |  | RA |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 8.69% | 11.14% | 11.64% | 81% | 89% |
| HLG422 | 8.69% | 10.30% | 10.13% | 83% | 91% |
| Overall | 8.69% | 10.72% | 10.88% | 82% | 90% |
| Overall HLG | 6.57% | 8.44% | 8.76% | 83% | 90% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 6.66% | 9.56% | 9.82% | 6.51% | 9.32% | 9.66% | 90% | 91% |
| PQ422 | 5.12% | 7.02% | 6.93% | 4.97% | 6.86% | 6.82% | 86% | 90% |
| Overall | 5.89% | 8.29% | 8.37% | 5.74% | 8.09% | 8.24% | 88% | 91% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 10.25% | 13.89% | 13.98% | 9.75% | 13.41% | 13.82% | 88% | 92% |
| PQ422 | 8.93% | 10.15% | 10.03% | 8.44% | 9.77% | 9.93% | 88% | 93% |
| Overall | 9.59% | 12.02% | 12.01% | 9.10% | 11.59% | 11.87% | 88% | 92% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 10.22% | 13.56% | 14.22% | 9.76% | 13.16% | 14.15% | 86% | 89% |
| PQ422 | 8.84% | 10.12% | 9.95% | 8.40% | 9.84% | 9.95% | 87% | 92% |
| Overall | 9.53% | 11.84% | 12.09% | 9.08% | 11.50% | 12.05% | 87% | 91% |
| Overall PQ | 8.34% | 10.72% | 10.82% | 7.97% | 10.39% | 10.72% | 88% | 91% |

Lossless:

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| PQ444 | 2.5 | 2.5 | 2.20% | 3.0 | 2.9 | 3.76% | 3.0 | 2.9 | 3.70% |
| PQ422 | 2.3 | 2.3 | 1.80% | 2.8 | 2.7 | 3.41% | 2.8 | 2.7 | 3.34% |
| Overall | 2.4 | 2.4 | 2.00% | 2.9 | 2.8 | 3.58% | 2.9 | 2.8 | 3.52% |
| Enc Time[%] | 99% |  |  | 75% |  |  | 76% |  |  |
| Dec Time[%] | 69% |  |  | 78% |  |  | 77% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| HLG444 | 1.7 | 1.7 | -0.06% | 1.9 | 1.9 | -0.54% | 1.9 | 1.9 | -0.54% |
| HLG422 | 1.6 | 1.6 | 0.11% | 1.9 | 1.9 | -0.37% | 1.9 | 1.9 | -0.39% |
| Overall | 1.7 | 1.7 | 0.02% | 1.9 | 1.9 | -0.46% | 1.9 | 1.9 | -0.46% |
| Enc Time[%] | 98% |  |  | 56% |  |  | 54% |  |  |
| Dec Time[%] | 63% |  |  | 70% |  |  | 70% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| SVT16 | 1.2 | 1.2 | -0.63% | 1.2 | 1.2 | -0.66% | 1.2 | 1.2 | -0.62% |
| SVT12 | 1.2 | 1.2 | -0.42% | 1.3 | 1.3 | -0.17% | 1.3 | 1.3 | -0.14% |
| Overall | 1.2 | 1.2 | -0.53% | 1.2 | 1.2 | -0.41% | 1.2 | 1.3 | -0.38% |
| Enc Time[%] | 92% |  |  | 74% |  |  | 78% |  |  |
| Dec Time[%] | 80% |  |  | 85% |  |  | 82% |  |  |

TEST 2

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [2]. The anchor is generated CE-1.5[3], the test is produced by the CE-1.5 software with proposed method.

Low QP:

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.5 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.32% | -0.54% | -0.56% | 78% | 81% |
| SVT12 | 0.37% | 0.38% | 0.36% | 82% | 86% |
| Overall | 0.03% | -0.08% | -0.10% | 80% | 84% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1.5 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.70% | -0.78% | -0.78% | 81% | 80% |
| SVT12 | 0.64% | 0.85% | 0.83% | 85% | 86% |
| Overall | -0.03% | 0.04% | 0.03% | 83% | 83% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1.5 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.68% | -0.77% | -0.77% | 85% | 86% |
| SVT12 | 0.76% | 1.01% | 0.99% | 87% | 86% |
| Overall | 0.04% | 0.12% | 0.11% | 86% | 86% |
| Overall RGB | 0.01% | 0.03% | 0.01% | 83% | 84% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.5 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 2.22% | 3.95% | 4.39% | 84% | 93% |
| HLG422 | 2.07% | 3.50% | 3.88% | 83% | 92% |
| Overall | 2.14% | 3.73% | 4.13% | 83% | 93% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1.5 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 8.26% | 10.84% | 11.57% | 83% | 92% |
| HLG422 | 8.33% | 10.26% | 10.18% | 85% | 94% |
| Overall | 8.29% | 10.55% | 10.87% | 84% | 93% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1.5 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 8.64% | 11.09% | 11.60% | 80% | 88% |
| HLG422 | 8.64% | 10.25% | 10.06% | 83% | 94% |
| Overall | 8.64% | 10.67% | 10.83% | 82% | 91% |
| Overall HLG | 6.36% | 8.32% | 8.61% | 83% | 92% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.5 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 6.62% | 9.60% | 9.87% | 6.42% | 9.34% | 9.70% | 87% | 94% |
| PQ422 | 5.15% | 7.18% | 7.07% | 4.96% | 7.02% | 6.96% | 88% | 93% |
| Overall | 5.88% | 8.39% | 8.47% | 5.69% | 8.18% | 8.33% | 87% | 94% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE-1.5 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 10.26% | 13.88% | 14.01% | 9.75% | 13.38% | 13.84% | 87% | 90% |
| PQ422 | 8.99% | 10.28% | 10.22% | 8.48% | 9.90% | 10.10% | 88% | 97% |
| Overall | 9.63% | 12.08% | 12.12% | 9.12% | 11.64% | 11.97% | 87% | 93% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE-1.5 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 10.24% | 13.55% | 14.23% | 9.76% | 13.14% | 14.16% | 89% | 96% |
| PQ422 | 8.87% | 10.19% | 10.06% | 8.41% | 9.92% | 10.04% | 86% | 93% |
| Overall | 9.55% | 11.87% | 12.14% | 9.08% | 11.53% | 12.10% | 87% | 94% |
| Overall PQ | 8.35% | 10.78% | 10.91% | 7.96% | 10.45% | 10.80% | 87% | 94% |

Lossless:

| PQ |  | All Intra |  |  |  |  |  | Low delay B |  |  |  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ratio |  |  | bit-rate savings |  |  | ratio |  |  |  | bit-rate savings |  | ratio |  | bit-rate savings |
|  | CE-1.5 | proposed |  | CE-1.5 | proposed |  | CE-1.5 | proposed |  |
| PQ444 | 2.6 | 2.5 | 1.67% | 3.1 | 2.9 | 3.32% | 3.1 | 2.9 | 3.32% |
| PQ422 | 2.4 | 2.3 | 1.31% | 2.8 | 2.7 | 3.01% | 2.9 | 2.8 | 2.97% |
| Overall | 2.5 | 2.4 | 1.49% | 2.9 | 2.8 | 3.17% | 3.0 | 2.8 | 3.15% |
| Enc Time[%] |  | 88% |  |  |  |  |  | 84% |  |  |  |  |  | 83% |  |  |
| Dec Time[%] |  | 71% |  |  |  |  |  | 88% |  |  |  |  |  | 79% |  |  |
| HLG |  | All Intra |  |  |  |  |  | Low delay B |  |  |  |  |  | Random Access |  |  |
|  |  | ratio |  |  | bit-rate savings |  |  | ratio |  |  |  | bit-rate savings |  | ratio |  | bit-rate savings |
|  | CE-1.5 | proposed |  | CE-1.5 | proposed |  | CE-1.5 | proposed |  |
| HLG444 | 1.8 | 1.8 | -0.12% | 1.9 | 1.9 | -0.29% | 1.9 | 1.9 | -0.29% |
| HLG422 | 1.7 | 1.7 | -0.22% | 1.9 | 1.9 | -0.31% | 1.9 | 1.9 | -0.32% |
| Overall | 1.7 | 1.7 | -0.17% | 1.9 | 1.9 | -0.30% | 1.9 | 1.9 | -0.31% |
| Enc Time[%] |  | 98% |  |  |  |  |  | 84% |  |  |  |  |  | 82% |  |  |
| Dec Time[%] |  | 78% |  |  |  |  |  | 78% |  |  |  |  |  | 75% |  |  |
| SVT |  | All Intra |  |  |  |  |  | Low delay B |  |  |  |  |  | Random Access |  |  |
|  |  | ratio |  |  | bit-rate savings |  |  | ratio |  |  |  | bit-rate savings |  | ratio |  | bit-rate savings |
|  | CE-1.5 | proposed |  | CE-1.5 | proposed |  | CE-1.5 | proposed |  |
| SVT16 | 1.2 | 1.2 | -0.47% | 1.2 | 1.2 | -0.91% | 1.2 | 1.2 | -0.92% |
| SVT12 | 1.3 | 1.3 | -0.50% | 1.3 | 1.3 | -1.07% | 1.3 | 1.3 | -1.07% |
| Overall | 1.2 | 1.2 | -0.49% | 1.3 | 1.3 | -0.99% | 1.3 | 1.3 | -0.99% |
| Enc Time[%] |  | 85% |  |  |  |  |  | 78% |  |  |  |  |  | 73% |  |  |
| Dec Time[%] |  | 76% |  |  |  |  |  | 89% |  |  |  |  |  | 80% |  |  |

Analysis

For SVT 16-bit sequences, the proposed method can achieve over 15% reduction in average decoding time with slightly coding gain for both low QP and lossless coding. For 12-bit SVT and PQ/HLG sequences, the proposed method also brings about 10% notable reduction in decoding time for low QP coding, but there is loss in coding efficiency. The main reason for this loss is that the coding bitrate is not high enough.

Conclusion

This contribution proposes a full-bypass mode in residual coding for high bit depth and high bit rate coding, aiming at high quality video coding applications. It is observed that the proposed method brings in both encoding and decoding time saving over 15%, even with a slight coding efficiency gain, from the 16-bit SVT sequences. It is recommended to conduct core experiment on and around this method.

References

[1] Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, Doc. JVET- T2001, 20th Meeting, by teleconference, 7 – 16 Oct. 2020

[2] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

[3] A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou “CE on entropy coding for high bit depth and high bit rate coding”, JVET-U2022

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).