| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0125_r1 |
| --- | --- |

| Title: | AHG12: Evaluation of GPM with MMVD for coding efficiency improvement over VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Hong-Jheng Jhu, C.-W. Kuo, W. Chen, Yi-Wen Chen, Xianglin Wang | Email: | {xiaoyuxiu, jhuhong-jheng, cheweikuo, chenwei06, yiwenchen, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, the coding performance of enhanced geometry partition mode (GPM) by further applying motion vector (MV) refinement to the unidirectional MV of each GPM partition is studied. To reduce the signaling overhead, the MV refinements are signaled based on the MV difference (MVD) signaling method of merge mode with MVD (MMVD). Compared to VTM-10.0 anchors, simulation results reportedly show that the method provides average {Y, U, V} BD-rate savings of {0.36%, 0.58%, 0.63%} and {0.66%, 1.12%, 0.78%} for RA and LD configurations, respectively. The corresponding encoding and decoding run-time are 103% and 97% for RA and 103% and 98% for LD, respectively.

# Introduction

In VVC standard [1], geometry partition mode (GPM) is one inter coding tool which splits one coding unit (CU) to one non-rectangular partitions based on sample-wise weighted combination of two uni-prediction signals. The couple of uni-directional motion vectors (MVs) used to generate two uni-prediction signals are obtained from merge candidate list. On the other hand, merge mode with MV difference (MMVD) is included in the VVC standard [1] which applies additional MVD to selected base motion of merge candidates. Specifically, when the MMVD is enabled, the corresponding MVD values are indicated in bit-stream by one distance index (that specifies the MMVD magnitude) and one direction index (that specifies the MMVD signs). Close to the end of the VVC version 1 development, evidence has been shown that additional coding gain can be achieved by further applying MV refinement on top of the existing GPM uni-directional MVs [2][3].

# Proposal

In this contribution, one enhancement method is proposed to further improve the coding efficiency of the GPM. Specifically, in the proposed scheme, the core GPM design (e.g. 64 GPM modes, uni-directional MV candidate generation, sample blending along GPM edge and motion field generation) is kept the same as the one in the VVC. However, to better capture the true motion for each GPM partition, additional MV refinement is allowed to be further applied on top of its existing uni-directional MV that is derived from merge candidate list. Additionally, to reduce the signaling cost, the MV refinements are signaled in the same manner of the MMVD mode, i.e., one distance index plus one direction index.

Table 1 illustrates the syntax elements that the proposed scheme introduces to the existing GPM signaling. As shown in Table 1, after the syntax element of the GPM mode (i.e., merge_gpm_partition_idx), two control flags, namely merge_gpm_mmvd_enable_flag0 and merge_gpm_mmvd_enable_flag1, are firstly signaled to indicate whether the MV refinement is applied to the first and second GPM partitions, respectively. When the corresponding flag of one partition is true, one distance index (i.e., merge_gpm_mmvd_distance_idx0 or merge_gpm_mmvd_distance_idx1) and one direction index (i.e., merge_gpm_mmvd_direction_idx0 or merge_gpm_mmvd_direction_idx1) are further signaled to specify the magnitude and the direction of the selected MV refinement. In the current design, the existing 8 distance and 4 directions that are applied by the MMVD in the VVC are reused in the proposed GPM scheme. Finally, the existing syntax elements merge_gpm_idx0 and merge_gpm_idx1 are signaled to indicate the index of the corresponding uni-directional MV of each partition.

Table 1 Illustration of the proposed syntax elements for the GPM enhancement

| } else { |  |
| --- | --- |
| if( sps_ciip_enabled_flag && sps_gpm_enabled_flag && sh_slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth >= 8 && cbHeight >= 8 && cbWidth < ( 8 * cbHeight ) && cbHeight < ( 8 * cbWidth ) && cbWidth < 128 && cbHeight < 128 ) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] ) { |  |
| merge_gpm_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_gpm_mmvd_enable_flag0[ x0 ][ y0 ] | ae(v) |
| if(merge_gpm_mmvd_enable_flag0) { |  |
| merge_gpm_mmvd_distance_idx0[ x0 ][ y0 ] | ae(v) |
| merge_gpm_mmvd_direction_idx0[ x0 ][ y0 ] | ae(v) |
| } |  |
| merge_gpm_mmvd_enable_flag1[ x0 ][ y0 ] | ae(v) |
| if(merge_gpm_mmvd_enable_flag1) { |  |
| merge_gpm_mmvd_distance_idx1[ x0 ][ y0 ] | ae(v) |
| merge_gpm_mmvd_direction_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| merge_gpm_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGpmMergeCand > 2 ) |  |
| merge_gpm_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

Additionally, to enable more flexible combinations of MV refinements for two GPM partitions, the following conditions are applied to the signaled values of merge_gpm_idx0 and merge_gpm_idx1:

- When both the first and second GPM partitions do not apply MV refinement (i.e., merge_gpm_mmvd_enable_flag0 and merge_gpm_mmvd_enable_flag1 are equal to 0), the merge indices of two GPM partitions are disallowed to be identical.

- When one of the two GPM partitions applies the MV refinement and the other does not (i.e., one of merge_gpm_mmvd_enable_flag0 and merge_gpm_mmvd_enable_flag1 is equal to 1 and the other is equal to 0), the merge indices of two GPM partitions are allowed to be identical.

- When both of two GPM partitions apply the MV refinement (i.e., merge_gpm_mmvd_enable_flag0 and merge_gpm_mmvd_enable_flag1 are equal to 1), the merge indices are allowed to be the same when the MV refinements of two partitions are different, and disallowed to be the same when the two MV refinements are identical.

# Simulation results

The proposed scheme was implemented based on VTM-10.0 reference software and tested by following the common test condition (CTC) as specified in [5]. In the simulations, both the temporal filter and GOP-32 are enabled in both anchor and test.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.23% | -0.40% | 101% | 97% |
| Class A2 | -0.36% | -0.54% | -0.45% | 103% | 96% |
| Class B | -0.28% | -0.47% | -0.56% | 103% | 96% |
| Class C | -0.62% | -1.00% | -1.04% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.58% | -0.63% | 103% | 96% |
| Class D | -0.66% | -1.19% | -1.12% | 105% | 96% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.45% | -0.85% | -0.91% | 103% | 98% |
| Class C | -0.88% | -1.41% | -1.49% | 102% | 100% |
| Class E | -0.70% | -1.17% | 0.41% | 104% | 97% |
| Overall | -0.66% | -1.12% | -0.78% | 103% | 98% |
| Class D | -1.09% | -1.17% | -1.51% | 105% | 98% |

Additional results are provided on top of the current EE2 software base. In the table, the anchor is the results of EE2 software (i.e., JVET-U0100 base) and the test is the anchor plus the proposed GPM-MMVD scheme.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over U0100 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.18% | 108% | 101% |
| Class A2 | -0.12% | -0.15% | -0.14% | 107% | 99% |
| Class B | -0.05% | -0.23% | -0.30% | 106% | 97% |
| Class C | -0.16% | -0.21% | -0.42% | 105% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.17% | -0.28% | 106% | 99% |
| Class D | -0.22% | -0.29% | -0.46% | 107% | 100% |
| Class F | 0.00% | -0.08% | -0.03% | 106% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over U0100 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.28% | -0.71% | 105% | 100% |
| Class C | -0.33% | -0.24% | -0.29% | 107% | 100% |
| Class E | -0.21% | -0.39% | -0.09% | 105% | 99% |
| Overall | -0.21% | -0.29% | -0.42% | 106% | 100% |
| Class D | -0.35% | -0.55% | -0.47% | 104% | 99% |
| Class F | -0.19% | -0.25% | 0.34% | 107% | 100% |

# Reference

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, June 2020, Teleconference.

K. Zhang, L. Zhang, Z. Deng, H. Liu, Y. Wang, “Geometric prediction mode with motion vector differences”, JVET-R0357, April. 2020, Teleconference.

K. Panusopone, S. Hong, L. Wang, “GEO with MMVD”, JVET-R0282, April. 2020, Teleconference.

VTM-10.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-10.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, Oct. 2020, Teleconference.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).