| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0126-v3 |
| --- | --- |

| Title: | EE-related: Modified OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes modified OBMC based on JVET-U0100 to improve coding efficiency, which includes fixing the RDO of OBMC for AMVR. Simulation results reportedly show proposed method achieves -0.04% BD-rate saving and 10% decoding time saving with 1% increasing encoding time increase compared to the EE common software base where OBMC is only enabled.

Introduction

In the previous meeting, JVET-U0100[1] proposed OBMC as one of the tools to improve the coding efficiency beyond VVC. Regarding RDO of OBMC for AMVR (IMV) in JVET-U0100, the RD cost check of OBMC-off is almost not checked. In EE common software base [2], this part is described as “to do” even with the latest version. Furthermore, it is also skipped in HASH_INTER and INTER_MULTIHYP.

//EncCu.cpp

if( currTestMode.type == ETM_INTER_ME )

{

#if ENABLE_OBMC

bool tryObmc = true;

#endif

if( ( currTestMode.opts & ETO_IMV ) != 0 )

{

const bool skipAltHpelIF = ( int( ( currTestMode.opts & ETO_IMV ) >> ETO_IMV_SHIFT ) == 4 ) && ( bestIntPelCost > 1.25 * bestCS->cost );

if (!skipAltHpelIF)

{

tempCS->bestCS = bestCS;

#if ENABLE_OBMC

tryObmc = xCheckRDCostInterIMV(tempCS, bestCS, partitioner, currTestMode, bestIntPelCost);

#else

xCheckRDCostInterIMV(tempCS, bestCS, partitioner, currTestMode, bestIntPelCost);

#endif

tempCS->bestCS = nullptr;

}

}

else

{

tempCS->bestCS = bestCS;

xCheckRDCostInter( tempCS, bestCS, partitioner, currTestMode );

tempCS->bestCS = nullptr;

}

#if ENABLE_OBMC

if (tryObmc && tempCS->cus.size() != 0)//todo

{

xCheckRDCostInterWoOBMC(tempCS, bestCS, partitioner, currTestMode);

}

#endif

}

else if (currTestMode.type == ETM_HASH_INTER)

{

xCheckRDCostHashInter( tempCS, bestCS, partitioner, currTestMode );

}

~~

#if MULTI_HYP_PRED

else if (currTestMode.type == ETM_INTER_MULTIHYP)

{

xCheckRDCostInterMultiHyp2Nx2N(tempCS, bestCS, partitioner, currTestMode);

Proposal

This contribution proposes fixing the RDO of OBMC for AMVR. In detail, the RD cost check of OBMC-off is added into AMVR, HASH_INTER, and INTER_MULTIHPY.

Experimental Results

The proposed method is implemented on the top of EE common software base [2] with the version released in Feb. 16, 2021. Simulation conditions basically follows JVET-U2024 [3] but only OBMC and/or 12TAP interpolation filter is enabled while the other tools proposed by JVET-U0100 are disabled to clarify the effect of the proposal compared to the original one. The anchor is EE common software base. Experimental results are summarized as in the following tables.

Table 1 The simulation results of the proposed method in RA 10 compared to the EE common software base with only OBMC.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE common software base with only OBMC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.07% | -0.05% | 101% | 92% |
| Class A2 | -0.07% | 0.02% | 0.04% | 102% | 96% |
| Class B | -0.04% | 0.02% | 0.09% | 101% | 87% |
| Class C | -0.04% | 0.05% | -0.10% | 101% | 88% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.01% | 0.00% | 101% | 90% |
| Class D | -0.10% | -0.05% | 0.04% | 101% | 91% |
| Class F | -0.22% | -0.20% | -0.10% | 100% | 86% |

Table 2 The simulation results of the proposed method in LDB 10 compared to the EE common software base with only OBMC.

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE common software base with only OBMC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.07% | 0.06% | 101% | 100% |
| Class C | 0.01% | -0.09% | 0.14% | 100% | 100% |
| Class E | -0.17% | 0.27% | -0.27% | 101% | 101% |
| Overall | -0.04% | 0.01% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.70% | 0.16% | 102% | 100% |
| Class F | -0.39% | -0.32% | -0.16% | 101% | 100% |

Table 3 The simulation results of the proposed method in RA 10 compared to the EE common software base with only OBMC and 12TAP.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE common software base with only OBMC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.06% | -0.04% | 101% | 96% |
| Class A2 | -0.08% | 0.04% | -0.11% | 102% | 98% |
| Class B | -0.06% | 0.01% | 0.04% | 101% | 99% |
| Class C | -0.05% | 0.05% | -0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.04% | -0.03% | 101% | 98% |
| Class D | -0.10% | -0.30% | -0.21% | 101% | 99% |
| Class F | -0.24% | -0.20% | -0.07% | 100% | 99% |

Table 4 The simulation results of the proposed method in LDB 10 compared to the EE common software base with only OBMC and 12TAP.

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over EE common software base with only OBMC and 12TAP |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.08% | -0.04% | 101% | 100% |
| Class C | -0.12% | 0.16% | -0.04% | 101% | 100% |
| Class E | -0.06% | -0.52% | -0.13% | 100% | 100% |
| Overall | -0.06% | -0.11% | -0.06% | 101% | 100% |
| Class D | -0.02% | -0.47% | -0.46% | 100% | 100% |
| Class F | -0.43% | -0.50% | 0.07% | 100% | 99% |

Conclusion

In this contribution, it was proposed that modified OBMC based on JVET-U0100 to improve coding efficiency.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

References

- Y.-J. Chang, C.-C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, and Z. Zhang, “Compression efficiency methods beyond VVC”, JVET-U0100, teleconference, 6–15 January 2021.

- https://vcgit.hhi.fraunhofer.de/jvet-u-ee2/VVCSoftware_VTM/-/tree/U0100

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, M. Winken, J. Ström, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability”, JVET-U2024, teleconference, 6–15 January 2021.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).