| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0127-v2 |
| --- | --- |

| Title: | An optimized VVC encoder implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jing Cui1, Yingming Fan1, Yuwen He2 Xiaolong Jiang1, Hongbin Liu1 Huade Shi1, Hao Yang1, Hua Yang2 Haibin Yin1, Jun Zhang1, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 228910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | +1-858-999-7093 lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the performance of an optimized VVC encoder, BVC, developed by Bytedance. Majority of the coding tools and features introduced in VVC have been integrated and optimized in the BVC encoder. In addition, it also includes real-world encoder features like scene change detection, rate control and multi-threading. When compared to HM and VTM, the corresponding global BD-rate gain and speedup factors under the Random Access configuration are summarized as follows:

- Fast preset (6 threads): 34.13%, 17x (HM), 131x (VTM)

- Slow preset (6 threads): 41.63%, 1x (HM), 8x (VTM)

# Introduction

The state-of-the-art video compression standard H.266/VVC [1] was finalized in July 2020. It has been demonstrated that 50% bitrate saving could be achieved compared to its predecessor, H.265/HEVC. To promote faster and more wide-spread market adoption of H.266/VVC, Bytedance developed a software VVC encoder, namely BVC, targeting fulfilling real-world encoding requirements. Some highlights of the BVC encoder are listed as follows:

- Self-developed framework for better support of parallel encoding, enabling task scheduling in the order of respective priorities

  - Task-level parallel processing, such as file I/O, video analysis, pre-processing, and encoding

  - Picture-level parallel encoding

  - Wavefront parallel encoding

- A variety of fast encoding algorithms optimized for each enabled coding tool at each preset

- Optimized SIMD implementation for computationally intensive modules wherein data can be processed simultaneously

- Pre-analysis features, such as video type detection (camera or SCC), adaptive GOP size determination, motion complexity analysis, and scene cut detection, etc.

- Pre-processing for noise detection and reduction

- Multiple coding modes, including Constant QP (CQP) and Constant Rate Factor (CRF)

- Encoder implementation with two predefined quality/speed presets

- Encoder usage support for both production users and experts

It is noted that the following coding tools/features have not been integrated into the BVC encoder yet:

- 10-bit encoding

- Subpictures/slices/tiles

- Wrap-around motion compensation

# Simulation Results

HM-16.19 [2] and VTM-11.0 [3] were used as benchmarks, and simulations were conducted under the Random-Access configuration following the common test conditions [4][5] except that:

- Sequences with 10-bit depth are rounded to 8-bit depth (by changing the internal bit depth from 10 to 8).

For the BVC encoder, both wavefront and picture-level parallel processing were enabled with multithreading applied. Two presets, including a fast preset and a slow preset, were tested. When calculating the global BD-rate saving, BD-rate savings of the Y, U and V component are calculated with Equation (1). PSNR_YUV is a weighted averaged with weighting factors 6, 1 and 1 for the PSNR of Y, U and V component, respectively.

PSNR_YUV=186*PSNR_Y+PSNR_U+PSNR_V           (1)

Table 1 - Table 3 present the simulation results of the BVC encoder at two presets and VTM-11.0 compared to HM-16.19, respectively. For those simulations, 6 threads are used for the BVC encoder. It could be seen that with the fast preset, the BD-rate gain over HM is still approx. 34.13% on average, but a speedup of 17 times and 131 times over HM and VTM, respectively, is observed. While with the slow preset, which is targeting at close performance to VTM, the BD-rate gain over HM is approx. 41.63% on average, but a speedup of 8x times over VTM could be achieved.

To further evaluate the impact of multiple threads, we tested our BVC encoder under different number of threads, including 6, 8, and 16, and the speedup factor compared to VTM-11.0 is shown in Figure 1. It could be easily noticed that with increased number of threads, the speedup ratio is also increased. When 16 threads are applied, the same gain over HM (i.e., 34.13%) could be maintained while the speedup compared to VTM is also nearly doubled, i.e., from 131 times to 255 times, compared to the case of 6 threads applied.

Table 1: Coding performance of the BVC encoder @ fast preset

| Class | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to HM |  |  |  | Speedup |  |
|  | Y (%) | U (%) | V (%) | YUV (%) | vs. HM | vs. VTM |
| A1 | -35.76 | -43.56 | -48.57 | -38.48 | 22.36 | 148.45 |
| A2 | -36.93 | -45.53 | -45.83 | -39.24 | 18.40 | 133.37 |
| B | -34.12 | -54.24 | -50.68 | -38.05 | 18.05 | 130.91 |
| C | -20.04 | -30.53 | -30.50 | -22.16 | 11.78 | 118.43 |
| Overall | -31.26 | -44.04 | -43.91 | -34.13 | 16.88 | 131.20 |

Table 2: Coding performance of the BVC encoder @ slow preset

| Class | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to HM |  |  |  | Speedup |  |
|  | Y (%) | U (%) | V (%) | YUV (%) | vs. HM | vs. VTM |
| A1 | -41.71 | -49.32 | -55.16 | -44.56 | 1.36 | 9.05 |
| A2 | -44.47 | -53.07 | -52.59 | -46.66 | 1.18 | 8.52 |
| B | -40.93 | -60.20 | -57.78 | -44.85 | 1.08 | 7.86 |
| C | -29.39 | -39.64 | -41.26 | -31.63 | 0.63 | 6.31 |
| Overall | -38.72 | -51.11 | -51.81 | -41.63 | 1.00 | 7.75 |

Table 3: Coding performance of VTM-11.0

| Class | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to HM |  |  |  | Speedup |  |
|  | Y (%) | U (%) | V (%) | YUV (%) | vs. HM | vs. VTM |
| A1 | -44.50 | -48.74 | -53.86 | -46.22 | 0.15 | N/A |
| A2 | -47.75 | -47.75 | -46.30 | -48.30 | 0.14 |  |
| B | -42.14 | -55.89 | -52.62 | -44.85 | 0.14 |  |
| C | -32.25 | -39.32 | -40.56 | -33.80 | 0.10 |  |
| Overall | -41.09 | -48.41 | -48.39 | -42.87 | 0.13 |  |

Figure 1. The speedup of the BVC encoder (fast preset) compared to VTM-11.0 under different number of threads.

# Conclusion

The BVC encoder, as an optimized VVC software encoder, provides multiple features and presets. Simulation results show that the fast preset could keep majority of the coding gain from the VTM and 255 times faster compared to VTM could be obtained via 16 threads. With multithreading, the BVC encoder is asserted to be efficient enough for video-on-demand applications.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, Oct. 2020.

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.19/

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0

- K. Sühring, K. Sharman, “Common Test Conditions for HM Video Coding Experiments,” JVET-U1100, Dec. 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video,” JVET-T2010, Oct. 2020.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).