| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0129 |
| --- | --- |

| Title: | AHG12: 3D Geometry for Global Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hossein Golestani Christian Rohlfing Mathias Wien | Tel: Email: | golestani@ient.rwth-aachen.de rohlfing@ient.rwth-aachen.de wien@lfb.rwth-aachen.de |
| Source: | RWTH Aachen |  |  |

_____________________________

# Abstract

This contribution reports a research project on developing 3D geometry-based global motion compensation for video sequences captured by monocular moving cameras. DayLightRoad, ParkRuning, and MarketPlace are examples of the target sequences. The main idea is to estimate 3D scene geometry and camera motion data from 2D video sequences and use them for global motion compensation. Since this 3D data provides a better scene understanding, integrating it into video coding can be beneficial. The proposed method applies Structure-from-Motion (SfM) to the target 2D video sequences to estimate their camera parameters and 3D scene geometry (in the form of point clouds or 3D meshes). The extracted 3D data guides a rendering engine to synthesize 3D-based virtual Reference Pictures (RPs). These RPs are offered to the VVC encoder by adding them into Reference Picture Lists (RPLs). This contribution states that the 3D-based rendered RP can provide better motion compensation performance compared to regular RPs.

The proposed method can be applied directly to video sequences captured by an unknown monocular moving camera. Any pre-knowledge about the camera (like focal length), having stereo/RGBD cameras, using additional sensors like GPS, Inertial Measurement Unit (IMU), or LiDAR sensors (in self-driving cars) might accelerate the procedure and improve the coding gain. In this document, the proposed method applied to 7 video sequences with camera motion. It is reported that the proposed method achieves around 3% bit-rate reduction over VTM10.0 on average on the used test sequences.

# Problem Statement

The majority of 2D videos are initially captured in a 3D environment, and some of them are recorded by a moving camera. It is well known that 2D motion models (e.g., translational or affine) perform well on CTC sequences. However, this contribution presents another direction: developing 3D-based global motion compensation by reconstructing the 3D scene and camera trajectories (figure 1). If one knows the 3D scene geometry and camera motion, they might be able to predict the position of stationary objects in upcoming frames more accurately. This contribution renders virtual Reference Pictures (RPs) for the forthcoming frames by warping regular RPs guided by the estimated 3D data.

Figure 1. The estimated 3D geometry and camera trajectories

The same 3D data must be available on both encoder and decoder sides to sync the encoder and decoder. For this reason, one may estimate the 3D data from decoded frames at both encoder and decoder sides. However, the quality of the extracted 3D data from the decoded material can be pretty low. Moreover, this solution can make the decoder too complex. The proposed method estimates the 3D data from uncompressed frames at the encoder side and transmits it to the decoder as side information. This overhead, however, could eat up the whole coding gain or even make the proposed system worse than the anchor. This document states that the problem can be solved by compressing camera parameters and employing 3D mesh processing techniques like mesh de-noising, mesh decimation, and mesh compression.

# Proposed Solution

In this section, the presented algorithm is reviewed. Figure 2 depicts the block diagram of the proposed method.

## Encoder

The camera displacement is continuously monitored by visual computer vision techniques or embedded sensors (if any). If the camera is static, the standard VVC encoder is called; otherwise, a flag triggers the proposed system. This flag (called flag_3D) has to be encoded and transmitted to the decoder. Henceforward, the proposed approach is described.

### Camera Localization and Mesh Reconstruction

Camera parameters (CPs), i.e., lens parameters and 3D poses, are estimated using Structure-from-Motion (SfM) [1], encoded with LZMA [2], and transmitted to the decoder side (bitstream B2.1 in figure 2). Dense reconstruction [3] generates the scene's 3D geometry from a subset of previous frames in the form of a point cloud and a list of visible 3D points per camera. Considering the visibility list, the estimated point cloud is converted into a triangular 3D mesh.

### Mesh Processing and Compression

The proposed method estimates the 3D data at the encoder side and send it to the decoder as an overhead. In order to reduce the overhead, the estimated mesh is processed, decimated, and compressed before the transmission.

Yes

No

Yes

No

Move?

Floating-point Mesh-based

Ref. Picture Synthesis

RPLs Modification

Mesh Processing & Compression

VVC Decoder

VVC Encoder

CPs Decoding

Geometry Encoding

B2.2

B2.3

Mesh Decimation

Mesh Decoding

B2.1

Display

Mesh Decoding

CPs Decoding

Video Encoder

RPLs Modification

Previously Encoded Frames

Floating-point Mesh-based

Ref. Picture Synthesis

Topology Encoding

Dense Reconstruction

& Delaunay Meshing

CPs Processing & Encoding

Camera Localization

Input

Video

Figure 2. The proposed VVC-based video coding structure: encoder (left) and decoder (right)

Previously Decoded Frames

Video Encoder

Pre-processing

VVC Encoder

VVC Decoder

Display

Flag_3D

Flag_3D

B1

Mesh Pre-Processing: the 3D meshes generated by automatic SfM-based pipelines can be pretty noisy. Noisy components of the mesh may harm local error metrics used for edge collapse in mesh decimation and damage mesh compression efficiency by increasing un-predictability. The proposed method tries to clean the 3D mesh by removing non-manifold and extra-large faces, detecting and dissolving vertices with extremely sharp and long edges, removing isolated or duplicated vertices/edges, and smoothing the mesh.

Mesh Decimation: mesh decimation is applied to the cleaned mesh to reduce the vertex/face count of the mesh with minimal shape changes. In this contribution, mesh decimation using quadric error metrics [4] is employed. The Decimation Factor (DF), i.e., the percentage of remaining vertices after decimation, determines the decimation's intensity. The lower the DF, the heavier the decimation. Different DFs are tested (DF ∈ {0.05, 0.1, 0.3, 0.5}) and DF=0.1 is selected since it has shown to provide a good compromise between mesh fidelity and mesh size in preliminary experiments. Some examples of original and decimated meshes are given in Figure 3.

Mesh Compression: a triangular mesh is represented by its geometry (i.e., point cloud) and topology (i.e., connectivity). A mesh compression method should provide solutions for both data types. This contribution uses Google Draco [5], an open-source library developed by Google. Draco is recently chosen as the anchor of new mesh compression activities by MPEG [6]. Draco employs a k-d tree for topology compression (like [7]) and Edgebreaker [8] for connectivity compression.

### Mesh-based Reference Picture Synthesis

For every frame, except IRAPs, a virtual RP is rendered by warping regular RPs to the to-be-encoded frame's camera position. The decoded CPs and 3D mesh guide the Depth Image-based Rendering (DIBR) procedure (like VSRS [9]). Unlike most DIBR scenarios where only quantized 2D depth maps are available, the proposed method has access to the estimated 3D mesh. This fact can expedite the rendering procedure by eliminating time-consuming steps of DIBR, e.g., virtual depth map synthesis and depth map median filtering.

Figure 3. The original and decimated meshes with DF=0.1

Reference Pictures Lists (RPLs) Manipulation

VVC has two RPLs and two active references per list, by default. Two modes have been defined in this document: Additional Reference (AR) and Replace Reference (RR). In AR mode, the synthesized RP is always added to RPLs as an additional active reference. However, in RR mode, the synthesized RP is replaced with the second active reference if two RPs exist or added as a further active reference if only one active RP exists. Compared to VTM10.0, AR mode has one more active reference picture.

The modified RPLs are then offered to VVC to be used in motion compensation. Note that VVC applies its 2D motion models to all RPs (including the 3D-based RP) and decides which reference should be selected for each CU through rate-distortion optimization. This way, VVC can take its own RP(s) if the synthesized RPs couldn't provide a better motion compensation performance.

## Decoder

If the flag-3D (proposed in Section 2.1) is zero, we run the regular decoding procedure; otherwise, the 3D-based decoding routine is called: First, CPs and 3D geometry are decoded by the corresponding decoders of LZMA [2] and Google Draco [5]. Then the regular RPs are warped to the position of the to-be-decoded frame, guided by decoded CPs and 3D geometry. The same RPLs modification strategy, as used by the encoder in Section 2.1.4, is applied here to update the RPLs. Finally, VVC decodes the input video considering the updated RPLs.

# Experimental Results

## A Visual Example

An example of a synthesized reference picture for random access main profile and GOP size of 16 is given in figure 4. In this example, frame number 80 (Fig. 4a) should be motion-compensated from reference frame number 64 (Fig. 4b). The proposed method takes frame number 64 as the regular RP and warps it to the position of frame number 80 to virtually synthesize a 3D-based RP for frame number 80 (Fig. 4e). It can be seen that the objects with valid depth data (e.g., static objects) are better synthesized than the moving objects (e.g., runners' legs), which do not have

Figure 4. A visual example of the 3D-based synthesized reference picture for ParkRunning sequence

reliable depth. Since the 3D-based synthesized RP is already geometrically motion-compensated, offering it to the VVC may improve the motion compensation performance.

## Video Coding Results

The proposed method applied to 3 CTC sequences (DayLightRoad, ParkRunning, and MarketPlace) and 4 non-CTC sequences (NetFlix-DrivingPOV, MountainBay, IceRock, and NetFlix-Aerial). Note that this contribution only targets video sequences with camera motion. VTM 10.0 in random access profile is used as the anchor. Since the rendering engine was initially configured to synthesize 8-bit color depth RPs, VTM 10.0 is used in 8-bit internal depth. The rendering engine may need to be re-configured to 10-bit later on aiming to run VTM in 10-bit internal depth. However, conceptually they are similar. Table 1 reports the overhead size in Kbyte (B2.1 and B2.2 in figure 2), BD-Rates before and after adding the overhead for two configurations: Replaced Reference and Additional Reference (section 2.1.4).

## Encoding/Decoding time

The encoding and decoding running times are reported in Table 2. The total run-time adds up the run-time of all blocks, e.g., camera calibration, mesh estimation, mesh decimation and compression, mesh-based RP synthesis, and VVC encoding/decoding. The reported simulation results state that the proposed method's total encoding time is almost equal to or less than the anchor. That can be justified by noting that the synthesized RPs are rendered based on the scene's geometry and camera motion; thus, they may offer a better motion compensation than the regular RPs. That may help the encoder to decide faster. Compared to the Replaced Reference (RR) mode, the encoder has more active reference pictures to be processed in the Additional Reference (AR) mode. That may increase the run-time.

Table 1. BD-Rates (%) and overheads (kB) for different configurations – Anchor: VTM 10.0

Table 2. Encoding and decoding time comparison – Anchor: VTM 10.0

The presented simulation results report that the proposed decoder is more complex than VTM 10.0. It’s mainly due to the non-optimized rendering engine. More investigations need to be carried out to further reduce the rendering run-time. For instance, the preliminary investigations states that there is no need to synthesize RPs for all frames, as some of them do not provide huge gains. Dropping these RPs accelerates the decoder. Of course, in this case, the encoder has to decide which RPs must be synthesized (a signalling might be required here). Also, the initial analysis express that encoder selects the 3D-based RPs more frequently in specific regions (e.g., the regions with almost static objects). This can be investigated more to define a measure to find regions where the synthesis is more profitable; the rest can be simply skipped.

# Reference

[1] J.L. Schönberger and J. Frahm, “Structure-from-Motion Revisited,” CVPR 2016, pp. 4104-4113.

[2] D. Salomon, "Data Compression: The Complete Reference," Fourth Edition, Springer, 2007.

[3] J.L. Schönberger, E. Zheng, JM. Frahm, M. Pollefeys, “Pixelwise View Selection for Unstructured Multi-View Stereo,” ECCV 2016.

[4] M. Garland, Y. Zhou, “Quadric-based simplification in any dimension”, ACM Trans. Graph- 24, 2 (2005), pp: 209-239.

[5] Draco, Accessed 01.10.2020 [Online], Available: https://github.com/google/draco.

[6] D. Graziosi, A Zaghetto, A. Tabatabai, [PCC-MESH] Anchor generation for the Call for Proposal on Dynamic Mesh Compression, ISO/IEC JTC1/SC29/WG07, 133rd meeting, m56120, Online, Jan. 2021.

[7] E. Hubo, T. Mertens, T. Haber, and P. Bekaert, "The Quantized kd-Tree: Efficient Ray Tracing of Compressed Point Clouds," 2006 IEEE Sym. Int. Ray Tracing, 2006, pp. 105-113.

[8] J. Rossignac, A. Safonova, and A. Szymczak, “Edgebreaker on a Corner Table: A simple technique for representing and compressing triangulated surfaces,” Hierarchical and geometrical methods in scientific visualizations, Springer, pp. 41-50, 2003.

[9] T. Senoh, N. Tetsutani, H. Yasuda, M. Teratani, “[MPEG-I Visual] Proposed View Synthesis Reference Software (pVSRS4.3) Manual,” ISO/IEC JTC1/SC29/WG11, M44031.v5, Oct. 2018, Macau.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).