| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0133 |
| --- | --- |

| Title: | AHG8: Content Adaptive Transform Precision for High Bit Depth Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser, Franck Galpin, Fabrice Le Leannec Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

The intermediate transform coefficient precision is fixed to 16 bit signed representation in VVC. Therefore, a pre-computed down-shift is performed after inverse quantization and inverse transform in order to avoid exceeding the 16 bit buffer. It is argued that this down-shift is too conservative as the prediction mechanism results in low magnitude residuals. For High Bit-Depth coding, the down-shift is increased to ensure 16 bit representation, which further decreases the quality of the inverse transformed coefficients. This contribution proposes to adaptively assign the down-shift according the contents.

# Introduction

As shown in Figure 1, the operation of the dequantization and inverse transform includes three shifts.

Figure 1 Down-shift in VVC

According to VVC specification, shift1 is computed as:

bdShift = BitDepth + rectNonTsFlag +		(1133)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + sh_dep_quant_used_flag

And Shift2 is fixed to 7:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(1163)

And Shift 3 is :

bdShift = ( nTbH > 1  &&  nTbW > 1 ) ? ( 20 − BitDepth ) : ( 21 − BitDepth )	(1164)

That is, shift1 is proportional to the input bit-depth. Thus, for high bit-depth coding, higher downshift is used after dequantization that leads to unnecessary decrease of the coefficient precision. The 2nd shift is constant whereas the third is a normalization shift to the original sample precision.

It is observed that the pre-computed downshifts are typically higher than what is required to restrict to 16 bit precision. This is especially true for high bit-depth sequences that require higher downshift although the residuals are of low magnitude.

# Proposed Method

It is proposed to adaptively assign the downshift by examining the values of upon dequantization. The method, named Content Adaptive Transform Precision (CATP), is illustrated in Figure 2.

Figure 2 proposed modified downshift

That is, before performing the first shift, it is proposed to use a buffer of 32 bits and compute the maximum value. The first downshift is then chosen to fit the current considered transform dynamic range internal precision (15 or bitDepth + 6).

The relation between the new shift and the old one is:

ShiftDequant+ShiftMid+ShiftNormalization=Shift1+Shift2+Shift3

Two versions of CATP are tested:

## CATP0

In this method, only the precision after the dequantization is adaptively set. Let M be the maximum absolute dequantized coefficient (before down shift), the first shift is computed as:

ShiftDequant=ceillog2M-1-TrDynamicRange

The second shift is modified according to the first shift as:

ShiftMid=Shift2+Shift1-ShiftDequant

And the last shift is computed as:

ShiftNormalization=Shift1-ShiftDequant+Shift2-ShiftMid+Shift3

## CATP1

The principle of adaptive shift is further extended to mid transform shift. That is, after computed the first shift as in CATP0:

ShiftDequant=ceillog2M-1-TrDynamicRange

The maximum coefficient after first inverse transform (before the downshift) is computed. Let B be this value, then the middle shift is computed as:

ShiftMid=ceillog2B-TrDynamicRange+Shift2

And the normalization shift remains the same:

ShiftNormalization=Shift1-ShiftDequant+Shift2-ShiftMid+Shift3

# Experimental results

Both CATP0 and CATP1 are implemented on top of VTM-12.0 following the common testing conditions defined in JVET-U2018. The simulation results are provided below.

## CATP0

### Low QP:

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall PQ | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.12% | -0.23% | -0.25% | 108% | 114% |
| HLG422 | -0.11% | -0.21% | -0.22% | 109% | 113% |
| Overall | -0.12% | -0.22% | -0.23% | 109% | 114% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.19% | -0.28% | -0.31% | 105% | 109% |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall HLG | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 112% | 122% |
| SVT12 | -0.12% | -0.30% | -0.30% | 110% | 126% |
| Overall | -0.06% | -0.15% | -0.15% | 111% | 124% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 106% | 108% |
| SVT12 | -0.16% | -0.27% | -0.28% | 107% | 109% |
| Overall | -0.08% | -0.14% | -0.14% | 106% | 108% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall RGB | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

### normal QP:

Random access

|  | Y | U | V | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class H1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class H2 |  |  |  | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

All Intra

|  | Y | U | V | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class H1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class H2 |  |  |  | 0.00% | -0.05% | -0.12% | 106% | 109% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## CATP1

### Low QP:

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall PQ | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.21% | -0.40% | -0.42% | 111% | 112% |
| HLG422 | -0.20% | -0.35% | -0.38% | 110% | 114% |
| Overall | -0.20% | -0.38% | -0.40% | 111% | 113% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.29% | -0.45% | -0.48% | 106% | 111% |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall HLG | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 113% | 120% |
| SVT12 | -0.23% | -0.46% | -0.46% | 105% | 122% |
| Overall | -0.11% | -0.23% | -0.23% | 109% | 121% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 105% | 106% |
| SVT12 | -0.26% | -0.36% | -0.36% | 107% | 105% |
| Overall | -0.13% | -0.18% | -0.18% | 106% | 106% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall RGB | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

### normal QP:

|  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |
|  | wPSNR |  |  | PSNR |  |  |  |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class H2 |  |  |  | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | All Intra |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |
|  | wPSNR |  |  | PSNR |  |  |  |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class H2 |  |  |  | 0.01% | -0.03% | -0.13% | 105% | 107% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Additional results Low QP:

It is noted that when coding SVT 16 bit contents, the internal precision is increased using the following equation:

TrDynamicRange = bitDepth + 6

That is, the effect of adaptive precision is less obvious as higher precision is used for transform coefficients. We therefore test the effect of reducing the TrDynamicRange for 16 bit contents. Specifically, the following dynamic range is tested for both anchor and test:

TrDynamicRange = bitDepth

## CATP0

### Low QP

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.41% | -0.62% | -0.61% | 111% | 114% |
| SVT12 | -0.12% | -0.30% | -0.30% | 110% | 126% |
| Overall | -0.26% | -0.46% | -0.46% | 110% | 120% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall RGB | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## CATP1

### Low QP

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.50% | -1.81% | -1.82% | 111% | 112% |
| SVT12 | -0.23% | -0.46% | -0.46% | 105% | 122% |
| Overall | -0.86% | -1.13% | -1.14% | 108% | 117% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall RGB | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution proposes a method for adaptive precision of transform coefficients for high bitdepth coding. The method provides bitrate saving especially when the transform precision is close to the input data depth. Thus, this allows increased precision while keeping the processing at lower bitdepth unchanged. It is recommended to further study this method within the core experiment on high bitdepth coding.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).