| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0137-v1 |
| --- | --- |

| Title: | EE1-1.1: neural network based in-loop filter using depthwise separable convolution and regular convolution (JVET-U0061) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zeqiang Li, Cheung Auyeung, Xiaozhong Xu, Wei Wang, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-857-869-3368 {zeqiangli, cauyeung, xiaozhongxu} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution presents the updated result of the neural-network-based in loop filter using depthwise separable convolution (DSC) in EE1.1 with new test condition that GOP=32, and QP= {22,27,32,37,42}. With VTM-11.0 as anchor, on average results from RA/AI configurations report gains for luma BD-Rate are 1.15% and 1.61% respectively.

# Introduction

This contribution provides updated results for DSC neural network in JVET-U0061[2] with the same model. With same code base with JVET-U0061[2] on VTM-10.0, The EE1-1.1 use new test condition with GOP=32, to be able to compare with results generated from VTM-11 as anchor.

The model is trained for QP= {22, 27, 32, 37}, during the inference stage, when QP > 37, the QP map will be cast to 37 as the input QP map. And same as JVET-U0061[2], the clipping is also intended in the training/test stage.

## Depthwise Separable convolution with Pytorch

In depthwise separable convolution (DSC), the output value of the layer with tensor input size (N,Cin,H,W) and output with size (N,Cout,Hout,Wout) can be achieved in two steps with depthwise convolution. in the first step followed by pointwise convolution in the second step.

In the first step, DSC applies depthwise convolution using a single convolutional filter per each input channel k:

outNi,k=biask+weightk ⋆inputNi,k

where ⋆ is the valid 2-D cross-correlation operator, N is a batch size, C denotes a number of channels, H is a height of input planes in pixels, and W is width in pixels.

Then in the second step, it applies pointwise convolution to create the output of the DSC as a linear combination of the outputs of the depthwise convolution:

outNi, Coutj=biasCoutj+ k=0Cin-1weightCoutj,k ⋅outNi,k

When the kernel size of the 2-D convolution is (h,w), the number of weight is Cin⋅h⋅w+1+Cout⋅(Cin+1) and the number MACs is Hout⋅Wout⋅(Cin⋅h⋅w+Cin+Cin⋅Cout+Cout),

# Description of the neural network based in-loop filter in JVET-U0060/ U0061 with depthwise separable convolution (DSC)

JVET-U0060 [1] reported the results of adding an additional filter called Dense Residual Convolutional Neural Network Based In-loop Filter (DRNLF) inserted between deblocking filter and SAO as shown in Figure 1. As shown in Figure 2, the DRNLF includes a QP Map Quantizer unit, a Pre-Processing unit, a Main-processing unit and a Post-Processing unit.

Figure 1: DRNLF in JVET-U0060 is an in-loop filter in between deblocking filter and SAO. The inputs to the DRMLF is the output of the deblocking filter and the QP map of the picture.

Figure 2: DRNLF in JVET-U0060 consists of a QP Map Quantizer, a Pre-Processing unit, a Main Processing unit and a Post-Processing unit.

In JVET-U0061[2], a clipping step is added to the Pre-Processing unit depicted in Figure 3 from JVET-U0060 [1] to obtain the Pre-Procssing unit in Figure 4 . The clipping is intended to impove the results in the training and the inference process of the neural network.

Figure 3: DRNLF Pre-Processing in JVET-U0060.

Figure 4:Clipping is added to the Pre-Processing to impove the results of the neural network.

# Network information in training stage

Table 1: Network information for the NN-based video coding tool testing with DSC in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Intel Core i7-8700 3.20 GHz (6 cores), 32GB RAM and one 11 GB Nvidia GTX 1080Ti |
|  | Framework: | PyTorch v1.4.0 |
|  | Number of GPUs per Task | 1 Nvidia GTX 1080Ti was used for training |
|  | Epoch: | 300 |
|  | Batch size: | 16 |
|  | Training time: | 48h |
|  | Training data information: | DIV2K still images |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37 |
| Optional | Learning rate | 1e-3 |
|  | Number of iterations |  |
|  | Patch size | 64x64 |
|  | Learning rate: | 1.00E-03 |
|  | Optimizer: | ADAM |
|  | Loss function: | L1 |
|  | Preprocessing: | randomly cropped and rotated |
|  | Other information: |  |

# Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage with DSC

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Pytorch 1.2.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 22371 |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | 0.085338593 |
|  | MAC (Giga) per 3840x2160 pixels | 186.64 |
| Optional |  |  |
|  | Total Conv. Layers | 13 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points | (e.g. ) |
|  | Peak Memory Usage |  |
|  | Other information: |  |

# Results

The software was integrated with VTM10.0 using DSC, but with GOP=32 and QP= {22,27,32,37,42} in order to be comparable with VTM11.0. Results are reported as follows:

Table 3. Performance of the proposed method on top of VTM11.0 (RA)

|  | Random access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -1.77% | -3.40% | -3.78% | -3.92% | -2.97% | -3.59% | 95% | 3849% |
| Class A2 | -1.19% | -4.41% | -2.92% | -0.97% | -3.45% | -1.60% | 94% | 2495% |
| Class B | -0.78% | -5.52% | -3.81% | -0.39% | -5.14% | -2.70% | 106% | 2535% |
| Class C | -1.11% | -5.76% | -4.36% | -0.77% | -5.42% | -3.63% | 105% | 3691% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.15% | -4.94% | -3.77% | -1.32% | -4.44% | -2.90% | 101% | 3037% |
| Class D | -1.40% | -4.20% | -4.43% | -0.40% | -4.77% | -4.64% | 104% | 2446% |
| Class F | -0.42% | -2.02% | -2.18% | -0.22% | -2.58% | -2.58% | 111% | 1634% |

Table 4. Performance of the proposed method on top of VTM11.0 (AI)

|  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -1.41% | -3.07% | -3.63% | -2.98% | -3.55% | -4.20% | 114% | 3712% |
| Class A2 | -1.34% | -3.81% | -2.97% | -1.09% | -3.21% | -1.87% | 107% | 2821% |
| Class B | -1.04% | -4.85% | -4.27% | -0.75% | -4.98% | -3.63% | 105% | 2870% |
| Class C | -2.04% | -6.53% | -5.57% | -1.34% | -6.22% | -5.43% | 103% | 3359% |
| Class E | -2.45% | -3.18% | -3.57% | -1.51% | -2.72% | -2.40% | 106% | 4997% |
| Overall | -1.61% | -4.48% | -4.12% | -1.44% | -4.35% | -3.63% | 107% | #DIV/0! |
| Class D | -2.49% | -4.82% | -5.81% | -1.31% | -5.08% | -5.99% | 103% | 3077% |
| Class F | -0.94% | -2.58% | -2.49% | -0.65% | -3.04% | -2.95% | 102% | 2432% |

# Conclusions

This contribution provides the updated results for the DSC loop filter tool from JVET-U0061[2]. The results from this contribution demonstrated that DSC still have 1.15% and 1.61% gain on BD-Rate for RA/AI configurations compare with anchor of VTM-11.0.

# References

- Cheung Auyeung, Wei Wang, Wei Jiang, Xiang Li, Shan Liu,”EE1.1: A comparison of depthwise separable convolution and regular convolution with the JVET-T0057 neural network based in-loop filter”, JVET-U0060, 21st Meeting, by teleconference, 6–15 Jan. 2021.

- Cheung Auyeung, Xiang Li, Shan Liu, “EE-1.1-related: BD-Rate improvements to JVET-T0057 neural network based in-loop filter using depthwise separable convolution and regular convolution” JVET-U0061, 21st Meeting, by teleconference, 6–15 Jan. 2021.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).