| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0146 |
| --- | --- |

| Title: | Crosscheck of Decoder Side Refinement Tools in JVET-V0120 and Additional Results |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Semih Esenlik Elena Alshina | Tel: Email: | semih.esenlik@huawei.com elena.alshina@huawei.cim |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results for tool-on tests related to decoder side MV refinement in JVET-V0120. The performance and runtimes are reportedly verified. This document also reports additional tests performed by the authors for better understanding of the tools.

# Introduction

Historically the decoder side MV refinement is considered to be a computationally complex method of coding, hence in the VVC a highly simplified version of DMVR is adopted that can be implemented using currently available hardware principles. Notably, the spatial MV prediction from refined MVs are disabled, which was considered the biggest obstacle for adoption of DMVR during the development of VVC. Furthermore special care was taken to limit the number of processing cycles necessary for application of DMVR on a coding block, which affects the decoding time and the depth of the decoding pipeline.

This crosscheck document reports the results of the following EE tests:

| 3.3 | Template matching (TM) |
| --- | --- |
| 3.4 | Multi-pass decoder-side motion vector refinement |
| 3.5 | Multi-pass decoder-side motion vector refinement with sample-based BDOF |
| 7.2 | Template + bilateral matching (3.3 + 3.5) |

In order to have a better understanding on the complexity-coding gain trade-off, addition to the results of the following aspects are reported:

| Additional test 1 | Test 7.2 with refinement of the motion vectors sequentially by Template Matching and DMVR on the same coding block is disabled (either only DMVR is applied or Template matching is applied). |
| --- | --- |
| Additional test 2 | The spatial prediction from refined motion vectors in VTM12 is enabled as in JVET-V0120. |

The 2 additional tests reported above are performed, since it is believed by the authors that those two aspects pose serious challenges to hardware implementations.

# Simulation results for EE2 tests

## Tool on results for Template matching, EE test 3.3

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over EE anchor |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.11% | -2.33% | -2.90% | 116% | 111% |
| Class A2 | -2.96% | -2.95% | -2.99% | 115% | 117% |
| Class B | -2.03% | -2.28% | -2.24% | 118% | 115% |
| Class C | -2.11% | -2.28% | -2.47% | 119% | 121% |
| Class E |  |  |  |  |  |
| Overall | -2.26% | -2.42% | -2.58% | 117% | 116% |
| Class D | -1.99% | -2.05% | -2.09% | 119% | 123% |
| Class F | -0.91% | -0.98% | -1.14% | 111% | 108% |

## Tool on results for Multi-Pass DMVR, EE test 3.4

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over EE anchor |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.75% | -1.70% | -1.78% | 114% | 185% |
| Class A2 | -2.91% | -2.66% | -2.83% | 113% | 238% |
| Class B | -1.41% | -1.30% | -1.27% | 113% | 204% |
| Class C | -1.44% | -1.37% | -1.32% | 110% | 200% |
| Class E |  |  |  |  |  |
| Overall | -1.78% | -1.67% | -1.70% | 112% | 205% |
| Class D | -1.57% | -1.15% | -1.22% | 109% | 206% |
| Class F | -0.55% | -0.75% | -0.72% | 108% | 159% |

## Tool on results for Multi-Pass DMVR and sample Based BDOF, EE test 3.5

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over EE Anchor |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.75% | -1.71% | -1.85% | 116% | 188% |
| Class A2 | -2.94% | -2.64% | -2.74% | 115% | 240% |
| Class B | -1.49% | -1.35% | -1.31% | 115% | 207% |
| Class C | -1.57% | -1.39% | -1.44% | 112% | 203% |
| Class E |  |  |  |  |  |
| Overall | -1.85% | -1.69% | -1.74% | 115% | 208% |
| Class D | -1.79% | -1.39% | -1.33% | 111% | 208% |
| Class F | -0.60% | -0.62% | -0.68% | 109% | 161% |

## Combination of 3.3 and 3.4, EE test 7.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.58% | -3.66% | -4.40% | 141% | 191% |
| Class A2 | -5.41% | -5.24% | -5.32% | 140% | 251% |
| Class B | -3.35% | -3.44% | -3.43% | 143% | 220% |
| Class C | -3.63% | -3.63% | -3.81% | 141% | 229% |
| Class E |  |  |  |  |  |
| Overall | -3.88% | -3.89% | -4.10% | 141% | 222% |
| Class D | -3.91% | -3.70% | -3.76% | 138% | 234% |
| Class F | -1.44% | -1.64% | -1.90% | 125% | 161% |

# Additional simulation results

## Disabling joint application of TM and DMVR on the same CU

In EE test 7.2, the DMVR and template matching methods are applied sequentially for the same CU. In other words the motion vector is refined 2 times, first by DMVR and secondly by template matching.

The additional test results below indicate that about 1% coding loss is observed (compared to EE test 7.2 reported above) if a coding block is restricted apply either TM or DMVR but not both. In the table below the anchor is EE anchor. In the test TM, Multi-Pass DMVR and sample based BDOF are enabled whereas a coding block is restricted to apply either template matching or DMVR.

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over EE anchor |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.60% | -2.69% | -3.18% | 137% | 174% |
| Class A2 | -4.01% | -3.95% | -3.96% | 136% | 226% |
| Class B | -2.45% | -2.49% | -2.42% | 138% | 196% |
| Class C | -2.59% | -2.51% | -2.67% | 138% | 203% |
| Class E |  |  |  |  |  |
| Overall | -2.83% | -2.83% | -2.95% | 137% | 199% |
| Class D | -2.74% | -2.51% | -2.60% | 136% | 213% |
| Class F | -1.13% | -1.25% | -1.41% | 124% | 153% |

## Enabling spatial MV prediction from refined MV in VTM 12

The spatial prediction from refined MVs are prohibited in VVC, due hardware implementation concerns. In the table below results of removing that restriction in VTM is reported.

In the table below the anchor is VTM12. In the test spatial prediction from refined MV is enabled.

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 12 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.46% | -0.46% | 101% | 101% |
| Class A2 | -1.17% | -0.89% | -0.96% | 101% | 101% |
| Class B | -0.41% | -0.29% | -0.32% | 101% | 104% |
| Class C | -0.37% | -0.28% | -0.15% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -0.44% | -0.43% | 101% | 103% |
| Class D | -0.38% | -0.25% | -0.08% | 101% | 104% |
| Class F | -0.09% | -0.06% | -0.12% | 101% | 105% |

The results above show that removing of the restriction on refined MV usage can already provide 0.6% coding gain on VTM 12. This aspect is suggested to be investigated carefully for template matching and DMVR tools in JVET-V0120.

# Conclusion

This contribution reports the crosscheck results for the EE2 tests: 3.3, 3.4, 3.5 and 7.2. The reported results are confirmed by the crosscheckers.

Additional results are reported for the aspects that are identified during this crosscheck to have serious implication on hardware implementations. It is suggested that these aspects are investigated very carefully by JVET.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).