| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0150 |
| --- | --- |

| Title: | AHG8: A study on bin rate of VTM-12.0 for high bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution reports the coding performance of VTM-12.0 relative to HM-16.22 for high bit depth coding under low QP setting (QP=-13,…,12 for 12-bit, QP=-33,…,-8 for 16-bit) and normal QP setting (QP=22, 27, 32, 37). The average results are summarized as follows:

In the normal QPs,

- VTM achieves BD-bitrate saving of -17.17% for AI and -31.12% for RA, respectively.

- BD-bin rate saving is -2.44% for AI and -23.78% for RA, respectively.

- The unweighted bin to bit ratios of VTM are roughly 13~15% higher than HM’s. The weighted ones are roughly 20% higher than HM’s.

In the low QPs,

- BD-bitrate reduction for AI/LDB/RA is {-6.79%, -6.61%, -6.81%} for PQ, {-3.68%, -5.14%, -5.72%} for HLG, and {18.22%, 8.65%, 7.36%} for SVT, respectively.

- BD-bin rate for AI/LDB/RA is increased by {10.26%, 7.11%, 7.17%} for PQ, {6.05%, 4.65%, 4.19%} for HLG, and {9.88%, 4.51%, 4.14%} for SVT, respectively.

- The unweighted bin to bit ratios of VTM for AI/LDB/RA are a bit higher than HM by {6.6%, 5.28%, 3.59%} for PQ, {3,18%, 3.89%, 2.87%} for HLG, and {-5.63%, -5.63%, -5.99%} for SVT, respectively. The weighted bin to bit ratios of VTM for AI/LDB/RA are higher than HM by {18.25%, 17.78%, 17.14%} for PQ, {11.40%, 12.75%, 12.38%} for HLG and {-10.87%, -5.25%, -6.47%} for SVT, respectively.

In addition, the coding performance of VTM-12.0 CE Anchor for low QP is reported as below.

- BD-bitrate reduction for AI/LDB/RA is {-7.24%, -6.68%, -6.93%} for PQ, {-4.09%, -5.12%, -5.73%} for HLG and {-0.92%, -1.91%, -2.29%} for SVT, respectively.

- BD-bin rate for AI/LDB/RA is {9.93%, 7.07%, 7.09%} for PQ, {5.58%, 4.76%, 4.27%} for HLG and {-3.36%, -2.50%, -2.32%} for SVT, respectively.

- The unweighted bin to bit ratios of VTM for AI/LDB/RA are a bit higher than HM by {6.70%, 5.40%, 3.69%} for PQ, {3.16%, 3.98%, 2.96%} for HLG, and {-5.13%, -5.15%, -5.52%} for SVT, respectively. The weighted bin to bit ratios of VTM for AI/LDB/RA are higher than HM by {18.47%, 18.12%, 17.58%} for PQ, {11.49%, 13.05%, 12.74%} for HLG and {-2.01%, -1.92%, -2.74%} for SVT, respectively.

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1). All the percentage numbers are measured against the HM-16.22.

A source code to measure bin rate is also attached.

Introduction

One of the scopes of AHG8: High bit depth, high bit rate, and high frame rate coding is as below.

- Study VVC entropy decoding throughput in the cases of high bit depth, high bit rate, and high frame rate coding.

To investigate CABAC throughput of VTM-12.0 [1] for high bit depth coding, a study was conducted to measure the bin to bit ratio and BD-bin rate following JVET-N0049 [4] , under low QP setting (QP=-13,…,12 for 12bit, QP=-33,…,-8 for 16bit) and normal QP setting (QP=22, 27, 32, 37) compared to HM-16.22 [2] . Additionally, delta bin-rate was also measured by using Bjøntegaard Delta-rate on bitrate vs bin rate. Tests for VTM-12.0 CE Anchor is also conducted for low QP setting.

Simulation results

Tests were conducted by using the VTM-12.0 and HM-16.22 for the low QPs (QP=-13,…,12 for 12bit, QP=-33,…,-8 for 16bit) and normal QPs (QP=22, 27, 32, 37) under the common test condition defined in [5] . For the low QPs, additional tests were conducted by using VTM-12.0 CE Anchor [3] . Note that to align GOP size between VTM-12.0 and HM-16.22, GOP size is set to 16.

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bitrate of a coded sequence

- The average context coded bin rate, bypass bin rate and bitrate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bitrate consumed for the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements for the peak case, for the average case and for the transform coefficients coding part are calculated based on the collected data:

- Weighted BD-bin rate

- {Weighted, Unweighted} bin to bit ratio

- Weighted bin rate-increase

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1).

The summary results for the normal QPs, low QPs and middle QPs are provided in Table 1, Table 2 and Table 3, respectively. For detailed results, please refer to the attached spreadsheets.

Table 1: Summary results of normal QPs (Anchor=HM-16.22 Tested=VTM-12.0, GOP16)

|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | Y | U | V | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| Class H1 | -22.51% | -66.14% | -76.91% | -14.21% | -60.08% | -72.23% | 10.65% | 1.19 | 1.33 | 12.16% | 0.88 | 1.05 | 19.71% | 369% | 171% |
| Class H2 | -39.73% | -60.33% | -59.80% | -33.35% | -55.29% | -54.66% | 14.40% | 1.21 | 1.37 | 13.31% | 0.92 | 1.08 | 17.80% | 366% | 161% |
| Overall | -31.12% | -63.24% | -68.36% | -23.78% | -57.69% | -63.44% | 12.52% | 1.20 | 1.35 | 12.74% | 0.90 | 1.07 | 18.73% | 368% | 166% |
|  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | Y | U | V | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| Class H1 | -11.88% | -67.99% | -54.93% | 3.18% | -59.81% | -49.10% | 17.36% | 1.21 | 1.37 | 13.35% | 0.92 | 1.11 | 20.51% | 2549% | 173% |
| Class H2 | -22.47% | -49.15% | -49.93% | -8.06% | -39.51% | -40.31% | 18.52% | 1.21 | 1.40 | 16.41% | 0.94 | 1.15 | 21.88% | 3194% | 170% |
| Overall | -17.17% | -58.57% | -52.43% | -2.44% | -49.66% | -44.71% | 17.94% | 1.21 | 1.39 | 14.88% | 0.93 | 1.13 | 21.20% | 2854% | 172% |

In the normal QPs (Table 1),

- VTM achieves BD-bitrate saving of -17.17% for AI and -31.12% for RA thanks to introducing new and more coding tools.

- BD-bin rate saving is -2.44% for AI and -23.78% for RA, respectively. It indicates VTM consumes less bins for the same PSNR values.

- The unweighted bin to bit ratios of VTM are roughly 13~15% higher than HM’s. The weighted ones are roughly 20% higher than HM’s.

- The weighted delta bin rate is roughly 17.94% for AI and 12.52% for RA, respectively. It indicates VTM consume more bins for the same bitrate values than HM’s. Note that in normal QPs, bitrate for RA covers roughly 300Kbps to 24Mbps.

Table 2: Summary results for low QPs (Anchor=HM-16.22 Tested=VTM-12.0)

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -5.42% | -6.08% | -6.69% | 12.83% | 12.03% | 11.47% | 18.06% | 1.19 | 1.28 | 6.89% | 0.76 | 0.90 | 18.74% | 3197% | 170% |
| PQ422 | -8.17% | -12.30% | -12.82% | 7.68% | 3.68% | 2.95% | 14.64% | 1.19 | 1.27 | 6.31% | 0.74 | 0.87 | 17.74% | 2718% | 167% |
| Overall | -6.79% | -9.19% | -9.75% | 10.26% | 7.85% | 7.21% | 16.35% | 1.19 | 1.27 | 6.60% | 0.75 | 0.88 | 18.25% | 2958% | 169% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -6.00% | -4.18% | -5.75% | 9.24% | 10.65% | 9.60% | 15.21% | 1.19 | 1.26 | 6.13% | 0.74 | 0.89 | 19.51% | 314% | 162% |
| PQ422 | -7.21% | -11.17% | -11.80% | 4.97% | 1.32% | 0.41% | 11.08% | 1.18 | 1.23 | 4.43% | 0.71 | 0.82 | 15.97% | 402% | 155% |
| Overall | -6.61% | -7.67% | -8.77% | 7.11% | 5.98% | 5.00% | 13.14% | 1.18 | 1.25 | 5.28% | 0.73 | 0.86 | 17.78% | 358% | 158% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -6.14% | -4.32% | -5.56% | 9.22% | 10.69% | 9.81% | 15.48% | 1.19 | 1.23 | 3.84% | 0.71 | 0.84 | 17.67% | 375% | 171% |
| PQ422 | -7.47% | -11.50% | -11.72% | 5.12% | 1.23% | 0.71% | 11.66% | 1.18 | 1.22 | 3.34% | 0.69 | 0.80 | 16.60% | 505% | 165% |
| Overall | -6.81% | -7.91% | -8.64% | 7.17% | 5.96% | 5.26% | 13.57% | 1.19 | 1.23 | 3.59% | 0.70 | 0.82 | 17.14% | 440% | 168% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -3.39% | -5.40% | -5.68% | 6.59% | 4.86% | 4.46% | 8.99% | 1.20 | 1.25 | 3.70% | 0.72 | 0.80 | 11.52% | 3821% | 191% |
| HLG422 | -3.97% | -6.73% | -6.70% | 5.50% | 3.25% | 3.14% | 8.26% | 1.20 | 1.23 | 2.66% | 0.69 | 0.77 | 11.28% | 3496% | 183% |
| Overall | -3.68% | -6.06% | -6.19% | 6.05% | 4.06% | 3.80% | 8.62% | 1.20 | 1.24 | 3.18% | 0.71 | 0.79 | 11.40% | 3658% | 187% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -5.36% | -7.49% | -6.79% | 5.54% | 3.45% | 4.05% | 9.68% | 1.20 | 1.25 | 4.49% | 0.72 | 0.82 | 12.99% | 459% | 169% |
| HLG422 | -4.92% | -8.86% | -7.23% | 3.76% | 0.01% | 1.29% | 7.32% | 1.20 | 1.23 | 3.28% | 0.69 | 0.78 | 12.50% | 583% | 179% |
| Overall | -5.14% | -8.18% | -7.01% | 4.65% | 1.73% | 2.67% | 8.50% | 1.20 | 1.24 | 3.89% | 0.71 | 0.80 | 12.75% | 521% | 174% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -5.92% | -8.08% | -7.19% | 5.08% | 2.93% | 3.78% | 9.94% | 1.20 | 1.24 | 3.44% | 0.70 | 0.79 | 12.64% | 520% | 162% |
| HLG422 | -5.51% | -9.19% | -7.44% | 3.29% | -0.32% | 1.08% | 7.56% | 1.20 | 1.23 | 2.30% | 0.68 | 0.76 | 12.11% | 713% | 167% |
| Overall | -5.72% | -8.64% | -7.32% | 4.19% | 1.31% | 2.43% | 8.75% | 1.20 | 1.23 | 2.87% | 0.69 | 0.77 | 12.38% | 616% | 165% |

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 36.32% | 34.27% | 34.20% | 20.63% | 19.20% | 19.16% | -1.98% | 1.19 | 1.11 | -6.50% | 0.46 | 0.41 | -10.87% | 5195% | 135% |
| SVT12 | 0.13% | 0.68% | 0.58% | -0.87% | -0.48% | -0.55% | -1.19% | 1.22 | 1.16 | -4.77% | 0.57 | 0.57 | -0.74% | 6195% | 155% |
| Overall | 18.22% | 17.48% | 17.39% | 9.88% | 9.36% | 9.31% | -1.59% | 1.21 | 1.14 | -5.63% | 0.52 | 0.49 | -5.25% | 5695% | 145% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 20.21% | 21.49% | 21.55% | 11.76% | 12.27% | 12.32% | -0.90% | 1.19 | 1.11 | -6.34% | 0.45 | 0.41 | -10.75% | 611% | 148% |
| SVT12 | -2.91% | -0.57% | -0.55% | -2.74% | -1.31% | -1.30% | -1.57% | 1.22 | 1.16 | -4.94% | 0.56 | 0.56 | -0.99% | 700% | 134% |
| Overall | 8.65% | 10.46% | 10.50% | 4.51% | 5.48% | 5.51% | -1.23% | 1.21 | 1.14 | -5.63% | 0.51 | 0.48 | -5.35% | 656% | 141% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 17.97% | 20.35% | 20.33% | 10.71% | 12.00% | 11.99% | -0.61% | 1.18 | 1.11 | -6.57% | 0.45 | 0.40 | -12.06% | 779% | 145% |
| SVT12 | -3.26% | -0.83% | -0.82% | -2.44% | -0.87% | -0.86% | -1.06% | 1.23 | 1.16 | -5.42% | 0.55 | 0.54 | -1.93% | 894% | 147% |
| Overall | 7.36% | 9.76% | 9.76% | 4.14% | 5.57% | 5.56% | -0.83% | 1.21 | 1.13 | -5.99% | 0.50 | 0.47 | -6.47% | 836% | 146% |

In the low QPs (Table2),

- BD-bitrate reduction for AI/LDB/RA is {-6.79%, -6.61%, -6.81%} for PQ, {-3.68%, -5.14%, -5.72%} for HLG, and {18.22%, 8.65%, 7.36%} for SVT, respectively. Compared to the normal QPs, BD-rate reduction for HDR sequences is degraded by roughly 10% for AI and 25% for RA. For RGB sequences (SVT), since VTM takes lower rice parameters than HM, it shows poor coding performance.

- BD-bin rate for AI/LDB/RA is increased by {10.26%, 7.11%, 7.17%} for PQ, {6.05%, 4.65%, 4.19%} for HLG, and {9.88%, 4.51%, 4.14%} for SVT, respectively. This indicates that VTM consumes a bit higher number of bins for the same PSNR values compared to HM at higher bitrate.

- The unweighted bin to bit ratios of VTM for AI/LDB/RA are a bit higher than HM by {6.6%, 5.28%, 3.59%} for PQ, {3,18%, 3.89%, 2.87%} for HLG, and {-5.63%, -5.63%, -5.99%} for SVT, respectively. The weighted bin to bit ratios of VTM for AI/LDB/RA are higher than HM by {18.25%, 17.78%, 17.14%} for PQ, {11.40%, 12.75%, 12.38%} for HLG and {-10.87%, -5.25%, -6.47%} for SVT, respectively.

- The weighted delta bin rate for AI/LDB/RA is {16.35%, 13.14%, 13.57%} for PQ, {8.62%, 8.50%, 8.75%} for HLG, and {-1.59%, -1.23%, -0.83%} for SVT, respectively. In low QPs, bitrate for RA covers roughly 50Mbps to 800Mbps for PQ, 100Mbps to 1Gbps for HLG, 800 Mbps to 2.3 Gbps for SVT 12-bit, and 2 Gbps to 5 Gbps for SVT 16-bit, respectively. It indicates that there is a cross point where bin rate of VTM is lower than HM’s at very high bitrate over 1 Gbps.

Table 3 shows the results of VTM-12.0 CE Anchor. It is observed that

- BD-bitrate reduction for AI/LDB/RA is {-7.24%, -6.68%, -6.93%} for PQ, {-4.09%, -5.12%, -5.73%} for HLG and {-0.92%, -1.91%, -2.29%} for SVT, respectively. As can be seen, by applying rice parameter fix, coding performance of VTM is drastically improved for SVT 16-bit sequence. However, coding performance of VTM is still lower by 0.7% for AI than HM.

- BD-bin rate for AI/LDB/RA is {9.93%, 7.07%, 7.09%} for PQ, {5.58%, 4.76%, 4.27%} for HLG and {-3.36%, -2.50%, -2.32%} for SVT, respectively.

- The unweighted bin to bit ratios of VTM for AI/LDB/RA are a bit higher than HM by {6.70%, 5.40%, 3.69%} for PQ, {3.16%, 3.98%, 2.96%} for HLG, and {-5.13%, -5.15%, -5.52%} for SVT, respectively. The weighted bin to bit ratios of VTM for AI/LDB/RA are higher than HM by {18.47%, 18.12%, 17.58%} for PQ, {11.49%, 13.05%, 12.74%} for HLG and {-2.01%, -1.92%, -2.74%} for SVT, respectively.

- The weighted delta bin rate for AI/LDB/RA is {16.46%, 13.18%, 13.62%} for PQ, {8.49%, 8.64%, 8.87%} for HLG, and {-2.95%, -1.59%, -1.17%} for SVT, respectively. In low QPs, bitrate for RA covers roughly 50Mbps to 800Mbps for PQ, 100Mbps to 1Gbps for HLG, 800 Mbps to 2.3 Gbps for SVT 12-bit, and 2 Gbps to 5 Gbps for SVT 16-bit, respectively. It indicates that there is a cross point where bin rate of VTM is lower than HM’s at very high bitrate over 1 Gbps.

Table 3: Summary results for low QPs (Anchor=HM-16.22 Tested=VTM-12.0 CE Anchor)

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | Weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -5.82% | -6.50% | -7.13% | 12.53% | 11.71% | 11.13% | 18.14% | 1.19 | 1.28 | 6.94% | 0.76 | 0.90 | 18.91% | 3067% | 156% |
| PQ422 | -8.66% | -12.78% | -13.32% | 7.33% | 3.33% | 2.58% | 14.78% | 1.19 | 1.27 | 6.46% | 0.74 | 0.87 | 18.03% | 2606% | 157% |
| Overall | -7.24% | -9.64% | -10.23% | 9.93% | 7.52% | 6.86% | 16.46% | 1.19 | 1.27 | 6.70% | 0.75 | 0.89 | 18.47% | 2837% | 156% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -6.06% | -4.23% | -5.79% | 9.23% | 10.64% | 9.60% | 15.27% | 1.19 | 1.26 | 6.19% | 0.74 | 0.89 | 19.78% | 298% | 150% |
| PQ422 | -7.30% | -11.23% | -11.86% | 4.90% | 1.28% | 0.37% | 11.10% | 1.18 | 1.24 | 4.60% | 0.71 | 0.83 | 16.38% | 381% | 142% |
| Overall | -6.68% | -7.73% | -8.82% | 7.07% | 5.96% | 4.98% | 13.18% | 1.18 | 1.25 | 5.40% | 0.73 | 0.86 | 18.12% | 340% | 146% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | -6.26% | -4.44% | -5.69% | 9.15% | 10.62% | 9.74% | 15.52% | 1.19 | 1.23 | 3.87% | 0.71 | 0.84 | 17.92% | 355% | 157% |
| PQ422 | -7.61% | -11.64% | -11.85% | 5.03% | 1.15% | 0.64% | 11.72% | 1.18 | 1.22 | 3.50% | 0.69 | 0.81 | 17.23% | 484% | 155% |
| Overall | -6.93% | -8.04% | -8.77% | 7.09% | 5.89% | 5.19% | 13.62% | 1.19 | 1.23 | 3.69% | 0.70 | 0.82 | 17.58% | 420% | 156% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -3.75% | -5.67% | -5.96% | 6.21% | 4.55% | 4.14% | 8.91% | 1.20 | 1.25 | 3.71% | 0.72 | 0.80 | 11.64% | 3823% | 187% |
| HLG422 | -4.44% | -7.11% | -7.10% | 4.96% | 2.77% | 2.64% | 8.06% | 1.20 | 1.23 | 2.61% | 0.69 | 0.77 | 11.34% | 3531% | 184% |
| Overall | -4.09% | -6.39% | -6.53% | 5.58% | 3.66% | 3.39% | 8.49% | 1.20 | 1.24 | 3.16% | 0.71 | 0.79 | 11.49% | 3677% | 186% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -5.34% | -7.53% | -6.84% | 5.63% | 3.53% | 4.13% | 9.80% | 1.20 | 1.25 | 4.58% | 0.72 | 0.82 | 13.25% | 463% | 164% |
| HLG422 | -4.90% | -8.88% | -7.29% | 3.88% | 0.14% | 1.40% | 7.47% | 1.20 | 1.24 | 3.38% | 0.69 | 0.78 | 12.84% | 566% | 172% |
| Overall | -5.12% | -8.20% | -7.06% | 4.76% | 1.84% | 2.76% | 8.64% | 1.20 | 1.24 | 3.98% | 0.71 | 0.80 | 13.05% | 514% | 168% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | -5.93% | -8.12% | -7.24% | 5.16% | 3.00% | 3.85% | 10.05% | 1.20 | 1.24 | 3.53% | 0.70 | 0.79 | 12.95% | 529% | 164% |
| HLG422 | -5.54% | -9.24% | -7.51% | 3.38% | -0.24% | 1.16% | 7.69% | 1.20 | 1.23 | 2.39% | 0.68 | 0.76 | 12.52% | 720% | 172% |
| Overall | -5.73% | -8.68% | -7.37% | 4.27% | 1.38% | 2.50% | 8.87% | 1.20 | 1.24 | 2.96% | 0.69 | 0.78 | 12.74% | 625% | 168% |

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.70% | 0.82% | 0.79% | -3.41% | -3.37% | -3.40% | -3.88% | 1.19 | 1.13 | -5.14% | 0.46 | 0.44 | -3.98% | 5062% | 108% |
| SVT12 | -2.54% | -1.95% | -2.03% | -3.30% | -2.90% | -2.96% | -2.02% | 1.22 | 1.16 | -5.11% | 0.57 | 0.57 | -0.43% | 5977% | 148% |
| Overall | -0.92% | -0.57% | -0.62% | -3.36% | -3.14% | -3.18% | -2.95% | 1.21 | 1.15 | -5.13% | 0.52 | 0.51 | -2.01% | 5519% | 128% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.52% | 1.13% | 1.13% | -1.76% | -0.86% | -0.86% | -1.49% | 1.19 | 1.13 | -4.94% | 0.45 | 0.44 | -3.51% | 588% | 134% |
| SVT12 | -3.31% | -1.05% | -1.02% | -3.23% | -1.91% | -1.90% | -1.70% | 1.22 | 1.16 | -5.35% | 0.56 | 0.56 | -0.63% | 708% | 144% |
| Overall | -1.91% | 0.04% | 0.05% | -2.50% | -1.39% | -1.38% | -1.59% | 1.21 | 1.14 | -5.15% | 0.51 | 0.50 | -1.92% | 648% | 139% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-bin rate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -0.86% | 0.91% | 0.91% | -1.63% | -0.62% | -0.62% | -1.13% | 1.18 | 1.12 | -5.10% | 0.45 | 0.43 | -4.07% | 749% | 129% |
| SVT12 | -3.72% | -1.32% | -1.29% | -3.02% | -1.52% | -1.51% | -1.21% | 1.23 | 1.15 | -5.93% | 0.55 | 0.54 | -1.65% | 913% | 152% |
| Overall | -2.29% | -0.21% | -0.19% | -2.32% | -1.07% | -1.06% | -1.17% | 1.21 | 1.14 | -5.52% | 0.50 | 0.49 | -2.74% | 831% | 141% |

Discussions

BD-bitrate for low QPs are significantly degraded by roughly 20% and 25%, respectively compared to normal QPs. Although rice parameter fix (CE Anchor) alleviate the coding loss for SVT 16-bit sequence, there is still performance gap between normal QPs and low QPs. It is necessary to further improve coding performance of VVC for high bit depth coding.

The weighted peak bin to bit ratio of VVC is generally higher than HEVC’s except ultra high bitrate over 1 Gbps. It implies VVC CABAC engine needs to deal with bins at faster pace than HEVC. It is also beneficial to improve CABAC throughput.

BD-bin rate for low QPs are significantly degraded by roughly 25%~30% compared to normal QPs; For the same PSNR values, VVC generally consumes more bins than HEVC except ultra high bit rate over 1 Gbps. If we want to improve BD-bin rate at high bitrate, it is necessary to improve further coding performance of VVC or reduce its bin rate with BD-bitrate kept.

It is recommended to continuingly study bin rate of VVC for high bit depth coding.

Reference

- VTM-12.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-12.0

- HM-16.22, https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tags/HM-16.22

- VTM-12.0 CE Anchor, https://vcgit.hhi.fraunhofer.de/jvet-u-ce/VVCSoftware_VTM/-/tree/CE-U2022

- M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19,” JVET-N0049, Mar. 2019

- A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding,” JVET-U2018, Jan. 2021

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).