| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0151 |
| --- | --- |

| Title: | Crosscheck of JVET-V0093 (AHG9: Film grain estimation and film grain synthesis for VVC Film grain characteristics SEI message, film grain estimation and film grain synthesis modules) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Peng Yin, Sean McCarthy | Email: | {pyin,Sean.McCarthy}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-V0093 (AHG9: Film grain estimation and film grain synthesis for VVC Film grain characteristics SEI message, film grain estimation and film grain synthesis modules). The cross-checkers checked the software implementation and performed the film grain estimation and synthesis test on several sequences. The software is functioning as described in JVET-V0093.

- Introduction

JVET-V0093 proposes a complete VTM software implementation of the film grain feature, including an estimation module at encoder side and a synthesis module at decoder side. The implementation is based on the SMPTE RDD 5 Film Grain Technology specification, with the adaptations to use VVC core DCT-II transform instead of the transform proposed in SMPTE RDD 5.

- Simulation results

The cross-checkers checked the software implementation, mainly on film grain estimation part. MCTF as in VTM12.0 software is used as it is to remove the film grain in the processing stage. The different between input frame and MCTF filtered frame is used to estimate the FG parameters. To avoid the interference of edge and complex features, only flat regions are used for FG parameter estimation. The software currently supports only one intensity level. The estimated parameters include the noise variance, the horizontal and vertical cutoff high frequencies.

The simulations are performed following SDR RA CTC, where FGC SEI message is sent for every IRAP. CrownRun sequence as in JVET-V0093 is used for simulation. The decoding is performed with and without FG synthesis. The subjective results are confirmed as shown in JVET-V0093.

Additional simulations are also performed for several other clips. The software is functioning but may still need further improvement, especially on the stability of FG look temporally and also among different QPs, when the considered video clip presents strong temporal variations.

The crosscheckers appreciate the proponents’ effort to complete an initial software implementation of end to end FG technology workflow and recommend the software to be available publicly in a fork of the VTM git repository for further study and improvement.

Some additional results are shown in Figure 1.

- decoded frame

- decoded frame + film grain insertion

Figure 1 BikeSparkler sequence: frame# 423, Qp=37

- Reference

- Miloš Radosavljević, Edouard François, “AHG9: Film grain estimation and film grain synthesis for VVC – SEI message characteristics, film grain estimation and film grain synthesis modules”, JVET-V0093. April. 2021