| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20-28th Apr. 2021 | Document: JVET-V0166 |
| --- | --- |

| Title: | Cross-check report of AHG8: Content Adaptive Transform Precision for High Bit Depth Coding (JVET-V0133) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for AHG8: Content Adaptive Transform Precision for High Bit Depth Coding (JVET-V0133). Proponents of JVET-V0133 consider bit depth precision of the inverse transform and suggest that a fixed, input bit depth dependent normalization shifts applied to prevent exceeding the bitdepth of inverse transform is too conservative. Instead, a content dependent down-shift operations are introduced. The amount of required right shift is derived from transform coefficient values being decoded at the decoder side. Simulation results in low QP configuration have been cross-checked. In terms of coding performance, results for those QP points reported by proponent were replicated during the cross-check.

# Introduction

This document reports results of the cross-check for JVET-V0166.  The following work was conducted:

- Cross-check of simulation results for method 1 (CAT0).

- Cross-check of simulation results for method 2 (CAT2).

# Cross-check results

In terms of coding performance, results, reported by proponent were replicated during the cross-check.

VTM12.0 in HBD  configuration has been used as an Anchor.

For the Method 2 (CATP2) results were fully replicated to what is reported by the proponents. For the method 1 (CATP0), a mismatch in results identified for some QP points for Method 1, the mismatch is within 0.02% of BD-rate.

Tables 1 present Method 1 results for HBD/HBR CTC in Low QP lossy coding configuration.

| HDR PQ |  |  |  |  |  | AI |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Over VTM12.0 |  |  |  |  |  |  |  |  |
|  |  | wPsnrY |  | wPsnrU |  | wPsnrV |  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| PQ444 |  | 0.01% |  | 0.00% |  | 0.00% |  | 0.01% |  | 0.01% |  | 0.00% | 108% | 0% |
| PQ422 |  | -0.01% |  | 0.01% |  | 0.00% |  | -0.01% |  | 0.00% |  | -0.01% | 110% | 0% |
| Overall |  | 0.00% |  | 0.01% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% | 109% | 0% |
|  |  |  |  |  |  | LDB |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM12.0 |  |  |  |  |  |  |  |  |
|  |  | wPsnrY |  | wPsnrU |  | wPsnrV |  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| PQ444 |  | -0.22% |  | -0.25% |  | -0.23% |  | -0.22% |  | -0.25% |  | -0.24% | 110% | #NUM! |
| PQ422 |  | -0.22% |  | -0.16% |  | -0.17% |  | -0.22% |  | -0.16% |  | -0.17% | 109% | #NUM! |
| Overall |  | -0.22% |  | -0.21% |  | -0.20% |  | -0.22% |  | -0.21% |  | -0.20% | 110% | #NUM! |
|  |  |  |  |  |  | RA |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM12.0 |  |  |  |  |  |  |  |  |
|  |  | wPsnrY |  | wPsnrU |  | wPsnrV |  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| PQ444 |  | -0.15% |  | -0.20% |  | -0.21% |  | -0.16% |  | -0.19% |  | -0.21% | 106% | #NUM! |
| PQ422 |  | -0.16% |  | -0.13% |  | -0.14% |  | -0.16% |  | -0.13% |  | -0.14% | 108% | #NUM! |
| Overall |  | -0.16% |  | -0.16% |  | -0.17% |  | -0.16% |  | -0.16% |  | -0.17% | 107% | #NUM! |
| Overall PQ |  | -0.13% |  | -0.12% |  | -0.12% |  | -0.13% |  | -0.12% |  | -0.13% | 109% | #NUM! |
| HDR HLG |  |  |  |  | AI |  |  |  |  |  |
|  |  |  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | psnrY |  | psnrU |  | psnrV |  | EncT |  | DecT |  |
| HLG444 | -0.12% |  | -0.24% |  | -0.25% |  | 109% |  | #NUM! |  |
| HLG422 | -0.10% |  | -0.21% |  | -0.22% |  | 109% |  | #NUM! |  |
| Overall | -0.11% |  | -0.22% |  | -0.23% |  | 109% |  | #NUM! |  |
|  |  |  |  |  | LDB |  |  |  |  |  |
|  |  |  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | psnrY |  | psnrU |  | psnrV |  | EncT |  | DecT |  |
| HLG444 | -0.20% |  | -0.27% |  | -0.30% |  | 106% |  | #NUM! |  |
| HLG422 | -0.17% |  | -0.19% |  | -0.23% |  | 105% |  | #NUM! |  |
| Overall | -0.19% |  | -0.23% |  | -0.27% |  | 105% |  | #NUM! |  |
|  |  |  |  |  | RA |  |  |  |  |  |
|  |  |  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | psnrY |  | psnrU |  | psnrV |  | EncT |  | DecT |  |
| HLG444 | -0.12% |  | -0.19% |  | -0.22% |  | 107% |  | #NUM! |  |
| HLG422 | -0.12% |  | -0.14% |  | -0.17% |  | 107% |  | #NUM! |  |
| Overall | -0.12% |  | -0.17% |  | -0.19% |  | 107% |  | #NUM! |  |
| Overall HLG | -0.14% |  | -0.21% |  | -0.23% |  | 107% |  | #NUM! |  |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 110% | #NUM! |
| SVT12 | -0.12% | -0.30% | -0.30% | 108% | #NUM! |
| Overall | -0.06% | -0.15% | -0.15% | 109% | #NUM! |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.01% | 0.00% | 0.00% | 110% | #NUM! |
| SVT12 | -0.17% | -0.26% | -0.28% | 111% | #NUM! |
| Overall | -0.09% | -0.13% | -0.14% | 110% | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 107% | #NUM! |
| SVT12 | -0.08% | -0.29% | -0.29% | 109% | #NUM! |
| Overall | -0.04% | -0.14% | -0.15% | 108% | #NUM! |
| Overall RGB | -0.06% | -0.14% | -0.15% | 109% | #NUM! |

Tables 2 present Method 2 results for HBD/HBR CTC in Low QP lossy coding configuration.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.51% | -0.88% | -0.96% | -0.50% | -0.89% | -0.97% | 109% | 108% |
| PQ422 | -0.49% | -0.64% | -0.70% | -0.48% | -0.63% | -0.69% | 110% | 107% |
| Overall | -0.50% | -0.76% | -0.83% | -0.49% | -0.76% | -0.83% | 110% | 107% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.39% | -0.44% | -0.47% | -0.39% | -0.43% | -0.47% | 106% | 109% |
| PQ422 | -0.38% | -0.32% | -0.34% | -0.38% | -0.31% | -0.33% | 107% | 114% |
| Overall | -0.39% | -0.38% | -0.40% | -0.38% | -0.37% | -0.40% | 107% | 111% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.31% | -0.38% | -0.41% | -0.31% | -0.37% | -0.41% | 107% | 110% |
| PQ422 | -0.31% | -0.28% | -0.29% | -0.31% | -0.27% | -0.29% | 107% | 109% |
| Overall | -0.31% | -0.33% | -0.35% | -0.31% | -0.32% | -0.35% | 107% | 110% |
| Overall PQ | -0.40% | -0.49% | -0.53% | -0.39% | -0.48% | -0.53% | 108% | 109% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.21% | -0.40% | -0.42% | 111% | 108% |
| HLG422 | -0.20% | -0.35% | -0.38% | 109% | 106% |
| Overall | -0.20% | -0.38% | -0.40% | 110% | 107% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.29% | -0.45% | -0.48% | 106% | 107% |
| HLG422 | -0.26% | -0.31% | -0.37% | 105% | 107% |
| Overall | -0.28% | -0.38% | -0.43% | 106% | 107% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.21% | -0.34% | -0.37% | 107% | 110% |
| HLG422 | -0.20% | -0.27% | -0.31% | 107% | 108% |
| Overall | -0.20% | -0.30% | -0.34% | 107% | 109% |
| Overall HLG | -0.23% | -0.35% | -0.39% | 108% | 108% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 110% | 110% |
| SVT12 | -0.23% | -0.46% | -0.46% | 108% | 109% |
| Overall | -0.11% | -0.23% | -0.23% | 109% | 110% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 105% | 108% |
| SVT12 | -0.26% | -0.36% | -0.36% | 109% | 112% |
| Overall | -0.13% | -0.18% | -0.18% | 107% | 110% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 108% | 108% |
| SVT12 | -0.17% | -0.37% | -0.37% | 110% | 107% |
| Overall | -0.08% | -0.18% | -0.19% | 109% | 108% |
| Overall RGB | -0.11% | -0.20% | -0.20% | 108% | 109% |

# References

[1] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu.

[2] JVET-V0166 AHG8: Content Adaptive Transform Precision for High Bit Depth Coding (JVET-V0133),  K. Naser, F. Galpin, F. Le Léannec, P. De Lagrange (InterDigital)