| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0174 |
| --- | --- |

| Title: | Report on VVC compression performance verification testing in the SDR HD Random Access, SDR HD Low Delay and 360 Category |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vittorio Baroncini Mathias Wien | Tel: Email: | baroncini@gmx.com wien@lfb.rwth-aachen.de |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Abstract

This document reports the second set of verification test results comparing VVC to its predecessor, HEVC. The compression performance capabilities of the two specifications are compared based on the HEVC reference software HM-16.22, and the VVC reference software VTM-11.0. In the SDR HD random access category, the open-source VVC implementation VVenC-0.3 is included in the evaluation. The results of a visual assessment of VVC compared to HEVC performing formal subjective assessment evaluation (with the participation of naïve test subjects) is reported. The assessment this time was made at HD resolution content and included three Categories: SDR encoded in random-access configuration, SDR encoded in Low Delay configuration and the 360° video category assessed through viewports. The measured MOS figures indicate a significant improvement of all the tested configurations over HEVC. A huge number video material was tested at this time, namely 60 test points for SDR RA, 60 test points for SDR SD and 80 test points for the 360° video category. This huge testing activities were conducted at Vabtech and GBtech labs, regardless the severe restrictions imposed by COVID pandemic, with the participation of more than 60 non-expert viewers.

# Introduction

A major design goal for the development of the VVC standard was to achieve a substantial improvement in compression capability relative to its predecessor, the HEVC standard. This document is the second verification test report in a series of reports addressing a variety of test categories and embracing some of the available versatile tools provided by the VVC standard. It reports the results of a verification test to confirm that this goal was achieved and to estimate the magnitude of that achievement.

A subjective evaluation was conducted at two test sites comparing the VVC Main 10 profile to the HEVC Main 10 profile for the SDR HD test category with random-access and low delay configurations, as well as for the 360 category.

# Verification test logistics

The SDR HD and 360 subjective tests were carried out at the following test sites:

- GBTech,

- Vabtech.

The tests were conducted using the DCR test method [9] with a 11-grade impairment scale. The verification test environment and the testing methodology is described in Annex A. The test set-up at the two test sites is shown in Table 1.

More than 60 test subjects participated to the test. All subjects were screened for vision acuity and color vision.

All subjects showed a recently done COVID test. The access to the test laboratory was allowed to two subjects at a time; one staying in the test room and one staying in the rest/ waiting room; the two subjects moved back and forth to the two rooms, trying to keeping in any time the COVID “social distance” of at least one meter.

Table 1: Test site information and set up.

| Test Site | GBTech | Vabtech |
| --- | --- | --- |
| Display, size (resolution setting) | LG 65” CX6LA (3840x2160) | LG 65” CX6LA (3840x2160) |
| Viewing distance | 1 viewer at 1,5H | 1 viewer at 1.5H |
| Viewing angle | 90° (at screen center) | 90° (at screen center) |
| Total number of viewers | 30 (16 females, 14 males; age 18-24), all screened for visual acuity and normal colour vision. | 29 (20 females, 9 males, age 18-24), all screened for visual acuity and normal colour vision. |

# Verification test generation

In the test, the HEVC bitstreams were encoded using the HEVC reference software HM16.22 [2]. For VVC, two encoder implementations were used. One set of bitstreams was encoded using the VTM-11.0 reference software [3], a second set of bitstreams was encoded using the open source VVC implementation VVenC-0.3 [4].

## Verification test content generation

According to the definition of the SDR HD RA tests in the verification test plan [1], motion compensated filtering was to be enabled for both, HM-16.22 and VTM-11.0. For HM-16.22, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_main10.cfg in the software package was used with TemporalFilter=1. For VTM-11.0, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_vtm.cfg  of the VTM-11.0 software package was employed. This configuration file enables temporal filtering by default. For the VVenc-0.3 software, the Medium configuration has been used. VVenC was operated with perceptual QP adaptation and without rate control. For the purpose of the verification tests, the QPs were selected such that the VVenC rate was approximately the same or lower than the VTM bitrate.

In the SDR HD low delay category, HEVC and VVC were compared using the HM-16.22 for HEVC and the VTM-11.0 test models. For HM-16.22, the low-delay configuration provided with the configuration file cfg/encoder_lowdelay_main10.cfg was used. For VTM-11.0, the corresponding configuration is provided in cfg/encoder_lowdelay_vtm.cfg.

In the 360° video category, the above-mentioned HM-16.22 and VTM-11.0 random-access configurations were used with TemporalFilter=0 for both software packages.

## Test sequences and rate points selection

The selected sequences and coding QPs are listed in Table 3, 4 and 5 for HD LD, HD RA and 360 respectively. Five rate points for each test sequence have been selected for the quality assessment of the test sequences. The rate points were chosen such that the VTM/HM pair for a rate point would represent approximately same quality while at the same time allow for approximate rate matching of the HM rate point with the next VTM rate point. Thereby both assessment of rate-savings at similar quality and assessment of quality improvement at similar rates are enabled.

The generation of the 360 test sequences required the decoding of the bitstreams downloaded from the JVET FTP site and the generation of HD resolution viewports video files. The viewpaths proposed in [16] have been applied.

This process was done thanks to the viewport generation package kindly provided by Alibaba experts (reference person: Ru-Ling Liao ruling.lrl@alibaba-inc.com).

All the other test sequences were obtained decoding the bit streams provided by JVET volunteers (name of the volunteers) and made available on the JVET FTP site.

Table 2: 360 test sequences.

| Test sequence | Resolution | Fps | Frames | md5 |
| --- | --- | --- | --- | --- |
| GT_Sheriff | 4320×2160 | 30 | 300 | 0f8ecc5f4976d7cf056cdf78c4dc3f2c |
| HarborBiking2 | 8192×4096 | 30 | 300 | 746b0aba98bca07dae646c75348eed87 |
| KiteFliteWalking2 | 8192×4096 | 30 | 300 | 8a102140ff5c9aa3b3f515c512511a59 |
| SkateBoardAtBridge | 6144×3072 | 30 | 300 | 3dd5bef4f2ab2c887d4730332ee4d5de |

Table 3: QP settings for the HM and VTM software for the 360° video test sequences using the PERP projection format

| Test sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| GT_Sheriff | 36, 32, 28, 24, 20 | 38, 34, 30, 26, 22 |
| HarborBiking2 | 35, 31, 27, 23, 20 | 38, 34, 30, 26, 22 |
| KiteFliteWalking2 | 40, 36, 32, 28, 24 | 42, 38, 34, 30, 26 |
| SkateBoardAtBridge | 39, 35, 31, 27, 24 | 42, 38, 34, 30, 26 |

Table 4: QP settings for the HM and VTM software for the 360° video test sequences using the PCMP / GCMP projection format

| Test sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| GT_Sheriff | 39, 35, 31, 27, 22 | 42, 38, 34, 30, 26 |
| HarborBiking2 | 35, 31,27, 23, 20 | 38, 34, 30, 26, 22 |
| KiteFliteWalking2 | 40, 36, 32, 28, 24 | 42, 38, 34, 30, 26 |
| SkateBoardAtBridge | 38, 34, 30, 27, 23 | 42, 38, 34, 30, 26 |

Table 4: SDR HD LD test sequences.

| Test sequence | Resolution | fps | Frames | md5 |
| --- | --- | --- | --- | --- |
| Beatriz | 1920×1080 | 50 | 500 | fe74cd5046fa033b4f743f42b29e69cd |
| OfficeWalkAtWall | 1920×1080 | 30 | 300 | 529c15491ea8e1eb0320244a6ff902bb |
| OfficeWalkCeiling | 1920×1080 | 30 | 300 | 265906bc8f65441fb53d167483b9f726 |
| DOTA2 | 1920×1080 | 60 | 600 | be1c5d02a8fb298e26e5c5b890451413 |
| EuroTruckSimulator2 | 1920×1080 | 60 | 600 | f6850dbfff967945c0a273a374f28abd |
| Starcraft | 1920×1080 | 60 | 600 | e3d024bd65f4483fd5895f0e0ca44433 |

Table 5: QP settings for HM, VTM for the SDR HD LD test sequences.

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| Beatriz | 41, 37, 33, 28, 23 | 42, 38, 34, 29, 24 |
| OfficeWalkAtWall_ | 46, 42, 38, 32, 26 | 47, 44, 40, 34, 27 |
| OfficeWalkCeiling | 45, 40, 34, 30, 24 | 46, 42, 36, 32, 26 |
| DOTA2 | 38,34,29,24,20 | 40,36,31,26,22 |
| EuroTruckSimulator2 | 45,40,36,32,26 | 46,41,37,33,27 |
| Starcraft | 38,34,30,26,21 | 40,36,32,28,23 |

Table 6: SDR RA test sequences.

| Test sequence | Resolution | fps | Frames | md5 |
| --- | --- | --- | --- | --- |
| BarScene | 1920×1080 | 60 | 600 | 89b3195543e8e9a4014d38d425d4c9a3 |
| DrivingPOV | 1920×1080 | 60 | 600 | 71aa6ab712e2152c1d4142db8350ca34 |
| Metro | 1920×1080 | 60 | 600 | 1030673bd52a31705ccbba36a2959f4f |
| Meridian | 1920×1080 | 60 | 600 | 514bdb1932c11352f024edc63e524d1d |

Table 7: QP settings for HM, VTM, VVenC for the SDR HD RA test sequences.

| Sequence | HM QPs | VTM QPs | VVenC QPs |
| --- | --- | --- | --- |
| BarScene | 42, 38, 33, 28, 23 | 43, 39, 35, 31, 26 | 47, 43, 39, 35, 30 |
| DrivingPOV | 39, 34, 30, 27, 24 | 42, 38, 34, 30, 26 | 43, 39, 35, 31, 27 |
| Meridian | 39, 34, 30, 27, 25 | 42, 38, 34, 30, 26 | 47, 44, 39, 35, 31 |
| Metro | 39, 35, 31, 27, 24 | 42, 38, 34, 30, 26 | 44, 39, 35, 31, 27 |
|  |  |  | VVenC QPs |

# Test method and test design

The test sequences were evaluated using the 11-grade scale as specified in ITU-R BT.500-14, shown in Figure 1 below.

Figure 1: Meaning of the 11 grades numerical scale as specified in ITU-R BT.500-14 Table 2-4 [8].

A total of 9 test sessions were designed: three for HD LD, three for HD RA and three for 360; all the test sessions were designed inserting a “stabilization phase” ass suggested in ITU-R BT.500-14, and were not longer than 12 minutes, to avoid fatigue impact.

# Verification test results

The results of the visual assessment in at the two test sites were evaluated by the verification test coordinators, and were found to be consistent in terms of quality range and quality progression over the rate points as voted by the test subjects. Therefore, the votes of both test sites were merged into a joint evaluation providing the results provided below. The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bitrate of the corresponding bitstream. The ±0.95% confidence intervals for the MOS values are indicated.

Here below the MOS data and relevant plots are provided.

## SDR RA MOS and plots

In a crosschecking activity for the bitstreams in the SDR HD RA category, it was revealed that for the test sequences BarScene, DrivingPOV and Metro, the temporal filter had not been activated for HM-16.22 as defined in the verification test plan. The data is provided here for information, nonetheless. For the test sequence Meridian, the correct configuration was confirmed.

Table 8: SDR HD RA MOS values

Figure 2: Plots of the SDR HD RA MOS results

## SDR LD MOS and plots

Table 8: SDR HD LD MOS values

Figure 2: Plots of the SDR HD LD MOS results

## 360 GCMP (PCMP) MOS and plots

Table 8: 360 GCMP (PCMP) MOS values for the sequences GTSheriff and HarbourBiking2

Figure 2: Plots of the 360 GCMP (PCMP) MOS results for the sequences GTSheriff and HarbourBiking2.

Table 8: 360 GCMP (PCMP) MOS values for the sequences KitFliteWalking2 and SkateBoardAtBridge

Figure 3: Plots of the 360 GCMP (PCMP) MOS results for the sequences KitFliteWalking2 and SkateBoardAtBridge.

## 360 PERP MOS and plots

Table 9: 360 PERP MOS values for the sequences GTSheriff and HarbourBiking2

Figure 2: Plots of the 360 PERP MOS results for the sequences GTSheriff and HarbourBiking2.

Table 8: 360 PERP MOS values for the sequences KitFliteWalking2 and SkateBoardAtBridge

Figure 3: Plots of the 360 PERP MOS results for the sequences KitFliteWalking2 and SkateBoardAtBridge.

# Conclusions

The MOS-over-rate plots provided indicate a significant subjective quality improvement of VVC over HEVC. It is recommended to produce updated results for the SDR HD RA category in order to achieve consistent encoder configurations for the HM-16.22 across all test sequences. It is further recommended to include a performance analysis following the analysis provided for the SDR UHD category based on the presented assessment results.

# References

- M. Wien, V. Baroncini, A. Segall, and Y. Ye, “VVC verification test plan (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, output document JVET-U2021, 21st JVET meeting, by teleconference, Jan. 2021.

- HEVC Test Model, https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tree/HM-16.22, online, accessed Jan. 2021.

- VVC Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0, online, accessed Jan. 2021.

- Fraunhofer HHI, “Fraunhofer Versatile Video Encoder (VVenC),” version 0.3, https://github.com/fraunhoferhhi/vvenc/tags, online, accessed Mar. 2021.

- C. Helmrich, B. Bross, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand, “Information on and analysis of the VVC encoders in the SDR UHD verification test,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, input document JVET-T0103, Oct. 2020.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.2100-2 (2018), Image parameter values for high dynamic range television for use in production and international programme exchange.

- SMPTE ST 2084, High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays, 2014.

- SMPTE ST 2036-1, Ultra High Definition Television – Image Parameter Values for Program Production, 2014.

- ETSI TS 101 154, Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications, 2019.

- K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, A. Tourapis, “Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3),” Joint Video Experts Team, doc. JVET-S2016, teleconference, July 2020.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” in ITU-T SG 16 Q.6 document VCEG-M33, 13th VCEG meeting, Austin, Texas, USA, Apr. 2001.

- S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, “Methodology and reporting template for neural network coding tool testing,” Joint Video Experts Team, doc. JVET-T0041, teleconference, Oct. 2020.

- J. Ström, K. Andersson, Rickard Sjöberg, A. Segall, F. Bossen, G. J. Sullivan, J.-R. Ohm, A. Tourapis, “Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 output document JVET-S2016, 19th JVET meeting by teleconference, June 2020.

- M. Wien, V. Baroncini, Y. Ye, “Status Report on 360º video Verification Test Preparation,” Joint Video Experts Team, Doc. JVET-S0146, teleconference, June 2020.

# Acknowldegements

The verification test coordinators wish to thank the organizations who contributed to the VVC verification tests. [List to be provided in an output version of the verification test report]

# Annex A

The same evaluation method as for the HEVC verification tests is adopted for the VVC verification tests. The following description is based on JCTVC-Q1011 [A4] with minor adaptations.

## A.1 Test method

The test method adopted for this evaluation is DCR (Degradation Category Rating) [A1].

## A.1.1 Degradation Category Rating (DCR)

This test method is commonly adopted when the material to be evaluated shows a range of visual quality that well distributes across all quality scales. All the video material used for these tests consist of video clips of 10 seconds duration.

This method has been used under the schema of evaluation of the quality; for this reason, a quality rating scale made of 11 levels was adopted, ranging from "0" (lowest quality) to "10" (highest quality), see also Figure 1.

The structure of the Basic Test Cell (BTC) of DCR method is made by two consecutive presentations of the video clip under test; at first the original version of the video clip is displayed, immediately afterwards the coded version of the video clip is presented; then a message displays for 5 seconds asking the viewers to vote. The presentation of the video clips is preceded by a mid-grey screen displaying “Source” for the original and “Test” for the coded version of the sequence under test for one second.

Figure 3 - DCR BTC

## A.2 How to express the visual quality opinion with DCR

The viewers were asked to express their vote putting a mark on a scoring sheet.

The scoring sheet for a DCR test is made of a section for each BTC; each section has a box wherein which the viewer shall write the score ranging from 0 to 10. By writing a score of “10”, the subject will express an opinion of “best” quality, while by writing a score of “0” the subject will express an opinion of “worst” quality, as shown in Figure 1.

The vote has to be written when the message "Vote N" appears on the screen. The number "N" is a numerical progressive indication on the screen aiming to help the viewing subjects to use the appropriate box of the scoring sheet.

## A.4 Training and stabilization phase

The outcome of a test is highly dependent on a proper training of the test subjects.

For this purpose, each subject has to be trained by means of a short practice (training) session demonstrating the range of qualities to be expected in the test.

The stabilization phase uses the test material of a test session; three BTCs, containing one sample of best quality, one of the worst qualities and one of medium quality, are duplicated at the beginning of the test session. By this way, the test subjects have an immediate impression of the quality range they are expected to evaluate during that session.

The scores of the stabilization phase are discarded.

## A.5 The laboratory set-up

The laboratory for a subjective assessment were set up according to [A1], except for the selection of the display and the video play-out server. Play-out of the UHD video clips was done at the native resolution.

The PCs used to play the video sequence supported the display of 10 bit UHD at 30 and 60 frames per second, without any limitation, or without introducing any additional temporal or visual degradation. At GBTech, the connection between the PC and the display was provided by a 10 bit-capable HDMI connection. At RWTH Aachen University, the display was connected via quad-link SDI.

## A.5.2 Viewing environment

The viewing distance was 1.5H, where H is equal to the height of the active part of the screen, depending on the size of the active part of the screen and its native resolution.

The test laboratories were protected from external visual or audio pollution. Internal general light was low (just enough to allow the viewing subjects to fill out the scoring sheets) and a uniform light was placed behind the monitor, in a way no direct light hits the viewing subjects seated in front of the screen; the light behind the monitor must be dimmed to an intensity as specified in Table 4 of Recommendation ITU-T P.911 (“Typical viewing and listening conditions as used in audio-visual quality assessment”). No other light source was admitted, and in particular any light source directed to the screen or creating reflections.

## A.6 Overall test effort and subjects’ involvement

Each viewing session did not run for more than 20 minutes and the same viewing subject did not participate to the test run for more than six hours in total. Young people were hired as test subjects, selecting them for an age from 16 to 30, mostly students of scientific faculties. Viewing subjects were compensated for their participation to the testing activities.

## A.7 Statistical analysis and presentation of the results

The data collected from the score sheets, filled out by the viewing subjects, were stored in an Excel spread sheet. For each coding condition the Mean Opinion Score (MOS) and associated Confidence Interval (CI) values were computed in the spread-sheets.

The MOS and CI values are used to draw graphs. The graphs are drawn grouping the results for each video test sequence. No graph grouping results from different video sequences is considered.

From the “raw” data subject reliability should be calculated and the method used to assess subject reliability should be reported. Some criteria for subjective reliability are given in [A2] and [A3].

## A.8 References

- International Telecommunication Union Standardization Sector; Recommendation ITU-T P.910 “Subjective video quality assessment methods for multimedia applications”

- Pseudo Isochromatic Plates, engraved and printed by The Beck Engraving Co., Inc., Philadelphia and New York, United States.

- KIRK (R.E.): Experimental Design – Procedures for the Behavioural Sciences, 2nd Editions, Brooks/Cole Publishing Co., California, 1982.

- Tan, T. K.; Mrak, M.; Baroncini, V. & Ramzan, N, “Report on HEVC compression performance verification testing,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 2014.