| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0176-v1 |
| --- | --- |

| Title: | AHG4: On ITU-R BT.2245 HDR Test Content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Kenneth Andersson | Tel: Email: | +13608178487 asegall@sharplabs.com wien@lfb.rwth-aachen.de kenneth.r.andersson@ericsson.com |
| Source: |  |  |  |

_____________________________

# Abstract

As part of the VVC HDR verification test activity, three test sequences were acquired from the ITE for evaluation.  These sequences are included in the ITU-R BT.2245 recommendation.  This document describes how the sequences were converted to raw YUV files and provides initial performance analysis.

# Introduction

As part of the VVC HDR verification test activity, three test sequences were acquired from the ITE for evaluation.  These sequences were initially selected by a small group of experts participating in the verification test activity.  The sequences are listed in the table below.

| Sequence Name | Resolution | Frame Rate | Bit-Depth | Number of Frames | Transfer Function |
| --- | --- | --- | --- | --- | --- |
| Drama Sea | 3840x2160 | 60 | 10 | 960 | HLG |
| SwimRaceBackStrock | 3840x2160 | 60 | 10 | 960 | HLG |
| PaddockFollow | 3840x2160 | 60 | 10 | 960 | HLG |

Table 1 – Summary of sequences

Note: The sequence length is 16 seconds.  However, the first second is a static title slide.

A visual example of each sequence is shown below.

Sequence 7 – Drama Sea

Sequence 11 – Swim Race Backstroke

Sequence 16 - Paddock

# Conversion

The sequences were provided in a 4:2:2 sampling format and encapsulated in the MOV file format.  The sequences were converted to a YUV using the following procedure:

- Using a script provided with the content, the YUV data was extracted from the original MOV files and stored as raw 4:2:2 files.

- Using HDRConvert, the 4:2:2 data was converted to 4:2:0.  The conversion file is provided with this contribution.

# Analysis

The three sequences were compressed using VTM-11.0 and HM-16.22 for further analysis.  And, the characteristics of the sequences are reported below on a per-RAP basis.  As can be seen from the Tables, the first second of the sequence does not require a high bit-rate to achieve high fidelity.

Additional conclusions include:

- The Drama Sea sequence has a significant change in coding characteristics between the 4th and 5th RAP.  While there is not a scene cut in the sequence, there is a significant change in dynamic range at this point.

- We see that the Swim Rack Backstroke sequence appears to be easier to compress toward the end of the sequence.  This is likely due to the camera pan becoming slower.

| RAP | Bitrate | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| --- | --- | --- | --- | --- | --- |
| 0 | 344.3668 | 62.8110 | 926.7605 | 927.0589 | 56.7688 |
| 1 | 505.0412 | 45.2369 | 48.0132 | 51.9706 | 46.2553 |
| 2 | 722.9317 | 45.5348 | 47.9979 | 51.9636 | 46.5071 |
| 3 | 717.312 | 45.3103 | 47.9887 | 51.7837 | 46.3106 |
| 4 | 817.0855 | 45.1538 | 47.9242 | 51.7453 | 46.1689 |
| 5 | 3693.3342 | 44.252 | 47.0235 | 51.1084 | 45.254 |
| 6 | 9635.1582 | 42.6899 | 45.7379 | 50.0811 | 43.7511 |
| 7 | 12809.2062 | 42.2137 | 45.118 | 49.6683 | 43.2477 |
| 8 | 14644.8517 | 41.8927 | 44.7508 | 49.371 | 42.928 |
| 9 | 14548.3569 | 41.8811 | 44.7223 | 49.3219 | 42.9136 |
| 10 | 15405.0462 | 41.726 | 44.7783 | 49.3493 | 42.7861 |
| 11 | 15615.4486 | 41.6768 | 44.8133 | 49.3183 | 42.7461 |
| 12 | 13621.056 | 41.9254 | 44.9226 | 49.4276 | 42.9781 |
| 13 | 12622.5526 | 42.0305 | 44.9741 | 49.5274 | 43.0802 |
| 14 | 11891.8725 | 41.8972 | 44.9696 | 49.4936 | 42.965 |

Table 2 – Drama Sea

| RAP | Bitrate | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| --- | --- | --- | --- | --- | --- |
| 0 | 4606.9514 | 61.2851 | 926.3575 | 926.5684 | 49.0683 |
| 1 | 23413.3514 | 36.5369 | 42.7102 | 45.6228 | 37.9078 |
| 2 | 24891.4043 | 36.4308 | 42.5194 | 45.4911 | 37.7945 |
| 3 | 26965.5138 | 36.2885 | 42.2684 | 45.2983 | 37.6421 |
| 4 | 26669.5015 | 36.403 | 42.4679 | 45.5347 | 37.7661 |
| 5 | 28308.4209 | 36.3082 | 42.273 | 45.4249 | 37.663 |
| 6 | 28374.0406 | 36.3065 | 42.2565 | 45.4246 | 37.6608 |
| 7 | 27842.208 | 36.3036 | 42.2022 | 45.3311 | 37.6521 |
| 8 | 25786.464 | 36.434 | 42.3965 | 45.452 | 37.7878 |
| 9 | 24826.656 | 36.3913 | 42.3355 | 45.316 | 37.7412 |
| 10 | 23600.7655 | 36.3956 | 42.3733 | 45.2942 | 37.7473 |
| 11 | 22572.576 | 36.3549 | 42.3312 | 45.1961 | 37.7048 |
| 12 | 21089.2283 | 36.4327 | 42.4156 | 45.2513 | 37.7832 |
| 13 | 20687.6898 | 36.4883 | 42.4816 | 45.3458 | 37.8409 |
| 14 | 18404.8875 | 36.227 | 41.9443 | 44.9003 | 37.5586 |

Table 3 – Swim Rack Backstroke

| RAP | Bitrate | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| --- | --- | --- | --- | --- | --- |
| 0 | 1931.9409 | 61.91 | 926.7076 | 927.0379 | 51.0014 |
| 1 | 8542.9145 | 38.4321 | 47.2246 | 51.5022 | 39.9953 |
| 2 | 8107.6578 | 38.4502 | 47.2345 | 51.4761 | 40.0122 |
| 3 | 7514.5477 | 38.4926 | 47.2008 | 51.4743 | 40.0541 |
| 4 | 8078.9612 | 38.4711 | 47.1477 | 51.4607 | 40.0327 |
| 5 | 8226.048 | 38.462 | 47.1095 | 51.4751 | 40.0221 |
| 6 | 7971.6258 | 38.5646 | 47.1983 | 51.5691 | 40.1247 |
| 7 | 7927.7022 | 38.5161 | 47.1594 | 51.5525 | 40.0762 |
| 8 | 7581.8437 | 38.5451 | 47.2678 | 51.53 | 40.1078 |
| 9 | 7793.4129 | 38.4586 | 47.2063 | 51.3891 | 40.0217 |
| 10 | 8299.2295 | 38.4013 | 47.1706 | 51.3456 | 39.9636 |
| 11 | 9047.9483 | 38.2329 | 47.1078 | 51.4522 | 39.8018 |
| 12 | 8606.3852 | 38.2159 | 47.1361 | 51.5095 | 39.7878 |
| 13 | 7983.744 | 38.1792 | 47.0882 | 51.3787 | 39.7505 |
| 14 | 7271.115 | 38.1192 | 47.0688 | 51.2136 | 39.6872 |

Table 4 – Paddock Follow

The behavior of the VTM-11.0 and HM-16.22 was also compared, and the results are shown below.  We note the following:

- The results exclude the first RAP (RAP0)

- The results are generated with the configuration provided with the VTM-11.0 and HM-16.22, respectively.  This means that the VTM-11.0 has the temporal pre-filter enabled while the HM-16.22 result does not.

As can be seen in the Figures, VTM-11.0 provides a BD-rate gain of 29.57%, 40.78%, and 39.9% for the Drama Sea, Swim Rack Backstroke and Paddock Follow sequences, respectively.

Sequence 7 – Drama Sea

Sequence 11 – Swim Race Backstroke

Sequence 16 - Paddock