| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V2022 |
| --- | --- |

| Title: | Core Experiment on Entropy Coding for High Bit Depth and High Bit Rate Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Karam Naser Dmytro Rusanovskyy Mohammed Golam Sarwer Fan Wang | Email: | Karam.Naser@InterDigital.com dmytror@qti.qualcomm.com m.sarwer@alibaba-inc.com wangfan6@oppo.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to conduct a study of coding tools for High Bitdepth and High Bitrate coding proposed at V meeting of JVET.

The software basis for this CE is VTM-13.0. For the test sequences, configurations and test conditions, the High Bit-depth CTC described in JVET-U2018 is used, unless otherwise specified in the CE description.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | M. G. Sarwer | Alibaba | m.sarwer@alibaba-inc.com |
| 2 | K. Naser | Interdigital | Karam.Naser@InterDigital.com |
| 3 | F. Wang | OPPO | wangfan6@oppo.com |

# Test conditions and evaluation criteria

The proposals will be tested under the High Bit-Depth CTC specified in [1] in both lossy and lossless configurations.

Planned tests in the CE shall be implemented on, and compared with, VTM-13.0 with the modification described in CTC [1] to enable high bit depth processing. For tests targeting high throughput profile, HM configured in HEVC high throughput profile as an additional reference point to be used.

In LowQP configuration, extended precision processing will be enabled, and all results will be compared with an anchor with the same setting and code base. For 12 bit testing in Normal QP configuration, extended precision will be disabled and the results will be compared with an anchor with extended precision disabled.

Proposals will be compared with respect to bit rate, objective quality and complexity. To provide an indication of complexity, comparative run-times for encoding and decoding will be used. In addition, throughput issues should be considered, in particular any change to the bin to bit ratio.

If a proposal changes coding for 8 or 10 bit sources, additional VVC 10 bit CTC results shall be provided.

# Proposals descriptions

The following contributions have been selected for CE study at the V meeting of JVET.

- JVET-V0121 AHG8: On coding of last significant coefficient position for high bit depth and high bit rate extensions [F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

The proposal of JVET-V0121 proposes a modification to the derivation method of last significant coefficient position in regular residual coding (RRC). In the current VVC specification, the coordinate of last significant coefficient (LastSignificantCoeffX, LastSignificantCoeffY) is measured in reference to the top-left corner of a transform block (TB) and coded with syntax elements last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix, and last_sig_coeff_y_suffix. It is observed that, when coding for high quality (or high bit-rate) with high bit-depth video, the number of significant coefficients become larger and the last significant coefficient often occur in the bottom-right portion of a TB. To accommodate this new distribution aspect, this contribution proposes to code the relative coordinates of last significant coefficient in reference to the bottom-right corner of a zero-out transform block. More specifically, the usage of the proposed alternative coordinates is signaled by a flag sh_reverse_last_sig_coeff_flag in the slice header. When sh_reverse_last_sig_coeff_flag is equal to 1, the last coefficient position is coded by using the proposed alternative coordinates for a whole slice. Otherwise, the current coordinates for the last coefficient position are used.

- JVET-V0133 AHG8: Content Adaptive Transform Precision for High Bit Depth Coding [K. Naser, F. Galpin, F. Le Léannec, P. De Lagrange (InterDigital)]

The intermediate transform coefficient precision is fixed to 16 bit signed representation in VVC. Therefore, a pre-computed down-shift is performed after inverse quantization and inverse transform in order to avoid exceeding the 16 bit buffer. It is argued that this down-shift is too conservative as the prediction mechanism results in low magnitude residuals. For High Bit-Depth coding, the down-shift is increased to ensure 16 bit representation, which further decreases the quality of the inverse transformed coefficients. This contribution proposes to adaptively assign the down-shift according the contents.

- JVET-V0178 AHG8: a combination of JVET-V0059 option 2 and JVET-V0122 for high bit depth and high bit rate extensions [F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)] [late]

- JVET-V0059 AHG8: CABAC-bypass alignment for high bit-depth coding [M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba)]

This contribution proposed to enable CABAC bypass alignment method in VVC High Throughput 4:4:4 16 Intra Profile. Same as JVET-U0069, this contribution proposes two alignment options. In the first option, the bypass alignment is applied only to coefficient coding within a coefficient group (CG), without affecting the coding of sb_coded_flag of a transform block (TB). In the second option, in addition to CABAC bypass alignment, it is also proposed to switch to bypass coding of sb_coded_flag after the limit of context coded bins has been reached for the current TB. It is asserted that with the second option, alignment is needed only once after the limit of the context coded bins has been reached for the TB.

- JVET-V0122, “AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions”, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

This contribution proposes a full-bypass mode in residual coding for VVC version 2. It is proposed to add this new mode in both regular residual coding (RRC) and transform skip residual coding (TSRC). When enabled, this new mode will code coefficient or residual levels only through the bypass mode of the arithmetic coding engine without using any context-coded bin. This mode is primarily designed to improve the throughput of arithmetic coding for high quality applications with high bit-depth video signals, such as content capture, creation, and editing. It is observed that the simulation results showed over 15% reduction in average decoding time, even with a slight coding efficiency gain, from the 16-bit SVT sequences. The tests were conducted by using the CE anchor software and running the latest CTC in JVET-U2018.

JVET-V0178 proposes a combination of JVET-V0059 option 2 and JVET-V0122, in which all the syntax elements in residual coding, except for last significant coefficient position in RRC, are coded through bypass mode and alignment is needed only once after the last significant coefficient position in RRC and at the very beginning of a TB in TSRC.

An SPS level syntax element sps_high_throughput_flag is used to enable/disable the proposed method. If sps_high_throughput_flag is equal to 0, the current RRC and TSRC in VVC remains unchanged. If sps_high_throughput_flag is equal to 1, the following aspects are changed:

- The initial value of remBinsPass1 in RRC and RemCcbs in TSRC are set to 0, i.e. the maximum number of context coded bins is set to 0. As a result the coefficient or residual levels at all positions are coded only in the bypass mode.

- The alignment happens before going into sub-block, i.e. after last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix and last_sig_coeff_y_suffix if any of residual_coding in RRC, and at the very beginning of residual_ts_coding in TSRC.

- All the syntax elements of sb_coded_flag are always coded in bypass mode.

# Planned tests

## Coding of the last significant coefficient position

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE1.1 | OPPO |  |
| CE1.2 |  |  |

## CE1.1: Method of JVET-V0121

In this test, the improvements of the proposed method will be tested. The encoding setting described in the JVET-V0121 will be used to determine the value of sh_reverse_last_sig_coeff_flag for each slice.

## Adaptive Transform Precision

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE2.1 | Interdigital |  |
| CE2.2 |  |  |
| CE2.3 |  |  |

Additional testing on extended precision flag enabled/disabled can be conducted.

## CE2.1: Method of enabling content adaptive transform precision of JVET-V0133

Description

## Bypass alignment and bypass coding

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE3.1 | Alibaba |  |
| CE3.2 | OPPO, Alibaba |  |

Additional testing on components of combined method or harmonization with existing tools can be tested as separate tests.

## Test 3.1: Option 2 described in JVET-V0059/JVET-V0178

In this test option 2 presented in JVET-V0059/JVET-V0178 will be tested. In order to guarantee that there is no context coded bins after reaching to the maximum limits, the sb_coded_flag (of both RRC and TSRC) be bypass coded after reaching to the number of context coded bins to the maximum limit. In order to reduce the number of bypass alignments, in option 2, the alignment method (described in option 1) is combined with sb_coded_flag coding method when CABAC bypass alignment is enabled.

## CE3.2: Method of JVET-V0178

In this test, the throughput benefits of the proposed method will be tested. When sps_cabac_bypass_alignment_enabled_flag is equal to 1:

- the corresponding remBinsPass1 and RemCcbs are set to be 0;

- the coding of last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix and last_sig_coeff_y_suffix remain the same (context coded);

- bypass alignment happens before the first bypass coded bin in both RRC and TSRC;

- all coefficient or residual levels are bypass coded with corresponding syntax elements for both RRC and TSRC;

- all sb_coded_flag are bypass coded as well.

In this test, the VTM throughput will be studied and tested in reference to HEVC. The measurements of bitrate, binrate, bin2bit ratio (weighted, unweighted, peak, average) and other relevant test results will be reported by using the templates in JVET-N0049 and JVET-V0150.

# Time-line and Responsibilities

T1: 2021-May-14:	   			   Version of CE description with final descriptions of tests is uploaded. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to marked as "withdrawn".

T2: VTM 13.0 + 2 week (not later than 2021-June-04):							Integration of initial CE test SW into a separate CE branch of the VTM is completed and initial study by cross-checkers begins.

T3: T2 + 2 (not later than 2021-June-18):

Final version of CE tests software and full results are provided, final cross-check begins.

T4: 2021-June-25: 			   		CE contribution documents including specification text and complete test results are uploaded to the JVET document repository. Work on CE report is starting.

# References

[1] A. Browne, T. Ikai, D.  Rusanovskyy, M.Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, JVET, 21th Meeting: by teleconference, 20-28 April. 2021

[2] AHG8: On coding of last significant coefficient position for high bit depth and high bit rate extensions, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO) JVET-V0121.

[3] AHG8: Content Adaptive Transform Precision for High Bit Depth Coding, K. Naser, F. Galpin, F. Le Léannec, P. De Lagrange (InterDigital), JVET-V0133.

[4] AHG8: CABAC-bypass alignment for high bit-depth coding, M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba), JVET-V0059.

[5] AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-V0122.

[6] AHG8: a combination of JVET-V0059 option 2 and JVET-V0122 for high bit depth and high bit rate extensions, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), JVET-V0178