| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0024-v1 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Semih Esenlik Fabrice Le Leannec Ling Li Jacob Strom Martin Winken Xiaoyu Xiu Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com semih.esenlik@huawei.com fabrice.leleannec@interdigital.com aurali@tencent.com jacob.strom@ericsson.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as partitioning, intra prediction, inter prediction, transform, in-loop filtering, and combination tests.

The software basis for this EE is ECM-1.0, released at https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tags/ECM-1.0

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-v-ee2/VVCSoftware_VTM/-/branches

ECM-1.0 is used as an anchor in the (tool-off) tests, and VTM-11.0 with the updated MCTF from JVET-V0056 is used as an anchor in the tool-on tests.

# List of tools

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |
| 1.1 | ABT | InterDigital JVET-W0084 |  |
| 1.2 | ABT with non-normative restrictions on partition splits | InterDigital JVET-W0084 |  |
| 1.3.a | UQT, high efficiency configuration | Bytedance JVET-W0086 | InterDigital JVET-W0137 |
| 1.3.b | UQT, low complexity configuration | Bytedance JVET-W0086 | InterDigital JVET-W0137 |
| 1.4.a | UQT+UBT, high efficiency configuration | Bytedance JVET-W0086 | InterDigital JVET-W0137 |
| 1.4.b | UQT+UBT, low complexity configuration | Bytedance JVET-W0086 | InterDigital JVET-W0137 |
| 1.5a | UQT+ABT (based on test 1.4) | Bytedance, Interdigital JVET-W0087 | Qualcomm |
| 1.5b | ABT (based on test 1.4) | Bytedance, Interdigital JVET-W0087 | Qualcomm |
| 2 Intra prediction |  |  |  |
| 2.1 | TIMD | Bytedance JVET-W0053 | Qualcomm JVET-W0157 |
| 2.2 | Additional blending DIMD modes | LGE JVET-W0054 |  |
| 3 Inter prediction |  |  |  |
| 3.1 | ARMC with template matching | Bytedance JVET-W0090 | Alibaba JVET-W0144 |
| 3.2 | ARMC with bilateral matching instead of TM | Bytedance JVET-W0090 | Alibaba JVET-W0144 |
| 3.3 | GPM with MMVD | Bytedance, Kwai JVET-W0088 | Qualcomm JVET-W0153 |
| 3.4 | GPM with template matching | Alibaba JVET-W0065 | Kwai |
| 3.5 | GPM with template matching and CU-level signaling | Qualcomm JVET-W0065 | Kwai |
| 3.7 | GPM with MMVD 3.3 and MV list construction from 3.5 | Bytedance, Kwai, Qualcomm JVET-W0089 | Alibaba JVET-W0143 |
| 3.6 | Extension of template matching to CIIP, GPM, Affine merge modes, and boundary sub-blocks | Qualcomm | withdrawn |
| 3.8 | Combination of 3.4 and 3.5 (GPM TM) | Alibaba, Qualcomm JVET-W0065 | Kwai |
| 3.9 | Combination of 3.3 and 3.8 (GPM MMVD and GPM TM), encoder option #1 | Kwai, Alibaba, Bytedance, Qualcomm JVET-W0097 | InterDigital |
| 3.10 | Combination of 3.3 and 3.8 (GPM MMVD and GPM TM), encoder option #2 | Kwai, Alibaba, Bytedance, Qualcomm JVET-W0097 | InterDigital |
| 4 Transform |  |  |  |
| 4.1 | Extended intra MTS | Qualcomm JVET-W0103 | LGE |
| 4.2 | Extended LFNST | Qualcomm JVET-W0103 | LGE |
| 4.3 | Extended LFNST - method 2 | Qualcomm | withdrawn |
| 4.4 | Extended intra MTS + extended LFNST | Qualcomm JVET-W0103 | LGE |
| 4.5 | Extended LFNST with large kernel – method A | LGE JVET-W0119 | Qualcomm JVET-W0120 |
| 4.6 | Extended LFNST with large kernel – method B | LGE JVET-W0119 | Qualcomm JVET-W0120 |
| 4.7 | Replace the existed LFNST with the proposed method A | LGE JVET-W0119 | Qualcomm JVET-W0120 |
| 4.8 | Replace the existed LFNST with the proposed method B | LGE JVET-W0119 | Qualcomm JVET-W0120 |
| 4.9 | Combination of 4.7 and 4.1 | LGE JVET-W0119 |  |
| 4.10 | Combination of 4.8 and 4.1 | LGE JVET-W0119 |  |
| 5 In-loop filtering |  |  |  |
| 5.1a | Cross-component SAO with separate clipping | Kwai JVET-W0066 | Qualcomm JVET-W0146 |
| 5.1b | Cross-component SAO with joint clipping | Kwai JVET-W0066 | Qualcomm JVET-W0146 |

# Description of tests

## Partitioning

### Tests 1.1 – 1.2: Asymmetric binary tree (ABT)

Four new asymmetric binary tree splitting modes are added to the multi-type tree structure of VVC shown on Figure 1.

Figure 1. Split modes supported in the proposed QTBT+TT+ABT framework

CU can be divided into 2 sub-CUs with 1/4 and 3/4 sizes. 6-point, 12-point, 24-point, and 48-point transforms/inverse-transforms are introduced

Test 1.1 Redundant partitions are normatively disallowed.

Test 1.2 Redundant partitions are disallowed at encoder side instead of by the syntax normatively

Implementation of these tests were provided on top of ECM with ECM tools disabled, the proponents suggest continuing EE on this topic.

### Tests 1.3 – 1.5: Unsymmetric quad (UQT) and binary (UBT) partitioning

Four types of unsymmetric quad tree partitions with 1/8, 1/2, 1/4, and 1/8 sizes as shown on Figure 2 are tested.

Figure 2. UQT partitions

Four types of unsymmetric binary tree partitions with 1/4 and 3/4 sizes as shown on Figure 3 are tested. 6-point, 12-point, 24-point, and 48-point forward and inverse transforms are introduced.

Figure 3. UBT partitions

Some normative constrains are imposed on partitioning to reduce redundancy and complexity.

Test 1.3a: UQT in high efficiency configuration

Test 1.3b: UQT in low complexity configuration

Test 1.4a: UQT+UBT in high efficiency configuration

Test 1.4b: UQT+UBT in low complexity configuration

Test 1.5a: UQT+ABT (Test 1.4b with different normative constraints from 1.2 and encoder optimization)

Test 1.5b: ABT (Test 1.4b with different normative constraints from 1.2 and encoder optimization)

## Intra prediction

### Test 2.1: Template-based intra mode derivation (TIMD) using MPMs

For each intra prediction mode a cost is calculated as the sum of absolute transformed differences (SATD) between the prediction and the reconstruction samples of the left and above templates shown on Figure 4.

Figure 4. Template and its reference samples used in TIMD

Primary and secondary MPMs are included into the TIMD cost estimation. DC, horizontal and vertical mode are also included into TIMD cost estimation, if not included in the primary or secondary MPM lists. Those modes are converted to twice dense angles from 67 to 131 intra mode range. Additionally, the adjacent MPM intra modes, i.e. MPM modes +-1 are included into the estimation. The intra mode with the smallest TIMD cost is selected for the intra prediction in the extended 131 intra mode range.

TIMD mode is not applied with DIMD or MIP, it is used with MRL and ISP, 4-tap intra interpolation filter is utilized to derive TIMD cost.

Intra mode propagation is applied for inter blocks.

### Test 2.2: Additional blending DIMD modes

In this test, two additional DIMD blending modes corresponding to the DIMD indices 1 and 2 are added as shown in Table 1. Planar weight is changed from 1/3 to 5/9 for the existed in ECM DIMD blending.

| DIMD Index | DIMD blend mode |
| --- | --- |
| 0 | No DIMD |
| 1 | Blend predictions of Mode1 and Mode2 |
| 2 | Blend predictions of Mode2 and Planar (wP = 1/3) |
| 3 | Blend predictions of Mode1, Mode2 and Planar (wP = 5/9) |

Table 1. DIMD blending modes

DIMD index is signalled instead of DIMD flag.

## Inter prediction

### Tests 3.1 – 3.2: Adaptive reordering of merge candidates (ARMC)

The reordering method is applied to regular merge mode, template matching (TM) merge mode, and affine merge mode (excluding the SbTMVP candidate). For the TM merge mode, merge candidates are reordered before the refinement process.

After a merge candidate list is constructed, merge candidates are divided into several subgroups. The subgroup size is set to 5 for regular merge mode and TM merge mode. The subgroup size is set to 3 for affine merge mode. Merge candidates in each subgroup are reordered in ascending order according to the cost values based on template matching or bilateral matching. For simplification, merge candidates in the last but not the first subgroup are not reordered.

Test 3.1: ARMC with template matching

Test 3.2: ARMC with bilateral matching

### Test 3.3, 3.7: GPM with MMVD

MV differences (MVDs) are applied to existing GPM merge candidates, MVDs are signaled in the same manner as in MMVD.

Two different MMVD sets are introduced, the set selection is done based on the resolution and is signalled in picture header:

0: 8 distances {1/4-pel, 1/2-pel, 1-pel, 2-pel, 4-pel, 8-pel, 16-pel, 32-pel} and 4 horizontal/vertical directions

1: 9 distances {1/4-pel, 1/2-pel, 1-pel, 2-pel, 3-pel, 4-pel, 6-pel, 8-pel, 16-pel} and 8 directions (4 horizontal/vertical directions plus 4 diagonal directions)

Two flags are signaled to indicate whether MVD is applied to each GPM partition. If MVD is applied, MMVD index is further signaled.

Additionally, an MV pruning procedure is introduced to construct the GPM merge candidate list when GPM with MMVD is applied.

The pruning method of 3.5 is tested in combination with MMVD, where MV pruning threshold is adapted based on the current CU size and zero MVs are added to the GPM candidate list if the list is incomplete.

Test 3.3: GPM with MMVD

Test 3.7: GPM with MMVD and MV list construction from 3.5

### Tests 3.4, 3.5, 3.8: GPM with template matching

Template matching is extended to GPM mode, where GPM MVs are further refined.

In test 3.4, a template is constructed using left, above or left (L) and above (A) neighboring samples according to partition angle as shown on Table 2 and each GPM partition can indicate whether template matching is applied. VVC GPM candidate list derivation is used.

| Partition angle | 0 | 2 | 3 | 4 | 5 | 8 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |
| Partition angle | 16 | 18 | 19 | 20 | 21 | 24 | 27 | 28 | 29 | 30 |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |

Table 2. GPM templates

In test 3.5, GPM template matching is enabled for a CU, i.e. for both partitions together. The existed left and above templates in ECM are reused, and the same template matching is used to refine MVs in both partitions.

A GPM candidate list is constructed as follows:

- Interleaved List-0 MV candidates and List-1 MV candidates are derived directly from the regular merge candidate list, where List-0 MV candidates have a higher priority than List-1 MV candidates.  A pruning method with an adaptive threshold based on the current CU size provided in Table 3 is applied to remove redundant MV candidates.

- Interleaved List-1 MV candidates and List-0 MV candidates are further derived directly from the regular merge candidate list, where List-1 MV candidates have a higher priority than List-0 MV candidates. The same pruning method with the adaptive threshold is also applied to remove redundant MV candidates.

- Zero MV candidates are padded until the GPM candidate list is full.

| CU area (w*h) | MV pruning threshold |
| --- | --- |
| area < 64 | 1/4-pel |
| 64 <= area < 256 | 1/2-pel |
| area >= 256 | 1-pel |

Table 3. MV pruning threshold

In a combination test 3.4 and 3.5, when a CU is coded in GPM, a CU-level flag is signaled to indicate whether TM is applied to both partitions. If applied, two uni-prediction motions are selected from the GPM candidate list which is constructed using the method as described in test 3.5. Then the two uni-prediction motions are refined using different templates according to the partition angle as described in test 3.4.

Test 3.4: GPM with template matching

Test 3.5: GPM with template matching and CU-level signaling

Test 3.8: Combination of 3.4 and 3.5

### Test 3.9 – 3.10: Combination of GPM with MMVD and template matching

This test is a combination of tests 3.3 and 3.8, the combination done as follows:

- The GPM-MMVD and GPM-TM are exclusively enabled to one GPM CU. This is done by firstly signaling the GPM-MMVD syntax. When both two GPM-MMVD control flags are equal to false (i.e., the GPM-MMVD are disabled for two GPM partitions), the GPM-TM flag is signaled to indicate whether the template matching is applied to the two GPM partitions. Otherwise (at least one GPM-MMVD flag is equal to true), the value of the GPM-TM flag is inferred to be false.

- The GPM merge candidate list generation methods in EE2-3.3 and EE2-3.8 are directly combined in a manner that the MV pruning scheme in EE2-3.8 (where the MV pruning threshold is adapted based on the current CU size) is applied to replace the default MV pruning scheme applied in EE2-3.3; additionally, as in EE2-3.8, multiple zero MVs are added until the GPM candidate list is fully filled.

Two encoder options are provided to trade-off the coding performance and encoding complexity.

Test 3.9: Combination of 3.3 and 3.8 (GPM MMVD and GPM TM), encoder option #1

Test 3.10: Combination of 3.3 and 3.8 (GPM MMVD and GPM TM), encoder option #2

## Transform

### Test 4.1: Extended intra MTS

Additional primary transforms including DCT5, DST4, DST1, and identity transform (IDT) are tested. MTS kernels are intra mode and TU size dependent.

### Test 4.2: Extended LFNST

In this test, the existed LFNST design in ECM-1.0 is extended by introducing the second set of LFNST8 kernels to be used for top left 8x8 regions of 16x16 and large blocks. Zero-out scheme is introduced for 8x8 blocks which results in 32 output coefficients, there is no zero-out in other cases.

The set of kernels can be represented as

(LFNST4, LFNST8, LFNST8*) = (16x16, 64x64, 64x64)

### Test 4.4: Extended intra MTS and extended LFNST

This is a combination of tests 4.1 and 4.2.

### Tests 4.5 – 4.8: Extended LFNST with large kernel

Three kernel sets LFNST4, LFNST8, and LFNST16 are tested which are applied to 4xN/Nx4 (N≥4), 8xN/Nx8 (N≥8), and MxN (M, N≥16), respectively.

LFNST16 is newly introduced kernel set to EMC-1.0 shown Figure 5. There are two methods with different LFNST8 kernel sizes shown on Figure 6:

A:  (LFNST4, LFNST8, LFNST16*) = (16x16, 16x48, 32x96)

B:  (LFSNT4, LFNST8*, LFNST16*) = (16x16, 32x64, 32x96)

Figure 5. The ROI for LFNST16

Figure 6.  The ROI for LFNST8 in the case of (a) Test 4.5 and 4.7 and (b) Test 4.6 and 4.8

LFNST kernels are intra mode dependent.

Test 4.5 and 4.6 reuse the existed in EMC-1.0 LFNST signaling and 35 intra modes intra mapping. Tests 4.7 and 4.8 change LFNST signaling, introduce 36th intra mode (WAIP) mapping, and have encoder modification.

Test 4.5: Extended LFNST with large kernel – method A

Test 4.6: Extended LFNST with large kernel – method B

Test 4.7: Replace the existed LFNST with the proposed method A

Test 4.8: Replace the existed LFNST with the proposed method B

Test 4.7: Combination of 4.7 and 4.1

Test 4.8: Combination of 4.8 and 4.1

### Test 5.1: Cross-component SAO

CCSAO utilizes all three components as shown on Figure 7 to classify the reconstructed samples into different bands.

Figure 7. CCSAO flowchart

The values of the selected Ycol, Ucol, and Vcol samples are classified into three different bands {bandY, bandU, bandV}, and a joint index i is used to indicate the category of the given sample. One offset is signaled and added to the reconstructed samples that fall into that category, class index and sample modification are derived as follows.

bandY=Ycol∙NY≫BDbandU=Ucol∙NU≫BDbandV=Vcol∙NV≫BDi=bandY∙NU∙NV+bandU∙NV+bandVC'rec=Clip1Crec+σCCSAOi                       (1)

The parameters for each classifier (i.e., the position of Ycol, NY, NU, NV, and offsets) are signaled at picture level, and which classifier to be used is explicitly signaled and switched at CTB level. For each classifier, the maximum of {NY, NU, NV} is set to {16, 4, 4}, and offsets are constrained to be within the range [-15, 15]. The maximum classifiers per frame is constrained to be 4.

Two places for CCSAO clipping is tested as shown on Figure 8.

Figure 8. CCSAO clipping separate (left) and joint (right)

Test 5.1: Cross-component SAO with separate clipping

Test 5.2: Cross-component SAO with joint clipping

# Test results

See the accompanied Excel table.

# EE related documents

JVET-W0067: EE2-related: Implicit derivation of DIMD blend modes

JVET-W0068: EE2-related: A combination of CIIP and DIMD/TIMD

JVET-W0069: [AHG12] On Intra TMP Boundary Conditions

JVET-W0079: AHG12: CTB level filter shape selection of CCALF

JVET-W0097: EE2-related: Combination of EE2-3.3, EE2-3.4 and EE2-3.5

JVET-W0098: Non-EE2: Bilateral Inloop Filter on Chroma

JVET-W0106: EE2-related: Bilateral matching AMVP-merge mode

JVET-W0107: EE2-related: Adaptive decoder side motion vector refinement

JVET-W0108: EE2-related: Low complexity sign prediction

JVET-W0110: AHG12: GPM with inter and intra prediction

JVET-W0112: AHG12: Diagonal MMVD with ARMC

JVET-W0122: EE2-related: On spatial MV propagation and neighboring template block access for template matching and multi-pass DMVR

JVET-W0123: EE2-related: Fusion for template-based intra mode derivation

JVET-W0124: EE2-related: Template based intra most probable modes sorting

JVET-W0126: EE2 Related – DIMD with implicitly derived multiple blending modes

JVET-W0128: AHG12: Alternative classifiers for ALF