| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0044-v3 |
| --- | --- |

| Title: | CE3.1: CABAC-bypass alignment for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Jie Chen Yan Ye Ru-Ling Liao 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution reports the results of CE3.1. In CE3.1, in order to improve the throughput of the CABAC, a bypass alignment method is tested. In addition to bypass alignment, coding of sb_coded_flag is switched from the context coding to bypass coding after the limit of context coded bins has been reached for the current TB. Similar to HEVC, it is proposed to enable the proposed method only in High Throughput 4:4:4 16 Intra Profile.

Following results are reported. Note that only SVT 16-bit sequences will be affected by the proposed changes as all other sequences are 12-bit sequences.

As compared to VTM-13.0:

- SVT-16-bit sequences (low QP):

  - All Intra: 0.28%(G), 0.33% (B), 0.33 % (R), 101% (EncT), 93%(DecT)

  - LDB: 0.46%(G), 0.47 % (B), 0.47 % (R), 101% (EncT), 96%(DecT)

  - Random access: 0.46%(G), 0.59 % (B), 0.49 % (R), 101% (EncT), 96%(DecT)

- SVT-16-bit sequences (lossless):

  - All Intra: 0.20%, 101% (EncT), 88%(DecT)

  - LDB: 0.26%, 102% (EncT), 88%(DecT)

  - Random access: 0.27%, 103% (EncT), 88%(DecT)

v3 of this document include estimated throughput performance.

# Problem Statement

The CABAC process in VVC is a sequential process where the evaluation of each iteration depends on the outcome of the previous iteration. At the higher bit depth and higher bit rate operating ranges (especially in 16-bit input), the serial nature of CABAC decoding process may impact the throughput of the codec. Since there is an upper-limit of the maximum number of context coded bins, at high bit-rate the bypass coding bins are the dominant portion in the bitstream, compared to the context coded bins.

The decoding process of a bypass coded bin is not so straight forward. In VVC, the status of the arithmetic decoding engine is represented by the variables ivlCurrRange and ivlOffset. In order to decode a bypass coded bin the parameter ivlCurrRange is updated first. Then, the updated ivlCurrRange is compared with the ivlOffset. If the ivlOffset is greater than or equal to ivlCurrRange, the bin value is equal to 1 and ivlOffset is updated; otherwise, the bin value is set to 0. This serial decoding of bypass coding may impact the throughput of the codec especially at high bit-rate.

In order to improve the throughput of the CABAC engine, HEVC range extension (the High Throughput 4:4:4 16 Intra Profile) supports aligning the CABAC process prior to coding of bypass data, which allows easy, simultaneous decoding of multiple bypass bins [1]. In the last JVET meeting, JVET-V0059 was proposed to add the same CABAC bypass alignment concept as that in HEVC range extension into VVC [2].

# Proposed method

The CABAC bypass alignment method of CE3.1 is same as the method proposed in JVET-V0059. Same as JVET-V0059, after reaching to the maximum limits of context coded bins, sb_coded_flag of a coefficient group (CG) is bypass coded. Following is the pseudo-code of CE3.1. The sps_cabac_bypass_alignment_enabled_flag is set to 1 to enable the proposed method. If the value of sps_cabac_bypass_alignment_enabled_flag is equal to 0, the proposed method is disabled and the behavior is same as current VVC. The variables remBinsPass1 and RemCcbs represent the remaining number of context coded bins available in the TB in regular and transform-skip residual coding, respectively. The red italic portions are the additional process as compared to the current VVC.

In case of regular residual coding:

- remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (remBinsPass1  >=  4 || sps_cabac_bypass_alignment_enabled_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (remBinsPass1  >=  4)

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

    - If (CABACBypasIntervalAlign == 0 && sps_cabac_bypass_alignment_enabled_flag)

      - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 2: abs_remainder, dec_abs_level

    - Pass 3: coeff_sign_flag

In case of transform-skip residual coding:

- RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (RemCcbs >= 4 || sps_cabac_bypass_alignment_enabled_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (RemCcbs >= 4)

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4)

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - If (CABACBypasIntervalAlign == 0 && sps_cabac_bypass_alignment_enabled_flag)

      - CABAC-bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 3: abs_remainder, coeff_sign_flag

Figure 1 shows an example of the bit-stream of a transform block of CE3.1. It shows that after the number of context coded bins has reach the limit for the current TB, only bypass coding bins are processed and the bypass alignment is necessary at most once per TB.

Figure 1: an example of bit-stream of CE3.1

The proposed CABAC bypass alignment (i.e. set the value of ivlCurrRange to 256) has benefits of simplifying high-throughput design of the entropy decoder, however, it may not be suitable for all application scenarios. It is proposed to enable this method in High Throughput 4:4:4 16 Intra Profile only.

# Simulation results

The CE3.1 is implemented on top of VTM-13.0. The performance is evaluated based on the test conditions suggested in JVET-V2022 [3]. Since the proposed CE test only impact the performance of the 16-bit sequences, the results of 16-bit sequences are presented in this section (the results of other sequences are included in the Appendix)

Table 1 and Table 2 shows the result of CE3.1 over VTM-13.0 under low QP and lossless test conditions, respectively. Table 3 shows the results of CE3.1 over HEVC High Throughput 4:4:4 16 Intra Profile.

Table 1: results of CE3.1 (low QP)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.28% | 0.33% | 0.33% | 101% | 93% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.46% | 0.47% | 0.47% | 101% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.46% | 0.49% | 0.49% | 101% | 96% |

Table 2: results of CE3.1 (lossless)

| SVT | All Intra | Low delay B | Random Access |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| SVT16 | 0.20% | 0.26% | 0.27% |
| Enc Time[%] | 101% | 101% | 102% |
| Dec Time[%] | 85% | 85% | 85% |

Table 3: results of CE3.1 (low QP, anchor: HM-16.23 high-throughput-RExt, Test: CE3.1)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 high-throughput-RExt |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.40% | -1.53% | -1.55% | 5841% | 210% |

Table 4 shows the supplemental result showing comparison between HEVC RExt and HEVC RExt high throughput profile.

Table 4: HEVC RExt high throughput profile vs RExt (Anchor: HM-16.23 RExt; Test: HM-16.23 high-throughput-RExt)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 RExt |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.31% | 0.29% | 0.29% | 100% | 75% |

## Estimated clock cycle

In order to measure the throughput performance of the proposed method, the clock cycle is estimated for both VTM-13.0 and the proposed method. At first the statistics of the number of context coded bins and number of bypass coded bins are collected from the decoder side for both anchor and test. The clock cycle is estimated as follows:

Clk Cycle in MHz = (((totalCtxBins + totalbypassBin/bypassWeight)* numCodedFrame)/framerate)/1000000

Where,

- totalCtxBins = Total number of context coded bins in the sequence

- totalbypassBin  = Total number of bypass coded bins

- bypassWeight is the number of bypass coded bins can be parsed per cycle. For anchor, bypassWeight is equal to 4. For proposed method, the bypassWeight is equal to 12.

- numCodedFrame is the total number of encoded frames

As it is mentioned earlier, the proposed method can process multiple bypass coded bins in parallel. For clock cycle estimation, it is assumed that the proposed method can process 12 bypass coded bin in one cycle. As it is shown in Table 5, the proposed method can save clockcycle of SVT-16 sequences, significantly.

Table 5: Estimated saving of clock cycle as compared to VTM-13.0

|  | HDR PQ | HDR HLG | SVT12 RGB | SVT16 RGB |
| --- | --- | --- | --- | --- |
| AI | 100% | 85% | 75% | 66% |

# Conclusions

In order to improve the throughput of the CABAC, CE3.1 proposes a bypass alignment method where the bypass bins are directly visible in ivlOffset. In the proposed method, N bypass coded bins can be decoded just by looking at the N most-significant-but-one bits of ivlOffset concatenated with the bit-stream which ultimately allows decoding of multiple bypass coded bins in parallel. It is proposed to enable the proposed method only in High Throughput 4:4:4 16 Intra Profile.

# References

K. Sharman, N. Saunders, J. Gamei, “AHG 5 and 18: Entropy Coding Throughput for High Bit Depths”, JCTVC-N0190, July 2013.

M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao, “AHG8: CABAC-bypass alignment for high bit-depth coding”, JVET-V0059, April 2021

K. Naser, D. Rusanovskyy, M.Sarwer, F. Wang, “Core Experiment on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-V2022, April- 2021.

# Appendix

- The simulation results of CE3.1 with both 12-bit and 16-bit sequences are presented in this section.

Table 6: Results of CE3.1 (low QP, PQ sequences)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.17% | 1.24% | 1.20% | 1.16% | 1.22% | 1.19% | 100% | 98% |
| PQ422 | 1.08% | 1.12% | 1.09% | 1.07% | 1.11% | 1.08% | 101% | 99% |
| Overall | 1.13% | 1.18% | 1.14% | 1.12% | 1.17% | 1.14% | 100% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.54% | 1.67% | 1.64% | 1.51% | 1.63% | 1.61% | 100% | 98% |
| PQ422 | 1.44% | 1.52% | 1.61% | 1.40% | 1.49% | 1.57% | 100% | 99% |
| Overall | 1.49% | 1.60% | 1.62% | 1.46% | 1.56% | 1.59% | 100% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.55% | 1.63% | 1.61% | 1.51% | 1.60% | 1.59% | 100% | 98% |
| PQ422 | 1.44% | 1.50% | 1.55% | 1.41% | 1.47% | 1.54% | 100% | 99% |
| Overall | 1.49% | 1.57% | 1.58% | 1.46% | 1.54% | 1.57% | 100% | 98% |
| Overall PQ | 1.37% | 1.45% | 1.45% | 1.34% | 1.42% | 1.43% | 100% | 98% |

Table 7: Results of CE3.1 (low QP, HLG sequences)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.90% | 0.97% | 0.98% | 101% | 99% |
| HLG422 | 0.83% | 0.89% | 0.91% | 100% | 99% |
| Overall | 0.87% | 0.93% | 0.94% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.47% | 1.46% | 1.50% | 99% | 98% |
| HLG422 | 1.39% | 1.47% | 1.51% | 99% | 99% |
| Overall | 1.43% | 1.47% | 1.51% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.47% | 1.46% | 1.52% | 99% | 98% |
| HLG422 | 1.39% | 1.48% | 1.49% | 99% | 99% |
| Overall | 1.43% | 1.47% | 1.51% | 99% | 99% |
| Overall HLG | 1.24% | 1.29% | 1.32% | 100% | 99% |

Table 8: Results of CE3.1 (low QP, SVT sequences)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.28% | 0.33% | 0.33% | 101% | 93% |
| SVT12 | 0.48% | 0.59% | 0.60% | 101% | 99% |
| Overall | 0.38% | 0.46% | 0.46% | 101% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.46% | 0.47% | 0.47% | 101% | 96% |
| SVT12 | 0.72% | 0.76% | 0.76% | 102% | 101% |
| Overall | 0.59% | 0.62% | 0.62% | 102% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.46% | 0.49% | 0.49% | 101% | 96% |
| SVT12 | 0.74% | 0.80% | 0.80% | 102% | 101% |
| Overall | 0.60% | 0.64% | 0.64% | 102% | 99% |
| Overall RGB | 0.52% | 0.57% | 0.57% | 101% | 98% |

Table 9: Results of CE3.1 (standard QP)

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| PQ-420 | 2.68% | 1.52% | 1.48% | 2.64% | 2.96% | 1.50% | 2.73% | 3.04% | 99% | 99% |
| HLG-420 |  |  |  |  |  | 2.02% | 2.50% | 2.40% | 99% | 99% |
| Overall | 2.68% | 1.52% | 1.48% | 2.64% | 2.96% | 1.76% | 2.62% | 2.72% | 99% | 99% |
|  | All intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| PQ-420 | 3.06% | 1.97% | 1.91% | 3.26% | 3.29% | 1.90% | 3.31% | 3.42% | 100% | 99% |
| HLG-420 |  |  |  |  |  | 2.15% | 2.56% | 3.12% | 99% | 98% |
| Overall | 3.06% | 1.97% | 1.91% | 3.26% | 3.29% | 2.03% | 2.94% | 3.27% | 100% | 98% |

Table 10: Results of CE3.1 lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | CE-3.1 |  | VTM13.0 | CE-3.1 |  | VTM13.0 | CE-3.1 |  |
| PQ444 | 2.6 | 2.6 | 0.56% | 3.1 | 3.1 | 0.69% | 3.1 | 3.1 | 0.72% |
| PQ422 | 2.4 | 2.4 | 0.54% | 2.9 | 2.8 | 0.65% | 2.9 | 2.9 | 0.66% |
| Overall | 2.5 | 2.5 | 0.55% | 3.0 | 3.0 | 0.67% | 3.0 | 3.0 | 0.69% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | CE-3.1 |  | VTM13.0 | CE-3.1 |  | VTM13.0 | CE-3.1 |  |
| HLG444 | 1.8 | 1.8 | 0.37% | 2.0 | 1.9 | 0.50% | 2.0 | 1.9 | 0.51% |
| HLG422 | 1.7 | 1.7 | 0.38% | 1.9 | 1.9 | 0.45% | 1.9 | 1.9 | 0.46% |
| Overall | 1.7 | 1.7 | 0.38% | 1.9 | 1.9 | 0.47% | 1.9 | 1.9 | 0.48% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 102% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  | 100% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | CE-3.1 |  | VTM13.0 | CE-3.1 |  | VTM13.0 | CE-3.1 |  |
| SVT16 | 1.2 | 1.2 | 0.20% | 1.2 | 1.2 | 0.26% | 1.2 | 1.2 | 0.27% |
| SVT12 | 1.3 | 1.3 | 0.29% | 1.3 | 1.3 | 0.39% | 1.3 | 1.3 | 0.40% |
| Overall | 1.2 | 1.2 | 0.25% | 1.3 | 1.3 | 0.33% | 1.3 | 1.3 | 0.34% |
| Enc Time[%] | 101% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 88% |  |  | 88% |  |  | 88% |  |  |

Table 11: results of CE3.1 (low QP, anchor: HM-16.23 high-throughput-RExt, Test: CE3.1)

| HDR PQ |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 high-throughput-RExt |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -5.38% | -6.26% | -6.98% | 3210% | 199% |
| PQ422 | -8.68% | -12.48% | -13.14% | 2697% | 193% |
| Overall | -7.03% | -9.37% | -10.06% | 2954% | 196% |

Table 12: results of CE3.1 (low QP, anchor: HM-16.23 high-throughput-RExt, Test: CE3.1)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 Rext High throughput profile |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -4.23% | -5.78% | -6.10% | 3771% | 198% |
| HLG422 | -5.02% | -7.22% | -7.24% |  |  |
| Overall | -4.62% | -6.50% | -6.67% |  |  |

Table 13: results of CE3.1 (low QP, anchor: HM-16.23 high-throughput-RExt, Test: CE3.1)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 Rext High throughput profile |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT12 | -4.03% | -2.86% | -2.94% | 5604% | 188% |

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).