| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0045-v2 |
| --- | --- |

| Title: | CE-3.2: a high throughput mode for high bit depth and high bit rate extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Zhihuang Xie Yue Yu Haoping Yu Dong Wang (OPPO) Mohammed Golam Sarwer (Alibaba) Jie Chen Yan Ye Ru-Ling Liao (Alibaba) | Email: | wangfan6@oppo.com m.sarwer@alibaba-inc.com |
| Source: | OPPO, Alibaba |  |  |

_____________________________

- Abstract

This contribution reports the results of Core Experiment (CE)-3.2. It is asserted that with the proposed method, only bypass-coded bins are involved in the residual coding process, except for the coding of last significant coefficient position in RRC, and the bit-alignment process is performed only once for each TB, which happens after coding the last significant coefficient position in RRC and at the very beginning of TB coding in TSRC. It is suggested to apply the proposed method in 16-bit high throughput configurations.

The following results for SVT 16-bit sequences are reported. It is asserted that the proposed method in this contribution can truly enable decoding of all the coefficients in a TB simply by using a shift register and multiple coded bins can be decoded in parallel.

BD-bitrate compared to VTM13 (low QP):

- AI:  0.06 % (G), 0.09 % (B), 0.07 % (R),  85 % (EncT), 76 % (DecT)

- LDB: 0.32 % (G), 0.36 % (B), 0.35 % (R),  86 % (EncT), 82 % (DecT)

- RA:  0.35 % (G), 0.41 % (B), 0.41 % (R),  87 % (EncT), 86 % (DecT)

BD-binrate and Bin2Bit compared to VTM13 (low QP):

- AI:  -37.53 % (weighted average Delta Binrate), -10.57 % (peak unweighted Bin2Bit), -37.45 % (peak weighted Bin2Bit)

- LDB: -40.03 % (weighted average Delta Binrate), -10.56 % (peak unweighted Bin2Bit), -37.16 % (peak weighted Bin2Bit)

- RA:  -40.40 % (weighted average Delta Binrate), -10.34 % (peak unweighted Bin2Bit), -36.32 % (peak weighted Bin2Bit)

BD-bitrate compared to HM16.23 (low QP):

- AI:  -2.30 % (G), -1.48 % (B), -1.51 % (R),  5257 % (EncT), 113 % (DecT)

- LDB: -1.97 % (G), -0.45 % (B), -0.44 % (R),  644 % (EncT), 125 % (DecT)

- RA:  -2.06 % (G), -0.56 % (B), -0.56 % (R),  726 % (EncT), 101 % (DecT)

BD-binrate and Bin2Bit compared to HM16.23 (low QP):

- AI:  -36.17 % (weighted average Delta Binrate), -13.11 % (peak unweighted Bin2Bit), -35.91 % (peak weighted Bin2Bit)

- LDB: -39.24 % (weighted average Delta Binrate), -12.90 % (peak unweighted Bin2Bit), -35.37 % (peak weighted Bin2Bit)

- RA:  -39.43 % (weighted average Delta Binrate), -12.75 % (peak unweighted Bin2Bit), -34.83 % (peak weighted Bin2Bit)

The full simulation results are shown below in Section 3 of this document.

Proposed method

Full-bypass mode

A high throughput mode for high bit depth and high bit rate coding was proposed in JVET-V0178 [1] and it is the subject of this core experiment (CE-3.2). It is asserted that with the proposed method, only bypass-coded bins are involved in the residual coding process, except for the coding of last significant coefficient position in RRC, and the bit-alignment process is performed only once for each TB, which happens after coding the last significant coefficient position in RRC and at the very beginning of TB coding in TSRC.

An SPS level syntax element sps_high_throughput_flag is used to enable/disable the proposed method. If sps_high_throughput_flag is equal to 0, the current RRC and TSRC in VVC remains unchanged. If sps_high_throughput_flag is equal to 1, the following adjustments are proposed:

- The initial value of remBinsPass1 in RRC and the initial value of RemCcbs in TSRC are set to 0, i.e. the maximum number of context coded bins is set to 0 for both remBinsPass1 and RemCcbs. As a result, the coefficient or residual levels at all positions are coded only in the bypass mode.

- The bit-alignment proccess happens right before going into sub-blocks and coding the residual levels, i.e. after coding last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix and last_sig_coeff_y_suffix in RRC, and at the very beginning of residual_ts_coding in TSRC.

- All the syntax elements of sb_coded_flag are always coded as bypass bins.

The pseudo-code below illustrates the proposed changes to the residual coding process.

For regular residual coding:

- For last significant coefficient position // no change

  - context_coding of last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, bypass-coding of last_sig_coeff_x_suffix and last_sig_coeff_y_suffix

- For remBinsPass1 and alignment // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

  - else

    - remBinsPass1 = 0

    - CABAC bypass alignment (i.e., set ivlCurrRange to 256)

- For each coefficient group // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1) // no change

    - if (remBinsPass1  >=  4)  // context-coded bins

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

        - set firstPosMode1

    - Pass 2:  // bypass-coded bins

      - if (n > firstPosMode1 && abs_level_gtx_flag[ ][ 1 ] == 1) abs_remainder

      - if (n <= firstPosMode1) dec_abs_level

    - Pass 3: coeff_sign_flag  // bypass-coded bins

For transform-skip residual coding:

- For RemCcbs and alignment // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

  - else

    - RemCcbs = 0

    - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

- For each coefficient group // changes are marked in red

  - if (sps_high_throughput_flag == 0)

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1) // no change

    - if (RemCcbs >= 4) // context-coded bins

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4) //context-coded bins

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - Pass 3: abs_remainder, coeff_sign_flag // bypass-coded bins

Figure 1 below shows an example of the bit-stream format using the proposed method in RRC.

Figure 1: proposed method in RRC

Figure 2 below shows an example of the bit-stream format using the proposed method in TSRC.

Figure 2: proposed method in TSRC

Harmonization with history-based rice parameter derivation method

In the history-based rice parameter derivation method introduced by JVET-V0106 [2] method, a single counter per color component StatCoeff [3] is utilized to maintain a history of the rice parameter. This counter is being updated once per TU by using the methods described in the 2 branches below:

[branch 1] when first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(rem)) + 2 ) >> 1

, wherein rem is abs_remainder[]

[branch 2] when first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as dec_abs_level, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(rem)) ) >> 1

, wherein rem is dec_abs_level[ ]

Prior to TU decoding, this counter StatCoeff[cIdx] will initialize the variable histValue as:

histValue = 1 << StatCoeff[cIdx].

This method dispatches TUs into 2 branches. For the same value of rem, StatCoeff[cIdx] in branch 1 is given a bigger value by 1, compared with that in branch 2. This means branch 1 should work better for TUs with bigger coefficient values. As the proposed high throughput mode is designed to improve the throughput for very high bitrate coding that usually ends up with many big coefficients in each TU, branch 1 would be a better choice when harmonizing the proposed method into VTM 13. However, since there is no syntax element abs_remainder in the proposed high throughput mode and every level is bypass-coded by dec_abs_level, an adjustment marked in green below has been made to the condition of branch 1 so that it also includes the high throughput mode.

[branch 1] When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, or sps_high_throughput_flag is equal to 1, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(rem)) + 2 ) >> 1

, wherein rem is abs_remainder[] or dec_abs_level[], respectively

Simulation results

Comparison with VTM13

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [3]. The anchor is generated by VTM13, and the test is produced by the proposed method.

Low QP:

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.06% | 0.09% | 0.07% | 85% | 76% |
| SVT12 | 1.51% | 1.70% | 1.68% | 86% | 81% |
| Overall | 0.79% | 0.89% | 0.88% | 86% | 78% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.32% | 0.36% | 0.35% | 86% | 82% |
| SVT12 | 3.00% | 3.29% | 3.27% | 81% | 83% |
| Overall | 1.66% | 1.82% | 1.81% | 83% | 82% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.35% | 0.41% | 0.41% | 87% | 86% |
| SVT12 | 3.40% | 3.69% | 3.67% | 82% | 86% |
| Overall | 1.87% | 2.05% | 2.04% | 84% | 86% |
| Overall RGB | 1.44% | 1.59% | 1.58% | 84% | 82% |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 11.24% | 13.81% | 14.13% | 11.03% | 13.59% | 14.01% | 93% | 88% |
| PQ422 | 9.43% | 11.33% | 11.26% | 9.23% | 11.16% | 11.17% | 89% | 87% |
| Overall | 10.33% | 12.57% | 12.70% | 10.13% | 12.38% | 12.59% | 91% | 88% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 17.84% | 21.14% | 21.28% | 17.30% | 20.66% | 21.12% | 88% | 92% |
| PQ422 | 16.97% | 19.40% | 19.43% | 16.44% | 18.95% | 19.30% | 84% | 92% |
| Overall | 17.40% | 20.27% | 20.36% | 16.87% | 19.81% | 20.21% | 86% | 92% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 17.62% | 20.50% | 21.33% | 17.13% | 20.12% | 21.27% | 87% | 90% |
| PQ422 | 16.61% | 19.01% | 19.06% | 16.15% | 18.70% | 19.05% | 84% | 91% |
| Overall | 17.11% | 19.76% | 20.20% | 16.64% | 19.41% | 20.16% | 86% | 90% |
| Overall PQ | 14.95% | 17.53% | 17.75% | 14.55% | 17.20% | 17.65% | 88% | 90% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 6.62% | 8.55% | 9.13% | 84% | 81% |
| HLG422 | 6.05% | 7.64% | 8.06% | 84% | 85% |
| Overall | 6.34% | 8.09% | 8.59% | 84% | 83% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 16.36% | 17.71% | 18.63% | 74% | 81% |
| HLG422 | 17.43% | 18.83% | 19.01% | 72% | 81% |
| Overall | 16.89% | 18.27% | 18.82% | 73% | 81% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 16.71% | 17.77% | 18.53% | 74% | 84% |
| HLG422 | 17.67% | 18.54% | 18.70% | 71% | 84% |
| Overall | 17.19% | 18.16% | 18.61% | 72% | 84% |
| Overall HLG | 13.47% | 14.84% | 15.34% | 77% | 82% |

Lossless:

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | vtm13 | proposed |  | vtm13 | proposed |  | vtm13 | proposed |  |
| PQ444 | 2.6 | 2.5 | 3.16% | 3.1 | 2.9 | 6.38% | 3.1 | 2.9 | 6.38% |
| PQ422 | 2.4 | 2.3 | 2.67% | 2.9 | 2.7 | 5.83% | 2.9 | 2.7 | 5.81% |
| Overall | 2.5 | 2.4 | 2.91% | 3.0 | 2.8 | 6.11% | 3.0 | 2.8 | 6.10% |
| Enc Time[%] | 106% |  |  | 93% |  |  | 91% |  |  |
| Dec Time[%] | 73% |  |  | 79% |  |  | 77% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | vtm13 | proposed |  | vtm13 | proposed |  | vtm13 | proposed |  |
| HLG444 | 1.8 | 1.8 | 0.99% | 2.0 | 1.9 | 1.32% | 2.0 | 1.9 | 1.33% |
| HLG422 | 1.7 | 1.7 | 0.88% | 1.9 | 1.9 | 1.26% | 1.9 | 1.9 | 1.27% |
| Overall | 1.7 | 1.7 | 0.93% | 1.9 | 1.9 | 1.29% | 1.9 | 1.9 | 1.30% |
| Enc Time[%] | 103% |  |  | 93% |  |  | 93% |  |  |
| Dec Time[%] | 76% |  |  | 71% |  |  | 74% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | vtm13 | proposed |  | vtm13 | proposed |  | vtm13 | proposed |  |
| SVT16 | 1.2 | 1.2 | -0.29% | 1.2 | 1.2 | -0.28% | 1.2 | 1.2 | -0.25% |
| SVT12 | 1.3 | 1.3 | 0.42% | 1.3 | 1.3 | 0.39% | 1.3 | 1.3 | 0.41% |
| Overall | 1.2 | 1.2 | 0.07% | 1.3 | 1.3 | 0.06% | 1.3 | 1.3 | 0.08% |
| Enc Time[%] | 93% |  |  | 91% |  |  | 89% |  |  |
| Dec Time[%] | 68% |  |  | 70% |  |  | 71% |  |  |

Normal QP:

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 10.79% | 7.92% | 7.75% | 11.38% | 11.83% | 7.49% | 11.87% | 12.46% | 93% | 101% |
| Class H2 |  |  |  |  |  | 10.42% | 6.67% | 7.51% | 89% | 100% |
| Overall | 10.79% | 7.92% | 7.75% | 11.38% | 11.83% | 8.96% | 9.27% | 9.98% | 91% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM-13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 11.73% | 8.87% | 9.07% | 12.86% | 13.64% | 8.80% | 13.29% | 14.34% | 91% | 97% |
| Class H2 |  |  |  |  |  | 15.56% | 8.76% | 9.14% | 91% | 100% |
| Overall | 11.73% | 8.87% | 9.07% | 12.86% | 13.64% | 12.18% | 11.02% | 11.74% | 91% | 98% |

Bin rate and bin to bit ratio (Low QP):

The relevant tests in this report were conducted following the method in JVET-N0049 [4] and JVET-V0150 [5].

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bitrate of a coded sequence

- The average context coded bin rate, bypass bin rate and bitrate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bitrate consumed for the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements for the peak case, for the average case and for the transform coefficients coding part are calculated based on the collected data:

- Weighted BD-bin rate

- {Weighted, Unweighted} bin to bit ratio

- Weighted bin rate-increase

In the weighted measurements, a bypass bin is counted at 0.25 context coded bins (1:4); in the unweight measurements, a bypass bin is counted at 1 context coded bin (1:1).

The summary results of proposed method compared with VTM13 for low QP are provided in the table below. For detailed results, please refer to the attached spreadsheets.

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.06% | 0.09% | 0.07% | -37.59% | -37.56% | -37.57% | -37.53% | 1.16 | 1.04 | -10.57% | 0.47 | 0.29 | -37.45% | 85% | 78% |
| SVT12 | 1.51% | 1.70% | 1.68% | -45.42% | -45.33% | -45.32% | -45.55% | 1.17 | 1.05 | -10.40% | 0.58 | 0.31 | -46.46% | 85% | 85% |
| Overall | 0.79% | 0.89% | 0.88% | -41.51% | -41.44% | -41.45% | -41.54% | 1.16 | 1.04 | -10.49% | 0.52 | 0.30 | -42.42% | 85% | 81% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.32% | 0.36% | 0.35% | -40.01% | -39.96% | -39.96% | -40.03% | 1.16 | 1.03 | -10.56% | 0.47 | 0.29 | -37.16% | 87% | 82% |
| SVT12 | 3.00% | 3.29% | 3.27% | -50.77% | -50.71% | -50.70% | -51.03% | 1.17 | 1.05 | -10.39% | 0.57 | 0.31 | -45.72% | 80% | 85% |
| Overall | 1.66% | 1.82% | 1.81% | -45.39% | -45.33% | -45.33% | -45.53% | 1.16 | 1.04 | -10.47% | 0.52 | 0.30 | -41.86% | 84% | 83% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | 0.35% | 0.41% | 0.41% | -40.39% | -40.30% | -40.30% | -40.40% | 1.15 | 1.03 | -10.34% | 0.46 | 0.29 | -36.32% | 88% | 67% |
| SVT12 | 3.40% | 3.69% | 3.67% | -51.70% | -51.61% | -51.61% | -52.04% | 1.17 | 1.05 | -10.26% | 0.56 | 0.31 | -44.23% | 80% | 69% |
| Overall | 1.87% | 2.05% | 2.04% | -46.05% | -45.95% | -45.95% | -46.22% | 1.16 | 1.04 | -10.30% | 0.51 | 0.30 | -40.65% | 84% | 68% |
| Overall RGB | 1.44% | 1.59% | 1.58% | -44.31% | -44.24% | -44.24% | -44.43% | 1.16 | 1.04 | -10.42% | 0.52 | 0.30 | -41.65% | 84% | 78% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 11.24% | 13.81% | 14.13% | -54.15% | -53.43% | -53.14% | -81.23% | 1.27 | 1.07 | -16.14% | 0.90 | 0.37 | -58.16% | 93% | 92% |
| PQ422 | 9.43% | 11.33% | 11.26% | -53.27% | -52.48% | -52.14% | -56.04% | 1.26 | 1.07 | -15.56% | 0.87 | 0.37 | -56.80% | 89% | 90% |
| Overall | 10.33% | 12.57% | 12.70% | -53.71% | -52.96% | -52.64% | -68.63% | 1.27 | 1.07 | -15.85% | 0.88 | 0.37 | -57.49% | 91% | 91% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 17.84% | 21.14% | 21.28% | -55.17% | -54.06% | -53.96% | -60.24% | 1.26 | 1.07 | -14.82% | 0.89 | 0.39 | -56.69% | 89% | 92% |
| PQ422 | 16.97% | 19.40% | 19.43% | -54.66% | -53.86% | -53.40% | -59.70% | 1.24 | 1.07 | -13.39% | 0.83 | 0.38 | -53.83% | 84% | 90% |
| Overall | 17.40% | 20.27% | 20.36% | -54.92% | -53.96% | -53.68% | -59.97% | 1.25 | 1.07 | -14.11% | 0.86 | 0.38 | -55.32% | 86% | 91% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 17.62% | 20.50% | 21.33% | -55.53% | -54.54% | -54.13% | -60.71% | 1.23 | 1.07 | -13.14% | 0.84 | 0.38 | -54.60% | 88% | 77% |
| PQ422 | 16.61% | 19.01% | 19.06% | -55.08% | -54.17% | -53.72% | -60.13% | 1.22 | 1.07 | -12.46% | 0.80 | 0.38 | -52.76% | 85% | 73% |
| Overall | 17.11% | 19.76% | 20.20% | -55.30% | -54.36% | -53.93% | -60.42% | 1.23 | 1.07 | -12.80% | 0.82 | 0.38 | -53.70% | 86% | 75% |
| Overall PQ | 14.95% | 17.53% | 17.75% | -54.64% | -53.76% | -53.42% | -63.01% | 1.25 | 1.07 | -14.27% | 0.85 | 0.38 | -55.55% | 88% | 86% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 6.62% | 8.55% | 9.13% | -55.02% | -54.38% | -53.96% | -57.26% | 1.24 | 1.05 | -15.12% | 0.80 | 0.34 | -57.51% | 84% | 86% |
| HLG422 | 6.05% | 7.64% | 8.06% | -52.51% | -51.86% | -51.46% | -54.51% | 1.23 | 1.06 | -13.97% | 0.77 | 0.34 | -55.26% | 85% | 85% |
| Overall | 6.34% | 8.09% | 8.59% | -53.77% | -53.12% | -52.71% | -55.88% | 1.24 | 1.06 | -14.55% | 0.78 | 0.34 | -56.41% | 85% | 85% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 16.36% | 17.71% | 18.63% | -62.98% | -62.67% | -62.34% | -67.44% | 1.25 | 1.06 | -15.08% | 0.81 | 0.35 | -57.13% | 75% | 86% |
| HLG422 | 17.43% | 18.83% | 19.01% | -60.78% | -60.46% | -60.16% | -65.69% | 1.23 | 1.06 | -13.72% | 0.78 | 0.35 | -54.51% | 72% | 82% |
| Overall | 16.89% | 18.27% | 18.82% | -61.88% | -61.57% | -61.25% | -66.56% | 1.24 | 1.06 | -14.40% | 0.80 | 0.35 | -55.85% | 73% | 84% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over VTM13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 16.71% | 17.77% | 18.53% | -63.02% | -62.84% | -62.65% | -67.61% | 1.24 | 1.06 | -14.25% | 0.79 | 0.34 | -56.40% | 76% | 72% |
| HLG422 | 17.67% | 18.54% | 18.70% | -60.95% | -60.87% | -60.62% | -65.96% | 1.22 | 1.06 | -12.99% | 0.75 | 0.35 | -53.53% | 73% | 70% |
| Overall | 17.19% | 18.16% | 18.61% | -61.98% | -61.86% | -61.63% | -66.78% | 1.23 | 1.06 | -13.62% | 0.77 | 0.35 | -55.00% | 74% | 71% |
| Overall HLG | 13.47% | 14.84% | 15.34% | -59.21% | -58.85% | -58.53% | -63.08% | 1.24 | 1.06 | -14.19% | 0.78 | 0.35 | -55.76% | 77% | 80% |

Comparison with HM16.23

The test conditions in this contribution followed the CTC for high bit depth and high bit rate video coding [3]. The anchor was generated by HM16.23, where the High Throughput 4:4:4 Intra profile mode was disabled. The same method in JVET-N0049 [4] and JVET-V0150 [5] described above is also used in the tests below.

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -2.30% | -1.48% | -1.51% | -37.42% | -36.97% | -36.99% | -36.17% | 1.19 | 1.04 | -13.11% | 0.46 | 0.29 | -35.91% | 5257% | 113% |
| SVT12 | -2.52% | -1.27% | -1.36% | -45.32% | -44.88% | -44.90% | -44.16% | 1.21 | 1.05 | -13.47% | 0.56 | 0.31 | -44.40% | 5480% | 124% |
| Overall | -2.41% | -1.37% | -1.44% | -41.37% | -40.92% | -40.94% | -40.17% | 1.20 | 1.04 | -13.29% | 0.51 | 0.30 | -40.55% | 5368% | 119% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -1.97% | -0.45% | -0.44% | -40.20% | -39.47% | -39.46% | -39.24% | 1.19 | 1.03 | -12.90% | 0.45 | 0.29 | -35.37% | 644% | 125% |
| SVT12 | -1.32% | 1.48% | 1.46% | -51.55% | -50.82% | -50.81% | -50.82% | 1.21 | 1.05 | -13.64% | 0.55 | 0.31 | -43.83% | 559% | 123% |
| Overall | -1.65% | 0.51% | 0.51% | -45.87% | -45.14% | -45.13% | -45.03% | 1.20 | 1.04 | -13.27% | 0.50 | 0.30 | -40.01% | 602% | 124% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -2.06% | -0.56% | -0.56% | -40.44% | -39.69% | -39.69% | -39.43% | 1.18 | 1.03 | -12.75% | 0.45 | 0.29 | -34.83% | 726% | 101% |
| SVT12 | -1.22% | 1.66% | 1.64% | -52.16% | -51.39% | -51.39% | -51.46% | 1.21 | 1.05 | -13.76% | 0.54 | 0.31 | -42.39% | 649% | 101% |
| Overall | -1.64% | 0.55% | 0.54% | -46.30% | -45.54% | -45.54% | -45.44% | 1.20 | 1.04 | -13.26% | 0.49 | 0.30 | -38.95% | 687% | 101% |
| Overall RGB | -1.90% | -0.10% | -0.13% | -44.51% | -43.87% | -43.87% | -43.55% | 1.20 | 1.04 | -13.27% | 0.50 | 0.30 | -39.85% | 2219% | 115% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 4.57% | 6.02% | 5.44% | -48.25% | -47.85% | -47.90% | -65.72% | 1.19 | 1.07 | -10.18% | 0.75 | 0.37 | -50.00% | 3335% | 147% |
| PQ422 | -0.66% | -3.42% | -4.27% | -49.85% | -50.82% | -50.90% | -49.18% | 1.19 | 1.07 | -10.00% | 0.73 | 0.37 | -48.83% | 2705% | 142% |
| Overall | 1.95% | 1.30% | 0.58% | -49.05% | -49.34% | -49.40% | -57.45% | 1.19 | 1.07 | -10.09% | 0.74 | 0.37 | -49.42% | 3020% | 144% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 10.00% | 15.21% | 13.37% | -50.92% | -49.16% | -49.65% | -53.91% | 1.18 | 1.07 | -9.30% | 0.74 | 0.39 | -47.59% | 286% | 155% |
| PQ422 | 7.56% | 5.06% | 4.14% | -52.21% | -53.05% | -53.16% | -54.50% | 1.18 | 1.07 | -9.16% | 0.70 | 0.38 | -45.41% | 360% | 154% |
| Overall | 8.78% | 10.13% | 8.76% | -51.56% | -51.10% | -51.40% | -54.20% | 1.18 | 1.07 | -9.23% | 0.72 | 0.38 | -46.53% | 323% | 155% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 9.62% | 14.44% | 13.67% | -51.32% | -49.68% | -49.72% | -54.36% | 1.18 | 1.07 | -9.24% | 0.71 | 0.38 | -46.12% | 331% | 130% |
| PQ422 | 6.92% | 4.27% | 3.96% | -52.61% | -53.46% | -53.35% | -54.77% | 1.18 | 1.07 | -9.18% | 0.68 | 0.38 | -44.53% | 432% | 132% |
| Overall | 8.27% | 9.35% | 8.81% | -51.96% | -51.57% | -51.54% | -54.56% | 1.18 | 1.07 | -9.21% | 0.70 | 0.38 | -45.33% | 382% | 131% |
| Overall PQ | 6.34% | 6.93% | 6.05% | -50.86% | -50.67% | -50.78% | -55.40% | 1.18 | 1.07 | -9.51% | 0.72 | 0.38 | -47.14% | 1241% | 143% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 1.94% | 2.06% | 2.25% | -52.41% | -52.43% | -52.21% | -53.23% | 1.20 | 1.05 | -12.08% | 0.71 | 0.34 | -52.56% | 3661% | 149% |
| HLG422 | 0.63% | -0.27% | 0.04% | -50.35% | -50.72% | -50.38% | -50.64% | 1.20 | 1.06 | -11.80% | 0.69 | 0.34 | -50.16% | 3138% | 143% |
| Overall | 1.28% | 0.89% | 1.14% | -51.38% | -51.57% | -51.29% | -51.94% | 1.20 | 1.06 | -11.94% | 0.70 | 0.34 | -51.38% | 3400% | 146% |
|  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 9.69% | 8.26% | 9.89% | -60.77% | -61.23% | -60.72% | -63.70% | 1.19 | 1.06 | -11.20% | 0.72 | 0.35 | -51.39% | 331% | 142% |
| HLG422 | 11.16% | 7.41% | 9.44% | -59.42% | -60.63% | -59.89% | -62.83% | 1.19 | 1.06 | -10.89% | 0.69 | 0.35 | -48.72% | 403% | 143% |
| Overall | 10.43% | 7.83% | 9.66% | -60.09% | -60.93% | -60.31% | -63.27% | 1.19 | 1.06 | -11.04% | 0.70 | 0.35 | -50.08% | 367% | 142% |
|  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over HM-16.23 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 9.37% | 7.59% | 9.21% | -60.98% | -61.63% | -61.18% | -63.87% | 1.20 | 1.06 | -11.43% | 0.70 | 0.34 | -50.88% | 363% | 112% |
| HLG422 | 10.70% | 6.80% | 8.96% | -59.72% | -61.14% | -60.41% | -63.03% | 1.20 | 1.06 | -11.10% | 0.67 | 0.35 | -47.89% | 455% | 113% |
| Overall | 10.04% | 7.19% | 9.08% | -60.35% | -61.39% | -60.79% | -63.45% | 1.20 | 1.06 | -11.27% | 0.69 | 0.35 | -49.42% | 409% | 112% |
| Overall HLG | 7.25% | 5.31% | 6.63% | -57.27% | -57.96% | -57.46% | -59.55% | 1.20 | 1.06 | -11.42% | 0.70 | 0.35 | -50.30% | 1392% | 134% |

Comparison with HM16.23 High Throughput 4:4:4 Intra profile

The test conditions in this contribution followed the CTC for high bit depth and high bit rate video coding [3]. The anchor was generated by HM16.23, where the High Throughput 4:4:4 Intra profile mode was enabled for all intra. The same method in JVET-N0049 [4] and JVET-V0150 [5] described above is also used in the tests below.

| SVT RGB |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.23 high throughput 444 intra 16 profile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrG | psnrB | psnrR | psnrG | psnrB | psnrR | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| SVT16 | -2.61% | -1.76% | -1.80% | -38.16% | -37.71% | -37.72% | -36.77% | 1.19 | 1.04 | -13.28% | 0.46 | 0.29 | -36.50% | 5314% | 139% |
| SVT12 | -3.04% | -1.79% | -1.89% | -46.36% | -45.93% | -45.96% | -45.06% | 1.21 | 1.05 | -13.78% | 0.56 | 0.31 | -45.14% | 5648% | 138% |
| Overall | -2.82% | -1.78% | -1.84% | -42.26% | -41.82% | -41.84% | -40.91% | 1.20 | 1.04 | -13.53% | 0.51 | 0.30 | -41.24% | 5481% | 139% |

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.23 high throughput 444 intra 16 profile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | PsnrY | PsnrU | PsnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| PQ444 | 3.74% | 5.17% | 4.60% | -49.38% | -48.99% | -49.04% | -67.10% | 1.19 | 1.07 | -10.69% | 0.76 | 0.37 | -50.65% | 3403% | 159% |
| PQ422 | -1.43% | -4.16% | -5.01% | -50.93% | -51.89% | -51.98% | -49.99% | 1.19 | 1.07 | -10.50% | 0.74 | 0.37 | -49.51% | 2757% | 156% |
| Overall | 1.15% | 0.51% | -0.21% | -50.16% | -50.44% | -50.51% | -58.54% | 1.19 | 1.07 | -10.59% | 0.75 | 0.37 | -50.09% | 3080% | 157% |

| HDR HLG |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.23 high throughput 444 intra 16 profile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Delta Binrate (average) | Bin2Bit (peak, unweighted) |  |  | Bin2Bit (peak, weighted) |  |  | Runtime |  |
|  | psnrY | psnrU | psnrV | psnrY | psnrU | psnrV | weighted | Reference | Tested | Diff (%) | Reference | Tested | Diff (%) | EncT | DecT |
| HLG444 | 1.19% | 1.31% | 1.50% | -53.38% | -53.41% | -53.19% | -53.96% | 1.21 | 1.05 | -12.51% | 0.72 | 0.34 | -53.17% | 3685% | 151% |
| HLG422 | -0.11% | -0.99% | -0.67% | -51.40% | -51.75% | -51.42% | -51.44% | 1.21 | 1.06 | -12.23% | 0.70 | 0.34 | -50.83% | 3153% | 146% |
| Overall | 0.54% | 0.16% | 0.42% | -52.39% | -52.58% | -52.31% | -52.70% | 1.21 | 1.06 | -12.37% | 0.71 | 0.34 | -52.02% | 3419% | 148% |

Analysis

The weighted BD-bin rate for low QP reduction is roughly 37%~63% compared with VTM13 and HM16.23. The unweighted BD-bin rate for low QP reduction is roughly 10%~34% compared with VTM13 and roughly 10%~19% compared with HM16.23. For the same PSNR values, the proposed method consumes much less bins than the current VVC version 2 and HEVC.

The weighted peak bin to bit ratio (Bin2Bit) reduction is roughly 36%~58% compared with VTM13 and roughly 35%~53% compared with HM16.23. The unweighted peak bin to bit ratio reduction is roughly 10%~16% compared with VTM13 and roughly 9%~14% compared with HM16.23. It implies that, the proposed method can significantly benefit the throughput and, with the proposed method, there is no need to increase the CABAC engine pace compared with HEVC.

The “weighted” results are much better than the “unweighted” ones because all the context coded syntax elements in residual level coding are left out and residual level coding accounts for a major portion of the bitstream in high bit depth, high bit rate coding. In the weighted measurements, a bypass bin is counted as 0.25 context coded bin according to [4][5] because the process of decoding a context coded bin is much more complicated than that for a bypass bin.

In the proposed method, all the coefficients in a TU are coded as bypass coded bins. As a result, the entropy coding complexity is significantly reduced, which is clearly shown by the processing time reduction in the simulation results. In addition, as the alignment process is performed prior to the coefficient coding, the proposed method enables decoding of all the coefficients in a TB simply by using a shift register and multiple coded bins can be decoded in parallel [6], which allows an easy hardware implementation of the proposed method.

The proposed method brings significant gain in BD-bin rate and bin to bit ratio in all low QP tests. In addition, when the bitrate is high enough the loss in BD-bitrate is negligible, for example SVT16 tests with AI 0.06% LD 0.32% RA 0.35% loss.

But if the bitrate is not high enough, for example PQ and HLG Low QP tests, the loss in BD-bitrate is certainly quite noticeable. The reason comes from 2 aspects. First, in PQ and HLG Low QP tests, it has been observed that a portion of coefficients are small numbers such as 0,1,2,3… In this case, the context coded syntax elements, sig_coeff_flag, abs_level_gtx_flag, par_level_flag, abs_level_gtx_flag, play an important role in terms of BD-bitrate savings. But when coefficients are usually big numbers like in the case with SVT16 tests, using these syntax elements won’t impact much to the BD-bitrate performance. Secondly, the current harmonization with the history based rice parameter derivation method makes it always use “+2” in the calculation of StatCoeff[cIdx], which works well for high bitrate cases, but not so much when bitrate is not high enough.

Conclusion

This contribution proposes a high throughput mode for high bit depth and high bit rate coding. It is asserted that with the proposed method, all the syntax elements in residual coding, except for last significant coefficient position in RRC, are coded through bypass mode and alignment is needed only once before these syntax elements in a transform block. It is believed that the proposed method significantly reduces the BD-binrate and bin to bit ratio and enables decoding of all the coefficients in a TB simply by using a shift register and multiple coded bins can be decoded in parallel. As a result, the goal of high coding throughput is achieved. It is suggested to apply the proposed method in 16-bit high throughput configurations.

References

[1] F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang, “AHG8: a combination of JVET-V0059 option 2 and JVET-V0122 for high bit depth and high bit rate extensions”, JVET-V0178.

[2] D. Rusanovskyy, T. Hsieh, L. Pham Van, M. Coban, M. Karczewicz, “CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths”, JVET-V0106.

[3] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018.

[4] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19,” JVET-N0049.

[5] T. Tsukuba, M. Ikeda, T. Suzuki, “AHG8: A study on bin rate of VTM-12.0 for high bit depth coding”, JVET-V0150.

[6] K. Sharman, N. Saunders, J. Gamei, “Entropy Coding Throughput for High Bit Depths”, JCTVC-N0190.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).