| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0064-v2 |
| --- | --- |

| Title: | AHG8: Constraints on transforms and high precision operation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tianyang Zhou Tomohiro Hashimoto | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to reduce complexity by utilizing exclusive usage on 64 point transform and LFNST for high bit depth coding. Specifically, 64 point transform and LFNST are normatively disabled when extended precision flag is equal to 1. In low QP HBD CTC experiment, the proposal shows up to 0.01 % loss as follows.

- 0.04 %, 0.06 %, 0.05 % in PQ

- 0.00%, 0.00% and -0.00% in HLG

- 0.00%, 0.00% and 0.00 % in SVT

The more QPs condition from QP = -13 to 27 was also conducted. It shows proposed constraint (disabling 64 point transform and LFNST) show 0.37 %, 2.12 % and 2.16 % loss in Y, U, and V at QP 12, 17, 22, 27 range in AI PQ 4:2:2. At the same time, this loss reduced to 0.11 %, 0.42 %, 0.27 % at QP 7, 12, 17, 22 by enabling 64 point transform and disabling extended precision (this combination remains valid in this proposal).

Considering performance and complexity balance, this contribution recommends exclusive usage.

v2 fixed PQ simulation results to include FireEater for average bd-rate value in accordance to JVET-U2018.

Introduction

When extended_precision_processing_flag is equal to 1, the transfrom coefficient range is extended from 15 to Min(BitDepth + 6, 20). In this case, transform implemetation needs to be extended. These extension could be straightforward but needs some more design cost. It is proposed to diable use of extended precision and 64 point transform as well as extended precision and LFNST combination to alleviate that issue.

Proposal

Option1: Syntax constraint

- 7.3.2.22	Sequence parameter set range extension syntax

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| if (sps_max_luma_transform_size_64_flag = = 0 &&sps_lfnst_enabled_flag = = 0) |  |
| extended_precision_processing_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_rrc_rice_extension_flag | u(1) |
| sps_persistent_rice_adaptation_enabled_flag | u(1) |
| } |  |

- 7.4.3.22 Sequence parameter set range extension semantics

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

Option2: Derivation constraint

7.4.3.4 Sequence parameter set RBSP semantics

MaxTbLog2SizeY = sps_max_luma_transform_size_64_flag && !extended_precision_processing_flag ? 6 : 5

7.4.12.5	Coding unit semantics

The variable ApplyLfnstFlag[ cIdx ] is derived as follows:

- When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the following applies:

ApplyLfnstFlag[ 0 ] = ( lfnst_idx > 0 ) && !extended_precision_processing_flag ? 1 : 0		(172)

- The following applies for cIdx = 1, 2:

ApplyLfnstFlag[ cIdx ] = ( lfnst_idx > 0  &&  treeType  = =  DUAL_TREE_CHROMA && !extended_precision_processing_flag) ? 1 : 0	(173)

Option3: Bitstream constraint

- 7.4.3.22 Sequence parameter set range extension semantics

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that when sps_max_luma_transform_size_64_flag is equal to 1 or sps_lfnst_enabled_flag is equal to 1, extended_precision_processing_flag shall be 0.

Experimental results

Anchor: extended_precision_processing_flag is equal to 1, 32x32 transform is off and LFNST off

|  | Extended precision | 64 point transform | LFNST |
| --- | --- | --- | --- |
| CTC | 1 | 0 (for RGB) 1 (for YCbCr) | 0 |
| Proposed | 1 | 0 | 0 |
| Test1 (disbaled by proposal) = CTC | 1 | 1 | 0 |
| Test2 (disbaled by proposal) | 1 | 0 | 1 |
| Test3 | 0 | 1 | 0 |
| Test4 | 0 | 0 | 1 |

Experimental results in CTC QP ranges

The proposal shows virtually no loss in lowQPs.

- 0.01%, 0.02%, 0.01% in PQ

- 0.00%, 0.00% and -0.00% in HLG

- 0.00%, 0.00% and 0.00 % in SVT

Proposal (disable 64 point transform)

It was observed that disabling 64 point transform shows up to 0.08 %, 0.13 % and 0.14 % loss in AI PQ. It is asserted the loss is small.

Table 4.0: --ExtendedPrecision=1 --Log2MaxTbSize=5 --LFNST=0

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.09% | 0.12% | 0.15% | 0.09% | 0.12% | 0.15% | 103% | 101% |
| PQ422 | 0.08% | 0.13% | 0.13% | 0.07% | 0.13% | 0.13% | 100% | 101% |
| Overall | 0.09% | 0.13% | 0.14% | 0.08% | 0.13% | 0.14% | 102% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.02% | 0.01% | 0.02% | 0.01% | 0.01% | 102% | 100% |
| PQ422 | 0.01% | 0.04% | 0.02% | 0.01% | 0.03% | 0.02% | 102% | 100% |
| Overall | 0.02% | 0.03% | 0.01% | 0.01% | 0.02% | 0.01% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.04% | -0.01% | 0.02% | 0.03% | 0.00% | 101% | 93% |
| PQ422 | 0.02% | 0.03% | 0.01% | 0.02% | 0.02% | 0.01% | 101% | 101% |
| Overall | 0.02% | 0.03% | 0.00% | 0.02% | 0.03% | 0.00% | 101% | 97% |
| Overall PQ | 0.04% | 0.06% | 0.05% | 0.04% | 0.06% | 0.05% | 101% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.01% | 0.00% | 101% | 100% |
| HLG422 | 0.01% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.01% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.00% | 0.02% | 102% | 100% |
| HLG422 | 0.01% | 0.00% | -0.02% | 101% | 100% |
| Overall | 0.01% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 101% | 92% |
| HLG422 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Overall HLG | 0.00% | 0.00% | 0.00% | 101% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 100% | 100% |

Note: For SVT, 64 point transform has been already disabled to ACT.

Test1 (enable 64x64 TUs, no Changes from CTC)

Test1’s result is the same as CTC for PQ/HLG and not applicable for RGB where 64 point transform is disabled due to ACT (and ACT’s gain is always better than 64 point transform). Thus this test was not conducted.

Test2 (enable LFNST) (disbaled by proposal)

It was observed that enabling LFNST shows loss up to 0.40 %, 0.04 % and 0.00 % in PQ AI.

Table 4.2: --ExtendedPrecision=1 --Log2MaxTbSize=5 --LFNST=1

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.44% | 0.06% | 0.05% | 0.44% | 0.06% | 0.05% | 114% | 102% |
| PQ422 | 0.35% | 0.02% | -0.05% | 0.35% | 0.03% | -0.04% | 108% | 102% |
| Overall |  |  | 0.39% |  | 0.04% |  | 0.00% |  | 0.40% |  | 0.04% |  | 0.00% |  | 111% |  | 102% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | -0.70% | -0.53% | 0.00% | -0.74% | -0.54% | 157% | 100% |
| PQ422 | 0.01% | -0.06% | -0.10% | 0.01% | -0.07% | -0.09% | 135% | 100% |
| Overall | 0.00% | -0.38% | -0.31% | 0.00% | -0.41% | -0.32% | 146% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.04% | -0.87% | -0.65% | 0.04% | -0.89% | -0.65% | 145% | 93% |
| PQ422 | 0.04% | -0.06% | -0.08% | 0.04% | -0.06% | -0.08% | 121% | 101% |
| Overall | 0.04% | -0.46% | -0.37% | 0.04% | -0.48% | -0.36% | 133% | 97% |
| Overall PQ | 0.15% | -0.27% | -0.23% | 0.15% | -0.28% | -0.22% | 130% | 100% |

Test3 (enable 64 point transform but disable extended_precision_flag)

It was observed that disabling extended precision flag shows loss up to 0.76 %, 1.04 % and 1.10% in PQ AI.

Table 4.3: --ExtendedPrecision=0 --Log2MaxTbSize=6 --LFNST=0

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.32% | 1.96% | 2.10% | 1.32% | 1.99% | 2.13% | 101% | 107% |
| PQ422 | 1.11% | 1.42% | 1.54% | 1.10% | 1.42% | 1.53% | 99% | 107% |
| Overall | 1.22% | 1.69% | 1.82% | 1.21% | 1.71% | 1.83% | 100% | 107% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.70% | 0.74% | 0.81% | 0.71% | 0.74% | 0.81% | 100% | 102% |
| PQ422 | 0.64% | 0.52% | 0.59% | 0.65% | 0.52% | 0.60% | 99% | 102% |
| Overall | 0.67% | 0.63% | 0.70% | 0.68% | 0.63% | 0.70% | 100% | 102% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.62% | 0.69% | 0.75% | 0.62% | 0.68% | 0.76% | 99% | 102% |
| PQ422 | 0.58% | 0.55% | 0.60% | 0.58% | 0.54% | 0.60% | 100% | 102% |
| Overall | 0.60% | 0.62% | 0.68% | 0.60% | 0.61% | 0.68% | 99% | 102% |
| Overall PQ | 0.68% | 0.76% | 0.83% | 0.68% | 0.76% | 0.83% | 99% | 103% |

Test4 (enable LFNST but disable extended_precision_flag)

It was observed that disabling extended precision flag and enabling LFNST shows loss up to 1.63 %, 1.67 % and 1.73 % in PQ AI.

Table 4.4: --ExtendedPrecision=0 --Log2MaxTbSize=5 --LFNST=1

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.78% | 1.93% | 2.05% | 1.79% | 1.96% | 2.07% | 113% | 108% |
| PQ422 | 1.48% | 1.37% | 1.38% | 1.48% | 1.39% | 1.38% | 107% | 108% |
| Overall | 1.63% | 1.65% | 1.72% | 1.63% | 1.67% | 1.73% | 110% | 108% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.67% | -0.01% | 0.25% | 0.68% | -0.05% | 0.23% | 155% | 101% |
| PQ422 | 0.67% | 0.48% | 0.52% | 0.67% | 0.47% | 0.51% | 136% | 102% |
| Overall | 0.67% | 0.24% | 0.38% | 0.68% | 0.21% | 0.37% | 145% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.66% | -0.22% | 0.09% | 0.66% | -0.25% | 0.08% | 145% | 94% |
| PQ422 | 0.62% | 0.48% | 0.53% | 0.61% | 0.46% | 0.52% | 121% | 103% |
| Overall | 0.64% | 0.13% | 0.31% | 0.64% | 0.11% | 0.30% | 133% | 98% |
| Overall PQ | 0.98% | 0.67% | 0.80% | 0.98% | 0.66% | 0.80% | 129% | 103% |

Experimental results in more QP ranges

We conducted experimental results from QP=-13 to QP 27. The experiment was done in the same low QP CTC conditions. The reported bd-rate was calculated by four QP points rather than six QP points in low QP CTC spread sheet.

It was observed that disabling 64 point transform, in the case of extended precision flag being equal to1, shows up to 0.37 %, 2.12 % and 2.16 % loss in AI at QP 12, 17, 22, 27 in average Y, U and V. However, in the case of extended precision flag being equal to 0, 64 point transform can be enabled (Test3), the loss was reduced to 0.11 %, 0.42 %, 0.27 % at QP 7, 12, 17, 22

Table 5.0: --ExtendedPrecision=1 --Log2MaxTbSize=5 --LFNST=0

| HDR PQ |  |  |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | qpMin | qpMax | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -13 | 2 | 0.09% | 0.00% | 0.04% | 0.09% | -0.01% | 0.04% | 102% | 101% |
|  | -8 | 7 | 0.15% | 0.06% | 0.13% | 0.15% | 0.06% | 0.13% | 103% | 101% |
|  | -3 | 12 | 0.13% | 0.23% | 0.26% | 0.12% | 0.23% | 0.26% | 104% | 102% |
|  | 2 | 17 | 0.11% | 0.43% | 0.40% | 0.12% | 0.40% | 0.40% | 102% | 102% |
|  | 7 | 22 | 0.15% | 0.62% | 0.56% | 0.12% | 0.88% | 0.57% | 100% | 102% |
|  | 12 | 27 | 0.38% | 0.77% | 1.12% | 0.39% | 0.15% | 1.04% | 97% | 103% |
| PQ422 | -13 | 2 | 0.08% | 0.01% | 0.05% | 0.09% | 0.01% | 0.05% | 100% | 101% |
|  | -8 | 7 | 0.11% | 0.08% | 0.13% | 0.11% | 0.08% | 0.13% | 101% | 101% |
|  | -3 | 12 | 0.09% | 0.24% | 0.25% | 0.08% | 0.24% | 0.23% | 101% | 101% |
|  | 2 | 17 | 0.09% | 0.56% | 0.45% | 0.09% | 0.42% | 0.52% | 100% | 102% |
|  | 7 | 22 | 0.14% | 1.03% | 0.91% | 0.10% | 2.44% | 0.72% | 100% | 102% |
|  | 12 | 27 | 0.35% | 2.20% | 2.04% | 0.37% | 2.12% | 2.16% | 100% | 103% |
| Overall | -13 | 27 | 0.16% | 0.52% | 0.53% | 0.15% | 0.58% | 0.52% | 101% | 102% |
| Overall min |  |  | 0.08% | 0.00% | 0.04% | 0.08% | -0.01% | 0.04% |  |  |
| Overall max |  |  | 0.38% | 2.20% | 2.04% | 0.39% | 2.44% | 2.16% |  |  |
|  |  |  |  |  | LDB |  |  |  |  |  |
|  |  |  |  |  | Over VTM13.0 |  |  |  |  |  |
|  |  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -13 | 2 | 0.01% | 0.01% | -0.01% | 0.01% | 0.01% | -0.01% | 102% | 100% |
|  | -8 | 7 | 0.02% | 0.02% | -0.01% | 0.02% | 0.02% | 0.00% | 102% | 100% |
|  | -3 | 12 | 0.03% | 0.03% | 0.02% | 0.03% | 0.02% | 0.02% | 102% | 100% |
|  | 2 | 17 | 0.07% | 0.06% | 0.11% | 0.05% | 0.02% | 0.07% | 101% | 101% |
|  | 7 | 22 | 0.14% | 0.08% | 0.30% | 0.10% | -0.01% | 0.19% | 100% | 100% |
|  | 12 | 27 | 0.20% | 0.07% | 0.63% | 0.17% | 0.05% | 0.52% | 101% | 100% |
| PQ422 | -13 | 2 | -0.01% | 0.01% | -0.01% | -0.01% | 0.01% | 0.00% | 102% | 100% |
|  | -8 | 7 | 0.02% | 0.03% | -0.01% | 0.01% | 0.02% | 0.00% | 102% | 101% |
|  | -3 | 12 | 0.04% | 0.06% | 0.04% | 0.02% | 0.05% | 0.06% | 101% | 101% |
|  | 2 | 17 | 0.07% | 0.17% | 0.21% | 0.05% | 0.17% | 0.15% | 101% | 101% |
|  | 7 | 22 | 0.17% | 0.57% | 0.66% | 0.12% | 0.38% | 0.46% | 101% | 101% |
|  | 12 | 27 | 0.34% | 1.36% | 1.03% | 0.28% | 0.95% | 0.78% | 100% | 101% |
| Overall | -13 | 27 | 0.09% | 0.21% | 0.25% | 0.07% | 0.14% | 0.19% | 101% | 100% |
| Overall min |  |  | -0.01% | 0.01% | -0.01% | -0.01% | -0.01% | -0.01% |  |  |
| Overall max |  |  | 0.34% | 1.36% | 1.03% | 0.28% | 0.95% | 0.78% |  |  |
|  |  |  |  |  | RA |  |  |  |  |  |
|  |  |  |  |  | Over VTM13.0 |  |  |  |  |  |
|  |  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -13 | 2 | 0.01% | 0.01% | -0.01% | 0.01% | 0.01% | 0.00% | 101% | 92% |
|  | -8 | 7 | 0.03% | 0.03% | -0.02% | 0.02% | 0.03% | 0.00% | 101% | 93% |
|  | -3 | 12 | 0.04% | 0.06% | -0.02% | 0.03% | 0.05% | -0.01% | 101% | 94% |
|  | 2 | 17 | 0.05% | 0.11% | 0.04% | 0.03% | 0.08% | 0.11% | 100% | 96% |
|  | 7 | 22 | 0.06% | 0.20% | 0.18% | 0.05% | 0.13% | 0.11% | 100% | 98% |
|  | 12 | 27 | 0.10% | 0.25% | 0.37% | 0.09% | 0.26% | 0.26% | 99% | 99% |
| PQ422 | -13 | 2 | 0.01% | 0.01% | -0.01% | 0.01% | 0.01% | 0.00% | 101% | 101% |
|  | -8 | 7 | 0.02% | 0.02% | 0.00% | 0.02% | 0.02% | 0.01% | 101% | 101% |
|  | -3 | 12 | 0.03% | 0.05% | 0.03% | 0.02% | 0.03% | 0.01% | 101% | 101% |
|  | 2 | 17 | 0.06% | 0.10% | 0.14% | 0.05% | 0.06% | 0.09% | 101% | 101% |
|  | 7 | 22 | 0.11% | 0.20% | 0.30% | 0.08% | 0.11% | 0.18% | 101% | 102% |
|  | 12 | 27 | 0.13% | 0.46% | 0.52% | 0.10% | 0.34% | 0.40% | 100% | 103% |
| Overall | -13 | 27 | 0.05% | 0.12% | 0.13% | 0.04% | 0.09% | 0.10% | 100% | 98% |
| Overall min |  |  | 0.01% | 0.01% | -0.02% | 0.01% | 0.01% | -0.01% |  |  |
| Overall max |  |  | 0.13% | 0.46% | 0.52% | 0.10% | 0.34% | 0.40% |  |  |
| Overall | -13 | 27 | 0.08% | 0.23% | 0.25% | 0.07% | 0.23% | 0.22% | 101% | 100% |
| Overall min |  |  | -0.01% | 0.01% | -0.02% | -0.01% | -0.01% | -0.01% |  |  |
| Overall max |  |  | 0.35% | 2.20% | 2.04% | 0.37% | 2.44% | 2.16% |  |  |

Table 4.3: --ExtendedPrecision=0 --Log2MaxTbSize=6 --LFNST=0

| HDR PQ |  |  |  |  |  |  |  |  |  |  |  | AI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Over VTM11.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | qpMin | qpMax | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT | CR ref | CR test |
| PQ444 | -13 | 2 | 2.11% | 2.84% | 3.02% | 2.12% | 2.89% | 3.06% | 100% | 110% | 6.9 | 6.8 |
|  | -8 | 7 | 1.05% | 1.56% | 1.69% | 1.06% | 1.58% | 1.72% | 102% | 106% | 9.8 | 9.7 |
|  | -3 | 12 | 0.46% | 0.89% | 0.92% | 0.47% | 0.91% | 0.95% | 103% | 103% | 16.0 | 15.9 |
|  | 2 | 17 | 0.22% | 0.56% | 0.53% | 0.22% | 0.58% | 0.59% | 101% | 102% | 33.7 | 33.7 |
|  | 7 | 22 | 0.11% | 0.30% | 0.27% | 0.11% | 0.42% | 0.27% | 98% | 101% | 69.0 | 68.9 |
|  | 12 | 27 | 0.05% | 0.11% | 0.11% | 0.04% | 0.06% | 0.14% | 95% | 100% | 133.0 | 132.9 |
| PQ422 | -13 | 2 | 1.77% | 2.02% | 2.07% | 1.76% | 2.03% | 2.08% | 99% | 109% | 5.6 | 5.6 |
|  | -8 | 7 | 0.89% | 1.10% | 1.17% | 0.90% | 1.11% | 1.17% | 100% | 105% | 7.9 | 7.8 |
|  | -3 | 12 | 0.38% | 0.61% | 0.68% | 0.38% | 0.61% | 0.68% | 100% | 103% | 12.6 | 12.6 |
|  | 2 | 17 | 0.18% | 0.42% | 0.39% | 0.18% | 0.42% | 0.44% | 100% | 101% | 25.2 | 25.2 |
|  | 7 | 22 | 0.09% | 0.26% | 0.18% | 0.09% | 0.22% | 0.11% | 100% | 100% | 49.6 | 49.5 |
|  | 12 | 27 | 0.03% | 0.17% | 0.13% | 0.04% | 0.17% | 0.11% | 99% | 100% | 93.4 | 93.4 |
| Overall | -13 | 27 | 0.61% | 0.90% | 0.93% | 0.61% | 0.92% | 0.94% | 100% | 103% |  |  |
| Overall min |  |  | 0.03% | 0.11% | 0.11% | 0.04% | 0.06% | 0.11% |  |  |  |  |
| Overall max |  |  | 2.11% | 2.84% | 3.02% | 2.12% | 2.89% | 3.06% |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | LDB |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Over VTM11.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT | CR ref | CR test |
| PQ444 | -13 | 2 | 1.09% | 1.18% | 1.18% | 1.11% | 1.20% | 1.21% | 100% | 103% | 11.5 | 11.4 |
|  | -8 | 7 | 0.43% | 0.47% | 0.46% | 0.45% | 0.47% | 0.48% | 100% | 101% | 18.6 | 18.6 |
|  | -3 | 12 | 0.21% | 0.21% | 0.24% | 0.22% | 0.19% | 0.25% | 100% | 100% | 39.8 | 39.8 |
|  | 2 | 17 | 0.09% | 0.08% | 0.13% | 0.10% | 0.08% | 0.19% | 100% | 100% | 99.2 | 99.2 |
|  | 7 | 22 | 0.04% | -0.07% | 0.01% | 0.04% | -0.14% | -0.05% | 100% | 99% | 235.7 | 235.7 |
|  | 12 | 27 | 0.03% | -0.33% | -0.17% | 0.03% | -0.20% | -0.19% | 100% | 99% | 551.6 | 551.9 |
| PQ422 | -13 | 2 | 1.00% | 0.81% | 0.77% | 1.02% | 0.82% | 0.79% | 99% | 103% | 9.7 | 9.7 |
|  | -8 | 7 | 0.43% | 0.31% | 0.34% | 0.44% | 0.32% | 0.37% | 100% | 101% | 16.0 | 16.0 |
|  | -3 | 12 | 0.21% | 0.12% | 0.23% | 0.21% | 0.14% | 0.27% | 100% | 101% | 33.0 | 33.0 |
|  | 2 | 17 | 0.10% | 0.04% | 0.18% | 0.10% | 0.07% | 0.14% | 100% | 101% | 76.2 | 76.2 |
|  | 7 | 22 | 0.06% | 0.19% | 0.30% | 0.05% | 0.15% | 0.21% | 100% | 100% | 171.2 | 171.3 |
|  | 12 | 27 | 0.07% | 0.60% | 0.28% | 0.05% | 0.43% | 0.20% | 100% | 99% | 389.1 | 389.3 |
| Overall | -13 | 27 | 0.31% | 0.30% | 0.33% | 0.32% | 0.30% | 0.32% | 100% | 101% |  |  |
| Overall min |  |  | 0.03% | -0.33% | -0.17% | 0.03% | -0.20% | -0.19% |  |  |  |  |
| Overall max |  |  | 1.09% | 1.18% | 1.18% | 1.11% | 1.20% | 1.21% |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Over VTM11.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT | CR ref | CR test |
| PQ444 | -13 | 2 | 0.94% | 1.06% | 1.06% | 0.95% | 1.06% | 1.08% | 98% | 102% | 12.1 | 12.1 |
|  | -8 | 7 | 0.40% | 0.44% | 0.43% | 0.41% | 0.45% | 0.46% | 99% | 101% | 20.4 | 20.4 |
|  | -3 | 12 | 0.20% | 0.21% | 0.21% | 0.20% | 0.21% | 0.25% | 99% | 101% | 43.2 | 43.2 |
|  | 2 | 17 | 0.10% | 0.07% | 0.12% | 0.10% | 0.08% | 0.17% | 99% | 100% | 104.8 | 104.7 |
|  | 7 | 22 | 0.07% | -0.01% | 0.06% | 0.07% | -0.01% | 0.07% | 99% | 100% | 239.1 | 239.1 |
|  | 12 | 27 | 0.08% | -0.20% | 0.07% | 0.07% | -0.13% | 0.03% | 99% | 100% | 534.9 | 534.4 |
| PQ422 | -13 | 2 | 0.87% | 0.78% | 0.77% | 0.88% | 0.79% | 0.79% | 99% | 103% | 10.3 | 10.2 |
|  | -8 | 7 | 0.39% | 0.35% | 0.37% | 0.39% | 0.35% | 0.39% | 100% | 102% | 17.4 | 17.4 |
|  | -3 | 12 | 0.20% | 0.17% | 0.22% | 0.21% | 0.18% | 0.25% | 100% | 101% | 35.8 | 35.8 |
|  | 2 | 17 | 0.11% | 0.06% | 0.15% | 0.11% | -0.03% | 0.10% | 100% | 101% | 80.6 | 80.6 |
|  | 7 | 22 | 0.08% | -0.04% | 0.07% | 0.07% | 0.01% | 0.05% | 100% | 100% | 174.8 | 174.8 |
|  | 12 | 27 | 0.02% | 0.00% | -0.14% | 0.02% | 0.04% | -0.13% | 99% | 100% | 379.6 | 379.6 |
| Overall | -13 | 27 | 0.29% | 0.24% | 0.28% | 0.29% | 0.25% | 0.29% | 99% | 101% |  |  |
| Overall min |  |  | 0.02% | -0.20% | -0.14% | 0.02% | -0.13% | -0.13% |  |  |  |  |
| Overall max |  |  | 0.94% | 1.06% | 1.06% | 0.95% | 1.06% | 1.08% |  |  |  |  |

Conclusion

Considering performance and complexity balance, this contribution recommends exclusive usage between extended precision and 64 point transform as well as extended precision and LFNST.

Reference

[1] K. Kondo, M. Ikeda, “A constraint of max transform size for high bit depth and high bit rate coding,” JVET-V0067

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).