| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0065 |
| --- | --- |

| Title: | EE2: Results of Test 3.4 and Test 3.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Yan Ye, Jie Chen, Xinwei Li (Alibaba) Yao-Jen Chang, Han Huang, Vadim Seregin, Chun-Chi Chen, Marta Karczewicz (Qualcomm) | Email: | {ruling.lrl, yan.ye}@alibaba-inc.com {yjchang, hanhuang, vseregin}@qti.qualcomm.com |
| Source: | Alibaba group, Qualcomm Incorporated |  |  |

_____________________________

- Abstract

This contribution reports the results of the combinations of template matching (TM) and geometric partition mode, which are studied in EE2 Test 3.4 and Test 3.5. In Test 3.4, each motion of geometric partition can be separately decided whether to use TM for refinement. When TM is applied, the template is constructed using one or both of left and above neighboring samples according to partition angle. On the other hand, in Test 3.5, both motions of geometric partitions are jointly decided whether to use TM. The template is constructed using both of the left and above samples when TM is chosen. Besides, a combination of Test 3.4 and Test 3.5 is studied in this contribution. In the combined test, both motions are jointly decided whether to be refined using TM, and the template is derived according to partition angle if TM is chosen. It is reported that on top of ECM-1.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is

- Test 3.4: RA {-0.20%, -0.35%, -0.37%, 104%, 100%}, LDB {-0.20%, -0.61%, -0.41%, 107%, 99%}

- Test 3.5: RA {-0.16%, -0.28%, -0.33%, 103%, 100%}, LDB {-0.18%, -0.47%, -0.20%, 103%, 97%}

- Combined: RA {-0.29%, -0.43%, -0.50%, 104%, 100%}, LDB {-0.34%, -0.80%, -0.80%, 105%, 96%}

Introduction

In ECM-1.0, template matching (TM) which refines the motions at the decoder side is applied to the regular merge mode. In the last JVET meeting, it was proposed to extend the TM to geometric partition mode (GPM)[1][2], and is studied in EE2 Test 3.4 and Test 3.5 [3]. This contribution reports the simulation results of the tests.

Proposed method

Test 3.4: GPM with TM and PU-level signaling

In Test 3.4, when a CU is coded in GPM, two motions for two geometric partitions are selected from a merge candidate list which is derived in the same way as that in VVC standard. Each motion for a geometric partition can decide whether to be refined using TM. When TM is chosen, a template is constructed using left, above or left and above neighboring samples according to partition angle, as shown in Table 1. The motion is then refined by minimizing the difference between the current template and the template in the reference picture using the same search pattern of merge mode with half-pel interpolation filter disabled. For the syntax design, when a CU coded in GPM, two additional flags, followed by the geometric partition mode indices and two merge indices, are signaled to indicate whether the motions are refined in two geometric partitions, respectively.

Table 1. Template for the 1st and 2nd geometric partitions, where A represents using above samples, L represents using left samples, and L+A represents using both left and above samples.

| Partition angle | 0 | 2 | 3 | 4 | 5 | 8 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |
| Partition angle | 16 | 18 | 19 | 20 | 21 | 24 | 27 | 28 | 29 | 30 |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |

Test 3.5: GPM with TM and CU-level signaling

In Test 3.5, when GPM mode is enabled for a CU, a CU-level flag is signaled to indicate whether TM is applied to both two geometric partition. When TM is used, two uni-prediction motion are selected from the GPM candidate list and are both refined using the same template matching refinement process as in the template matching of ECM-1.0. The template matching is searched using left and above neighboring samples.

A GPM candidate list is constructed as follows:

- Interleaved List-0 MV candidates and List-1 MV candidates are derived directly from the regular merge candidate list, where List-0 MV candidates are higher priority than List-1 MV candidates.  A pruning method with an adaptive threshold based on the current CU size is applied to remove redundant MV candidates.

- Interleaved List-1 MV candidates and List-0 MV candidates are further derived directly from the regular merge candidate list, where List-1 MV candidates are higher priority than List-0 MV candidates. The same pruning method with the adaptive threshold is also applied to remove redundant MV candidates.

- Zero MV candidates are padded until the GPM candidate list is full.

Test 3.4 + Test 3.5

A combination of Test 3.4 and Test 3.5 is proposed in this contribution. When a CU is coded in GPM, a CU-level flag is signaled to indicate whether TM is applied to both two partitions or not. When TM is enabled, two uni-prediction motions are selected from the GPM candidate list which is constructed using the method as described in Test 3.5. Then two uni-prediction motions are refined using different templates according to the partition angle as described in Test 3.4.

Simulation results

The described methods are implemented on top ECM-1.0 software and are tested under CTC for enhanced compression tool testing [4]. The test results are summarized in Table 2, and the results for individual test are shown in Table 3, Table 4 and Table 5.

Table 2. Summary of simulation results

|  | Random Access Main 10 |  |  |  |  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test 3.4 | -0.20% | -0.35% | -0.37% | 104% | 100% | -0.20% | -0.61% | -0.41% | 107% | 99% |
| Test 3.5 | -0.16% | -0.28% | -0.33% | 103% | 100% | -0.18% | -0.47% | -0.20% | 103% | 97% |
| Test 3.4 + Test 3.5 | -0.29% | -0.43% | -0.50% | 104% | 100% | -0.34% | -0.80% | -0.80% | 105% | 96% |

Table 3. Simulation results of Test 3.4

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.34% | -0.30% | 103% | 100% |
| Class A2 | -0.24% | -0.29% | -0.32% | 104% | 100% |
| Class B | -0.15% | -0.30% | -0.28% | 104% | 100% |
| Class C | -0.26% | -0.45% | -0.57% | 105% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.35% | -0.37% | 104% | 100% |
| Class D | -0.24% | -0.24% | -0.34% | 105% | 99% |
| Class F | -0.08% | -0.13% | -0.26% | 103% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.40% | -0.22% | 106% | 100% |
| Class C | -0.27% | -0.61% | -0.60% | 108% | 99% |
| Class E | -0.25% | -0.96% | -0.49% | 106% | 99% |
| Overall | -0.20% | -0.61% | -0.41% | 107% | 99% |
| Class D | -0.14% | 0.04% | -0.79% | 107% | 99% |
| Class F | -0.08% | -0.12% | -0.35% | 105% | 100% |

Table 4. Simulation results of Test 3.5

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.27% | -0.43% | 103% | 101% |
| Class A2 | -0.18% | -0.21% | -0.16% | 103% | 100% |
| Class B | -0.14% | -0.21% | -0.22% | 103% | 100% |
| Class C | -0.22% | -0.45% | -0.53% | 102% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.28% | -0.33% | 103% | 100% |
| Class D | -0.19% | -0.27% | -0.23% | 100% | 96% |
| Class F | -0.10% | -0.12% | -0.19% | 100% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.27% | 0.06% | 104% | 98% |
| Class C | -0.21% | -0.45% | -0.28% | 102% | 95% |
| Class E | -0.32% | -0.82% | -0.52% | 102% | 97% |
| Overall | -0.18% | -0.47% | -0.20% | 103% | 97% |
| Class D | -0.06% | -0.60% | -0.99% | 103% | 97% |
| Class F | -0.06% | 0.14% | -0.70% | 101% | 100% |

Table 5. Simulation results of Test 3.4 + Test 3.5

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.31% | -0.45% | 103% | 100% |
| Class A2 | -0.33% | -0.47% | -0.38% | 104% | 100% |
| Class B | -0.24% | -0.37% | -0.42% | 105% | 101% |
| Class C | -0.36% | -0.57% | -0.73% | 105% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.43% | -0.50% | 104% | 100% |
| Class D | -0.27% | -0.33% | -0.34% | 104% | 99% |
| Class F | -0.16% | -0.22% | -0.22% | 103% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -0.46% | -0.63% | 106% | 98% |
| Class C | -0.44% | -1.07% | -0.86% | 104% | 93% |
| Class E | -0.46% | -1.02% | -1.00% | 104% | 97% |
| Overall | -0.34% | -0.80% | -0.80% | 105% | 96% |
| Class D | -0.28% | -0.31% | 0.06% | 104% | 96% |
| Class F | -0.28% | -0.39% | -0.40% | 102% | 99% |

References

- R.-L. Liao, et. al., “EE2-related: Combination of GPM and template matching,” JVET-V0117, April 2021.

- C.-C. Chen, et. al., “EE2-related: Extension of template matching to Affine, CIIP, GPM merge modes, and boundary sub-blocks,” JVET-V0118, April 2021.

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-V2024, April 2021.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-V2017, April 2021.

Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).