| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-0075-v2 |
| --- | --- |

| Title: | Subpicture-wise reference picture resampling and scalability for Game/E-sport contents streaming |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Xiang Li Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

For Game/E-sport contents streaming use cases including,

- Local enhancement of visual quality or resolution

- Customized layout of the played video

- Update of layout within a coded video sequence,

we propose the following modifications of VVC high-level syntax:

- Signaling a scaling window per subpicture, when more than one subpicture is present
: For subpicture-wise reference picture resampling in a layer or across layers, scaling windows need to be signaled in PPS for each subpicture that has a different scaling ratio with the reference subpicture.

- Signaling the updated subpicture partitioning information in PPS
: To enable the picture-level updates of subpicture partitioning information, the subpicture partitioning information needs to be signaled in PPS, optionally. When the subpicture partitioning information is updated, some subpictures with particular subpicture IDs may appear or disappear, and the spatial resolution and the vertical and/or horizontal offset values of each subpicture may be changed.

- Permitting that each layer may have a different subpicture partitioning layout.
: In VVC version 1, all layers belonging to the same dependency tree shall have the same subpicture partitioning layout. In VVC next versions, it is proposed to relax this constraint to support the argued use cases that need different subpicture layouts across layers.

- Permitting the combination of subpicture partitioning and reference picture resampling
: In VVC version 1, the combination of subpicture partitioning and reference picture resampling is not supported. We propose to relax this constraint to support region-wise quality/resolution enhancement.

# Motivation

Versatile video coding (VVC) version 1 has been developed to be widely used for various applications including UHD video streaming and immersive media solution (360, point cloud, 6 degrees-of-freedom). Game/e-sport is another key application of video streaming whose revenue hit more than $1 billion in this year [4]. E-Sport is a new class of live broadcast/real-time streaming events. E-Sport as an online application is a form of competitive video gaming with multiple players battling against each other, usually in teams, often in matches that are streamed live to throngs of young fans. E-Sport as technology is a great opportunity for companies to display their most advanced products and technologies to potential consumers. With the cloud-based game streaming solution (e.g. Tencent’s Start), we expect E-Sport market will be rapidly emerging, because professional gamers can remotely play the match simultaneously and the game play with commenter’s video and audio will be broadcasted to all audiences in real-time.

In JVET, expecting the VVC standard to be successfully used for game industries, the computer-generated (CG) test sequences have been studied, including the most popular mobile game content (a.k.a. Arena of Valor). CG content may have unique characteristics distinguished from the natural images captured by a camera. Through the in-depth studies by JVET experts, several coding tools (e.g. IBC), specialized for those game content sequences, have been developed and adopted. In addition, JVET has developed useful functionalities and system-interfaces, with new designs of high-level syntax. Beyond VVC V1, toward the following addition use cases for game/e-sport contents streaming, we propose several modifications to VVC V1, considering a new (gaming mode) profile. However, the technical scopes for the application are not limited to the suggested topics in this document. More input contributions for game/e-sport applications are encouraged.

# Use cases – 1: Local enhancement of visual quality or resolution

There are many interesting live videos that audiences do not want to miss in the E-Sport matches. For example, images of commentators, players, the scoreboard and the screen of the game. In order to display multi-images on the player’s screen at the same time, multiple windows for picture-in-picture (PIP) provide a great advantage for E-Sport broadcasting. Fig. 2 shows an example image of multiple PIPs in E-Sport broadcasting.

E-Sport broadcasting system may be able to mix or switch multiple video inputs, which include team-players, commentators, and the game video. The main broadcasting video mixing the most interesting input videos to a video sequence will be encoded as the main bitstream and streamed to all remote audiences. While the main bitstream includes popular input videos, the visual quality of each input video may be relatively low, depending on its picture resolution or the bitrate.

Whereas, each input video can be additionally encoded with a higher quality and/or resolution and transmitted to each audience, as a separated bitstream.  An individual video can be dependently encoded by referencing the main video, where the main video includes the individual video as PIP. In this case, the individual video can be encoded as an enhancement layer of a local region of the main video (considered as the base layer), by referencing the decoded pictures in the main video. Otherwise, the enhancement layer may be independently encoded without any decoding dependency to the main video and is transmitted as a simulcast.

Figure 2. Datavideo’s E-Sport broadcasting system [6]

In terms of user’s experience, once a client receives a main video and one or more enhanced videos of local regions, the audience would select one of them.  If the audience chooses the main video, all PIPs within a background picture will be displayed. If the audience chooses a specific PIP, the PIP will be enlarged and displayed with an enhanced visual quality than the decoded corresponding region of the main video. Fig. 3 illustrates the above operations.

Figure 3. Example of the enhanced display of a local region

As a possible technical implementation in VVC, each rectangular region of the main video corresponding to an input video may be encoded as a VVC sub-picture. In this case, the coded bitstream of the main video may consist of multiple sub-pictures. The sub-bitstream corresponding to a sub-picture is extractible from the main bitstream and independently decodable. Each sub-bitstream may be composed of one or more layers with different qualities, or may be referenced by another sub-bitstream including a sub-picture layer with a higher resolution. Fig. 4 illustrates a systematic design of such case, with sub-pictures and multi-layers.

Figure 4. Example of video composition with multi-layered sub-pictures.

# Use cases – 2: Customized layout of played video

A number of players are playing simultaneously in a collaborative and interactive game. At the same time, several audiences are following the game in real-time. Each observer may watch a combination of views from their favorable players, observer views, and/or physical camera views according to a director’s intention [7]. For this use case, a server may generate transmit all interesting layouts of multiple input videos depending on director’s cuts. The layout information is represented by subpicture partitioning information in SPS or additionally delivered as metadata in file format. Once each audience chose one of them by preference, or additionally customized it due to its preference, each local region of the decoded picture would be relocated in the displayed picture depending to the selected layout information. After decoding and reorganizing the main video and one or more enhanced videos, the customized video would be created in cloud. Such a reorganization process for the customization of video may be processed in a cloud server with network-based media processing (NBMP [8]), and the resulting combined video is streamed to a client. The layout information can be generated by a user and can be shared with other users to enjoy the same experience, as well.

# Use cases – 3: Update of layout within a coded video sequence

In the use case -2, each local region can be mapped to a subpicture whose layout is signaled in SPS. In VVC version 1, the subpicture layout can be updated only at the start of the coded video sequence (CVS). It degrades the feature of flexible layout representation for game/esports content composition, because the same layout shall be used within the same CVS. To enable more flexible composition of output images within a CVS, the instant update of subpicture layout is desirable. Fig. 5 illustrates subpicture layout updates within a CVS, which enable composition layout changes without delay or coding gain loss. By signaling the subpicture layout and subpicture ID mapping information in PPS, the subpicture layout can be updated in picture-level. To minimize the scarification of coding efficiency when the subpicture layout is changed, motion compensated prediction is enabled within the same subpicture, with a subpicture-specific scaling window.

Figure 5. Example of video composition with multi-layered sub-pictures.

# Solutions

We suggest the AHG to hold a technical discussion on the useful technologies for the suggested use cases. The proposed changes include the following aspects:

- Signaling a scaling window per subpicture, when more than one subpicture is present
: For subpicture-wise reference picture resampling in a layer or across layers, scaling windows need to be signaled in PPS for each subpicture that has a different scaling ratio with the reference subpicture.

- Signaling the updated subpicture partitioning information in PPS
: To enable the picture-level updates of subpicture partitioning information, the subpicture partitioning information needs to be signaled in PPS, optionally. When the subpicture partitioning information is updated, some subpictures with particular subpicture IDs may appear or disappear, and the spatial resolution and the vertical and/or horizontal offset values of each subpicture may be changed.

- Permitting that each layer may have a different subpicture partitioning layout.
: In VVC version 1, all layers belonging to the same dependency tree shall have the same subpicture partitioning layout. In VVC next versions, it is proposed to relax this constraint to support the argued use cases that need different subpicture layouts across layers.

- Permitting the combination of subpicture partitioning and reference picture resampling
: In VVC version 1, the combination of subpicture partitioning and reference picture resampling is not supported. We propose to relax this constraint to support region-wise quality/resolution enhancement.

# Proposed specification text

##### Picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| pps_subpic_rpr_extension_flag | u(1) |
| pps_extension_7bits | u(7) |
| } |  |
| if( pps_subpic_rpr_extension_flag ) |  |
| pps_ subpic_rpr_extension( ) |  |
| if( pps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

##### Picture parameter set subpicture and reference picture resampling syntax

| pps_ subpic_rpr_extension( ){ | Descriptor |
| --- | --- |
| pps_subpic_update_info_present_flag | u(1) |
| pps_subpic_scaling_win_info_present_flag | u(1) |
| if( pps_subpic_update_info_present_flag ¦¦ pps_subpic_scaling_win_info_present_flag ) |  |
| pps_updated_num_subpics_minus1 | ue(v) |
| if( pps_subpic_update_info_present_flag ) { |  |
| if( pps_updated_num_subpics_minus1 > 0 ) { |  |
| pps_independent_subpics_flag | u(1) |
| pps_subpic_same_size_flag | u(1) |
| } |  |
| for( i = 0; pps_updated_num_subpics_minus1 > 0 && i <= pps_ updated_num_subpics_minus1; i++ ) { |  |
| if( !pps_subpic_same_size_flag ¦ ¦ i = = 0 ) { |  |
| if( i > 0 && pps_pic_width_ in_luma_samples > CtbSizeY ) |  |
| pps_subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pps_pic_height_in_luma_samples > CtbSizeY ) |  |
| pps_subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < pps_updated_num_subpics_minus1 && pps_pic_width_in_luma_samples > CtbSizeY ) |  |
| pps_subpic_width_minus1[ i ] | u(v) |
| if( i < pps_ updated_num_subpics_minus1 && pps_pic_height_in_luma_samples > CtbSizeY ) |  |
| pps_subpic_height_minus1[ i ] | u(v) |
| } |  |
| if( !pps_independent_subpics_flag) { |  |
| pps_subpic_treated_as_pic_flag[ i ] | u(1) |
| pps_loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| pps_updated_subpic_id_len_minus1 | ue(v) |
| pps_ updated_subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( pps_updated_subpic_id_mapping_explicitly_signalled_flag ) { |  |
| pps_updated_subpic_id_mapping_present_flag | u(1) |
| if( pps_updated_subpic_id_mapping_present_flag ) |  |
| for( i = 0; i <= pps_ updated_num_subpics_minus1; i++ ) |  |
| pps_updated_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| pps_subpic_scaling_win_info_present_flag | u(1) |
| if( pps_subpic_scaling_win_info_present_flag ) { |  |
| for( i = 0; i <= pps_ updated_num_subpics_minus1; i++ ) { |  |
| pps_subpic_scaling_win_left_offset[ i ] | se(v) |
| pps_subpic_scaling_win_right_offset[ i ] | se(v) |
| pps_subpic_scaling_win_top_offset[ i ] | se(v) |
| pps_subpic_scaling_win_bottom_offset[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

pps_subpic_rpr_extension_flag equal to 1 specifies that the pps_ subpic_rpr_extension( ) syntax structure is present in the PPS RBSP syntax structure. pps_subpic_rpr_extension_flag equal to 0 specifies that this syntax structure is not present. When not present, the value of pps_subpic_rpr_extension_flag is inferred to be equal to 0.

pps_subpic_update_info_present_flag equal to 1 specifies that the subpicture partitioning layout information is present in PPS. pps_subpic_update_info_present_flag equal to 0 specifies that the subpicture partitioning layout information is not present in PPS.

pps_updated_num_subpics_minus1 plus 1 specifies the number of subpictures of each picture that refers to the PPS. The value of pps_num_subpics_minus1 shall be in the range of 0 to MaxSlicesPerAu − 1, inclusive, where MaxSlicesPerAu is specified in Annex A. When not present, the value of pps_updated_num_subpics_minus1 is inferred to be equal to sps_num_subpics_minus1.

pps_independent_subpics_flag equal to 1 specifies that all subpicture boundaries of each picture that refers to the PPS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. pps_independent_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of pps_independent_subpics_flag is inferred to be equal to 1.

pps_subpic_same_size_flag equal to 1 specifies that all subpictures of each picture that refers to the PPS have the same width specified by pps_subpic_width_minus1[ 0 ] and the same height specified by pps_subpic_height_minus1[ 0 ]. pps_subpic_same_size_flag equal to 0 does not impose such a constraint. When not present, the value of pps_subpic_same_size_flag is inferred to be equal to 0.

Let the variable tmpWidthVal be set equal to ( pps_pic_width_ in_luma_samples + CtbSizeY − 1 ) / CtbSizeY, and the variable tmpHeightVal be set equal to ( pps_pic_height_ in_luma_samples + CtbSizeY − 1 ) / CtbSizeY.

pps_subpic_ctu_top_left_x[ i ] specifies horizontal position of top-left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( tmpWidthVal ) ) bits.

When not present, the value of pps_subpic_ctu_top_left_x[ i ] is inferred as follows:

–	If pps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of pps_subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

–	Otherwise, the value of pps_subpic_ctu_top_left_x[ i ] is inferred to be equal to ( i % numSubpicCols ) * ( pps_subpic_width_minus1[ 0 ] + 1 ).

When pps_subpic_same_size_flag is equal to 1, the variable numSubpicCols, specifying the number of subpicture columns in each picture in the CLVS, is derived as follows:

numSubpicCols = tmpWidthVal / ( pps_subpic_width_minus1[ 0 ] + 1 )	(37)

When pps_subpic_same_size_flag is equal to 1, the value of numSubpicCols * tmpHeightVal / ( pps_subpic_height_minus1[ 0 ] + 1 ) − 1 shall be equal to pps_updated_num_subpics_minus1.

pps_subpic_ctu_top_left_y[ i ] specifies vertical position of top-left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( tmpHeightVal ) ) bits.

When not present, the value of pps_subpic_ctu_top_left_y[ i ] is inferred as follows:

–	If pps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of pps_subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

–	Otherwise, the value of pps_subpic_ctu_top_left_y[ i ] is inferred to be equal to ( i / numSubpicCols )* ( pps_subpic_height_minus1[ 0 ] + 1 ).

pps_subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( tmpWidthVal ) ) bits.

When not present, the value of pps_subpic_width_minus1[ i ] is inferred as follows:

–	If pps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of pps_subpic_width_minus1[ i ] is inferred to be equal to tmpWidthVal − pps_subpic_ctu_top_left_x[ i ] − 1.

–	Otherwise, the value of pps_subpic_width_minus1[ i ] is inferred to be equal to pps_subpic_width_minus1[ 0 ].

When pps_subpic_same_size_flag is equal to 1, the value of tmpWidthVal % ( pps_subpic_width_minus1[ 0 ] + 1 ) shall be equal to 0.

pps_subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( tmpHeightVal ) ) bits.

When not present, the value of pps_subpic_height_minus1[ i ] is inferred as follows:

–	If pps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of pps_subpic_height_minus1[ i ] is inferred to be equal to tmpHeightVal − pps_subpic_ctu_top_left_y[ i ] − 1.

–	Otherwise, the value of pps_subpic_height_minus1[ i ] is inferred to be equal to pps_subpic_height_minus1[ 0 ].

When pps_subpic_same_size_flag is equal to 1, the value of tmpHeightVal % ( pps_subpic_height_minus1[ 0 ] + 1 ) shall be equal to 0.

It is a requirement of bitstream conformance that the shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

For each subpicture with subpicture index i in the range of 0 to pps_num_subpics_minus1, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true:

–	The value of ( pps_subpic_ctu_top_left_x[ i ] * CtbSizeY ) shall be less than ( pps_pic_width_ in_luma_samples − pps_conf_win_right_offset * SubWidthC ).

–	The value of ( ( pps_subpic_ctu_top_left_x[ i ] + pps_subpic_width_minus1[ i ] + 1 ) * CtbSizeY ) shall be greater than ( pps_conf_win_left_offset * SubWidthC ).

–	The value of ( pps_subpic_ctu_top_left_y[ i ] * CtbSizeY ) shall be less than ( pps_pic_height_ in_luma_samples − pps_conf_win_bottom_offset * SubHeightC ).

–	The value of ( ( pps_subpic_ctu_top_left_y[ i ] + pps_subpic_height_minus1[ i ] + 1 ) * CtbSizeY ) shall be greater than ( pps_conf_win_top_offset * SubHeightC ).

pps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture that refers to the PPS is treated as a picture in the decoding process excluding in-loop filtering operations. pps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of pps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

pps_loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations across subpicture boundaries is enabled and might be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. pps_loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations across subpicture boundaries is disabled and are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of pps_loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 0.

pps_updated_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element pps_updated_subpic_id[ i ], the syntax elements pps_updated_subpic_id[ i ], when present, and the syntax element sh_subpic_id, when present. The value of pps_updated_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive. The value of 1  <<  ( pps_updated_subpic_id_len_minus1 + 1 ) shall be greater than or equal to pps_updated_num_subpics_minus1 + 1.

pps_updated_subpic_id_mapping_explicitly_signalled_flag equal to 1 specifies that the subpicture ID mapping is explicitly signalled, in the PPSs referred to by coded pictures of the CLVS. pps_updated_subpic_id_mapping_explicitly_signalled_flag equal to 0 specifies that the subpicture ID mapping is not explicitly signalled for the CLVS. When not present, the value of pps_updated_subpic_id_mapping_explicitly_signalled_flag is inferred to be equal to 0.

pps_updated_subpic_id_mapping_present_flag equal to 1 specifies that the subpicture ID mapping is signalled in the PPS when pps_updated_subpic_id_mapping_explicitly_signalled_flag is equal to 1. pps_updated_subpic_id_mapping_present_flag equal to 0 specifies that subpicture ID mapping is signalled in the PPSs referred to by coded pictures of the CLVS when pps_subpic_id_mapping_explicitly_signalled_flag is equal to 1.

pps_updated_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_updated_subpic_id[ i ] syntax element is pps_updated_subpic_id_len_minus1 + 1 bits.

pps_subpic_scaling_win_info_present_flag equal to 1 specifies that the subpicture-wise scaling window information is present in PPS. pps_subpic_scaling_win_info_present_flag equal to 0 specifies that the subpicture-wise scaling window information is not present in PPS.

pps_subpic_scaling_win_left_offset[ i ], pps_subpic_scaling_win_right_offset[ i ], pps_subpic_scaling_win_top_offset[ i ], and pps_subpic_scaling_win_bottom_offset[ i ] specify the offsets that are applied to the subpicture size of the i-th subpicture for scaling ratio calculation. When not present, the values of pps_subpic_scaling_win_left_offset[ i ], pps_subpic_scaling_win_right_offset[ i ], pps_ subpic_scaling_win_top_offset[ i ], and pps_ subpic_scaling_win_bottom_offset[ i ] are inferred to be equal to pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset, respectively.

#### 7.4.3.4 Sequence parameter set RBSP semantics

sps_subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there might be one or more than one subpicture in each picture of the CLVS. sps_subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS.

When sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_subpic_info_present_flag shall be equal to 0.

NOTE 5 – When a bitstream is the result of a subpicture sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the subpicture sub-bitstream extraction process, it might be required to set the value of sps_subpic_info_present_flag equal to 1 in the RBSP of the SPSs.

8.3.2	Decoding process for reference picture lists construction

…

- When sps_num_subpics_minus1 is greater than 0 and the current subpicture with subpicture index subpicIdx has sps_subpic_treated_as_pic_flag[ subpicIdx ] equal to 1, it is a requirement of bitstream conformance that both of the following conditions shall be true:

  - Exactly one and not both of the following two conditions shall be true:

    - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] and the current picture have the same value for each of the following:

      - pps_pic_width_in_luma_samples

      - pps_pic_height_in_luma_samples

      - sps_num_subpics_minus1

      - sps_log2_ctu_size_minus5

      - sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ], sps_subpic_height_minus1[ i ], and sps_subpic_treated_as_pic_flag[ i ], respectively, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive

      - SubpicIdVal[ subpicIdx ]

    - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP entry for which the value of sps_num_subpics_minus1 is equal to 0.

  - The current picture and any picture that is in a different layer than the current layer and has the current picture in an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] have the same value for each of the following:

    - pps_pic_width_in_luma_samples

    - pps_pic_height_in_luma_samples

    - sps_num_subpics_minus1

    - sps_log2_ctu_size_minus5

    - sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ], sps_subpic_height_minus1[ i ], sps_subpic_treated_as_pic_flag[ i ], SubpicIdVal[ i ], respectively, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] https://start.qq.com/

[2] https://www.inverse.com/article/54194-google-stadia-price-launch-date-features-games-hints

[3] https://news.xbox.com/en-us/2019/03/12/project-xcloud-choice-for-how-and-when-you-play/

[4] https://www.reuters.com/article/us-videogames-outlook/global-esports-revenues-to-top-1-billion-in-2019-report-idUSKCN1Q11XY.

[5] G. J. Sullivan, J.-R. Ohm, “Meeting Report of the 13th JVET Meeting,” JVET-M1000.

[6] https://www.datavideo.com/article/The+Right+Switcher+for+the+Esport+Industry

[7] P. Kolan, et al, “Streaming E-Sports Event,” MPEG M46551

[8] ISO 23090-8 CD, “Coded representation of immersive media (MPEG-I) — Part 8: Network Based Media Processing”, WG11 N18236