| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0093-v1 |
| --- | --- |

| Title: | AHG8: On significance, GT1, and GT2 flag coding for high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document describes a study of the use of significance, GT1 and GT2 flags in regular residual coding for high bit depths at low QP and lossless operating points. An analysis using VTM 13.0 is provided which lists the number of these flags used and the fraction of them that are set for each QP in the low QP high bit depth CTC. This analysis outlines how as QPs decrease, the fraction of the flags that are set increases to almost 100%. A modification is described which effectively shifts the significance, GT1 and GT2 tests left by a number of bits signalled from the encoder. A non-normative algorithm is also described which computes a suitable value for this shift in a pre-encode step. The analysis, previously done for VTM 13.0, is repeated for the modification and indicates that the fraction of flags that are set does not rise as QPs decrease, unlike VTM 13.0.

The following results are reported for the modification against an anchor of VTM 13.0:

| Lossless | AI | LDB | RA |
| --- | --- | --- | --- |
| PQ | -0.36% | -0.41% | -0.41% |
| HLG | -0.77% | -1.13% | -1.13% |
| SVT-12 | -1.10% | -1.33% | -1.34% |
| SVT-16 | -1.12% | -1.45% | -1.46% |

| Low QP | AI |  |  | LDB |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | U/B | V/R | Y/G | U/B | V/R | Y/G | U/B | V/R |
| PQ | 0.05% | 0.05% | 0.06% | 0.03% | 0.03% | 0.04% | 0.03% | 0.04% | 0.04% |
| HLG | -0.08% | -0.10% | -0.10% | 0.02% | 0.03% | 0.03% | 0.01% | 0.03% | 0.03% |
| SVT-12 | -0.25% | -0.23% | -0.25% | -0.09% | -0.06% | -0.07% | -0.10% | -0.05% | -0.06% |
| SVT-16 | -0.87% | -0.89% | -0.89% | -0.87% | -0.80% | -0.80% | -0.85% | -0.77% | -0.77% |

If GolombRiceParameterAdaptation is disabled for both VTM 13.0 and the modification the following results are reported:

| Lossless | AI | LDB | RA |
| --- | --- | --- | --- |
| PQ | -1.00% | -1.06% | -1.00% |
| HLG | -2.74% | -2.65% | -2.51% |
| SVT-12 | -3.16% | -2.91% | -2.90% |
| SVT-16 | -2.81% | -2.41% | -2.42% |

| Low QP | AI |  |  | LDB |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | U/B | V/R | Y/G | U/B | V/R | Y/G | U/B | V/R |
| PQ | 0.02% | 0.04% | 0.05% | 0.03% | 0.03% | 0.03% | 0.02% | 0.02% | 0.03% |
| HLG | -0.30% | -0.16% | -0.17% | -0.01% | 0.01% | 0.01% | -0.01% | 0.01% | 0.01% |
| SVT-12 | -0.82% | -0.55% | -0.56% | -0.38% | -0.34% | -0.35% | -0.38% | -0.28% | -0.29% |
| SVT-16 | -2.33% | -2.08% | -2.06% | -1.78% | -1.82% | -1.82% | -1.76% | -1.69% | -1.69% |

# Introduction

JVET-V0122[1] proposed a bypass mode for coefficient coding which when enabled removed the context coded bin allowance for coefficient coding. When this mode is enabled, the significance, GT1, GT2 and parity flags are not used for both RRC and TSRC. As expected this resulted in losses for the high bit depth CTC[2] for most content over the standard and low QP ranges. However, for SVT-16 content at low QPs, and both HLG and SVT content in the lossless CTC, small gains were reported when comparing results generated using the anchor of the V CE. These results have been confirmed using VTM13.0.

The gains reported by JVET-V0122 indicate that at the affected operating points the coding of coefficient flags offers no benefit. The purpose of this contribution is to report on the investigation of these losses and propose a possible alternative method of coding the coefficient flag which alleviates these losses and offers gains for the affected content.

# Analysis of VTM13.0

By analysing the use of significance, GT1 and GT2 flags at different QPs for both 12 and 16 bit processing it can be seen from Tables 1 and 2 that the ratio of bins that are set increases as QP decreases.

For this sequence (CrowdRun), the ratio of GT1 bins set increases from 52.3% to 96.7%. For each set GT1 bin, two further bins (parity and GT2) are coded. As the total number of coded coefficient bins has a fixed maximum, the increase in GT2 and parity flags reduces the number of coefficients than can be coded with context coded bins. From Table 1, it can be seen that this leads to a reduction in the number of coefficients coded in this way of almost 1/3rd. Each of the coefficients not coded using context-coded bins must then be escape-coded.

The increase in the ratio of set bins is even greater for the GT2 flag, which increases from under 25% at QP12 to over 93% at QP-13. For each of these values, the remainder must be escape coded.

| QP | Coded Bins (millions) |  |  | Set Bins (millions) |  |  | %Set |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 |
| 12 | 49.50 | 43.50 | 22.77 | 42.05 | 22.77 | 5.59 | 85.0% | 52.3% | 24.6% |
| 7 | 42.37 | 40.06 | 28.01 | 38.42 | 28.01 | 12.04 | 90.7% | 69.9% | 43.0% |
| 2 | 38.04 | 37.71 | 31.17 | 35.88 | 31.17 | 19.76 | 94.3% | 82.7% | 63.4% |
| -3 | 35.88 | 36.50 | 32.54 | 34.47 | 32.54 | 24.85 | 96.1% | 89.1% | 76.4% |
| -8 | 34.53 | 35.78 | 33.62 | 33.75 | 33.62 | 29.25 | 97.7% | 94.0% | 87.0% |
| -13 | 33.29 | 35.27 | 34.10 | 32.85 | 34.10 | 31.79 | 98.7% | 96.7% | 93.2% |

Table 1. Analysis of bin usage for CrowdRun (12-bit AI) for VTM 13.0

For 16-bit processing (Table 2), the results continue the trend seen for 12-bit until virtually all significance, GT1 and GT2 context coded bins are set. The results shown in Table 1 and 2 are summarized in Figure 1.

| QP | Coded Bins (millions) |  |  | Set Bins (millions) |  |  | %Set |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 |
| -8 | 34.31 | 35.78 | 33.55 | 33.49 | 33.55 | 29.01 | 97.6% | 93.8% | 86.5% |
| -13 | 33.76 | 35.43 | 34.18 | 33.27 | 34.18 | 31.68 | 98.6% | 96.5% | 92.7% |
| -18 | 33.46 | 35.27 | 34.54 | 33.16 | 34.54 | 33.17 | 99.1% | 97.9% | 96.0% |
| -23 | 32.61 | 34.64 | 34.20 | 32.43 | 34.20 | 33.45 | 99.4% | 98.7% | 97.8% |
| -28 | 31.83 | 34.37 | 34.08 | 31.71 | 34.08 | 33.67 | 99.6% | 99.2% | 98.8% |
| -33 | 31.30 | 34.37 | 34.18 | 31.23 | 34.18 | 33.93 | 99.8% | 99.5% | 99.3% |

Table 2. Analysis of bin usage for CrowdRun (16-bit AI) for VTM 13.0

Figure 1. Analysis of bin usage for CrowdRun (12 and 16 bit AI) for VTM 13.0

From these results it is clear that as QPs decrease the number of coefficient values with the GT2 flag set, and thus requiring further Rice coding, increases to nearly 100%. The corollary of this is that the number of coefficient values that can be completely coded using context coded bins reduces to near zero.

It is suggested that at the QPs used in the low QP range of the CTC, the individual bins are coded with increased efficiency as the QP is reduced, but the efficiency of the coding of the actual coefficient values decreases as more and more values need to be escape coded. Thus as QPs decrease the information about the coefficient values provided by the significance, GT1 and GT2 flags also decreases.

# Proposal

The aim of this proposal is to improve the coding of coefficients using context-coded bins in RRC for low (and negative) QPs for high bit depths. In addition this improvement should apply to high bit depth lossless coding in VVC which at present gives only small gains over HEVC.

The chosen technique attempts to reduce the percentage of significance, GT1 and GT2 flags that are set, more closely matching the percentages reported at higher QPs. By making better use of these flags this technique should allow more coefficients to be coded without requiring Golomb-Rice coding.

The effect of the technique is to shift left the significance, GT1, GT2 and parity tests used in regular residual coding by n bits as shown in in Figure 2 and Table 3, with a shift of 0 being equivalent to VVC v1.

Figure 2. Significance test shifted left by n bits.

| Shift | Flag |  |  |  |
| --- | --- | --- | --- | --- |
|  | Sig Map | GT1 | GT2 | Parity |
| 0 | value ≥ 1 | value ≥ 2 | value ≥ 4 | (value & 1) ≠ 0 |
| 1 | value ≥ 2 | value ≥ 4 | value ≥ 8 | (value & 2) ≠ 0 |
| 2 | value ≥ 4 | value ≥ 8 | value ≥ 16 | (value & 4) ≠ 0 |
| .. |  |  |  |  |
| n | value ≥ 2n | value ≥ 2n+1 | value ≥ 2n+2 | (value & 2n) ≠ 0 |

Table 3 Tests for flags with shift left of n bits applied

Thus every shift left of a bit doubles the thresholds for the tests which causes a reduction the number of coefficients passing each test. Using a suitably chosen value for n, the percentages of coefficients passing these tests at low QPs can be made similar to those for higher QPs.

Rather than actually changing the sub-block coding of residuals and the significance, GT1, GT2 and parity tests used within them, the proposal is implemented by introducing a shift left of n bits of all coded values at the conclusion of residual coding for a block. The sub-block coded value thus becomes the MSBs of the full coefficient value. Once all sub-blocks are coded, the lowest n bits of the value of each coefficient in the block are then coded using EP-bins. An additional EP-bin is used to code the sign bit for coefficients with non-zero LSBs and an MSB of zero.

If n is non-zero, before any sub-block or the last x-y position is coded, a context coded bin is used to signal if this is significant block. If set this flag indicates that at least one coefficient exists in the block which has a non-zero MSB and thus sub-block coding is required. If the flag is not set this indicates that all MSBs are zero, and coefficient values can be derived from the LSBs and sign bits only. In this case, the coding of the last x-y position and the sub-blocks is skipped. In the proposal, the context coded bin for the significant block flag is counted in the number of context coded bins for the block. Therefore there is no increase in the maximum number of context-coded bins for residual coding.

An additional advantage of the proposal is the shift left of the parity flag. At the lowest QPs for high bit depths using the existing coding scheme, this flag is effectively coding noise, as the lowest bit typically does not contain structural information. This can be seen by analyzing the efficiency of parity flag coding in VTM13.0 which typically uses more than one bit to code the parity flag at low QPs. With the modification in the proposal, the parity flag has been found to require less than one bit, making context coding worthwhile.

# Determination of the value of n

The value of n, the size of the shift left of the significance test, is signalled in the slice header. Separate shift values are used for luma and chroma values. The shift values are constant for the slice and not varied in the decoder.

For some applications the values of n could be set using domain-specific rules, but for this proposal, an algorithm has been developed to determine these values which uses an estimate of the residual calculated in a pre-encode phase. This allows the shift values to be signalled for the frame being currently being processed rather than the following frame unlike other approaches[3][4] which are embedded in the actual encoder. One advantage of using this pre-encode step is that it may be applied to still pictures and handles events such as scene changes better than techniques which signal a value based on a previous frame.

The first phase of the algorithm computes an estimate of the residual for each pixel which is defined as the minimum absolute difference between the actual pixel value for the channel and two estimates of the pixel value. These two estimates are the values of the pixels immediately to the left and immediately above.

Once each residual value is computed this is used to choose one of 96 (16 bit content) or 72 (12 bit content) histogram bins using the following formula, and the number of entries in that bin is incremented:

bin = round(log2(residual) * 6)

If the value of the residual is 0, the 0th bin is automatically chosen.

Once this bin is calculated, a QP adjustment may be computed using the bit depth content and the slice QP as follows:

baseQP = 4 – (bitdepth -8) * 6

adjustment = sliceQP – baseQP

adjustedBin = max(0, bin - adjustment)

The effect of this change can be seen as a histogram and a cumulative frequency in Figure 3.

Figure 3. Histogram and cumulative frequency for CrowdRun (16 bit AI) with and without QP adjustment.

Note: for chroma, the histogram and cumulative frequency contain entries from both chroma channels.

From the cumulative frequency the value of n is computed using three percentiles of the cumulative frequency as shown in Figure 4.

Figure 4. Computation of n from the cumulative frequency

The algorithm for computing the value of n from the histogram is given below:

targetB = (total * 3) / 8;   // total is number of entries in histogram

targetA = targetB - total / 8;

targetC = targetB + total / 8;

a = -1;

b = -1;

current = 0;

for (i = 0; i < 96; i++)  {

current += hist[i] // hist is the histogram computed for the slice/frame

if (current >= targetA && a == -1)    { a = i } // reached intercept a

if (current >= targetB && b == -1)    { b = i } // reached intercept b

if (current >= targetC) {                                   // reached intercept c

c = i

n = (((a + 2 * b + c) / 4) / 6)

break

}

}

As the determination of n is non-normative this algorithm is just one example method of determining n.

# Signalling the value of n

At present the two values of n (one each for luma and chroma) are signalled in the slice header as follows in clause 7.3.7 of the VVC Operation Range Extensions[5] (change shown in yellow) :

| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| --- | --- |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if( !sh_ts_residual_coding_disabled_flag && sps_ts_residual_coding_rice_present_in_sh_flag ) |  |
| sh_ts_residual_coding_rice_idx_minus1 | u(3) |
| slice_flagshift_luma | u(4) |
| slice_flagshift_chroma | u(4) |
| if( pps_slice_header_extension_present_flag ) { |  |
| sh_slice_header_extension_length | ue(v) |

In future these additions should be moved into the slice header extension and renamed.

# Modification to the level of baseLevel

The size of the shift left of the significance test, can be used as a guide to the size of coefficients to be coded, and this may be used to modify the baseLevel used in clause 9.3.3.11 of the VVC Operation Range Extensions[5] in the calculation of the Rice parameter (change shown in yellow):

The variable baseLevel is derived as follows:

- If sps_rrc_rice_extension_flag is equal to 0, baseLevel is set equal to 4.

- Otherwise (sps_rrc_rice_extension_flag is equal to 1), the following applies:

baseLevel = ( BitDepth > 12 ) ? ( ( sh_slice_type  = =  I ) ? 1 : 2 ) : ( ( sh_slice_type  = =  I ) ? 2 : 3 )	(X)

baseLevel += (n + 1) >> 1

The rice parameter cRiceParam is derived as follows:

Where n is slice_flagshift_luma for luma channels and slice_flagshift_chroma for chroma channels.

The results section contains separate tables with and without this modification of baseLevel.

# Analysis of Proposal

The aim of this proposal is to improve the coding of coefficient values, and it can be seen that for this sequence (CrowdRun), the technique leads to gains for both 12 and 16 bit processing at all QPs where a shift is selected by the algorithm described above.

| QP | Shift n |  |  | Reference |  |  |  | Tested |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma/G | Chroma/B+R | kbps | G psnr | B psnr | R psnr | kbps | G psnr | B psnr | R psnr |
| -13 | 3 | 4 | 298058.67 | 75.34 | 74.77 | 74.76 | 295841.22 | 75.41 | 74.79 | 74.78 |
| -8 | 2 | 3 | 263499.91 | 70.50 | 70.01 | 70.00 | 262270.71 | 70.53 | 70.01 | 70.00 |
| -3 | 2 | 2 | 230487.68 | 65.78 | 65.04 | 65.03 | 229709.94 | 65.79 | 65.03 | 65.02 |
| 2 | 1 | 1 | 203669.13 | 61.79 | 61.03 | 61.02 | 203834.53 | 61.81 | 61.04 | 61.04 |
| 7 | 0 | 0 | 170526.38 | 56.86 | 56.03 | 56.02 | 170526.43 | 56.86 | 56.03 | 56.02 |
| 12 | 0 | 0 | 137664.09 | 51.91 | 51.10 | 51.10 | 137664.14 | 51.91 | 51.10 | 51.10 |

Table 4. Analysis of result for proposal (CrowdRun 12bit)

| QP | Shift n |  |  | Reference |  |  |  | Tested |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma/G | Chroma/B+R | kbps | G psnr | B psnr | R psnr | kbps | G psnr | B psnr | R psnr |
| -33 | 7 | 7 | 429790.41 | 95.51 | 94.99 | 94.97 | 425539.35 | 95.49 | 94.97 | 94.95 |
| -28 | 6 | 6 | 395775.45 | 90.89 | 90.05 | 90.02 | 392050.17 | 90.88 | 90.03 | 90.01 |
| -23 | 5 | 5 | 362083.39 | 85.87 | 85.13 | 85.11 | 358784.82 | 85.86 | 85.10 | 85.09 |
| -18 | 4 | 4 | 328822.80 | 81.02 | 80.04 | 80.03 | 326129.84 | 80.99 | 80.03 | 80.01 |
| -13 | 3 | 4 | 296391.52 | 76.01 | 75.08 | 75.07 | 293415.19 | 75.97 | 75.04 | 75.03 |
| -8 | 2 | 3 | 264099.11 | 71.09 | 70.12 | 70.10 | 261690.38 | 71.04 | 70.07 | 70.05 |

Table 5. Analysis of result for proposal (CrowdRun 16bit)

If the bin usage is studied in the same way as for VTM13.0 as shown in Table 6 and 7, the number of bins of each type (significance, GT1 and GT2) used and the percentage they are set is kept within bounds throughout the range of QPs. This is shown more clearly in Figure 5. In addition, the number of coefficients coded with context coded bins does not drop as QP rates reduce, unlike VTM 13.0. It is suggested that this implies the coefficients are coded more efficiently, even if the context coded bins themselves require more bits. The savings are in the reduced amount of Rice coding required.

| QP | Coded Bins (millions) |  |  | Set Bins (millions) |  |  | %Set |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 |
| 12 | 49.50 | 43.50 | 22.77 | 42.05 | 22.77 | 5.59 | 85.0% | 52.3% | 24.6% |
| 7 | 42.37 | 40.06 | 28.01 | 38.42 | 28.01 | 12.04 | 90.7% | 69.9% | 43.0% |
| 2 | 46.78 | 39.24 | 25.23 | 37.48 | 25.23 | 10.33 | 80.1% | 64.3% | 40.9% |
| -3 | 51.90 | 36.27 | 22.41 | 36.27 | 22.41 | 7.73 | 69.9% | 61.8% | 34.5% |
| -8 | 51.00 | 37.80 | 23.06 | 35.72 | 23.06 | 8.79 | 70.0% | 61.0% | 38.1% |
| -13 | 51.44 | 37.28 | 22.10 | 34.68 | 22.10 | 8.10 | 67.4% | 59.3% | 36.7% |

Table 3. Analysis of bin usage for CrowdRun (12-bit AI) for proposal

| QP | Coded Bins (millions) |  |  | Set Bins (millions) |  |  | %Set |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 | Sig Map | GT1 | GT2 |
| -8 | 52.03 | 37.73 | 22.83 | 35.87 | 22.83 | 8.89 | 68.9% | 60.5% | 39.0% |
| -13 | 54.06 | 37.22 | 21.77 | 35.37 | 21.77 | 8.22 | 65.4% | 58.5% | 37.8% |
| -18 | 47.67 | 37.22 | 25.28 | 35.24 | 25.28 | 11.46 | 73.9% | 67.9% | 45.3% |
| -23 | 49.18 | 37.12 | 24.18 | 34.98 | 24.18 | 10.19 | 71.1% | 65.2% | 42.1% |
| -28 | 49.81 | 36.77 | 22.82 | 34.11 | 22.82 | 8.83 | 68.5% | 62.1% | 38.7% |
| -33 | 50.40 | 36.49 | 21.67 | 33.41 | 21.67 | 7.77 | 66.3% | 59.4% | 35.9% |

Table 4. Analysis of bin usage for CrowdRun (16-bit AI) for proposal

Figure 5. Analysis of bin usage for CrowdRun (12 and 16 bit AI) for proposal

# Results

## Compared with VTM 13.0

### With baseLevel modification

This section reports results for the proposal against an anchor of VTM13.0 with the baseLevel modification described in section 6.

Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| PQ444 | 2.6 | 2.6 | -0.33% | 3.1 | 3.1 | -0.36% | 3.1 | 3.1 | -0.36% |
| PQ422 | 2.4 | 2.4 | -0.38% | 2.9 | 2.9 | -0.45% | 2.9 | 2.9 | -0.46% |
| Overall | 2.5 | 2.5 | -0.36% | 3.0 | 3.0 | -0.41% | 3.0 | 3.0 | -0.41% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 99% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| HLG444 | 1.8 | 1.8 | -0.77% | 2.0 | 2.0 | -1.04% | 2.0 | 2.0 | -1.04% |
| HLG422 | 1.7 | 1.7 | -0.77% | 1.9 | 2.0 | -1.22% | 1.9 | 2.0 | -1.22% |
| Overall | 1.7 | 1.8 | -0.77% | 1.9 | 2.0 | -1.13% | 1.9 | 2.0 | -1.13% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| SVT16 | 1.2 | 1.2 | -1.10% | 1.2 | 1.2 | -1.33% | 1.2 | 1.2 | -1.34% |
| SVT12 | 1.3 | 1.3 | -1.12% | 1.3 | 1.3 | -1.45% | 1.3 | 1.3 | -1.46% |
| Overall | 1.2 | 1.3 | -1.11% | 1.3 | 1.3 | -1.39% | 1.3 | 1.3 | -1.40% |
| Enc Time[%] | 104% |  |  | 100% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 103% |  |  |

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 99% |
| PQ422 | 0.04% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 99% |
| Overall | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 101% | 98% |
| PQ422 | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 102% | 99% |
| Overall | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% | 0.04% | 101% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 102% | 100% |
| PQ422 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 100% |
| Overall | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 100% |
| Overall PQ | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 102% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.06% | -0.08% | -0.08% | 103% | 100% |
| HLG422 | -0.10% | -0.11% | -0.12% | 103% | 99% |
| Overall | -0.08% | -0.10% | -0.10% | 103% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.03% | 102% | 101% |
| HLG422 | 0.02% | 0.03% | 0.03% | 102% | 101% |
| Overall | 0.02% | 0.03% | 0.03% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.02% | 102% | 100% |
| HLG422 | 0.02% | 0.03% | 0.03% | 102% | 100% |
| Overall | 0.01% | 0.03% | 0.03% | 102% | 100% |
| Overall HLG | -0.02% | -0.01% | -0.01% | 102% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.87% | -0.89% | -0.89% | 104% | 100% |
| SVT12 | -0.25% | -0.23% | -0.25% | 104% | 102% |
| Overall | -0.56% | -0.56% | -0.57% | 104% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.87% | -0.80% | -0.80% | 106% | 106% |
| SVT12 | -0.09% | -0.06% | -0.07% | 104% | 103% |
| Overall | -0.48% | -0.43% | -0.44% | 105% | 104% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.85% | -0.77% | -0.77% | 105% | 106% |
| SVT12 | -0.10% | -0.05% | -0.06% | 102% | 102% |
| Overall | -0.48% | -0.41% | -0.41% | 104% | 104% |
| Overall RGB | -0.51% | -0.47% | -0.47% | 104% | 103% |

Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 103% | 100% |
| Class H2 |  |  |  |  |  | 0.04% | 0.05% | 0.05% | 102% | 100% |
| Overall | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 102% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 102% |

The losses reported in this section for RA are entirely down to the cost of signalling the shift values in the slice header. An improved signalling mechanism could eradicate this losses.

### Without baseLevel modification

This section reports results for the proposal against an anchor of VTM13.0 without the baseLevel modification described in section 6.

Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| PQ444 | 2.6 | 2.6 | -0.27% | 3.1 | 3.1 | -0.34% | 3.1 | 3.1 | -0.33% |
| PQ422 | 2.4 | 2.4 | -0.31% | 2.9 | 2.9 | -0.45% | 2.9 | 2.9 | -0.45% |
| Overall | 2.5 | 2.5 | -0.29% | 3.0 | 3.0 | -0.39% | 3.0 | 3.0 | -0.39% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 99% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| HLG444 | 1.8 | 1.8 | -0.67% | 2.0 | 2.0 | -1.01% | 2.0 | 2.0 | -1.01% |
| HLG422 | 1.7 | 1.7 | -0.69% | 1.9 | 2.0 | -1.21% | 1.9 | 2.0 | -1.21% |
| Overall | 1.7 | 1.7 | -0.68% | 1.9 | 2.0 | -1.11% | 1.9 | 2.0 | -1.11% |
| Enc Time[%] | 98% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  | 99% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| SVT16 | 1.2 | 1.2 | -0.96% | 1.2 | 1.2 | -1.31% | 1.2 | 1.2 | -1.33% |
| SVT12 | 1.3 | 1.3 | -1.06% | 1.3 | 1.3 | -1.45% | 1.3 | 1.3 | -1.46% |
| Overall | 1.2 | 1.3 | -1.01% | 1.3 | 1.3 | -1.38% | 1.3 | 1.3 | -1.39% |
| Enc Time[%] | 103% |  |  | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  | 104% |  |  |

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.05% | 0.06% | 0.06% | 0.04% | 0.06% | 0.06% | 101% | 101% |
| PQ422 | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 101% |
| Overall | 0.05% | 0.05% | 0.06% | 0.04% | 0.05% | 0.06% | 102% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 101% |
| PQ422 | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 101% | 99% |
| Overall | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% | 0.04% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 102% | 101% |
| PQ422 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 101% | 100% |
| Overall | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 101% |
| Overall PQ | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 102% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.06% | -0.06% | 102% | 101% |
| HLG422 | -0.08% | -0.09% | -0.10% | 104% | 103% |
| Overall | -0.06% | -0.08% | -0.08% | 103% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.02% | 0.03% | 102% | 101% |
| HLG422 | 0.02% | 0.03% | 0.03% | 101% | 100% |
| Overall | 0.02% | 0.03% | 0.03% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.02% | 102% | 102% |
| HLG422 | 0.02% | 0.03% | 0.04% | 101% | 100% |
| Overall | 0.01% | 0.03% | 0.03% | 102% | 101% |
| Overall HLG | -0.01% | -0.01% | -0.01% | 102% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.63% | -0.62% | -0.62% | 105% | 101% |
| SVT12 | -0.20% | -0.14% | -0.15% | 103% | 101% |
| Overall | -0.41% | -0.38% | -0.39% | 104% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.75% | -0.68% | -0.69% | 106% | 108% |
| SVT12 | -0.06% | -0.03% | -0.04% | 104% | 106% |
| Overall | -0.41% | -0.36% | -0.36% | 105% | 107% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.73% | -0.65% | -0.65% | 106% | 108% |
| SVT12 | -0.08% | -0.02% | -0.03% | 103% | 104% |
| Overall | -0.41% | -0.33% | -0.34% | 105% | 106% |
| Overall RGB | -0.41% | -0.36% | -0.36% | 105% | 105% |

Standard QP

As the shift mechanism is never used in this QP range, the relative results are identical to those in section 8.1.1.

## With GolombRiceParameterAdaptation set to 0

### With baseLevel modification

This section reports results for the proposal against an anchor of VTM13.0 with the baseLevel modification described in section 6. Both the test results and the anchor results have been generated with the “history” component of the RRC rice modification[6] disabled using GolombRiceParameterAdaptation=0.

This section reports results for the proposal against an anchor of VTM13.0 without the baseLevel modification described in section 6.

Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| PQ444 | 2.5 | 2.6 | -0.88% | 3.0 | 3.1 | -1.00% | 3.1 | 3.1 | -0.95% |
| PQ422 | 2.3 | 2.4 | -1.12% | 2.8 | 2.9 | -1.12% | 2.8 | 2.9 | -1.06% |
| Overall | 2.4 | 2.5 | -1.00% | 2.9 | 3.0 | -1.06% | 2.9 | 3.0 | -1.00% |
| Enc Time[%] | 103% |  |  | 116% |  |  | 122% |  |  |
| Dec Time[%] | 111% |  |  | 105% |  |  | 108% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| HLG444 | 1.7 | 1.8 | -2.77% | 1.9 | 2.0 | -2.52% | 1.9 | 2.0 | -2.38% |
| HLG422 | 1.7 | 1.7 | -2.71% | 1.9 | 2.0 | -2.78% | 1.9 | 2.0 | -2.64% |
| Overall | 1.7 | 1.7 | -2.74% | 1.9 | 2.0 | -2.65% | 1.9 | 2.0 | -2.51% |
| Enc Time[%] | 92% |  |  | 179% |  |  | 189% |  |  |
| Dec Time[%] | 134% |  |  | 126% |  |  | 124% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  | VTM13.0 | RRCShift |  |
| SVT16 | 1.2 | 1.2 | -2.81% | 1.2 | 1.2 | -2.41% | 1.2 | 1.2 | -2.42% |
| SVT12 | 1.3 | 1.3 | -3.16% | 1.3 | 1.3 | -2.91% | 1.3 | 1.3 | -2.90% |
| Overall | 1.2 | 1.3 | -2.98% | 1.2 | 1.3 | -2.66% | 1.3 | 1.3 | -2.66% |
| Enc Time[%] | 112% |  |  | 201% |  |  | 195% |  |  |
| Dec Time[%] | 138% |  |  | 119% |  |  | 118% |  |  |

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0_NoHist |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.04% | 0.05% | 0.01% | 0.04% | 0.05% | 102% | 99% |
| PQ422 | 0.02% | 0.04% | 0.05% | 0.01% | 0.04% | 0.05% | 101% | 98% |
| Overall | 0.02% | 0.04% | 0.05% | 0.01% | 0.04% | 0.05% | 102% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0_NoHist |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 99% |
| PQ422 | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% | 0.04% | 102% | 99% |
| Overall | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 102% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0_NoHist |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.02% | 0.03% | 0.02% | 0.02% | 0.03% | 101% | 99% |
| PQ422 | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 101% | 98% |
| Overall | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 101% | 99% |
| Overall PQ | 0.02% | 0.03% | 0.04% | 0.02% | 0.03% | 0.04% | 101% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.28% | -0.14% | -0.14% | 103% | 101% |
| HLG422 | -0.33% | -0.19% | -0.19% | 103% | 100% |
| Overall | -0.30% | -0.16% | -0.17% | 103% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.01% | 0.00% | 102% | 100% |
| HLG422 | 0.00% | 0.01% | 0.01% | 102% | 99% |
| Overall | -0.01% | 0.01% | 0.01% | 102% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.01% | 0.01% | 103% | 101% |
| HLG422 | -0.01% | 0.01% | 0.01% | 103% | 101% |
| Overall | -0.01% | 0.01% | 0.01% | 103% | 101% |
| Overall HLG | -0.11% | -0.05% | -0.05% | 103% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.33% | -2.08% | -2.06% | 111% | 121% |
| SVT12 | -0.82% | -0.55% | -0.56% | 110% | 113% |
| Overall | -1.58% | -1.31% | -1.31% | 111% | 117% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.78% | -1.82% | -1.82% | 122% | 121% |
| SVT12 | -0.38% | -0.34% | -0.35% | 110% | 109% |
| Overall | -1.08% | -1.08% | -1.09% | 116% | 115% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.76% | -1.69% | -1.69% | 121% | 118% |
| SVT12 | -0.38% | -0.28% | -0.29% | 110% | 109% |
| Overall | -1.07% | -0.99% | -0.99% | 115% | 113% |
| Overall RGB | -1.24% | -1.13% | -1.13% | 114% | 115% |

Standard QP

As the shift mechanism is never used in this QP range, the relative results are identical to those in section 8.1.1.

### Without baseLevel modification

This section reports results for the proposal against an anchor of VTM13.0 without the baseLevel modification described in section 6. Both the test results and the anchor results have been generated with the “history” component of the RRC rice modification[6] disabled using GolombRiceParameterAdaptation=0.

This section reports results for the proposal against an anchor of VTM13.0 without the baseLevel modification described in section 6.

Lossless

| PQ | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  |
| PQ444 | 2.5 | 2.6 | -0.86% |
| PQ422 | 2.3 | 2.4 | -1.12% |
| Overall | 2.4 | 2.5 | -0.99% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 116% |  |  |
| HLG | All Intra |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  |
| HLG444 | 1.7 | 1.8 | -2.75% |
| HLG422 | 1.7 | 1.7 | -2.69% |
| Overall | 1.7 | 1.7 | -2.72% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 139% |  |  |
| SVT | All Intra |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM13.0 | RRCShift |  |
| SVT16 | 1.2 | 1.2 | -2.76% |
| SVT12 | 1.3 | 1.3 | -3.14% |
| Overall | 1.2 | 1.3 | -2.95% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 140% |  |  |

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0_NoHist |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.04% | 0.05% | 0.01% | 0.04% | 0.05% | 102% | 100% |
| PQ422 | 0.02% | 0.04% | 0.05% | 0.01% | 0.04% | 0.05% | 101% | 98% |
| Overall | 0.02% | 0.04% | 0.05% | 0.01% | 0.04% | 0.05% | 102% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.28% | -0.13% | -0.13% | 104% | 102% |
| HLG422 | -0.32% | -0.18% | -0.18% | 103% | 101% |
| Overall | -0.30% | -0.15% | -0.16% | 103% | 102% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0_NoHist |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.24% | -1.92% | -1.89% | 113% | 125% |
| SVT12 | -0.79% | -0.47% | -0.49% | 110% | 113% |
| Overall | -1.52% | -1.20% | -1.19% | 112% | 119% |

Standard QP

As the shift mechanism is never used in this QP range, the relative results are identical to those in section 8.1.1.

# Conclusions

This document has described a modification to the coding of residual data in which the significance, GT1, and GT2 tests are effectively shifted left by a number of bits. A non-normative algorithm has also been described which computes a suitable value for this shift in a pre-encode step. An analysis of the modification has been provided which indicates how the number of significance, GT1 and GT2 flags that are set is reduced when compared with VTM 13.0 .This increases the efficiency of coefficient coding as the flags though less efficiently coded in themselves, give more information about the coefficient value. The reduction in the fraction of set bins also leads to a decrease in the number of coefficients which require Golomb-Rice coding.

Results are reported which show gains for lossless coding for all classes of content in the high bit depth CTC, and for lossy SVT 12 and 16 bit content in the low QP range of this CTC. If the history-based Golomb-Rice parameter adaptation mechanism is disabled in both VTM 13.0 and the modification, these gains are increased to the range of 2-3% for both lossy SVT-16 coding, and lossless HLG and SVT coding.

It is suggested that the modification is considered for adoption into the high bit depth extensions for version 2 of VVC.

# Patent rights declaration(s)

Sony Group Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions”, JVET-V0122, F, Wang, Z. Xie, Y. Yu, H. Yu, D. Wang

[2] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu

[3] “AHG8: on coding of last significant coefficient position for high bit depth and high bit rate extensions”, JVET-V0121, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang

[4] “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-V0054, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang

[5] “VVC operation range extensions (Draft 3)”, JVET-V2005, F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, Y.-K. Wang

[6] “CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths”, JVET-V0106, D. Rusanovskyy, T. Hsieh, L. Pham Van, M. Coban, M. Karczewicz