| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0095-v1 |
| --- | --- |

| Title: | AHG9: Bit-accurate grain blending process for film grain characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen Dolby Laboratories, Inc. Edouard François, Miloš Radosavljević InterDigital Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam Ittiam Systems | Email: | {sean.mccarthy, pyin}@dolby.com {edouard.francois,milos.radosavljevic}@interdigital.com {vijayakumar.gr, kaustubh.patankar}@ittiam.com |
| Source: | Dolby Laboratories, Inc. InterDigital Ittiam Systems |  |  |

_____________________________

# Abstract

This contribution proposes to add semantics to specify a bit-accurate grain blending process to the film grain characteristics (FGC) SEI message in the next version of VSEI (ITU-T H.274 | ISO/IEC 23002-7). No change in syntax is proposed.

The proposed bit-accurate grain blending process is identical to the process specified in SMPTE RDD 5, Film Grain Technology, except for two modifications: 1) use of a transform already specified in VVC instead of the tramsform specified in SMPTE RDD 5; and 2) increase in grain block size for resolutions greater than 1920x1080.

# Problem Statement

As currently specified, the FGC SEI message does not provide a means by which an implementation of a grain blending process could be known to produce numerically identical output pictures compared to another implementation of a grain blending process.

SMPTE RDD 5 [1] specifies a bit-accurate grain blending process. However, SMPTE RDD 5 restricts grain block size to 8x8, which is not implementation friendly for resolutions above 1920x1080.

# Proposal

The proposed bit-accurate grain blending process addresses the implementation and quality problems associated with fixed grain block size for resolutions greater than 1920x1080.

The proposed bit-accurate grain blending process is identical to the process specified in SMPTE RDD 5, Film Grain Technology, except for two modifications:

- The transformation process as specified in VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4, with trType inferred to equal 0) is used [2] instead of the transform specified in SMPTE RDD 5. The VVC transform is more mature than the RDD 5 transform. The VVC transform may also be already available on decoders and may thus be considered implementation friendly.

- For picture resolutions greater than 1920x1080, the film grain block size is increased proportionally from the fixed 8x8-size in RDD 5 to 16x16 for resolutions up to 3840x2160 (4K) and 32x32 for resolutions greater than 4K including 8K. Use of larger block sizes for larger resolutions reduces implementation complexity and makes the grain pattern block size proportionally consistent across 2k, 4k, and 8k resolutions.

As is the case for SMPTE RDD5, bitstreams conforming to the bit-accurate grain blending process shall obey the following constraints:

–	ChromaFormatIdc shall equal 1

–	fg_model_id equal to 0.

–	fg_separate_colour_description_present_flag equal to 0.

–	fg_blending_mode_id equal to 0.

–	fg_log2_scale_factor in the range 2 to 7, inclusive.

–	fg_intensity_interval_upper_bound[ c ][ i ] less than fg_intensity_interval_lower_bound[ c ][ i + 1 ].

–	fg_num_model_values_minus1[ c ] in the range 0..2, inclusive.

–	fg_comp_model_value[ c ][ i ][ 0 ] in the range 0..255, inclusive.

–	fg_comp_model_value[ c ][ i ][ 1 ] in the range 2..14, inclusive.

–	fg_comp_model_value[ c ][ i ][ 2 ] in the range 2..14, inclusive.

Also as is the case for SMPTE RDD5, the bit-accurate grain blending process requires defintion of a picture order count, which we propose to denote as PicOrderCnt and set equal to the value of PicOrderCntVal as specified in VVC.

# References

- “RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier, “AHG9: Film grain estimation and film grain synthesis for VVC – Film grain characteristics SEI message characteristics, film grain estimation and film grain synthesis modules”, JVET-V0093, Teleconference, 20-28 April, 2021.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).