### Bit-accurate Grain blending process

The bit-accurate grain blending process can be used to produce numerically identical cropped decoded output pictures when the all of the following conditions apply:

–	ChromaFormatIdc is equal to 1

–	fg_model_id is equal to 0.

–	fg_separate_colour_description_present_flag equal is to 0.

–	fg_blending_mode_id is equal to 0.

–	fg_log2_scale_factor is in the range 2 to 7, inclusive.

–	fg_intensity_interval_upper_bound[ c ][ i ] is less than fg_intensity_interval_lower_bound[ c ][ i + 1 ].

–	fg_num_model_values_minus1[ c ] is in the range 0..2, inclusive.

–	fg_comp_model_value[ c ][ i ][ 0 ] is in the range 0..( 255  <<  ( fgBitDepth[ c ] – 8) ), inclusive.

–	fg_comp_model_value[ c ][ i ][ 1 ] is in the range 2..14, inclusive.

–	fg_comp_model_value[ c ][ i ][ 2 ] is in the range 2..14, inclusive.

Inputs to the process are the decoded picture sample arrays before grain blending decPictureL, decPictureCb, and decPictureCr.

Outputs of this process are modified decoded picture sample array after grain blending blendPictureL, blendPictureCb, and blendPictureCr.

The variables minPicDim and maxPicDim are derived as follows:

minPicDim = Min( PicHeightInLumaSamples , PicWidthInLumaSamples )

maxPicDim = Max( PicHeightInLumaSamples , PicWidthInLumaSamples )

The variable BlockSize is derived as follows:

BlockSize = Min( 1  <<  ( 3 + Ceil( Log2 (Max ( Ceil( minPicDim ÷ 1080 ),
	 Ceil( maxPicDim ÷ 1920 ) ) ) ) ), 32 )

The grain blending process is derived as the following ordered steps:

–	The grain pattern database generation is invoked in clause 1.1.3.1 with a 13x13x64x64 array GrainDb as output.

–	The grain blending process as specified in clause 1.1.3.2 is invoked with decPictureL, decPictureCb, and decPictureCr. as input, and blendPictureL, blendPictureCb, and blendPictureCr as output.

#### Grain pattern database generation

Output of this process is a 13x13x64x64 grain pattern database array GrainDb.

The function Prng( x ), with x = 0.. 232 − 1, is defined as follows:

Prng( x ) = ( x  <<  1 ) + ( ( 1 + ( ( x & ( 1  <<  2 ) ) > 0 ) + ( ( x & ( 1  <<  30 ) ) > 0 ) ) % 2 )

The pseudo-random number generator array SeedLUT[ i ] with i = 0..255 is specified as follows:

SeedLUT[ i ] =

{

747538460 1088979410 1744950180 1767011913 1403382928  521866116 1060417601 2110622736

1557184770  105289385  585624216 1827676546 1191843873 1018104344 1123590530  663361569

2023850500   76561770 1226763489   80325252 1992581442  502705249  740409860  516219202

557974537 1883843076  720112066 1640137737 1820967556   40667586  155354121 1820967557

1115949072 1631803309   98284748  287433856 2119719977  988742797 1827432592  579378475

1017745956 1309377032 1316535465 2074315269 1923385360  209722667 1546228260  168102420

135274561  355958469  248291472 2127839491  146920100  585982612 1611702337  696506029

1386498192 1258072451 1212240548 1043171860 1217404993 1090770605 1386498193  169093201

541098240 1468005469  456510673 1578687785 1838217424 2010752065 2089828354 1362717428

970073673  854129835  714793201 1266069081 1047060864 1991471829 1098097741  913883585

1669598224 1337918685 1219264706 1799741108 1834116681  683417731 1120274457 1073098457

1648396544  176642749   31171789  718317889 1266977808 1400892508  549749008 1808010512

67112961 1005669825  903663673 1771104465 1277749632 1229754427  950632997 1979371465

2074373264  305357524 1049387408 1171033360 1686114305 2147468765 1941195985  117709841

809550080  991480851 1816248997 1561503561  329575568  780651196 1659144592 1910793616

604016641 1665084765 1530186961 1870928913  809550081 2079346113   71307521  876663040

1073807360  832356664 1573927377  204073344 2026918147 1702476788 2043881033   57949587

2001393952 1197426649 1186508931  332056865  950043140  890043474  349099312  148914948

236204097 2022643605 1441981517  498130129 1443421481  924216797 1817491777 1913146664

1411989632  929068432  495735097 1684636033 1284520017  432816184 1344884865  210843729

676364544  234449232   12112337 1350619139 1753272996 2037118872 1408560528  533334916

1043640385  357326099  201376421  110375493  541106497  416159637  242512193  777294080

1614872576 1535546636  870600145  910810409 1821440209 1605432464 1145147393  951695441

1758494976 1506656568 1557150160  608221521 1073840384  217672017  684818688 1750138880

16777217  677990609  953274371 1770050213 1359128393 1797602707 1984616737 1865815816

2120835200 2051677060 1772234061 1579794881 1652821009 1742099468 1887260865   46468113

1011925248 1134107920  881643832 1354774993  472508800 1892499769 1752793472 1962502272

687898625  883538000 1354355153 1761673473  944820481 2020102353   22020353  961597696

1342242816  964808962 1355809701   17016649 1386540177  647682692 1849012289  751668241

1557184768  127374604 1927564752 1045744913 1614921984   43588881 1016185088 1544617984

1090519041  136122424  215038417 1563027841 2026918145 1688778833  701530369 1372639488

1342242817 2036945104  953274369 1750192384   16842753  964808960 1359020032 1358954497

},

The Gaussian pseudo-random array gaussianLUT[m], with m=0..2047 is specified as follows:

gaussianLUT[ m ] =

{

-11   12  103  -11   42  -35   12   59   77   98  -87    3   65  -78   45   56

-51   21   13  -11  -20  -19   33 -127   17   -6 -105   18   19   71   48  -10

-38   42   -2   75  -67   52  -90   33  -47   21   -3  -56   49    1  -57  -42

-1  120 -127 -108  -49    9   14  127  122  109   52  127    2    7  114   19

30   12   77  112   82  -61 -127  111  -52  -29    2  -49  -24   58  -29  -73

12  112   67   79   -3 -114  -87   -6   -5   40   58  -81   49  -27  -31  -34

-105   50   16  -24  -35  -14  -15 -127  -55  -22  -55 -127 -112    5  -26  -72

127  127   -2   41   87  -65  -16   55   19   91  -81  -65  -64   35   -7  -54

99   -7   88  125  -26   91    0   63   60  -14  -23  113  -33  116   14   26

51  -16  107   -8   53   38  -34   17   -7    4  -91    6   63   63  -15   39

-36   19   55   17  -51   40   33  -37  126  -39 -118   17  -30    0   19   98

60  101  -12  -73  -17  -52   98    3    3   60   33   -3   -2   10  -42 -106

-38   14  127   16 -127  -31  -86  -39  -56   46  -41   75   23  -19  -22  -70

74  -54   -2   32  -45   17  -92   59  -64  -67   56 -102  -29  -87  -34  -92

68    5  -74  -61   93  -43   14  -26  -38 -126  -17   16 -127   64   34   31

93   17  -51  -59   71   77   81  127  127   61   33 -106  -93    0    0   75

-69   71  127  -19 -111   30   23   15    2   39   92    5   42    2   -6   38

15  114  -30  -37   50   44  106   27  119    7  -80   25  -68  -21   92  -11

-1   18   41  -50   79 -127  -43  127   18   11  -21   32  -52   27  -88  -90

-39  -19  -10   24 -118   72  -24  -44    2   12   86 -107   39  -33 -127   47

51  -24  -22   46    0   15  -35  -69   -2  -74   24   -6    0   29   -3   45

32  -32  117  -45   79  -24  -17 -109  -10  -70   88  -48   24  -91  120  -37

50 -127   58   32  -82  -10  -17   -7   46 -127  -15   89  127   17   98  -39

-33   37   42  -40  -32  -21  105  -19   19   19  -59   -9   30    0 -127   34

127  -84   75   24  -40  -49 -127 -107  -14   45  -75    1   30  -20   41  -68

-40   12  127   -3    5   20  -73  -59 -127   -3   -3  -53   -6 -119   93  120

-80  -50    0   20  -46   67   78  -12  -22 -127   36  -41   56  119   -5 -116

-22   68  -14  -90   24  -82  -44 -127  107  -25  -37   40   -7   -7  -82    5

-87   44  -34    9 -127   39   70   49  -63   74  -49  109  -27  -89  -47  -39

44   49   -4   60  -42   80    9 -127   -9  -56  -49  125  -66   47   36  117

15  -11  -96  109   94  -17  -56   70    8  -14   -5   50   37  -45  120  -30

-76   40  -46    6    3   69   17  -78    1  -79    6  127   43   26  127 -127

28  -55  -26   55  112   48  107   -1  -77   -1   53   -9  -22  -43  123  108

127  102   68   46    5    1  123  -13  -55  -34  -49   89   65 -105   -5   94

-53   62   45   30   46   18  -35   15   41   47  -98  -24   94  -75  127 -114

127  -68    1  -17   51  -95   47   12   34  -45  -75   89 -107   -9  -58  -29

-109  -24  127  -61  -13   77  -45   17   19   83  -24    9  127  -66   54    4

26   13  111   43 -113  -22   10  -24   83   67  -14   75 -123   59  127  -12

99  -19   64  -38   54    9    7   61  -56    3  -57  113 -104  -59    3   -9

-47   74   85  -55  -34   12  118   28   93  -72   13  -99  -72  -20   30   72

-94   19  -54   64  -12  -63  -25   65   72  -10  127    0 -127  103  -20  -73

-112 -103   -6   28  -42  -21  -59  -29  -26   19   -4  -51   94  -58  -95  -37

35   20  -69  127  -19 -127  -22 -120  -53   37   74 -127   -1  -12 -119  -53

-28   38   69   17   16 -114   89   62   24   37  -23   49 -101  -32   -9  -95

-53    5   93  -23  -49   -8   51    3  -75  -90  -10  -39  127  -86  -22   20

20  113   75   52  -31   92  -63    7  -12   46   36  101  -43  -17  -53   -7

-38  -76  -31  -21   62   31   62   20 -127   31   64   36  102  -85  -10   77

80   58  -79   -8   35    8   80  -24   -9    3  -17   72  127   83  -87   55

18 -119 -123   36   10  127   56  -55  113   13   26   32  -13  -48   22  -13

5   58   27   24   26  -11  -36   37  -92   78   81    9   51   14   67  -13

0   32   45  -76   32  -39  -22  -49 -127  -27   31   -9   36   14   71   13

57   12  -53  -86   53  -44  -35    2  127   12  -66  -44   46 -115    3   10

56  -35  119  -19  -61   52  -59 -127  -49  -23    4   -5   17  -82   -6  127

25   79   67   64  -25   14  -64  -37 -127  -28   21  -63   66  -53  -41  109

-62   15  -22   13   29  -63   20   27   95  -44  -59 -116  -10   79  -49   22

-43  -16   46  -47 -120  -36  -29  -52  -44   29  127  -13   49   -9 -127   75

-28  -23   88   59   11  -95   81  -59   58   60  -26   40  -92   -3  -22  -58

-45  -59  -22  -53   71  -29   66  -32  -23   14  -17  -66  -24  -28  -62   47

38   17   16  -37  -24  -11    8  -27  -19   59   45  -49  -47   -4  -22  -81

30  -67 -127   74  102    5  -18   98   34  -66   42  -52    7  -59   24  -58

-19  -24 -118  -73   91   15  -16   79  -32  -79 -127  -36   41   77  -83    2

56   22  -75  127  -16  -21   12   31   56 -113 -127   90   55   61   12   55

-14 -113  -14   32   49  -67  -17   91  -10    1   21   69  -70   99  -19 -112

66  -90  -10   -9  -71  127   50  -81  -49   24   61  -61 -111    7  -41  127

88  -66  108 -127   -6   36  -14   41  -50   14   14   73 -101  -28   77  127

-8 -100   88   38  121   88 -125  -60   13  -94 -115   20  -67  -87  -94 -119

44  -28  -30   18    5  -53  -61   20  -43   11  -77  -60   13   29    3    6

-72   38  -60  -11  108  -53   41   66  -12 -127 -127  -49   24   29   46   36

91   34  -33  116  -51  -34  -52   91    7  -83   73  -26 -103   24  -10   76

84    5   68  -80  -13  -17  -32  -48   20   50   26   10   63 -104  -14   37

127  114   97   35    1  -33  -55  127 -124  -33   61   -7  119  -32 -127  -53

-42   63    3   -5  -26   70  -58  -33  -44  -43   34  -56 -127  127   25  -35

-11   16  -81   29  -58   40 -127 -127   20  -47  -11  -36  -63  -52  -32  -82

78  -76  -73    8   27  -72   -9  -74  -85  -86  -57   25   78  -10  -97   35

-65    8  -59   14    1  -42   32  -88  -44   17   -3   -9   59   40   12 -108

-40   24   34   18  -28    2   51 -110   -4  100    1   65   22    0  127   61

45   25  -31    6    9   -7  -48   99   16   44   -2  -40   32  -39  -52   10

-110  -19   56 -127   69   26   51   92   40   61  -52   45  -38   13   85  122

27   66   45 -111  -83   -3   31   37   19  -36   58   71   39  -78  -47   58

-78    8  -62  -36  -14   61   42 -127   71   -4   24  -54   52 -127   67   -4

-42   30  -63   59   -3   -1  -18  -46  -92  -81  -96  -14  -53  -10  -11  -77

13    1    8  -67 -127  127  -28   26  -14   18  -13  -26    2   10  -46  -32

-15   27  -31  -59   59   77 -121   28   40  -54  -62  -31  -21  -37  -32   -6

-127  -25  -60   70 -127  112 -127  127   88   -7  116  110   53   87 -127    3

16   23   74 -106  -51    3   74  -82 -112  -74   65   81   25   53  127  -45

-50 -103  -41  -65  -29   79  -67   64  -33  -30   -8  127    0  -13  -51   67

-14    5  -92   29  -35   -8  -90  -57   -3   36   43   44  -31  -69   -7   36

39  -51   43  -81   58    6  127   12   57   66   46   59  -43  -42   41  -15

-120   24    3  -11   19  -13   51   28    3   55  -48  -12   -1    2   97  -19

29   42   13   43   78  -44   56 -108  -43  -19  127   15  -11  -18  -81   83

-37   77 -109   15   65  -50   43   12   13   27   28   61   57   30   26  106

-18   56   13   97    4   -8  -62 -103   94  108  -44   52   27  -47   -9  105

-53   46   89  103  -33   38  -34   55   51   70  -94  -35  -87 -107  -19  -31

9  -19   79  -14   77    5  -19 -107   85   21  -45  -39  -42    9  -29   74

47  -75   60 -127  120 -112  -57  -32   41    7   79   76   66   57   41  -25

31   37  -47  -36   43  -73  -37   63  127  -69  -52   90  -33  -61   60  -55

44   15    4  -67   13  -92   64   29  -39   -3   83   -2  -38  -85  -86   58

35  -69  -61   29  -37  -95  -78    4   30   -4  -32  -80  -22   -9  -77   46

7  -93  -71   65    9  -50  127  -70   26  -12  -39 -114   63 -127 -100    4

-32  111   22  -60   65 -101   26  -42   21  -59  -27  -74    2  -94    6  126

5   76  -88   -9  -43 -101  127    1  125   92  -63   52   56    4   81 -127

127   80  127  -29   30  116  -74  -17  -57  105   48   45   25  -72   48  -38

-108   31  -34    4  -11   41 -127   52 -104  -43  -37   52    2   47   87   -9

77   27  -41  -25   90   86  -56   75   10   33   78   58  127  127   -7  -73

49  -33 -106  -35   38   57   53  -17   -4   83   52 -108   54 -125   28   23

56  -43  -88  -17   -6   47   23   -9    0  -13  111   75   27  -52  -38  -34

39   30   66   39   38  -64   38    3   21  -32  -51  -28   54  -38  -87   20

52  115   18  -81  -70    0  -14  -46  -46   -3  125   16  -14   23  -82  -84

-69  -20  -65 -127    9   81  -49   61    7  -36  -45  -42   57  -26   47   20

-85   46  -13   41  -37  -75  -60   86  -78 -127   12   50    2   -3   13   47

5   19  -78  -55  -27   65  -71   12 -108   20  -16   11  -31   63  -55   37

75  -17  127  -73  -33  -28 -120  105   68  106 -103 -106   71   61    2   23

-3   33   -5  -15  -67  -15  -23  -54   15  -63   76   58 -110    1   83  -27

22   75  -39  -17  -11   64  -17 -127  -54  -66   31   96  116    3 -114   -7

-108  -63   97    9   50    8   75  -28   72  112  -36 -112   95  -50   23  -13

-19   55   21   23   92   91   22  -49   16  -75   23    9  -49  -97  -37   49

-36   36 -127  -86   43  127  -24  -24   84   83  -35  -34  -12  109  102  -38

51  -68   34   19  -22   49  -32  127   40   24  -93   -4   -3  105    3  -58

-18    8  127  -18  125   68   69  -62   30  -36   54  -57  -24   17   43  -36

-27  -57  -67  -21  -10  -49   68   12   65    4   48   55  127  -75   44   89

-66  -13  -78  -82  -91   22   30   33  -40  -87  -34   96  -91   39   10  -64

-3  -12  127  -50  -37  -56   23  -35  -36  -54   90  -91    2   50   77   -6

-127   16   46   -5  -73    0  -56  -18  -72   28   93   60   49   20   18  111

-111   32  -83   47   47  -10   35  -88   43   57  -98  127  -17    0    1  -39

-127   -2    0   63   93    0   36  -66  -61  -19   39 -127   58   50  -17  127

88  -43 -108  -51  -16    7  -36   68   46  -14  107   40   57    7   19    8

3   88  -90  -92  -18  -21  -24   13    7   -4  -78  -91   -4    8  -35   -5

19    2 -111    4  -66  -81  122  -20  -34  -37  -84  127   68   46   17   47

},

The variable fgCutFreqH[ h ] is set equal to ( ( h + 3 ) << 2) – 1, with h = 0..12.

The variable fgCutFreqV[ v ] is set equal to ( ( v + 3 ) << 2) – 1, with v = 0..12.

The variable fgCoeffs[ h ][ v ][ i ][ j ] is initially set equal to 0, with h = 0..12, v = 0..12, i = 0..63, j = 0..63, and is derived as follows:

for(h = 0; h < 13 , h++ ) {
	for(v = 0; v < 13 , v++ ) {
		prngVal = SeedLUT( h + 13 * v ) 
		for( j = 0; j <= fgCutFreqV[ v ] ; j++ ) {
			for( i = 0; i <= fgCutFreqH[ h ] ; i + = 4 ) {
				for( k = 0; k < 4; k++ )
					fgCoeffs[ h ][ v ][ i + k ][ j ] = gaussianLUT[ ( prngVal + k ) % 2048 ]
				prngVal = Prng( prngVal )
			}
		}
		fgCoeffs[ h ][ v ][ 0 ][ 0 ] = 0
	}
}

For given h, and v, with h = 0..12, v = 0..12, the 64x64 array GrainDb[ h ][ v ][ x][ y ] is derived as follows:

- The variables nonZeroW, nonZeroH, nTbW and nTbH are all set to equal to 64.

- Each (vertical) column of scaled transform coefficients fgCoeffs[ h ][ v ][ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list fgCoeffs[ h ][ v ][ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to 0 as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- The intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = ( e[ x ][ y ] + 256 )  >>  9	 	(xxx)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in in Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to 0 as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

- The nTbW x nTbH array GrainDb[ h ][ v ][ x ][ y ] of grain database samples with x = 0..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

GrainDb[ h ][ v ][ x ][ y ] = Clip3( -127, 127 , ( r[ x ][ y ] + 256 )  >>  9 )	(xxx)

The grain pattern database GrainDb is further refined as follows:

deblockFactor[ v ] = { 64, 71, 77, 84, 90, 96, 103, 109, 116, 122, 128, 128, 128 }	(xx)
for( j = 0; j < 64 ; j += BlockSize ) {
	for( i = 0; i < 64 ; i++ ) {
		GrainDb[ h ][ v ][ i ][ j ] = ( GrainDb[ h ][ v ][ i ][ j ] * deblockFactor[ v ] )  >>  7	(xx)
		GrainDb[ h ][ v ][ i ][ j+ BlockSize – 1  ] = 
			( GrainDb[ h ][ v ][ i ][ j+ BlockSize – 1 ] * deblockFactor[ v ] )  >>  7
		}
	}
}

#### Grain blending process

Inputs to this process are the decoded picture prior to grain blending, i.e., the arrays decPictureL, decPictureCb and decPictureCr.

Outputs of this process are the modified decoded picture after grain blending, i.e., the array blendPictureL, blendPictureCb and blendPictureCr.

For cIdx = 0..2, the following applies:

- If cIdx is equal to 0, decSamples, PicWidth, PicHeight, PicWidthInBlock and PicHeightInBlock are derived as follows.

  - decSamples corresponds to the decoded picture sample array decPictureL and blendSamples corresponds to the blended sample array blendPictureL

  - PicWidth is set equal to PicWidthInLumaSamples

  - PicHeight is set equal to PicHeightInLumaSamples

  - PicWidthInBlock is set equal to Ceil( PicWidthInLumaSamples ÷ BlockSize )

  - PicHeightInBlock is set equal to Ceil( PicHeightInLumaSamples ÷ BlockSize )

- Otherwise (cIdx is not equal to 0), decSamples, blendSamples, PicWidth, PicHeight, PicWidthInBlock and PicHeightInBlock are derived as follows.

  - If cIdx is equal to 1, decSamples corresponds to the decoded picture sample array decPictureCb and blendSamples corresponds to the blended sample array blendPictureCb

  - Otherwise (cIdx is not equal to 1), decSamples corresponds to the decoded picture sample array decPictureCr and blendSamples corresponds to the blended sample array blendPictureCr.

  - PicWidth is set equal to PicWidthInLumaSamples / 2

  - PicHeight is set equal to PicHeightInLumaSamples / 2

  - PicWidthInBlock is set equal to Ceil( PicWidthInLumaSamples ÷ 2 ÷ BlockSize )

  - PicHeightInBlock is set equal to Ceil( PicHeightInLumaSamples ÷ 2 ÷ BlockSize )

- If fg_comp_model_present_flag[ cIdx ] is equal to 0, blendSamples[ i ][ j ] is set equal to decSamples[ i ][ j ], with i=0.. PicWidth, j=0..PicHeight

- Otherwise (fg_comp_model_present_flag[ cIdx ] is not equal to 0), the following applies:

  - The seed initialization for current picture is invoked in clause 1.1.3.2.1 with cIdx as inputs, and array prngArray as output.

  - The grain blending process is performed on a block basis as follows. For every block with block location ( rx, ry ), where rx = 0..PicWidthInBlock − 1 and ry = 0..PicHeightIn Block − 1, the grain block blending process is invoked as specified in clause 1.1.3.2.2, with the location ( xCurr, yCurr ) set equal to ( rx * BlockSize,  ry * BlockSize ), cIdx assigned to cIdx, ( xIdx, yIdx ) set equal to ( rx, ry ), the array prngArray and decSamples as inputs, and blendSamples as outputs.

##### Seed initialization for current picture

Input to this process is a variable cIdx specifying the colour component.

The output of this process are picture seed array prngArray.

The function MSB16( x ) with x = 0..232 – 1, is defined as follows:

MSB16( x ) = ( ( x & 0xFFFF0000 )  >>  16 )

The function LSB16(x) with x = 0..232 – 1, is defined as follows:

LSB16( x ) = ( x & 0x0000FFFF )

The function BIT0( x ) with x = 0..232 – 1, is defined as follows:

BIT0(x) = ( x & 0x1 )

The variable picOffset is set equal to PicOrderCntVal as specified in VVC

The variable cOffset is set equal to ( cIdx  ==  0) ? 0 : ( cIdx  == 1 ) ? 85 : 170 )

The variable prngInit is set equal to SeedLUT[ ( picOffset + cOffset) % 256 ]

The array prngArray[ i ][ j ] with i = 0.. PicWidthInBlock – 1, j = 0.. PicHeightInBlock – 1 is derived as follows:

- The variable prngVal is set equal to prngInit

- The variable bIdc is set equal to ( ( BlockSize  ==  8 ) ? 2 : 1 )

- The following applies,

for( j = 0; j < PicHeightInBlock / bIdc ; j ++ ) {
	for( i = 0; i < PicWidthInBlock / bIdc; i ++ ) {
		if (bIdc  = =  2) {
			prngArray[ 2 * i ][ 2 * j ] = prngVal
			prngArray[ 2 * i + 1 ][ 2 * j ] = prngVal
			prngArray[ 2 * i ][ 2 * j + 1 ] = prngVal
			prngArray[ 2 * i + 1 ][ 2 * j + 1] = prngVal
		}else
			prngArray[i][j] = prngVal
		prngVal = Prng(prngVal)
}

##### Grain block blending process

Inputs to this process are:

- a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component,

- a variable cIdx specifying the colour component of the current block,

- a block index ( xIdx, yIdx ) specifying the block index of the current block,

- a picture seed array prngArray,

- a decoded sample array decSamples.

Outputs to this process are blended sample array blendSamples.

The variable picWidth is set equalt to ( cIdx  = =  0 ) ? PicWidthInLumaSamples : PicWidthInLumaSamples / 2

The variable picHeight is set equal to ( cIdx  = =  0 ) ? PicHeightInLumaSamples : PicHeightInLumaSamples / 2

The variable intensityIntevalIdx for the current block is initialized to be – 1 and is derived as follows:

sumBlock = 0
avgRatio = Log2( BlockSize / 8)
for( i = 0; i < BlockSize ; i++ )
	for( j = 0; j < BlockSize ; j++ )
		sumBlock += decSamples[ Min( picWidth − 1, xCurr + i ) ][ Min( picHeight − 1, yCurr + j ) ]	(xx)
bavg = Clip3( 0, 255, ( sumBlock + ( 1  <<  ( fgBitDepth[ cIdx ] + 2 * avgRatio − 3 ) ) )  >>  ( fgBitDepth[ cIdx ] + 2 * avgRatio − 2 )

for( i = 0; i  <=  fg_num_intensity_intervals_minus1[ cIdx ]; i++ )
	if( bavg  >=  fg_intensity_interval_lower_bound[ cIdx ][ i ]  &&
			bavg  <=  fg_intensity_interval_upper_bound[ cIdx ][ i ] ) {
		intensityIntervalIdx = i		(xx)
		break
}

The derivation of decSamples for the current block is as follows:

- If intensityIntervalIdx is equal to – 1, blendSamples[ xCurr + i ][ yCurr + j ] is set equal to decSamples[ xCurr + i ][ yCurr + j ], for i=0..Min(picWidth – xCurr – 1, BlockSize – 1), j=0.. Min(picHeight – yCurr – 1, BlockSize – 1;

- Otherwise (intensityIntervalIdx is not equal to – 1), the following applies:

  - The variable scaleFactor is set equal to fg_comp_model_value[ cIdx ][ intensityIntervalIdx ][ 0 ] * ( 1 – 2 * BIT0( prngArray[ xIdx ][ yIdx ] )

  - The variable hFreq is derived as follows:

hFreq = fg_comp_model_value[ cIdx ][ intensityIntervalIdx ][ 1 ]

hFeq = (cIdx  = =  0) ? hFreq : Clip3( 2, 14, ( hFreq  <<  1 ) )

hFeq = hFreq – 2

  - The variable vFreq is derived as follows:

vFreq = fg_comp_model_value[ cIdx ][ intensityIntervalIdx][ 2 ]

vFeq = (cIdx  = =  0) ? vFreq : Clip3( 2, 14, ( vFreq  <<  1 ) )

vFeq = vFreq – 2

  - The variable horzDenom = (BlockSize  = =  32)? 36: 52

  - The variable vertDenom = (BlockSize  = =  32)? 40: 56

  - The variable horzOffset is derived as follows:

horzOffset = ( MSB16( prngArray[ xIdx ][ yIdx ] )  %  horzDenom )

horzOffset  & =  0xFFFC

horzOffset  + =  ( BlockSize  = =  8 ) ? ( xIdx & 0x0008 ) : 0

  - The variable vertOffset is derived as follows:

vertOffset = ( LSB16( prngArray[ xIdx ][ yIdx ] )  %  vertDenom )

vertOffset  & =  0xFFF8

vertOffset  + =  ( BlockSize  = =  8 ) ? ( yIdx & 0x0008) : 0

  - grainSamples is derived as follows, for i = 0..Min( picWidth – xCurr – 1, BlockSize – 1 ), j = 0.. Min( picHeight – yCurr – 1, BlockSize – 1 ):

grainSamples[ xCurr + i ][ yCurr + j ] = 
		( scaleFactor * GrainDb[ hFreq ][ vFreq ][ i + horzOffset ][ j + vertOffset] )  >>  ( fg_log2_scale_factor + 6 )

- if xCurr > 0, the following applies:

for( k = 0; k < BlockSize; k++ ) {
  	l1 = grainSamples[ xCurr – 2 ][ k ]
  	l0 = grainSamples[ xCurr – 1 ][ k ]
  	r0 = grainSample[ xCurr ][ k ]
  	r1 = grainSample[ xCurr + 1 ][ k ]
  grainSamples[ xCurr ][ k ] = ( ( l0 + ( r0  <<  1 ) + r1 )  >>  2 )
  grainSamples[ xCurr – 1 ][ k ] = ( ( l1 + ( l0  <<  1 ) + r0 )  >>  2 )
}

  - blendSamples[ xCurr + i ][ yCurr + j ] is derived as follows, for i = 0..Min( picWidth – xCurr – 1, BlockSize – 1 ), j = 0.. Min( picHeight – yCurr – 1, BlockSize – 1 )

blendSample[ xCurr + i ][ yCurr + j ] = Clip3( 0, ( 1  <<  fgBitDepth[ cIdx ] ) − 1,		(xx)
 decodedSamples[ xCurr + i ][ yCurr + j ] + grainSamples[ xCurr + i ][ yCurr + j ] )

