| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23nd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0099-v3 |
| --- | --- |

| Title: | AHG11: CNN-based Super Resolution for Video Coding Using Decoded Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Li Zhang2, Kai Zhang2, Yue Li2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | linchaoyi.cy@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com yue.li@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents a convolutional neural network-based super-resolution for video coding. In the proposed design, the decoded information is fed into the up-sampling network. In addition, different up-sampling networks are designed for the luma and chroma components. Compared with VTM-11.0 with new MCTF enabled, the proposed method reportedly shows on average {-12.00%, 14.12%, -4.78%} BD-rate changes for {Y, U, V} components under all intra configurations, respectively.

# Introduction

Recently, the convolutional Neural Network (CNN)-based super-resolution for video coding in VVC, especially with the reference picture resampling (RPR) mechanism, has demonstrated significant coding gains over the conventional up-sampling methods. However, there is still room to further improve the coding performance, especially for the chroma components. For example, the decoded information and cross-component correlation have not been utilized by the network in the prior art.

# Proposed method

In this contribution, it is proposed to fed the decoded information into the up-sampling network and separate the networks for luma and chroma components. To be more specified, for the luma up-sampling network, the sequence-level QP and the prediction signal are used as auxiliary information, while for the chroma up-sampling network, the luma information is utilized to boost the performance of the up-sampling CNN for chroma components.

Detailed information of the up-sampling network for luma and chroma components are described in the following sub-sections, respectively.

## Model for the luma component

The up-sampling model for the luma component is illustrated in Fig. 1. The input to the model consists of three parts, i.e., the low-resolution reconstruction YLRrec, the luma component of prediction frame YLRpred, and the QP map. The QP map is a matrix filled with the QP value. Those three parts are concatenated together and then fed into the first convolutional layer. The output from the first layer further goes through several residual blocks and one additional convolutional layer. Then, a shuffle layer generates the high-resolution reconstruction from the output of the last convolutional layer.

Fig. 1 The proposed model for luma up-sampling.

## Model for chroma components

Fig. 2 shows the up-sampling model for the chroma components, which resembles the luma up-sampling model except the input part. To utilize the cross-component correlation, the luma component of reconstruction is taken as one input. Specifically, the luma component YLRrec is first down-sampled to the same resolution as chroma components by a convolution layer with the stride of 2 for the 4:2:0 color format. The features extracted from the luma component are then concatenated with the features extracted from the chroma components along the channel dimension.

Fig. 2 The proposed model for chroma up-sampling.

## Inference

PyTorch is used for performing the inference of the proposed CNN-based super resolution in VTM. The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | GPU Type | Tesla V100-SXM2 |
|  | Framework: | PyTorch v1.7 |
|  | Number of GPUs per Task | 1 |
|  | Total Parameter Number | 4.72 M for the luma model, 1.19M for the chroma model |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 18.88 MB for the luma model，4.76MB for the chroma model. 2 models in total. |
|  | MAC (Giga) | 1442 kMAC/pixel |
| Optional | Total Conv. Layers | 34 for up-sampling the luma, 36 for up-sampling the chroma |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

## Training details

DIV2K [2] dataset is used for training. The VTM-11.0 [3] + new MCTF [4] is utilized to compress these images with QPs {22, 27, 32, 37, 42}. In the context of RPR scheme, the training data will be down-sampled by 2x before encoding. Then, we use the decoded low-resolution images and the corresponding original images for training.

Table 2 Network information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.7 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 |
|  | Batch size: | 16 |
|  | Training time: | 96h/model |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

The proposed CNN-based up-sampling filter is tested on top of VTM-11.0 + new MCTF, according to the common test conditions defined in [5]. For the test, the QP values are {22, 27, 32, 37, 42}. The test results are shown in Table 3. Under AI configurations, the proposed method can bring {-12.00%, 14.12%, -4.78%} BD-rate changes on average for {Y, U, V} components, respectively. For the encoding time and decoding time, the proposed method is tested both on CTU and GPU. The compression process is performed on CTU and the super-resolution process is performed on GPU.

Table 3 Results of the proposed method for AI configurations.

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 (QP 27,32,37,42,47) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -12.77% | -13.49% | -11.29% | 99% | 197% |
|  | FoodMarket4 | -6.87% | 2.90% | 1.77% | 104% | 195% |
|  | Campfire | -19.85% | 111.56% | -10.24% | 55% | 170% |
| Cass A2 4K | CatRobot1 | -11.22% | -15.33% | -9.13% | 78% | 182% |
|  | DaylightRoad2 | -7.63% | -18.06% | -5.37% | 81% | 188% |
|  | ParkRunning3 | -13.65% | 17.11% | 5.55% | 61% | 148% |
| Average on A1 |  | -13.17% | 33.66% | -6.59% | 80% | 185% |
| Average on A2 |  | -10.83% | -5.43% | -2.98% | 68% | 169% |
| Overall |  | -12.00% | 14.12% | -4.78% | 74% | 177% |

# Conclusion

This contribution presents a CNN-based super resolution method for the up-sampling filter of RPR. Different networks are designed for the luma and chroma components, respectively. The proposed method shows a promising performance.

# References

- Exploration experiments on Neural Network-based Video Coding, JVET-V2023, E. Alshina et al.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/, R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- GOP-based temporal filter improvements, JVET-V0056, P. Wennersten, C. Hollmann, J. Ström .

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-V2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).