| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0112 |
| --- | --- |

| Title: | AHG12: Diagonal MMVD with ARMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes a diagonal merge with motion vector difference (DMMVD) with adaptive reordering merge candidate (ARMC) to enhance the coding performance beyond VVC. With the method, diagonal motion vector difference (MVD) can be selected in addition to horizontal and vertical MVD, and the MMVD-applicable merge candidates are reordered with template matching.

On top of the VTM-11.0+MCTF which are defined as the anchor for tool-on test in EE2 of JVET-V2024, simulation results of the proposed method are reported as follows:

- Only DMMVD

  - RA: BD-rate YUV: -0.11%, -0.10%, -0.09%; EncT: 107%; DecT: 100%

  - LB: BD-rate YUV: -0.08%, -0.07%, -0.06%; EncT: 104%; DecT: 100%

- Only ARMC

  - RA: BD-rate YUV: -0.06%, -0.06%, -0.08%; EncT: 100%; DecT: 106%

  - LB: BD-rate YUV: 0.16%, 0.01%, -0.12%; EncT: 101%; DecT: 106%

- DMMVD + AMRC

- RA: BD-rate YUV: -0.18%, -0.21%, -0.19%; EncT: 108%; DecT: 106%

- LB: BD-rate YUV: 0.10%, 0.01%, -0.26%; EncT: 105%; DecT: 108%

Introduction

In the current MMVD, the direction of MVD by MMVD is restricted in the horizontal or vertical axes with the origin at the motion vector of the regular merge mode, and the MMVD-applicable merge candidates are also limited to 0th or 1st candidates [1]. However, MMVD has room to improve coding performance if the diagonal direction of MVD can be selected as reported in the CE test of JVET-N0142 [2], and if the optimal candidate for MMVD can be chosen.

Proposal

In this contribution, a diagonal MMVD (DMMVD) with adaptive reordering merge candidate (ARMC) based on template matching is proposed. With the method, the direction of MVD is specified with Direction IDX and the corresponding motion direction table shown in Table 1. Moreover, the spatial merge candidates are reordered with template matching before the motion vector refinement by MMVD.

Table 1. Motion direction

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +1 | –1 | 0 | 0 | +1 | -1 | -1 | +1 |
| y-axis | 0 | 0 | +1 | –1 | +1 | -1 | +1 | -1 |

Simulation Results

The proposed method was implemented on the top of VTM-11.0+MCTF [3], which are defined as the anchor software in EE2 for tool-on tests  [4]. Simulation conditions are followed by the common test condition (CTC) [5]. In the simulations, the following three tests are conducted for ablation study of the proposed methods.

- Test 1: DMMVD.

- Test 2: ARMC for spatial candidate

- Test 3: Test 1 + Test 2

Tables 1, 2 and 3 show the Test 1, Test 2, and Test 3 results over VTM-11.0+MCTF.

Table 2: Simulation results of Test 1 over VTM-11.0+MCTF.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.20% | -0.17% | 106% | 100% |  |  |  |  |  |
| Class A2 | -0.14% | -0.13% | -0.15% | 106% | 101% |  |  |  |  |  |
| Class B | -0.09% | -0.03% | -0.04% | 107% | 100% | -0.10% | -0.05% | -0.14% | 104% | 100% |
| Class C | -0.10% | -0.11% | -0.04% | 107% | 100% | -0.07% | -0.08% | 0.00% | 104% | 100% |
| Class E |  |  |  |  |  | -0.06% | -0.11% | 0.00% | 103% | 100% |
| Overall | -0.11% | -0.10% | -0.09% | 107% | 100% | -0.08% | -0.07% | -0.06% | 104% | 100% |
| Class D | -0.17% | -0.20% | -0.31% | 106% | 100% | -0.10% | 0.59% | -0.35% | 105% | 100% |
| Class F | 0.01% | -0.03% | -0.16% | 105% | 100% | -0.25% | -0.23% | -0.28% | 103% | 99% |

Table 2: Simulation results of Test 2 over VTM-11.0+MCTF.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.04% | -0.02% | 100% | 103% |  |  |  |  |  |
| Class A2 | -0.04% | -0.06% | -0.06% | 100% | 105% |  |  |  |  |  |
| Class B | -0.11% | -0.09% | -0.13% | 100% | 106% | 0.08% | -0.07% | -0.12% | 101% | 107% |
| Class C | -0.09% | -0.05% | -0.06% | 101% | 109% | 0.08% | -0.03% | -0.38% | 101% | 111% |
| Class E |  |  |  |  |  | 0.40% | 0.17% | 0.21% | 101% | 106% |
| Overall | -0.06% | -0.06% | -0.08% | 100% | 106% | 0.16% | 0.01% | -0.12% | 101% | 108% |
| Class D | -0.17% | -0.23% | -0.15% | 101% | 111% | 0.20% | 0.70% | -0.13% | 101% | 113% |
| Class F | -0.18% | -0.17% | -0.25% | 100% | 104% | -0.34% | -0.12% | -0.35% | 100% | 105% |

Table 3: Simulation results of Test 3 over VTM-11.0+MCTF.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.22% | -0.16% | 107% | 103% |  |  |  |  |  |
| Class A2 | -0.16% | -0.19% | -0.22% | 107% | 105% |  |  |  |  |  |
| Class B | -0.18% | -0.19% | -0.15% | 108% | 105% | 0.02% | -0.03% | -0.43% | 105% | 107% |
| Class C | -0.24% | -0.25% | -0.24% | 108% | 109% | 0.04% | 0.09% | -0.25% | 106% | 111% |
| Class E |  |  |  |  |  | 0.34% | -0.03% | 0.04% | 105% | 105% |
| Overall | -0.18% | -0.21% | -0.19% | 108% | 106% | 0.10% | 0.01% | -0.26% | 105% | 108% |
| Class D | -0.28% | -0.34% | -0.53% | 108% | 111% | 0.07% | -0.21% | -0.90% | 106% | 113% |
| Class F | -0.19% | -0.19% | -0.22% | 106% | 104% | -0.31% | 0.01% | -0.35% | 104% | 105% |

Conclusion

In this contribution, a diagonal MMVD with ARMC was proposed to improve coding efficiency beyond VVC. Simulation results reportedly show the proposed methods provide potentially a -0.18% BD-rate savings of luma component with increment encoding/decoding runtime by 108%/106% over VTM-11.0+MCTF in the RA conditions.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- Recommendation ITU-T H.266, "Versatile video coding," in Edition 1.0, August 2020.

- T. Hashimoto, E. Sasaki, and T. Ikai, “CE4-3.4: Diagonal direction candidate for MMVD,” JVET-N0142, Geneva, 19-27 March 2019.

- VTM-11.0+MCTF: https://vcgit.hhi.fraunhofer.de/chollmann/VVCSoftware_VTM/-/tree/MCTF_VTM11

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-V2024, by teleconference, 20–28 Apr. 2021.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, by teleconference, 7-16 October. 2020.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).