| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0126 |
| --- | --- |

| Title: | EE2 Related – DIMD with implicitly derived multiple blending modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seethal Paluri Seung-Hwan Kim | Tel: Email: | +1 (858) 635-5300 jie.zhao@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In this contribution, a method to implicitly derive the DIMD blending modes is presented. It utilizes techniques in both EE2-2.2 -- DIMD with multiple blending modes and EE2-2.1 – template-based decoder mode derivation (TIMD).  Instead of selecting the blending mode by full RDO and explicitly signaling it, now the blending mode is derived by template-based approach as in TIMD at both encoder and decoder.

On top of ECM-1.0, the coding performance is

AI: -0.04%, 0.00%, -0.03%, 1xx% (Enc), 1xx% (Dec)

RA: -0.xx%, -0.xx%, -0.xx%, 1xx% (Enc), 1xx% (Dec).

# Introduction

In EE2 common software ECM1.0 [1], one of the intra tools is Decoder Side Intra Mode Derivation (DIMD). Two intra modes are derived from the reconstructed neighboring samples based on Histogram of Gradients (HoG). The DIMD intra prediction is computed by blending the predictions from the two intra modes and planar mode. Planar prediction is assigned a weight of (1/3) and the remaining (2/3) is split between the first mode and second mode in proportion to their HoG amplitudes. A flag dimd_flag is signalled to indicate if a block is coded in the DIMD mode or not.

EE2 Test2.2 [2] extends the DIMD with multiple blending modes where 3 blending modes are used. The 3 blending modes are different weighted combinations of planar and the two derived intra modes by HoG. The best blending mode is selected at encoder side based on RDO decision. A DIMD index representing {0,1,2,3} is signalled in the bitstream.

EE2 Test.2.1 [3] introduces template-based decoder side intra mode derivation (TIMD) using most probable modes (MPMs). As shown in Figure 1, the prediction samples of the template is generated using the reference samples of the template for each candidate mode. For each intra prediction mode in MPMs, the sum of absolute transformed differences (SATD) between the prediction and reconstruction samples of a template is calculated. The intra prediction mode with the minimum SATD is selected as the TIMD mode and used for intra prediction of current block.  TIMD flag is signalled to indicate whether TIMD is being used.

Both EE2-2.1 and EE2-2.2 increase the number of encoder full RDO checks, therefore they both have large encoding complexity, encoding time are increased 20% or more.

Figure 1. Template and its reference samples used in TIMD

# Algorithm Description

The proposed method is based on top of both of EE2-2.2 -- DIMD with multiple blending modes and EE2-2.1 – template-based decoder mode derivation (TIMD). It shows another way of combining these two technologies.

Same as EE2-2.2, DIMD has 3 blending modes. They are different weighted combinations among planar and the two derived intra modes by HoG analysis. The 3 blending modes used in our test are:

- Blend predictions of Mode1, Mode2 and Planar(wP = 1/3). This is the same as existing DIMD blending method in ECM1.0.

- Blend predictions of Mode2 and Planar (wP = 1/3)

- Blend predictions of Mode1 and Mode2

For the multiple blending modes, instead of performing full RDO checks to select the best one, and signal it in the bitstream, now the best blending mode is derived by template-based method as in EE2.1 at both encoder and decoder side. Template and reference samples used are the same as those in EE2-2.1, as shown in Fig. 1 above. Given the template, for each blending mode, prediction is performed by blending the predictions from the two intra modes and planar mode. A cost is calculated as the SATD between the prediction and the reconstruction samples of the template. The best blending mode is selected based on the cost. In our test, the cost of existing DIMD blending mode is given a factor less than 1 to make it being chosen a little bit more than the  other two blending modes.

# Results

The proposed method is implemented on top of ECM1.0 and based on the EE2-2.1 and EE2-2.2 code.

Tables below show the performance of proposed method comparing to ECM1.0. Preliminary results show that there are small gains of -0.04% in AI with little encoding time increase, and xxx% decoding time increase.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | -0.11% | 101% | 107% |
| Class A2 | -0.02% | -0.03% | -0.05% | #NUM! | #NUM! |
| Class B | -0.04% | 0.00% | -0.07% | 101% | 111% |
| Class C | -0.01% | -0.03% | -0.11% | 102% | 119% |
| Class E | -0.11% | -0.20% | -0.03% | 102% | 114% |
| Overall | -0.04% | -0.04% | -0.08% | #NUM! | #NUM! |
| Class D | 0.00% | -0.22% | 0.08% | 102% | 122% |
| Class F | 0.05% | -0.02% | 0.03% | 100% | 106% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.03% | 0.00% | 0.01% | 99% | 100% |
| Class C | -0.01% | 0.08% | -0.27% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.03% | 0.07% | 100% | 101% |
| Class F | 0.05% | -0.29% | -0.05% | 99% | 100% |

# Conclusion

This documents presented a method to allow multiple DIMD blending mode by implicitly deriving it with TIMD approach.  It shows another way of combing techniques in both EE2-2.2 -- DIMD with multiple blending modes and EE2-2.1 – template-based decoder mode derivation (TIMD).

Preliminary results show that on average there is a small gain of 0.04% in AI with xx% encoding time increase and xxx% decoding time increase. We think there is more potential to improve this method, and would like to further study on this approach.

# References

[1] ECM1.0, available at https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tags/ECM-1.0

[2] J. Zhao, etc. “EE2-Related: Improvements of Decoder-Side Intra Mode Derivation,” JVET-V0087, Meeting by Teleconference, April 2021.

[3] Y. Wang, etc, “EE2-related: Template-based intra mode derivation using MPMs,” JVET-V0098, Meeting by Teleconference, April 2021.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).