| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0128-v3 |
| --- | --- |

| Title: | AHG12: Alternative classifiers for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In adaptive loop filter process of ECM-1.0, when a signalled filter set is applied to a luma coding tree block, a classifier is applied to each 2x2 luma block. For each class, a specific filter is applied among the signalled filter set. Based on the classification, a geometric transformation can also be applied. In this contribution, a two-classifier method is proposed and five different second classifier options are tested. For a luma filter set, a flag is signalled to indicate whether the classifier in ECM-1.0 or the proposed alternative classifier is applied. On top of ECM-1.0 with common test condition, simulation results of the proposed methods are reported as below.

|  | All intra |  |  | Random access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT |
| Test 1 | -0.05% | 100% | 97% | -0.18% | 99% | 99% | -0.29% | 102% | 98% |
| Test 2 | -0.09% | 105% | 96% | -0.30% | 103% | 99% | -0.44% | 110% | 101% |
| Test 3 | -0.11% | 100% | 97% | -0.26% | 100% | 99% | -0.44% | 100% | 99% |
| Test 4 | -0.14% | 106% | 97% | -0.36% | 104% | 99% | -0.60% | 110% | 98% |
| Test4 fast | -0.14% | 102% | 97% | -0.35% | 101% | 100% | -0.61% | 102% | 100% |
| Test 5 | -0.22% | 101% | 96% | -0.43% | 100% | 99% | -0.73% | 101% | 96% |

# Introduction

# In adaptive loop filter (ALF) process of ECM-1.0, a luma coding tree block (CTB) can reference a filter set in an adaptation parameter set (APS). Based on local gradients, classification is applied to each 2x2 luma block in the CTB. For each class, a specific filter is applied among the different filters in the filter set. Based on this classification, geometric transformation, such as 90-degree rotation, diagonal or vertical flip, of the coefficients within a diamond filter shape can also be applied.

# Proposed methods

To extend the classification in ALF, in this contribution, two-classifier based ALF is applied. For a signalled luma filter set, a flag is signalled to indicate whether the classifier in ECM-1.0 or one of the proposed new classifiers is applied.

## Alternative classifier 1

In the second classifier, all samples are classified into the same class and geometric transformation is not applied.

## Alternative classifier 2

In the second classifier, a 2x2 luma block is classified into a class in the same way as ECM-1.0 but without geometric transformation.

## Alternative classifier 3

In this band-based classifier as the second classifier, the sum of sample values of a 2x2 luma block is calculated at first. Then the class index is calculated as below,

class_index = (sum * 8) >> (sample bit depth + 2).

## Alternative classifier 4

This classifier is similar to the classifier 3, whereas the number of bands is 25. And the class index is calculated as

class_index = (sum * 25) >> (sample bit depth + 2).

## Alternative classifier 5

In this band-based classifier as the second classifier, the class index of a luma sample is calculated as

class_index = (luma sample * 25) >> (sample bit depth).

# Results

The proposed methods are implemented on top of ECM-1.0 and tested with common test condition (CTC) described in JVET-V2017.

## Test 1: Alternative classifier 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.03% | 104% | 99% |
| Class A2 | 0.00% | -0.01% | -0.01% | 102% | 97% |
| Class B | -0.03% | -0.06% | -0.06% | 99% | 95% |
| Class C | -0.17% | -0.03% | -0.03% | 98% | 95% |
| Class E | -0.05% | -0.05% | -0.05% | 100% | 99% |
| Overall | -0.05% | -0.04% | -0.04% | 100% | 97% |
| Class D | -0.03% | -0.02% | -0.02% | 99% | 98% |
| Class F | -0.04% | -0.05% | -0.05% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.15% | -0.02% | 101% | 99% |
| Class A2 | -0.02% | 0.02% | 0.02% | 101% | 99% |
| Class B | -0.32% | 0.00% | 0.03% | 99% | 99% |
| Class C | -0.17% | -0.01% | -0.16% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.03% | -0.03% | 99% | 99% |
| Class D | -0.05% | 0.04% | 0.14% | 98% | 98% |
| Class F | -0.07% | -0.13% | -0.03% | 102% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.19% | -0.34% | 103% | 101% |
| Class C | -0.22% | -0.44% | -0.18% | 98% | 96% |
| Class E | -0.21% | -0.07% | -0.43% | 105% | 95% |
| Overall | -0.29% | -0.24% | -0.31% | 102% | 98% |
| Class D | 0.03% | 0.07% | -0.23% | 102% | 103% |
| Class F | -0.17% | 0.07% | -0.41% | 103% | 97% |

## Test 2: Alternative classifier 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.00% | 0.00% | 108% | 99% |
| Class A2 | -0.01% | 0.00% | 0.00% | 103% | 101% |
| Class B | -0.11% | 0.02% | 0.02% | 105% | 96% |
| Class C | -0.19% | 0.00% | 0.00% | 103% | 92% |
| Class E | -0.08% | 0.01% | 0.01% | 108% | 97% |
| Overall | -0.09% | 0.01% | 0.01% | 105% | 96% |
| Class D | -0.03% | 0.00% | 0.00% | 108% | 98% |
| Class F | -0.13% | 0.03% | 0.03% | 104% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.15% | -0.01% | 102% | 100% |
| Class A2 | -0.18% | 0.08% | 0.03% | 102% | 100% |
| Class B | -0.51% | 0.13% | 0.16% | 104% | 100% |
| Class C | -0.20% | 0.08% | -0.07% | 105% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | 0.05% | 0.04% | 103% | 99% |
| Class D | -0.07% | 0.16% | 0.13% | 111% | 96% |
| Class F | -0.11% | -0.05% | 0.10% | 107% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.61% | -0.20% | 0.15% | 106% | 102% |
| Class C | -0.27% | -0.55% | -0.27% | 110% | 101% |
| Class E | -0.37% | -0.34% | -0.02% | 115% | 100% |
| Overall | -0.44% | -0.35% | -0.03% | 110% | 101% |
| Class D | 0.01% | 0.04% | 0.02% | 117% | 99% |
| Class F | -0.22% | -0.80% | -0.63% | 110% | 96% |

## Test 3: Alternative classifier 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.01% | 0.00% | 104% | 97% |
| Class A2 | -0.03% | -0.02% | -0.02% | 101% | 96% |
| Class B | -0.11% | -0.02% | -0.02% | 99% | 96% |
| Class C | -0.16% | -0.02% | -0.02% | 99% | 97% |
| Class E | -0.11% | 0.04% | 0.04% | 102% | 102% |
| Overall | -0.11% | -0.01% | -0.01% | 100% | 97% |
| Class D | -0.03% | 0.00% | 0.00% | 99% | 100% |
| Class F | -0.08% | -0.02% | -0.03% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | 0.05% | -0.09% | 101% | 99% |
| Class A2 | -0.15% | 0.07% | 0.03% | 101% | 99% |
| Class B | -0.41% | 0.05% | 0.02% | 100% | 99% |
| Class C | -0.17% | 0.01% | -0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | 0.04% | -0.05% | 100% | 99% |
| Class D | -0.09% | 0.05% | 0.16% | 99% | 97% |
| Class F | -0.12% | -0.08% | 0.06% | 103% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.68% | -0.39% | -0.21% | 101% | 100% |
| Class C | -0.20% | -0.31% | -0.31% | 100% | 98% |
| Class E | -0.35% | -0.75% | -0.15% | 108% | 101% |
| Overall | -0.44% | -0.45% | -0.23% | 102% | 100% |
| Class D | -0.07% | 0.77% | 0.19% | 103% | 104% |
| Class F | -0.41% | 0.12% | -0.11% | 101% | 94% |

## Test 4: Alternative classifier 4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.04% | 0.04% | 108% | 98% |
| Class A2 | -0.09% | 0.03% | 0.03% | 104% | 98% |
| Class B | -0.16% | 0.04% | 0.04% | 104% | 96% |
| Class C | -0.18% | 0.00% | 0.00% | 106% | 97% |
| Class E | -0.12% | 0.06% | 0.06% | 110% | 97% |
| Overall | -0.14% | 0.03% | 0.03% | 106% | 97% |
| Class D | -0.03% | 0.00% | 0.00% | 109% | 101% |
| Class F | -0.17% | 0.07% | 0.07% | 106% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.01% | 0.00% | 102% | 100% |
| Class A2 | -0.29% | 0.08% | 0.06% | 102% | 99% |
| Class B | -0.55% | 0.10% | 0.23% | 103% | 100% |
| Class C | -0.21% | 0.01% | -0.11% | 106% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | 0.05% | 0.06% | 104% | 99% |
| Class D | -0.09% | -0.04% | 0.19% | 113% | 99% |
| Class F | -0.21% | 0.00% | 0.13% | 110% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.91% | -0.30% | -0.35% | 106% | 99% |
| Class C | -0.29% | -0.56% | -0.14% | 110% | 97% |
| Class E | -0.49% | 0.11% | -0.12% | 116% | 99% |
| Overall | -0.60% | -0.28% | -0.22% | 110% | 98% |
| Class D | -0.08% | 0.48% | 0.17% | 120% | 102% |
| Class F | -0.47% | -0.12% | -0.55% | 114% | 99% |

## Test 4 fast: Alternative classifier 4 with fast methods

On top of test 4, encoder speed up is applied to clipping parameter derivation.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.04% | 0.04% | 105% | 99% |
| Class A2 | -0.09% | 0.03% | 0.03% | 102% | 97% |
| Class B | -0.16% | 0.04% | 0.04% | 101% | 98% |
| Class C | -0.18% | 0.00% | 0.00% | 100% | 96% |
| Class E | -0.12% | 0.06% | 0.06% | 101% | 98% |
| Overall | -0.14% | 0.03% | 0.03% | 102% | 97% |
| Class D | -0.03% | 0.00% | 0.00% | 98% | 97% |
| Class F | -0.16% | 0.07% | 0.07% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.02% | -0.02% | 103% | 102% |
| Class A2 | -0.29% | 0.10% | 0.12% | 102% | 101% |
| Class B | -0.54% | 0.14% | 0.17% | 101% | 100% |
| Class C | -0.21% | 0.01% | -0.09% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | 0.07% | 0.05% | 101% | 100% |
| Class D | -0.09% | -0.03% | 0.19% | 96% | 97% |
| Class F | -0.21% | -0.02% | 0.03% | 103% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.95% | -0.23% | -0.06% | 102% | 100% |
| Class C | -0.29% | -0.62% | -0.20% | 100% | 101% |
| Class E | -0.48% | 0.15% | -0.19% | 106% | 99% |
| Overall | -0.61% | -0.26% | -0.14% | 102% | 100% |
| Class D | -0.07% | 0.35% | 0.25% | 98% | 102% |
| Class F | -0.49% | -0.06% | -0.34% | 102% | 98% |

## Test 5: Alternative classifier 5 with fast methods

In this test, encoding speed up is applied to clipping parameter derivation.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | 0.06% | 0.06% | 104% | 96% |
| Class A2 | -0.16% | 0.04% | 0.04% | 101% | 99% |
| Class B | -0.25% | 0.08% | 0.08% | 100% | 96% |
| Class C | -0.21% | 0.01% | 0.01% | 99% | 93% |
| Class E | -0.25% | 0.14% | 0.13% | 101% | 97% |
| Overall | -0.22% | 0.06% | 0.06% | 101% | 96% |
| Class D | -0.06% | 0.03% | 0.03% | 97% | 99% |
| Class F | -0.90% | 0.30% | 0.30% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | 0.05% | 0.06% | 101% | 99% |
| Class A2 | -0.40% | 0.16% | 0.09% | 101% | 100% |
| Class B | -0.66% | 0.32% | 0.33% | 100% | 99% |
| Class C | -0.23% | 0.07% | 0.06% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | 0.17% | 0.15% | 100% | 99% |
| Class D | -0.08% | 0.30% | 0.26% | 94% | 98% |
| Class F | -0.89% | 0.19% | 0.22% | 101% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.11% | -0.12% | -0.03% | 103% | 102% |
| Class C | -0.32% | -0.43% | -0.08% | 96% | 91% |
| Class E | -0.64% | -0.10% | -0.37% | 103% | 95% |
| Overall | -0.73% | -0.22% | -0.13% | 101% | 96% |
| Class D | -0.21% | -0.16% | 0.11% | 94% | 101% |
| Class F | -1.10% | -0.57% | -0.31% | 100% | 98% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).