| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0131-v1 |
| --- | --- |

| Title: | EE1-related: Neural Network-based in-loop filter with constrained computational complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Jianle Chen Kevin Reuze Anand Meher Kotra Marta Karczewicz | Email: | hongtaow@qti.qualcomm.com cjianle@qti.qualcomm.com kreuz@qti.qualcomm.com akotra@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This EE related contribution studies the performance of the network structure tested in EE (JVET-W0130) with a different complexity configuration. The proposed configuration is roughly determined based on extrapolating computational capabilities of potential future processing units. As done in the EE test, one NN-based filter can be selected out of four candidates as an in-loop filter for each picture. The deblocking boundary strength information of VVC is also used as additional input to the NN filter but the actual deblocking filtering process is bypassed. The total number of parameters in each model is ~108K, which result in roughly 20 times lower MAC/pixel compared to the models tested in EE. The simulation results reportedly show 5.78%, 6.80% and 5.03% BD rate saving for RA and 5.16%, 5.93%, 5.19% BD rate saving for AI, for Y, Cb and Cr components respectively.

# Introduction

This contribution studies the network structure tested in JVET-W0130 [1] configured with a different computational complexity. The network structure is illustrated in Figure 1. It contains 13 filter blocks, including 11 filter blocks used as hidden layers. Each hidden filter block consists of two 1x1 convolution layers with a leaky ReLU as activation layer between them, and followed by a 3x3 convolution layer, where the value of M is set to be larger than the value of K. Compared to that in JVET-W0130, the neural network studied in this contribution has about 108K parameters. At the same time, luma and chroma components are filtered by a single model jointly. The changes result in roughly 20 times lower computational complexity (measured by MAC/pixel) compared to the solution in JVET-W0130.

Figure 1 Proposed network architecture

The input of the NN filtering process is a CTU and 8 neighboring samples to each side of the CTU. The luma samples are interleaved into four 72x72 sized blocks before being used as inputs of the filtering process. The output tensor corresponds to the filtered CTU samples (64x64x6), including 4 luma blocks and 2 chroma blocks.

Additionally, the QP and boundary strength (BS) information is used as additional input planes to the network.

## Scaling of NN filter residues

When the proposed NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in picture headers. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples.

# Simulation results

Simulations are performed based on the common test conditions for neural network-based video coding technology[4]. The results reportedly show 5.78%, 6.80% and 5.03% BD rate saving for RA, X.XX%, X.XX%, X.XX% for LB and 5.16%, 5.93%, 5.19% BD rate saving for AI, for Y, Cb and Cr components respectively.

Table 1 Simulation results

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -5.95% | -4.46% | -3.98% | 131% | 4371% | 0% |
| Class A2 | -5.96% | -5.91% | -3.61% | 120% | 3776% | 0% |
| Class B | -5.51% | -7.19% | -5.43% | 123% | 4311% | 0% |
| Class C | -5.88% | -8.72% | -6.38% | 118% | 3886% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -5.78% | -6.80% | -5.03% | 123% | 4095% | 0% |
| Class D | -7.21% | -8.37% | -6.50% | 115% | 4539% | 1% |
| Class F | -2.80% | -5.78% | -4.42% | 127% | 5012% | 0% |
| Class H | -5.56% | -5.35% | -5.47% | 129% | 5100% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 21% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 28% |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 17% |
| Class C | -5.85% | -9.17% | -5.43% | 116% | 3749% | 2% |
| Class E | -5.53% | -2.65% | -1.13% | 141% | 6732% | 2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 8% |
| Class D | -7.19% | -9.41% | -6.45% | 116% | 4157% | 2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 6% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 26% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.72% | -4.62% | -4.38% | 120% | 2599% | 0% |
| Class A2 | -4.77% | -5.71% | -4.31% | 127% | 2362% | 0% |
| Class B | -4.75% | -6.27% | -5.33% | 115% | 2108% | 0% |
| Class C | -5.01% | -6.74% | -5.41% | 117% | 1832% | 0% |
| Class E | -6.85% | -5.79% | -6.34% | 122% | 2784% | 0% |
| Overall | -5.16% | -5.93% | -5.19% | 119% | 2298% | 0% |
| Class D | -5.37% | -5.25% | -4.83% | 119% | 2134% | 0% |
| Class F | -3.49% | -5.98% | -4.53% | 113% | 2194% | 0% |
| Class H | -5.40% | -4.54% | -3.96% | 134% | 2285% | 0% |

# Training and inference information

Similar to JVET-W0130. The models are trained based on DIV2K[3] and BVI-DVC data set[2]. The raw image/video data was compressed by the base software used for NN technology and AI and RA (BVI-DVC only) configurations are used. In this test, joint models are used for luma and chroma components, 2 models are trained using the AI compressed data and 2 additional models are trained using the RA compressed data. Figure 2 shows the training and validation loss curve.

Figure 2 Training/Validation losses of model 1

More information related to the training & inference of models can be found in the tables below.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | Tensorflow 2.2.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~165 |
|  | Batch size: | 82Kx16 |
|  | Training time: |  |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (is different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size |  |
|  | Learning rate: | 2.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Tensorflow 2.2.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 107670x4 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 0.43x4 |
|  | MAC (Giga) | 3.36e-5 Giga/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 35 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage | 1.43MB (processing unit size 128x128) |
|  | Other information: |  |

# References

- EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer, JVET-W0130, H. Wang, J. Chen, K. Reuzé, A. M. Kotra, M. Karczewicz

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-V2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).