| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0172 |
| --- | --- |

| Title: | On conformance point of high throughput profile |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Kyohei Unno 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes an additional conformance point for high throughput conditions in operation-range extensions profile. The current buffer model with hypothetical reference decoder in VVC is well defined for both theoretical and implementation point-of-view. When the VVC is used for mezzanine compression and/or video editing, decoding latency may be unstable due to the CABAC throughput for some implementation. In this contribution, an additional conformance point for the high throughput cases is proposed based on a bit allocation of each frame.

Problem statement

In the current VVC specification, conformance check is realized by the buffer model with hypothetical reference decoder (HRD) [1]. This model is well defined for a typical bitrate range such as secondary distribution and consumer usage. Various picture structures are also well handled such as IBBP, hierarchical B, low-delay, and so on. Required CABAC throughput is also reasonable for hardware implementation. The bin-to-bit ratio is well controlled at the design level. For the ultra-low delay case, the decoding unit is also defined to reduce the codec delay without a large bit bucket. Theoretically speaking, HRD can work independently to the bitrate.

For the professional user, a required bitrate level may increase because users desire to maintain the content quality as primary distribution or mezzanine compression level. For video editing use-cases, frame-level access is also required. Decoding delay is dominated by CABAC throughput because the following decoding process is independent of the bitrate. A bypass bin mechanism is included in the throughput. When the bit amounts are equally allocated to each frame, a decoding delay is also stable. On the other hand, the unequal bit amounts cause unstable decoding delay. Such a situation is undesirable for professional use cases.

To solve the above situation, an additional conformance point is useful for high throughput cases. Furthermore, required CABAC throughput in the worst case can be reduced than the existing HRD model.

Proposal

In this contribution, we propose an additional conformance point for the bit allocation of each frame. To harmonize the existing HRD model, a difference of buffer amount among several pictures is introduced. The difference is sufficiently small, such pictures can be decoded under a certain latency caused by the CABAC latency.

In detail, the difference is defined as B_tn – B_t0 where B, t0, tn are buffer amounts at the timing t before withdrawing a bit of picture, the current timing, and n-previous picture timing. When the difference is zero or negative value, the average bits of each picture are smaller than that of other periods. Therefore, the difference should be equal to or smaller than a predefined positive threshold.

Figure 1 An example of buffer model with difference d.

This mechanism can be applied not only for the access unit model but also decoding unit. In this case, a buffer model with the predefined threshold is considered as each decoding unit. A proposed specification text will be attached.

Conclusion

In this contribution, the additional conformance point was proposed for high throughput cases. Regarding the conformance check point-of-view, the difference of buffer amount among several pictures was introduced.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- Recommendation ITU-T H.266, "Versatile video coding," in Edition 1.0, August 2020.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).