| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W2022-v1 |
| --- | --- |

| Title: | Core Experiment on Film Grain Synthesis |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Sean McCarthy Miloš Radosavljević Jay Shingala | Email: | sean.mccarthy@dolby.com milos.radosavljevic@interdigital.com jay.shingala@ittiam.com |
| Source: | CE |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to conduct a study of the grain blending processes for the film grain characteristics SEI message proposed at the V and W meetings of JVET. The purpose of the CE is to show that the proposed technologies have comparable performance compared to SMPTE RDD 5, but with lower complexity in implementation.

The software basis for this CE is VTM-13.0. The test sequences, configurations and test conditions are specified in the CE descriptions.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | S. McCarthy | Dolby | sean.mccarthy@dolby.com |
| 2 | M. Radosavljević | InterDigital | milos.radosavljevic@interdigital.com |
| 3 | J. Shingala | Ittiam | jay.shingala@ittiam.com |
| 4 | R.-L. Liao | Alibaba | ruling.lrl@alibaba-inc.com |
| 5 | D. Grois | Comcast | dan_grois@comcast.com |
| 6 | F. Pu | Dolby | Fangjun.pu@dolby.com |
| 7 | P. Yin | Dolby | pyin@dolby.com |
| 8 | W. Husak | Dolby | wjh@dolby.com |
| 9 | E. Francois | InterDigital | Edouard.Francois@InterDigital.com |
| 10 | A. Luthra | Picsel Labs | ajay@picsellabs.com |

# Test conditions and evaluation criteria

The proposals will be tested under the Random Access settings specified in [1]. The QPs and test clips will be specified in each CE description. The anchor is using SMPTE RDD 5 [2] grain blending technology. The test is using specified technology in the CE descriptions.

Planned tests in the CE shall be implemented on VTM-13.0 with the modifications described in references [3], [4] and [5].

Proposals will be evaluated based on subjective and complexity comparison.

- Proposals descriptions

The following contributions have been selected for CE study.

- JVET-V0093 AHG9: Film grain estimation and film grain synthesis for VVC – SEI message characteristics, film grain estimation and film grain synthesis modules [M. Radosavljević, E. François (InterDigital)]

- JVET-W0095 AHG9: Fixed-point grain blending process for film grain characteristics SEI message [S. McCarthy, P. Yin, W. Husak, F. Pu, T. Lu, T. Chen (Dolby), E. François, M. Radosavljević (InterDigital), V. G R, K. Patankar, S. Kadaramandalgi, Ajayshyam (Ittiam)]

JVET-V0093 proposed a software implementation to illustrate the use of the film grain for VVC. Specifically, the software illustrates:

- Film grain characteristics SEI message for VVC specified in H.SEI | ISO/IEC 23002-7;

- Film grain synthesis module based on frequency filtering model specified in SMPTE RDD 5;

- Film grain analysis (parameter estimation) module used to estimate film grain parameters according to SMPTE RDD 5 model.

JVET-V0093 proposed to use the VVC core 64x64 DCT-2 transform instead of the 64x64 transform specified in SMPTE RDD 5.

JVET-W0095 proposed an example of SMPTE RDD 5, Film Grain Technology, as a fixed-point process with support for 4K and 8K, and VVC DCT2. The process is identical to the SMPTE RDD 5 process already referenced in the FGC SEI message, except for two modifications:

- The transformation process as specified in VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4, with trType inferred to equal 0) is used instead of the transform specified in SMPTE RDD 5. The VVC transform is more mature than the SMPTE RDD 5 transform. The VVC transform may also be already available on decoders and may thus be considered implementation friendly.

- For picture resolutions greater than 1920x1080, the grain block size is increased proportionally from the fixed 8x8-size in SMPTE RDD 5 to 16x16 for resolutions up to 3840x2160 (4K) and 32x32 for resolutions greater than 4K including 8K. Use of larger block sizes for larger resolutions reduces implementation complexity and makes the grain pattern block size proportionally consistent across 2k, 4k, and 8k resolutions.

- Background of SMPTE RDD-5

The anchor of the CE is to use grain blending technology specified in SMPTE RDD 5. The below is a short summary. The film grain simulation and blending workflow for SMPTE RDD 5 is illustrated in Figure 1. SMPTE RDD 5 film grain simulation is accomplished by specifying: a database of film grain patterns (Database generation is either performed once per sequence or can be pre-generated and stored); and then for each picture, grain block generation and blending.

Figure 1 Film grain simulation and blending workflow for SMPTE RDD 5

Database generation: for every horizontal cutoff frequency h and vertical cutoff frequency v, Database[h][v][64][64] is created as follows: 1) generate a 64x64 block image from a predefined Gaussian random number LUT, 2) compute a 64x64 inverse integer DCT2 transform on 64x64 block image; 3) deblock horizontal 8x8 block edge.

Grain generation and blending: 1) select grain parameters based on intensity value which is computed based on 8x8 block; 2) create grain block of 8x8 samples from grain database with pseudo random number generator. 3) deblock vertical edges between adjacent blocks. 4) grain blending with decoded 8x8 block.

CE1 replaces SMPTE RDD 5 inverse DCT2 (64x64) transform with VVC inverse DCT2 (64x64) transform for database generation. CE2 applies adaptive grain block size based on picture resolution for intensity value computation and creation of grain block.

- Planned tests

  - Comparison of SMPTE RDD 5 transform and VVC DCT2 (JVET-W0095)

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE1.1 | InterDigital | Dolby |

This test compares the complexity and quality of using the VVC 64x64 DCT2 to the complexity and quality of using the 64x64 transform specified in SMPTE RDD 5.

The anchor of the CE uses SMPTE RDD 5 64x64 transform. The test in CE1.1 uses VVC 64x64 DCT2.

  - Comparison of grain block size (JVET-W0095)

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE2.1 | Dolby | Alibaba |
| CE2.2 | Dolby | InterDigital |

In JVET-W0095, instead of using fixed 8x8 block size for grain block generation and blending, the following changes are proposed.

PicSizeinLumaSamples = PicHeightInLumaSamples * PicWidthInLumaSamples

if PicSizeinLumaSamples <= (1920 * 1080)
    BlockSize = 8
else if PicSizeinLumaSamples <= (3840 * 2160)
    BlockSize = 16
else
    BlockSize = 32

The anchor of the CE is SMPTE RDD 5 with fixed 8x8 grain block size for all resolutions (HD, 4K and 8K, if available). The test of the CE is adaptive grain block size based on picture resolution as described in JVET-W0095. Two sub-CEs are proposed.  CE2.1 is to test adaptive block size with SMPTE RDD 5 DCT2 inverse transform for the grain database generation. CE2.2 is to test adaptive block size with VVC DCT2 inverse transform in CE1 for the grain database generation. The same film grain parameters are applied for both the anchor and the test candidates.

The subjective evaluation is used and complexity comparison is done using worst case multiplications, additions and memory access for different block sizes. Indicative software complexity is also compared by measuring the percentage runtime of the grain blending process relative to the VTM-13.0 decode time

- Timeline and Responsibilities

T1: 2021-July-30:	   				Version of CE description with final descriptions of tests is uploaded. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to marked as "withdrawn".

T2: T1 + 2 weeks 					Iinitial CE test SW

T3: JVET meeting start – 4 weeks : 	Final version of CE tests software and full results are provided, final cross-check begins.

T4: 2021-(TBD at plenary): 			CE contribution documents including specification text and complete test results are uploaded to the JVET document repository. Work on CE report is starting.

- References

- Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li, Vadim Seregin, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Teleconference, 7-16 Oct. 2020.

- RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier, “AHG9: Film grain estimation and film grain synthesis for VVC – Film grain characteristics SEI message characteristics, film grain estimation and film grain synthesis modules”, JVET-V0093, Teleconference, 20-28 April, 2021.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy and Guillaume Gautier, “AHG9: Enhancement of film grain parameter estimation for different intensity intervals”, JVET-W0072, Teleconference, 7-16 July, 2021.

- Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen, Edouard François, Miloš Radosavljević, Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam, “AHG9: Fixed-point grain blending process for film grain characteristics SEI message”, JVET-W0095, Teleconference, 7-16 July, 2021.