| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Leannec Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com fabrice.leleannec@interdigital.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 23rd and 24th JVET meetings.

# Introduction

The mandates for the AHG are as follows.

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-2.0 software version and the reference configuration encodings according to JVET-W2017 common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software was decoupled from VTM and a new repository was created located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

The following adopted aspects were integrated into ECM-2.0:

JVET-W0066: Cross-component SAO (CCSAO)

JVET-W0069: TMP boundary handling

JVET-W0090: Adaptive reordering of merge candidates with template matching (ARMC)

JVET-W0097: GPM with MMVD and template matching GPM

JVET-W0103: Extended intra MTS

JVET-W0119: LFNST extension with large kernel (test 4.6)

JVET-W0123: TIMD with fusion

Software improvements and bug fixes:

Align config settings with VTM related to https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/1439

Fix OBMC memory

Fix a crash with disabled TM_AMVP macro

Fix ECM encoder crash on small sequences

Add initialization for TU 256

Bug fix for bilateral filter when LMCS is disabled.

ECM-2.0 was tagged on August 2, 2021.

## CTC Performance

In this section, ECM-2.0 test results following ECM CTC configuration descried in JVET-W2017 are summarized.

The following tables show ECM-2.0 performance over ECM-1.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.04% | -1.43% | -2.23% | 140% | 109% |
| Class A2 | -1.01% | -2.27% | -0.94% | 139% | 113% |
| Class B | -0.97% | -3.01% | -3.99% | 137% | 116% |
| Class C | -0.72% | -1.49% | -1.62% | 133% | 123% |
| Class E | -1.10% | -2.01% | -2.79% | 131% | 119% |
| Overall | -0.96% | -2.12% | -2.46% | 136% | 116% |
| Class D | -0.88% | -1.53% | -1.07% | 132% | 128% |
| Class F | -0.55% | -1.97% | -2.33% | 114% | 110% |
| Class TGM | -1.08% | -1.61% | -1.63% | 110% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.45% | -3.15% | -5.45% | 123% | 111% |
| Class A2 | -1.07% | -3.59% | -2.70% | 120% | 107% |
| Class B | -1.07% | -5.84% | -6.26% | 131% | 114% |
| Class C | -1.26% | -3.64% | -3.56% | 132% | 117% |
| Class E |  |  |  |  |  |
| Overall | -1.20% | -4.26% | -4.66% | 127% | 113% |
| Class D | -1.08% | -3.33% | -2.88% | 128% | 118% |
| Class F | -1.22% | -3.94% | -3.77% | 126% | 126% |
| Class TGM | -2.02% | -3.79% | -4.08% | 133% | 121% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.86% | -12.85% | -12.12% | 127% | 114% |
| Class C | -0.87% | -7.38% | -6.20% | 130% | 113% |
| Class E | -1.46% | -4.02% | -6.78% | 133% | 123% |
| Overall | -1.01% | -8.82% | -8.81% | 130% | 116% |
| Class D | -0.66% | -6.60% | -5.70% | 125% | 113% |
| Class F | -0.84% | -7.38% | -7.04% | 125% | 134% |
| Class TGM | -3.45% | -8.88% | -8.91% | 136% | 131% |

Next tables show ECM-2.0 performance over VTM-11.0 + JVET-V0056 (https://vcgit.hhi.fraunhofer.de/chollmann/VVCSoftware_VTM/-/tree/MCTF_VTM11) anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.76% | -10.85% | -12.55% | 306% | 235% |
| Class A2 | -6.43% | -9.83% | -6.78% | 294% | 226% |
| Class B | -5.92% | -9.95% | -11.25% | 337% | 248% |
| Class C | -6.73% | -8.79% | -9.19% | 329% | 243% |
| Class E | -7.23% | -9.70% | -9.20% | 329% | 286% |
| Overall | -6.54% | -9.78% | -9.92% | 321% | 247% |
| Class D | -5.70% | -7.02% | -6.59% | 332% | 256% |
| Class F | -10.50% | -13.32% | -14.04% | 244% | 285% |
| Class TGM | -15.50% | -17.44% | -17.29% | 233% | 290% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -13.50% | -15.91% | -20.31% | 342% | 504% |
| Class A2 | -14.37% | -17.39% | -16.47% | 321% | 584% |
| Class B | -12.47% | -17.52% | -17.43% | 355% | 548% |
| Class C | -14.37% | -16.46% | -16.52% | 351% | 488% |
| Class E |  |  |  |  |  |
| Overall | -13.56% | -16.89% | -17.57% | 345% | 529% |
| Class D | -15.35% | -16.36% | -15.88% | 358% | 530% |
| Class F | -13.20% | -16.71% | -16.88% | 319% | 438% |
| Class TGM | -14.41% | -17.97% | -18.41% | 325% | 307% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -10.33% | -21.17% | -20.57% | 301% | 326% |
| Class C | -11.78% | -16.02% | -16.53% | 326% | 286% |
| Class E | -10.56% | -13.79% | -14.32% | 266% | 296% |
| Overall | -10.87% | -17.61% | -17.66% | 300% | 305% |
| Class D | -13.96% | -17.39% | -17.06% | 321% | 298% |
| Class F | -11.65% | -16.78% | -16.78% | 291% | 311% |
| Class TGM | -13.73% | -20.40% | -21.15% | 260% | 244% |

# Recommendations

The AHG recommends to:

- Continue to develop ECM software

- Improve software documentation

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues

- Encourage people to submit merge requests fixing identified bugs