| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0008-v2 |
| --- | --- |

| Title: | JVET AHG Report: High bit depth, high bit rate, and high frame rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomohiro Ikai Dmytro Rusanovskyy Mohammed Golam Sarwer Xiaoyu Xiu Yue Yu | Email: | adrian.browne@sony.com ikai.tomohiro@sharp.co.jp dmytror@qti.qualcomm.com m.sarwer@alibaba-inc.com xiaoyuxiu@kwai.com yue.yu@oppo.com |
| Source: | AHG8 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG8: High bit depth, high bit rate, and high frame rate coding between the 23rd meeting by teleconference (7-16 July 2021) and the 24th meeting by teleconference (6-15 October 2021).

# Mandates

The AHG was established with the following mandates:

- Study the benefits and characteristics of VVC coding tools for high bit depth, high bit rate, and high frame rate coding.

- Study lossless coding characteristics of VVC.

- Identify technologies for future extension of VVC to support such application usage.

- Study the JVET-U2018 testing conditions for high bit depth, high bit rate, and high frame rate coding, and suggest improvements as applicable.

- Contribute to the development of conformance testing for operation range extensions in coordination with AHG5.

- Identify suitable test material for testing of high bit depth, high bit rate, and high frame rate coding in coordination with AHG4 and AHG7.

- Study VVC entropy decoding throughput and latency in the cases of high bit depth, high bit rate, and high frame rate coding.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG8] in message headers, but no correspondence marked as AHG8 was sent between the 23rd and 24th meetings.

The major area of work for the AHG in this meeting cycle has been the development of conformance testing. A revision of JVET-W2026 “Conformance testing for VVC operation range extensions (Draft 1)” was uploaded on 1st September and a notification was sent to the reflector. The AHG would like to thank Iole Moccagatta for assistance with these activities.

In total there are 19 high bit depth related contributions. The following section lists these contributions.

# Contributions

The contributions can be split into two sections. There are 9 contributions related to high bit depth profiles and HLS, and 5 contributions are related to tools for high bit depths. In addition, 5 cross-checks have been registered.

## Profiles / HLS

JVET-X0050	“AHG2: On editing notes for the 2nd edition draft text of VVC”, Y.-K. Wang (Bytedance), M. Zhou (Broadcom)

JVET-X0073	“AHG2: On specifying the range extensions profiles”, Y.-K. Wang (Bytedance)

JVET-X0075	“AHG8: Level refinement for VVC operation range extension profiles”, T. Ikai, T. Chujoh, T. Aono (Sharp)

JVET-X0076	“AHG8: GCI flags for VVC operation range extension profiles”, T. Ikai, T. Chujoh, T. Aono (Sharp)

JVET-X0093	“AHG2/AHG8: Comments on VVC operation range extensions”, B. Choi, S. Wenger, S. Liu (Tencent)

JVET-X0095	“AHG8: General Constraints Information (GCI) flags for operation range extensions”, B. Choi, S. Wenger, S. Liu (Tencent)

JVET-X0106	“On constraints for intra profiles in VVC”, T. Tsukuba, S. Keating (Sony)

JVET-X0108	“Coded Picture Buffer sizes and MinCr for high bit-depth profiles”, S. Keating, A. Browne, K. Sharman (Sony)

JVET-X0109	“On maximum bit-rates for high bit-depth profiles”, S. Keating, A. Browne, K. Sharman (Sony)

## Tools

JVET-X0127	“AHG8: Modification of History Based Rice Parameter Derivation”, Y. Yu, H. Yu, Z. Xie, F. Wang, L. Xu, D. Wang(OPPO)

JVET-X0128	“AHG8: On History-Based Rice Parameter Derivations for Wavefront Parallel Processing”, Y. Yu, H. Yu, Z. Xie, F. Wang, L. Xu, D. Wang(OPPO)

JVET-X0129	“AHG8: Independent Rice Parameter Derivation for high bit depth and high bit rate extensions”, Y. Yu, H. Yu, Z. Xie, F. Wang, L. Xu, D. Wang(OPPO)

JVET-X0136	“AHG8: On significance, GT1, and GT2 flag coding for high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

JVET-X0137	“AHG8 and AHG10: On derivation of sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1”, A. Browne, S. Keating, K. Sharman (Sony)

## Cross-checks

JVET-X0157	“Cross-check report of JVET-X0127: AHG8: Modification of History Based Rice Parameter Derivation”, D. Rusanovskyy

JVET-X0158	“Cross-check report of JVET-X0128: AHG8: On History-Based Rice Parameter Derivations for Wavefront Parallel Processing”, D. Rusanovskyy

JVET-X0159	“Cross-check report of JVET-X0129: AHG8: Independent Rice Parameter Derivation for high bit depth and high bit rate extensions”, D. Rusanovskyy

JVET-X0173	“Cross-check of JVET-X0136: AHG8: On significance, GT1, and GT2 flag coding for high bit depths”, Z. Xie, Y. Yu (OPPO)

JVET-X0174	“Cross-check of JVET-X0137: AHG8 and AHG10: On derivation of sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1”, Z. Xie, Y. Yu (OPPO)

# Benchmarks

## VTM 14.0 vs VTM13.0

### Standard QP Range

The standard QP range is from QP 22 to QP 37. The most significant changes for this QP range were the adoption of JVET-W0129 which enables using true original samples for ALF, and the adoption of JVET-W0136 which disables the use of VVC v2 coding tools for the Main 12 profile which is automatically selected for these simulations.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.20% | -0.21% | -0.18% | 0.06% | 0.55% | -0.15% | 0.08% | 0.50% | 103% | 102% |
| Class H2 |  |  |  |  |  | -0.12% | 0.03% | 0.38% | 102% | 102% |
| Overall | 0.20% | -0.21% | -0.18% | 0.06% | 0.55% | -0.13% | 0.06% | 0.44% | 103% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 98% | 99% |
| Class H2 |  |  |  |  |  | 0.02% | 0.03% | 0.03% | 98% | 99% |
| Overall | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 98% | 99% |

### Low QP Range

The low QP range is from QP -13 to QP 12 for 12 bit processing and QP -33 to QP -8 for 16 bit processing. The gains in this QP range can be attributed to the adoption of JVET-W0046 which allows the signalling of the last significant coefficient position to be reversed.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.03% | -0.03% | -0.02% | -0.03% | -0.03% | 101% | 100% |
| PQ422 | -0.03% | -0.05% | -0.06% | -0.03% | -0.05% | -0.06% | 99% | 98% |
| Overall | -0.02% | -0.04% | -0.05% | -0.02% | -0.04% | -0.05% | 100% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.04% | 0.02% | 0.00% | 0.03% | 0.01% | 100% | 98% |
| PQ422 | -0.02% | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Overall | -0.01% | 0.02% | 0.01% | -0.01% | 0.02% | 0.01% | 100% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | -0.01% | 0.00% | 0.02% | -0.01% | 0.00% | 100% | 101% |
| PQ422 | 0.00% | 0.01% | 0.04% | 0.00% | 0.01% | 0.03% | 100% | 100% |
| Overall | 0.01% | 0.00% | 0.02% | 0.01% | 0.00% | 0.02% | 100% | 100% |
| Overall PQ | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | 100% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.21% | -0.24% | -0.30% | 101% | 98% |
| HLG422 | -0.23% | -0.27% | -0.32% | 101% | 98% |
| Overall | -0.22% | -0.25% | -0.31% | 101% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.17% | -0.16% | -0.24% | 102% | 101% |
| HLG422 | -0.20% | -0.20% | -0.32% | 101% | 100% |
| Overall | -0.18% | -0.18% | -0.28% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.17% | -0.16% | -0.23% | 101% | 100% |
| HLG422 | -0.22% | -0.22% | -0.35% | 101% | 100% |
| Overall | -0.19% | -0.19% | -0.29% | 101% | 100% |
| Overall HLG | -0.20% | -0.21% | -0.29% | 101% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.29% | -0.28% | -0.28% | 99% | 98% |
| SVT12 | -0.33% | -0.29% | -0.29% | 99% | 99% |
| Overall | -0.31% | -0.28% | -0.29% | 99% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.28% | -0.26% | -0.26% | 104% | 102% |
| SVT12 | -0.29% | -0.26% | -0.26% | 103% | 100% |
| Overall | -0.28% | -0.26% | -0.26% | 104% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.27% | -0.25% | -0.25% | 103% | 101% |
| SVT12 | -0.26% | -0.23% | -0.23% | 101% | 100% |
| Overall | -0.27% | -0.24% | -0.24% | 102% | 100% |
| Overall RGB | -0.29% | -0.26% | -0.26% | 102% | 100% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM 13.0 | VTM 14.0 |  | VTM 13.0 | VTM 14.0 |  | VTM 13.0 | VTM 14.0 |  |
| PQ444 | 2.6 | 2.6 | -0.32% | 3.1 | 3.1 | -0.41% | 3.1 | 3.1 | -0.39% |
| PQ422 | 2.4 | 2.4 | -0.30% | 2.9 | 2.9 | -0.44% | 2.9 | 2.9 | -0.42% |
| Overall | 2.5 | 2.5 | -0.31% | 3.0 | 3.0 | -0.42% | 3.0 | 3.0 | -0.41% |
| Enc Time[%] | 104% |  |  | 99% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 97% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM 13.0 | VTM 14.0 |  | VTM 13.0 | VTM 14.0 |  | VTM 13.0 | VTM 14.0 |  |
| HLG444 | 1.8 | 1.8 | -0.31% | 2.0 | 2.0 | -0.43% | 2.0 | 2.0 | -0.42% |
| HLG422 | 1.7 | 1.7 | -0.29% | 1.9 | 1.9 | -0.47% | 1.9 | 1.9 | -0.46% |
| Overall | 1.7 | 1.7 | -0.30% | 1.9 | 2.0 | -0.45% | 1.9 | 2.0 | -0.44% |
| Enc Time[%] | 102% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  | 101% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM 13.0 | VTM 14.0 |  | VTM 13.0 | VTM 14.0 |  | VTM 13.0 | VTM 14.0 |  |
| SVT16 | 1.2 | 1.2 | -0.15% | 1.2 | 1.2 | -0.20% | 1.2 | 1.2 | -0.19% |
| SVT12 | 1.3 | 1.3 | -0.25% | 1.3 | 1.3 | -0.26% | 1.3 | 1.3 | -0.26% |
| Overall | 1.2 | 1.2 | -0.20% | 1.3 | 1.3 | -0.23% | 1.3 | 1.3 | -0.22% |
| Enc Time[%] | 100% |  |  | 103% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 102% |  |  |

## VTM 14.0 vs HM 16.23

### Standard QP Range

The standard QP range is from QP 22 to QP 37. HM uses a GOP size of 16 whilst VTM uses a GOP size of 32 for this QP range.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM16.23 |  |  |  |  |
|  | PSNR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H1 | -30.26% | -69.93% | -79.50% | 367% | 176% |
| Class H2 | -33.60% | -57.88% | -56.84% | 356% | 169% |
| Overall | -31.93% | -63.90% | -68.17% | 362% | 173% |
|  | All Intra |  |  |  |  |
|  | Over HM16.23 |  |  |  |  |
|  | PSNR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H1 | -11.85% | -67.95% | -54.93% | 2488% | 184% |
| Class H2 | -22.45% | -49.09% | -49.93% | 2965% | 171% |
| Overall | -17.15% | -58.52% | -52.43% | 2716% | 178% |

### Low QP Range

The low QP range is from QP -13 to QP 12 for 12 bit processing and QP -33 to QP -8 for 16 bit processing. Both HM and VTM use a GOP size of 16 for this QP range.

| HDR PQ |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -7.30% | -8.46% | -9.19% | 3249% | 168% |
| PQ422 | -9.97% | -14.18% | -14.80% | 2714% | 161% |
| Overall | -8.63% | -11.32% | -12.00% | 2981% | 165% |
|  |  |  | LDB |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.73% | -4.97% | -6.61% | 313% | 165% |
| PQ422 | -7.96% | -11.75% | -12.42% | 413% | 164% |
| Overall | -7.34% | -8.36% | -9.52% | 363% | 164% |
|  |  |  | RA |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.86% | -5.17% | -6.48% | 361% | 165% |
| PQ422 | -8.21% | -12.15% | -12.42% | 500% | 163% |
| Overall | -7.54% | -8.66% | -9.45% | 431% | 164% |
| Overall PQ | -7.84% | -9.45% | -10.32% | 1258% | 164% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM 16.23 | VTM 14.0 |  | HM 16.23 | VTM 14.0 |  | HM 16.23 | VTM 14.0 |  |
| PQ444 | 2.5 | 2.6 | -1.58% | 3.1 | 3.1 | 1.18% | 3.1 | 3.1 | 0.96% |
| PQ422 | 2.3 | 2.4 | -2.72% | 2.9 | 2.9 | -0.21% | 2.9 | 2.9 | -0.36% |
| Overall | 2.4 | 2.5 | -2.15% | 3.0 | 3.0 | 0.49% | 3.0 | 3.0 | 0.30% |
| Enc Time[%] | 7610% |  |  | 1063% |  |  | 1100% |  |  |
| Dec Time[%] | 171% |  |  | 160% |  |  | 159% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM 16.23 | VTM 14.0 |  | HM 16.23 | VTM 14.0 |  | HM 16.23 | VTM 14.0 |  |
| HLG444 | 1.8 | 1.8 | -0.95% | 1.9 | 2.0 | -0.76% | 2.0 | 2.0 | -0.83% |
| HLG422 | 1.7 | 1.7 | -1.40% | 1.9 | 1.9 | -1.11% | 1.9 | 1.9 | -1.18% |
| Overall | 1.7 | 1.7 | -1.17% | 1.9 | 2.0 | -0.94% | 1.9 | 2.0 | -1.00% |
| Enc Time[%] | 8084% |  |  | 1119% |  |  | 1118% |  |  |
| Dec Time[%] | 164% |  |  | 148% |  |  | 150% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM 16.23 | VTM 14.0 |  | HM 16.23 | VTM 14.0 |  | HM 16.23 | VTM 14.0 |  |
| SVT16 | 1.2 | 1.2 | -0.43% | 1.2 | 1.2 | -0.01% | 1.2 | 1.2 | -0.06% |
| SVT12 | 1.3 | 1.3 | -0.75% | 1.3 | 1.3 | -0.21% | 1.3 | 1.3 | -0.27% |
| Overall | 1.2 | 1.2 | -0.59% | 1.3 | 1.3 | -0.11% | 1.3 | 1.3 | -0.16% |
| Enc Time[%] | 8781% |  |  | 1158% |  |  | 1139% |  |  |
| Dec Time[%] | 142% |  |  | 149% |  |  | 150% |  |  |

- Recommendations

The AHG recommends the following:

- To review all related contributions;

- To continue high bit depth, high bit rate, and high frame rate studies.