| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li K. Naser H. Yang | Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@tencent.com karam.naser@interdigital.com haitao.yang@huawei.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 23rd meeting by teleconference, 7-16 July 2021, and the 24th meeting by teleconference, 6–15 October 2021.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM2 algorithm description JVET-W2025.

- Study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Refine test conditions in JVET-W2017, generate anchors, identify new test sequences to be added, especially high resolution ones in 8K, in coordination with AHG4.

- Analyse the results of exploration experiments described in JVET-W2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The Common Test Conditions were updated to reduce simulation time for low delay configuration scenarios (JVET-W2017). The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-W2024). The indication of the achievable improvement over VVC are the results of the EE2 software base (ECM-2.0) as agreed at JVET-W meeting. The combined improvement of the ECM-2.0 over VTM-11.0 with the improved MCTF from JVET-V0056 for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.76% | -10.85% | -12.55% | 306% | 235% |
| Class A2 | -6.43% | -9.83% | -6.78% | 294% | 226% |
| Class B | -5.92% | -9.95% | -11.25% | 337% | 248% |
| Class C | -6.73% | -8.79% | -9.19% | 329% | 243% |
| Class E | -7.23% | -9.70% | -9.20% | 329% | 286% |
| Overall | -6.54% | -9.78% | -9.92% | 321% | 247% |
| Class D | -5.70% | -7.02% | -6.59% | 332% | 256% |
| Class F | -10.50% | -13.32% | -14.04% | 244% | 285% |
| Class TGM | -15.50% | -17.44% | -17.29% | 233% | 290% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -13.50% | -15.91% | -20.31% | 342% | 504% |
| Class A2 | -14.37% | -17.39% | -16.47% | 321% | 584% |
| Class B | -12.47% | -17.52% | -17.43% | 355% | 548% |
| Class C | -14.37% | -16.46% | -16.52% | 351% | 488% |
| Class E |  |  |  |  |  |
| Overall | -13.56% | -16.89% | -17.57% | 345% | 529% |
| Class D | -15.35% | -16.36% | -15.88% | 358% | 530% |
| Class F | -13.20% | -16.71% | -16.88% | 319% | 438% |
| Class TGM | -14.41% | -17.97% | -18.41% | 325% | 307% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -10.33% | -21.17% | -20.57% | 301% | 326% |
| Class C | -11.78% | -16.02% | -16.53% | 326% | 286% |
| Class E | -10.56% | -13.79% | -14.32% | 266% | 296% |
| Overall | -10.87% | -17.61% | -17.66% | 300% | 305% |
| Class D | -13.96% | -17.39% | -17.06% | 321% | 298% |
| Class F | -11.65% | -16.78% | -16.78% | 291% | 311% |
| Class TGM | -13.73% | -20.40% | -21.15% | 260% | 244% |

# Contributions

In addition to thirteen EE2 contributions, 40 related contributions were received which can be subdivided as follows:

## In Loop Filters (3)

JVET-X0086, "EE2-related: Adaptive Filter Shape Selection for ALF", W. Yin, K. Zhang, L. Zhang (Bytedance), N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-X0105, "AHG12: Edge Classifier for Cross-component Sample Adaptive Offset (CCSAO)”, A. M. Kotra, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-X0152, "AHG12: CCSAO classification with edge information", C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, N. Yan, X. Wang (Kwai)

## Intra (14)

JVET-X0072, "EE2-related: PDPC-skip scheme for angular intra modes", Jinrong Zhang, Chuan Zhou, Zhuoyi Lv (Vivo)

JVET-X0099, "Non-EE2: Extension of TIMD to intra chroma coding", Y. Wang, K. Zhang, L. Zhang, H. Liu (Bytedance)

JVET-X0100, "EE2-related: On propagating intra prediction mode for IBC", L.-F. Chen, X. Li, L. Li, S. Liu (Tencent)

JVET-X0114, "EE2-related: Fix on issues of TIMD mode", C. Zhou, Z. Lv, J. Zhang (vivo)

JVET-X0115, "EE2-related: Optimization on the second mode derivation of DIMD blending mode", C. Zhou, Z. Lv, J. Zhang (vivo)

JVET-X0122, "Non-EE2: Unification of negative modes processing in TIMD", A. Filippov, V. Rufitskiy, K. Goswami, D. Ruiz Coll, Y.Y. Lee, T.M. Bae, E. Dinan (Ofinno)

JVET-X0124, "AHG12: On signalling of intra template matching", Z. Xie, Y. Yu, H. Yu, L. Xu, F. Wang, D. Wang (OPPO)

JVET-X0131, "Non-EE2: Low-Complexity Improvements of Intra Coding for Screen Content", D. Ruiz Coll, T.M. Bae, A. Filippov, V. Rufitskiy, Y.Y. Lee, K. Goswami (Ofinno)

JVET-X0135, "Non-EE2: Adaptive intra MTS",	B. Ray, V. Seregin, M. Karczewicz (Qualcomm)

JVET-X0139, "AHG12: removing a discontinuity in the discrete angle comparison in DIMD", T. Dumas, P. Bordes, F. Galpin, F. Le Leannec (InterDigital)

JVET-X0142, "Non-EE2: Extended MRL candidate list", K. Cao, Y.-J. Chang, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)

JVET-X0148, "AHG12: On the PDPC handling in DIMD and TIMD", H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, N. Yan, X. Wang (Kwai)

JVET-X0149, "AHG12: Removal of floating operations in DIMD and TIMD", X. Xiu, J.-H. Jhu, W. Chen, C.-W. Kuo, N. Yan, Y.-W. Chen, X. Wang (Kwai)

JVET-X0156, "Non-EE2: Fix on issues of DIMD Histogram of Gradient generation", K. Cao, V. Seregin, M. Karczewicz (Qualcomm)

## Inter (20)

JVET-X0056, "EE2-Related: Complexity reduction for decoder side motion derivation", H. Huang, V. Seregin, Z. Zhang, C.-C. Chen, M. Karczewicz (Qualcomm)

JVET-X0058, "AHG12: Bilateral matching SMVD mode", D. Kim, H. Kwon, J. Kim, W. Lim, S. Y. Jeong (ETRI)

JVET-X0078, "EE2-related: Modified GPM with inter and intra prediction", Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-X0085, "Non-EE2: Template Matching-based Reordering for Extended MMVD Design", M. Salehifar, Y. He, K. Zhang, N. Zhang, L. Zhang (Bytedance)

JVET-X0087, "Non-EE2: Template Matching Based Merge Candidate List Construction (TM-MCLC)", L. Zhao, K. Zhang, N. Zhang, L. Zhang (Bytedance)

JVET-X0088, "Non-EE2: History-based Affine Model Inheritance", K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang (Bytedance)

JVET-X0089, "Non-EE2: Modifications of IBC Merge/AMVP List Construction", N. Zhang, K. Zhang, L. Zhang, J. Xu (Bytedance)

JVET-X0090, "Non-EE2: On combination of CIIP, OBMC and LMCS",	R.-L. Liao, X. Li, J. Chen, Y. Ye (Alibaba)

JVET-X0091, "Non-EE2: On TMVP improvement", R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)

JVET-X0119, "Non-EE2: On pairwise merge candidate", Laroche G., Onno P., Bellessort R. (Canon)

JVET-X0121, "EE2-related: bug fixes for enabling RPR in ECM", P. Bordes, F. Galpin, F. Leleannec, E. Francois (InterDigital)

JVET-X0132, "EE2-Related: On MVD sign prediction", Y. Zhang, B. Ray, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-X0133, "Non-EE2: MV candidate type-based ARMC", Y.-J. Chang, H. Huang, V. Seregin, C.-C. Chen, M. Karczewicz (Qualcomm)

JVET-X0134, "Non-EE2: On the number of TM merge candidates", Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-X0141, "EE2-3.1-related: CIIP with template matching", Z. Deng, K. Zhang, L. Zhang (Bytedance), X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-X0145, "EE2-related: Template matching CIIP on top of EE2-3.1", Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-X0146, "Non-EE2: Decoder side motion derivation using sample's spatial correlation", H. Huang, V. Seregin, C.-C. Chen, M. Karczewicz (Qualcomm)

JVET-X0147, "EE2-related: intra mode derivation based on TIMD for GPM inter/intra", H. Jang, S. Kim, J. Lim (LGE), Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-X0151, "AHG12: Non-adjacent spatial neighbors for affine merge mode", W. Chen, X. Xiu, Y.-W. Chen, H.-J. Jhu, C.-W. Kup, N. Yan, X. Wang (Kwai)

JVET-X0166, "EE2-related: Combination of JVET-X0078 (Test 7/8), JVET-X0147 (Proposal-2), and GPM direct motion storage", Y. Kidani, H. Kato, K. Kawamura (KDDI), H. Jang, S. Kim, J. Lim (LGE), Z. Deng, K. Zhang, L. Zhang (Bytedance)

## Coefficient Coding (2)

JVET-X0120, "AHG12: On sign prediction", M. G. Sarwer, Y. Yan, J. Chen, R. -L. Liao (Alibaba)

JVET-X0150, "AHG12: Enhanced sign prediction", X. Xiu, Y.-W. Chen, N. Yan, C.-W. Kuo, H.-J. Jhu, W. Chen, X. Wang (Kwai)

## Test Conditions (1)

JVET-X0138, “Proposal for a new Low Latency & Controlled Complexity (LLCC) common test conditions", Gaelle Martin-Cocher, Tangi Poirier, Saurabh Puri (Interdigital)

# Recommendations

The AHG recommends to:

- To review all the related contributions.

- To encourage contribution of new test sequences, especially 8k resolution, which might be included in future test conditions.