| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0041 |
| --- | --- |

| Title: | AHG11: CNN-based In-Loop Filter with Knowledge Distillation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyun-Ku Kang, Dong-Wook Kim, Seung-Won Jung, Hyoungjin Kwon, Se-Yoon Jeong | Tel: Email: | hyunku96@korea.ac.kr kimdongwook@korea.ac.kr swjung83@korea.ac.kr kwonjin@etri.re.kr jsy@etri.re.kr |
| Source: | Korea University, Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network (CNN)-based in-loop filtering method with knowledge distillation (KD). The proposed KD-based training strategy for in-loop filtering CNN enables a small-sized student network to perform favorably to a large-size teacher network. Compared with VTM-11.0-NNVC, the proposed method shows on average {7.43%, 9.71%, and 12.76%}, {5.91%, 9.52%, and 9.97%}, and {4.93%, -0.06%, and -0.50%} BD-rate reductions for {Y, Cb, Cr} on C and D classes, under AI, RA, and LDB configurations, respectively.

Introduction

In JVET-W0011 [1], most of the neural network-based in-loop filtering methods reported enormous decoding time, which may hinder the deployment of neural network-based video coding standards. Meanwhile, KD has shown its feasibility for model compression while preserving the performance of the compressed model. This contribution uses CNN as an in-loop filter with a small-size student network rather than a large-size teacher network via KD. Furthermore, dedicated networks are trained for different types of slices and quantization parameters (QP).

Proposed method

Architecture

The architecture of the proposed CNN filter is shown in Fig.1. The backbone network is based on grouped residual dense network (GRDN) [2] [3], consisting of 4 grouped residual dense blocks (GRDBs) containing N residual dense blocks (RDBs). Each RDB has M cascaded convolutional layers followed by the ReLU activation layer. Convolutional block attention module (CBAM) [4] is adopted for emphasizing spatial and channel-wise feature refinement.

The input size of the proposed CNN filter is equal to the size of the reconstructed frame before in-loop filtering process. The CNN filter restores the reconstructed input frame for each component (Y, U, V).

In KD, a large network, called teacher, transfers its knowledge to a small network, called student. Therefore, the teacher and the student with different block sizes are used during training. In this contribution, the teacher has 4 RDBs (N=4) in each GRDB and 8 convolution layers (M=8) in each RDB. On the other hand, the student has 2 RDBs (N=2) and 4 convolution layers (M=4) in each RDB. Both teacher and student have convolution layers with 64 channels. With this network design, the number of parameters of student is about seven times less than

Fig. 1. (a) Architecture of GRDN [2] [3]. GRDN consists of 4 GRDBs. Conv denotes 3ⅹ3 convolution layer. (b) Architecture of GRDB. GRDB contains N RDBs. 1ⅹ1 Conv denotes 1ⅹ1 convolution. (c) Architecture of RDB. Each Conv has 64 channels.

the one of the teacher’s parameters.

Knowledge Distillation

For training the proposed CNN filter, KD is utilized to enable student to learn knowledge from teacher. The overall training process is depicted in Fig. 2. The proposed training strategy with KD consists of two training rounds.

LAT=i QTi-QSi2,		                   (1)

At the first training round, the teacher is pre-trained. Then, the student is trained using the attention transfer (AT) loss [5]. Specifically, intermediate features of both student and teacher are averaged along the channel axis to obtain one-channel attention maps. For KD, the student is trained to mimic the attention map obtained from the teacher. Features after every GRDB are mapped to an attention map for effective KD. The objective for this first round is formulated below.

where Q is the attention map, i denotes an attention map index, and the subscripts T and S indicate the teacher and the student, respectively.

LFinetune=LReconstruction+α* LImitation.		    (2)

At the second training round, the first trained student is fine-tuned using the output of teacher as well as the original frame as a supervision signal. Both supervisions are utilized to measure L1 loss, each of which is named as imitation loss (w.r.t. output of teacher) and reconstruction loss (w.r.t. original frame), respectively. The objective for this second training round is formulated below.

The effect of the two loss terms is adjusted using a hyperparameter α, which is empirically set to 1.

Fig. 2. (a) First round of training. Student is supervised by attention map from teacher. (b) Second round of training. Pre-trained student is supervised not only from the original but also from the output of teacher.

Inference

Different CNN filters are selected based on coding condition. Filters are dedicated to QPs = {22, 27, 32, 37, 42}, color components = {Luma, Chroma}, and slice types = {IntraA, IntraB, Inter}. Because of encoding QP fluctuation, low delay B (LDB) and random access (RA) utilize Intra slice filters trained with different QPs = {19, 24, 29, 34, 39} for each QP configurations. CTU level on/off control is conducted based on the rate-distortion cost at the encoder side. The flag of on/off control is signaled into the bitstream.

LibTorch is used for performing the inference of the proposed CNN filters in VTM. Only CPU is used during performing the inference of the proposed CNN filters. The network information in the inference stage is provided in Table 1, as suggested by [6].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | LibTorch v1.8.1 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 3.28M/model |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | ~13.3M/model，30 models in total |
|  | MAC (Giga) | 869.94K/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training framework. BVI-DVC [7] dataset is adopted to train the CNN filters of I slices and B slices, Luma and Chroma components, respectively. Different CNN filters are trained to adapt to different QP points. The network information in the training stage is provided in Table 2, as suggested by [6].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: TITAN RTX D6 24GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 40+40 |
|  | Batch size: | 16 |
|  | Training time: | 120h/model |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 96×96 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is evaluated on top of VTM-11.0-NNVC [8] according to the common test conditions defined in [6]. Deblocking filter is disabled while SAO and ALF (and CCALF) are placed after the proposed CNN-based filtering.

Fig. 3. (a) Teacher and student (without KD) networks are trained separately. (b) Proposed student network is trained by KD.

Experimental results are shown in Tables 3, 4, and 5. For the ablation study, the results of the teacher and the student without KD (trained from scratch) are evaluated in Tables 4 and 5, respectively. Fig. 3 depicts training settings for each experiment.

Under AI configurations, the proposed method can bring 7.43%, 9.71%, and 12.76% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 5.91%, 9.52%, and 9.97% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring 4.93%, -0.06%, and -0.50% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 3. Performance of the student network with KD on top of VTM-11.0-NNVC

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -7.39% | -10.65% | -13.41% | 106% | 5953% | 1% |
| Class D | -7.47% | -8.78% | -12.11% | 106% | 7667% | 1% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -5.03% | -10.33% | -10.94% | 118% | 8798% | 0% |
| Class D | -6.79% | -8.72% | -9.01% | 108% | 17483% | 1% |

|  | Low Delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -4.65% | -4.84% | -3.19% | 119% | 9066% | 1% |
| Class D | -5.22% | 4.72% | 4.20% | 109% | 16848% | 1% |

Table 4. Performance of the teacher network on top of VTM-11.0-NNVC

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -8.44% | -10.95% | -13.75% | 124% | 21273% | 1% |
| Class D | -8.25% | -8.87% | -12.45% | 129% | 27823% | 1% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -5.55% | -10.51% | -11.47% | 145% | 30519% | 0% |
| Class D | -7.24% | -8.79% | -9.67% | 145% | 62693 | 1% |

|  | Low Delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -5.14% | -4.57% | -3.78% | 145% | 30136% | 1% |
| Class D | -5.71% | 4.17% | 4.72% | 144% | 59902% | 2% |

Table 5. Performance of the student without KD on top of VTM-11.0-NNVC

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -6.51% | -8.51% | -11.40% | 106% | 5409% | 1% |
| Class D | -6.81% | -6.67% | -9.70% | 105% | 7068% | 1% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -4.35% | -8.88% | -9.54% | 119% | 8648% | 0% |
| Class D | -5.98% | -7.77% | -7.78% | 108% | 17001% | 1% |

|  | Low Delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Bit Diff |
| Class C | -4.31% | -4.18% | -1.39% | 120% | 8830% | 1% |
| Class D | -4.76% | 4.90% | 5.59% | 109% | 16276% | 1% |

Conclusions

This contribution presents a CNN-based in-loop filtering method. The student network using the proposed KD-based training method showed better performance in terms of the processing time than the teacher network while yielding a minimal BD-rate decrease.

References

- E. Alshina, S.Liu, A. Segall, J. Chen, F. Galpin, J. Pfaff, S. S. Wang, Z. Wang, M. Wien, P. Wu, J. Xu. JVET AHG report: Neural network-based video coding (AHG11), JVET-W0011(2021)

- D.-W. Kim, J.-R. Jung, and S.-W. Jung. “GRDN: Grouped residual dense network for real image denoising and GAN-based real-world noise modeling.” In Proc. CVPR Workshops (2019)

- S. Cho, J. Lee, J. Kim, Y. Kim, D.-W. Kim, J.-R. Jung, and S.-W. Jung. “Low bit-rate image compression based on post-processing with grouped residual dense network.” In Proc. CVPR Workshops (2019)

- S. Woo, J. Park, J.-Y. Lee, I. Kweon. “CBAM: Convolutional block attention module.” In Proc. ECCV (2018)

- S. Zagoruyko, N. Komodakis. “Paying more attention to attention: Improving the performance of convolutional neural networks via attention transfer.” In Proc. ICLR (2017)

- S. Liu, A. Segall, E. Alshina, R. -L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-V2016(2021)

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A training database for deep video compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

Patent rights declaration(s)

Korea Univ. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).