| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0047-v1 |
| --- | --- |

| Title: | EE2-4.7: Joint test on CCALF and chroma-bilateral filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Ru-Ling Liao Jie Chen Yan Ye Xinwei Li 525 Almanor Ave, Sunnyvale, CA Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com yinwenbin.hit@bytedance.com |
| Source: | Alibaba Group, Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of test 4.7. Test 4.7 is the combination of chroma bilateral filter and CCALF tests described in test 4.1 and 4.2/4.3, respectively. Following results are reported:

- Test 4.7.a: Test 4.1 + Test 4.2.a

  - AI: 0.03% (Y), -2.60% (U), -3.35% (V),100% (EncTime), 102% (DecTime)

  - RA: 0.02% (Y),	-2.28% (U),	-2.43% (V),	100% (EncTime), 101% (DecTime)

  - LB: 0.01% (Y),	-3.69% (U),	-3.83% (V),	101% (EncTime), 101% (DecTime)

- Test 4.7.b: Test 4.1 + Test 4.3.a

  - AI: 0.05% (Y),	-2.71% (U), -3.52% (V), 100% (EncTime), 103% (DecTime)

  - RA: 0.01% (Y),	-2.19% (U), -2.37% (V), 100% (EncTime), 101% (DecTime)

  - LB: -0.01% (Y), -3.44% (U), -3.34% (V), 101% (EncTime), 102% (DecTime)

# Introduction

In last JVET meeting, the improvement of CCALF methods were proposed in JVET-W0079. On the other hand, the chroma bilateral filter is introduced in JVET-W0098. The modification of chroma bilateral filter and CCALF are studied in test 4.1 and 4.2/4.3, respectively. In this contribution, both methods are combined.

# Test 4.1: Chroma bilateral filter

In test 4.1, Bilateral Inloop filter on Chroma (BIF-chroma) is proposed. Same as BIF-luma, the BIF-chroma is also performed in parallel with the SAO and CCSAO process as shown in Figure 2-1. BIF-chroma, CCSAO and SAO use the same chroma samples produced by the deblocking filter as input and generate three offsets per chroma sample in parallel. Then these three offsets are added to the input chroma sample to obtain a sum, which is then clipped to form the final output chroma sample value. The proposed BIF-chroma provides an on/off control mechanism on CTU level and slice level.

Figure 2-1 Filtering stage of BIF-chroma

# Test 4.2.a: Multi-shape CCALF filters

In test 4.2.a, two filter shapes are proposed. The selection of shape is in the CTB level. In this test, for a given slice, the encoder can collect the statistics of the slice, analyze those and can signal up to 16 filters through APS. For each filter, the shape index (shape_idx in Figure 2) and associated filter coefficients are signalled. For each CTB, an index is signalled to the decoder to specify which filter shape and coefficients are used for that CTB.

Figure 3-1: CCALF filter shapes used in test 4.2

# Test 4.3.a: Long tap CCALF filter

In test 4.3, a 25-tap large filter is used. For a given slice, the encoder can collect the statistics of the slice, analyze those and can signal up to 16 filters through APS.

Figure 4-1: A 25-tap long tap filter

# Test 4.7: Combined test

Following two tests are conducted.

- Test 4.7.a: test 4.1 + Test 4.2.a

- Test 4.7.b: test 4.1 + Test 4.3.a

The simulation results are shown in Table 1 and Table 2.

Table 1: results of test 4.7.a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -1.76% | -4.31% | 100% | 101% |
| Class A2 | 0.14% | -3.44% | -4.80% | 100% | 101% |
| Class B | 0.08% | -4.19% | -3.42% | 100% | 102% |
| Class C | -0.05% | -1.44% | -1.46% | 100% | 103% |
| Class E | -0.02% | -1.49% | -3.34% | 101% | 104% |
| Overall | 0.03% | -2.60% | -3.35% | 100% | 102% |
| Class D | -0.08% | -1.55% | -1.90% | 100% | 103% |
| Class F | 0.02% | -2.74% | -1.93% | 99% | 103% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -1.23% | -2.19% | 100% | 100% |
| Class A2 | 0.01% | -2.72% | -3.96% | 99% | 100% |
| Class B | 0.03% | -3.17% | -2.50% | 100% | 100% |
| Class C | -0.01% | -1.63% | -1.37% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -2.28% | -2.43% | 100% | 101% |
| Class D | 0.05% | -1.84% | -1.90% | 100% | 102% |
| Class F | 0.03% | -1.59% | -1.37% | 100% | 102% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -4.24% | -3.54% | 99% | 100% |
| Class C | -0.08% | -3.01% | -2.63% | 102% | 103% |
| Class E | 0.07% | -3.70% | -5.91% | 102% | 102% |
| Overall | 0.01% | -3.69% | -3.83% | 101% | 101% |
| Class D | 0.19% | -3.74% | -3.16% | 102% | 102% |
| Class F | -0.08% | -3.97% | -2.38% | 101% | 102% |
| Class TGM |  |  |  |  |  |

Table 2: results of test 4.7.b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -1.70% | -4.61% | 100% | 102% |
| Class A2 | 0.16% | -3.76% | -5.48% | 100% | 102% |
| Class B | 0.09% | -4.18% | -3.33% | 100% | 102% |
| Class C | -0.04% | -1.44% | -1.56% | 100% | 104% |
| Class E | 0.00% | -1.90% | -3.38% | 100% | 105% |
| Overall | 0.05% | -2.71% | -3.52% | 100% | 103% |
| Class D | -0.07% | -1.38% | -1.77% | 100% | 104% |
| Class F | 0.03% | -2.49% | -1.75% | 99% | 103% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -1.13% | -2.28% | 100% | 100% |
| Class A2 | 0.03% | -2.69% | -4.27% | 100% | 100% |
| Class B | 0.00% | -3.05% | -2.16% | 100% | 100% |
| Class C | 0.00% | -1.52% | -1.26% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -2.19% | -2.37% | 100% | 101% |
| Class D | 0.03% | -1.15% | -1.57% | 101% | 102% |
| Class F | 0.02% | -1.35% | -0.99% | 100% | 102% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -3.85% | -3.12% | 100% | 101% |
| Class C | -0.09% | -2.81% | -2.34% | 103% | 103% |
| Class E | 0.04% | -3.61% | -5.02% | 102% | 102% |
| Overall | -0.01% | -3.44% | -3.34% | 101% | 102% |
| Class D | 0.14% | -2.90% | -3.00% | 102% | 102% |
| Class F | -0.06% | -3.55% | -2.03% | 101% | 103% |
| Class TGM |  |  |  |  |  |

# Conclusions

This contribution presents a combination test of EE2-4.1 and EE2-4.2/4.3. The proposed methods contribute promising coding gains on chroma components while almost no harm on luma component. The complexity increase is limited. It is recommended to adopt them into ECM.

# References

M. G. Sarwer, R. -L. Liao, J. Chen, Y. Ye, X. Li, “AHG12: CTB level filter shape selection of CCALF”, JVET-W0079, July 2021.

V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-W2024, July 2021.

W. Yin, K. Zhang, L. Zhang, “Non-EE2: Bilateral Inloop Filter on Chroma”, JVET-W0098, July 2021.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).