| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0049-v1 |
| --- | --- |

| Title: | EE2: Adaptive decoder side motion vector refinement (test 3.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Zhi Zhang, Vadim Seregin, Wei-Jung Chien, Chun-Chi Chen, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports test results of EE2 test 3.4 for adaptive decoder side motion vector refinement, where the MV refinement at PU level is done for either L0 or L1 direction of bi prediction. The first test was conducted on top of ECM-2.0, and it reports -0.27%, -0.18%, -0.20% BD rate for luma and chroma components, respectively, with 103% encoder and 95% decoder run time in random access configurations of common test condition. The second test was conducted on top of ECM-2.0 with TM related tools disabled, and it reports -0.84%, -0.70%, -0.75% BD rate for luma and chroma components, respectively, with 106% encoder and 97% decoder run time in random access configurations.

# Introduction

In ECM-2.0, a multi-pass decoder-side motion vector refinement (DMVR) method is applied in regular merge mode if the selected merge candidate meets the DMVR conditions. In the first pass, bilateral matching (BM) is applied to the coding block. In the second pass, BM is applied to each 16x16 subblock within the coding block. In the third pass, MV in each 8x8 subblock is refined by applying bi-directional optical flow (BDOF).

Similar to the DMVR in VVC, the BM refined a pair of motion vectors MV0 and MV1 under the constrain that MVD0 (MV0’-MV0) is just the opposite sign of MVD1 (MV1’-MV1), as illustrated in Fig. 1.

Fig. 1 Example of bilateral matching

# Proposed method

As was proposed earlier in JVET-W0107, adaptive decoder side motion vector refinement method consists of the two new merge modes introduced to refine MV only in one direction, either L0 or L1, of the bi prediction for the merge candidates that meet the DMVR conditions. The multi-pass DMVR process is applied for the selected merge candidate to refine the motion vectors, however either MVD0 or MVD1 is set to zero in the 1st pass (i.e. PU level) DMVR.

Like the regular merge mode, merge candidates for the proposed merge modes are derived from the spatial neighboring coded blocks, TMVPs, non-adjacent blocks, HMVPs, and pair-wise candidate.  The difference is that only those meet DMVR conditions are added into the candidate list. The same merge candidate list is used by the two proposed merge modes and merge index is coded as in regular merge mode.

# Test results

The first test was conducted on top of ECM-2.0, and it reports -0.27%, -0.18%, -0.20% BD rate for luma and chroma components, respectively, with 103% encoder and 95% decoder run time in random access configurations of common test condition.

The second test was conducted on top of ECM-2.0 with TM related tools (TM_AMVP, TM_MRG, JVET_W0097_GPM_MMVD_TM, and JVET_W0090_ARMC_TM) disabled, and it reports -0.84%, -0.70%, -0.75% BD rate for luma and chroma components, respectively, with 106% encoder and 97% decoder run time in random access configurations.

Details of the test results are shown in Table 1 and Table 2, respectively.

Table 1. Proposed method

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.15% | -0.26% | 103% | 96% |
| Class A2 | -0.27% | -0.18% | -0.16% | 103% | 94% |
| Class B | -0.21% | -0.13% | -0.17% | 103% | 95% |
| Class C | -0.33% | -0.27% | -0.23% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.18% | -0.20% | 103% | 95% |
| Class D | -0.34% | -0.41% | -0.11% | 102% | 94% |
| Class F | -0.10% | -0.10% | -0.07% | 103% | 98% |

Table 2. Proposed method with template matching tools disabled

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 TM Off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.81% | -0.53% | -0.68% | 106% | 98% |
| Class A2 | -0.97% | -0.69% | -0.78% | 105% | 96% |
| Class B | -0.68% | -0.67% | -0.61% | 106% | 97% |
| Class C | -0.96% | -0.88% | -0.95% | 106% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.84% | -0.70% | -0.75% | 106% | 97% |
| Class D | -1.07% | -0.91% | -1.16% | 105% | 97% |
| Class F | -0.32% | -0.29% | -0.26% | 104% | 100% |

In the implementation of EE2-3.4, a lossless software optimization was applied. The impact of that software optimization applied to the anchor is provided in Table 3.

Table 3. Impact of the software optimization

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class B | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 98% |

# Conclusion

In this contribution, the test results of EE2-3.4 are reported: -0.27%, -0.18%, -0.20% BD rate for luma and chroma components, respectively, with 103% encoder and 95% decoder run time in random access configurations of common test condition. It is recommended to adopt the proposed method in the next version of ECM.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).