| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24rd Meeting, by teleconference, 7–16 Oct. 2021 | Document: JVET-X0058-v1 |
| --- | --- |

| Title: | AHG12: Bilateral matching SMVD mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Donghyun Kim Hyung Jin Kwon Jongho Kim Woong Lim Se Yoon Jeong | Tel: Email: | kimddng@etri.re.kr kwonjin@etri.re.kr pooney@etri.re.kr woong.lim@etri.re.kr jsy@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

- Abstract

In this contribution, bilateral matching Symmetric MVD mode (BM-SMVD) is proposed as a sub-mode of SMVD. In the proposed method, only MVP indices for list0 and list1 are signaled and MVs are refined by minimizing bilateral matching error. Performance of the proposed method on top of VTM-13.0 and ECM-2.0 are reported as follows.

- Test 1. BM-SMVD on VTM-13.0
 : -0.08% (Y), -0.03% (U), - 0.08% (V), 101% (EncT), 100% (DecT)

- Test 2. BM-SMVD on ECM-2.0 (TM-off)
 : -0.xx% (Y), -0.xx% (U), - 0.xx% (V), xxx% (EncT), xxx% (DecT)

Introduction

Since SMVD and BM have similar enabling condition and common properties as below, this contribution proposes BM-SMVD mode as a sub-mode of SMVD mode.

- Both SMVD and BM requires reference pictures with different temporal directions and equal POC distance.

- For SMVD, MVD0 is signaled and MVD1 is set equal to (-MVD0).

- For BM, MV_offset is derived by minimizing bilateral matching (BM) error. MV0’ and MV1’ are set equal to (MV0 + MV_offset) and (MV1 – MV_offset), respectively.

Proposed method

Whereas mvp_l0_flag, mvp_l1_flag, and MVD0 are signaled in the current SMVD of VVC, only mvp_l0_flag and mvp_l1_flag are explicitly signaled in BM-SMVD. In BM-SMVD mode, starting from MVP, the bilateral matching refinement is applied and MV with the minimum bilateral matching cost is selected as a final MV. In the proposed mode, the indices of AMVR and BCW are set to default indices (IMV_OFF and BCW_DEFAULT, respectively).

Test 1: On VTM-13.0

On VTM-13.0, decoder-side motion vector refinement (DMVR) of the VVC without weighting on DMVR cost at initial MV is used for BM refinement and Bi-Directional Optical Flow (BDOF) is used for further refinement. BM-SMVD mode is only enabled when the current block has a width and height are equal to or greater than 8. To avoid a latency issue due to the inter-dependency between the current block and its neighboring ones [1], MVP is used instead of the refined motion vector (MV) for the spatial MV propagation.

Test 2: On ECM-2.0

On ECM-2.0, Multi-pass DMVR (MPDMVR) is used for BM refinement. BM-SMVD is enabled without any constraint on block size. Unlike BM-SMVD on VTM-13.0, refined MVs are used for both spatial and temporal motion vector propagation. LIC and OBMC are disabled when the current block is BM-SMVD mode.

Simulation results

Performances were evaluated for random access configuration under common test conditions (CTC) [2,3].

- Test 1. BM-DMVR over VTM-13.0.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.11% | -0.05% | 101% | 100% |
| Class A2 | -0.09% | 0.00% | -0.14% | 101% | 100% |
| Class B | -0.08% | -0.02% | -0.02% | 102% | 100% |
| Class C | -0.06% | 0.01% | -0.14% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.03% | -0.08% | 101% | 100% |
| Class D | -0.11% | -0.13% | -0.07% | 100% | 99% |
| Class F |  |  |  |  |  |

- Test 2. BM-DMVR over ECM-2.0 (both anchor and test set TM_AMVP, TM_MRG, JVET_W0097_GPM_MMVD_TM, JVET_W0090_ARMC_TM to 0)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

References

[1] C.-C. Chen, etc. “EE2-related: On spatial MV propagation and neighboring template block access for template matching and multi-pass DMVR,” JVET-W0122, Meeting by Teleconference, July 2021.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, by teleconference, 7-16 October. 2020.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).