| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0059-v1 |
| --- | --- |

| Title: | AHG2: Comments on the 2nd edition draft text for VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution mainly discusses the editing notes for the 2nd edition draft text of VSEI, along with a few other assertedly minor issues, mainly for triggering discussions at the meeting, with suggestions for some of the items.

# On persistency flag for the annotated regions SEI message

At the end of clause 8.18.1 is the following editing note:

[Ed. (YK): A persistency flag was added to the annotated regions SEI message syntax in HEVC. Shouldn't the same be added herein?]

It was was agreed at the JVET-T meeting in October 2020 not to add the persistency flag. Thus this editing note can be removed.

# On semantics of the alpha channel information SEI message

Clause 8.22.2 has the following text, including an editing note:

...

When an access unit contains an auxiliary picture picA in a layer that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

[Ed. (YK): Check the consistency between these four items and the three items below in the semantics of alpha_channel_cancel_flag.]

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

...

The four items are for specifying the persistency scope of the alpha channel sample values, while the three items are for specifying the persistency scope of the SEI message. The persistency scope of the alpha channel sample values uses four items because there is one item based on the associated primary layer, while the other three items are aligned with the other case. Thus there is no issue, and the editing note can be removed.

On the other hand, a minor bug was noticed, on the use of the variable nuhLayerIdA without definition. The following highlighted addition would fix that:

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

# On semantics of alpha_channel_use_idc

In the semantics of alpha_channel_use_idc, there are two editing notes, as follows:

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. [Ed. (GJS): Should we have a specific blending formula for this? I think we had a formula in some standard before - perhaps in H.263 or AVC.] Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 2. [Ed. (GJS): Is the value 2 useful?]

Regarding the first editing note: Indeed, in AVC, clause 7.4.2.1.2, Sequence parameter set extension RBSP semantics, the alpha blending composition using some equations is described in a NOTE.

Regarding the second editing note: Good question. The value 2 alpha_channel_use_idc does not seem to be really useful. If the encoder puts the value to be 2 such that the SEI message is not useful anyway, then it would probably have been better that the encoder did not include the SEI message at the first place. However, since this is already the case in HEVC, maybe it's slightly better not to change it.

# On semantics of alpha_channel_bit_depth_minus8, alpha_transparent_value, and alpha_opaque_value

The semantics of these three syntax elements are as follows, with two editing notes:

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 plus 8 shall be equal to the bit depth of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9. [Ed. (GJS): Couldn’t we use one less bit than this?]

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9. The value of alpha_opaque_value shall be greater than or equal to alpha_transparent_value. [Ed. (GJS): Please check the added sensibility constraint.]

Regarding the first editing note: Good question again. There seems to be more than one aspect problematic regarding the semantics of these three syntax elements: 1) The length of alpha_channel_bit_depth_minus8 is 3 bits, thus its value range is 0..7, therefore the bit depth of the auxiliary picture can only be 8..15 bits, not possible to support 16 bits. 2) The value range for alpha_transparent_value, and alpha_opaque_value is 9..16; seems not making sense not to be aligned with the bit depth range for the auxiliary picture itself.

Regarding the second editing note: Yes, the added sensibility constraint looks correct and good to me.

# On semantics of alpha_channel_clip_type_flag

The semantics of are as follows, with two editing notes:

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value − alpha_transparent_value ) / 2 [Ed. (GJS): Shouldn’t that minus be a plus?] is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value − alpha_transparent_value ) / 2 [Ed. (GJS): Shouldn’t that minus be a plus?] is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary picture luma sample.

Regarding both editing notes: Indeed, it seems a plus would make more sense in both places.

# On semantics of the multiview view position SEI message

The beginning of the semantics is as follows:

The multiview view position SEI message specifies the relative view position along a single horizontal axis of views within a CVS. When present, the multiview view position SEI message shall be associated with an IRAP access unit. The information signalled in this SEI message applies to the entire CVS.

When a CVS does not contain an SDI SEI message, the CVS shall not contain a multiview view position SEI message.

Due to the nature of this SEI message being close to that of the MAI SEI message, in the sense that their persistency scope is CVS, including all the views, it is suggested to make the following changes to align the description:

The multiview view position SEI message specifies the relative view position along a single horizontal axis of views within a CVS.

When present, the multiview view position SEI message shall be associated with an IRAP access unit. The information signalled in this SEI message applies to the entire CVS.

When a multiview view position SEI message is present in any AU of a CVS, a multiview view position SEI message shall be present for the first AU of the CVS. All multiview view position SEI messages in a CVS shall have the same content.

When a CVS does not contain an SDI SEI message, the CVS shall not contain a multiview view position SEI message.

When an AU contains both an SDI SEI message and a multiview view position SEI message, the SDI SEI message shall precede the multiview view position SEI message in decoding order.

Some of the views for which the view position information is included in a multiview view position SEI message may not be present in the current CVS.

# Patent rights declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the authors).