| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0067-v1 |
| --- | --- |

| Title: | EE2-4.1: Bilateral Inloop Filter on Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of EE2-4.1, extending Bilateral In-loop Filter (BIF) in ECM to chroma components. BIF-chroma keeps the same design as BIF-luma in ECM. A diamond 5×5 filtering kernel and a CTU level on/off control mechanism are used. The filtering coefficients are retrained for chroma components and the filtering strength for chroma components is decided based on the chroma block size as well as the corresponding luma block size.

Simulation results are reported as below:

Tool-off tests (ECM-2.0 as the anchor):

EE2-Test 4.1:

AI: -0.01%, -0.73%, -0.93%, 100%, 102%;

RA: 0.00%, -0.51%, -0.45%, 100%, 101%;

LB: -0.02%, -0.91%, -1.07%, 100%, 100%.

# Introduction

In JVET-W0098 [1], Bilateral In-loop filter on chroma (BIF-chroma) was proposed. The EE test on BIF-Chroma is reported in this proposal.

# Proposed method

Same as BIF-luma, proposed BIF-chroma is also performed in parallel with the SAO and CCSAO process as shown in Fig.1. BIF-chroma, CCSAO and SAO use the same chroma samples produced by the deblocking filter as input and generate three offsets per chroma sample in parallel. Then these three offsets are added to the input chroma sample to obtain a sum, which is then clipped to form the final output chroma sample value. The proposed BIF-chroma provides an on/off control mechanism on CTU level and slice level.

Figure 1 Filtering stage of BIF-Chroma.

The filtering process of BIF-chroma is similar to that of BIF-luma. For a chroma sample, a 5×5 diamond shape filter is used for generating the filtering offset. The difference between the central sample and each surrounding sample is calculated first. The coefficient for each reference sample is extracted from a pre-defined look-up-table based on the calculated difference directly. The coefficients used for chroma components are retrained, different from those from BIF-luma. In the BIF-luma design, the block-level filtering strength parameter c is determined based on luma TU size and CU mode. While in the BIF-chroma design, the parameter for chroma components is determined based the chroma TU size and mode when dual-tree partitioning is enabled for the current slice, and based on the corresponding luma TU size and mode when dual-tree partitioning is disabled.

# Experimental results

The proposed 	BIF-Chroma method is tested on top of ECM-2.0 [2], according to the common test conditions defined in [3].

Table 1: Performance of the tool-off test of EE2-Test 4.1.

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.55% | -1.15% | 99% | 102% | 0.01% | -0.46% | -0.51% | 101% | 100% |  |  |  |  |  |
| Class A2 | 0.06% | -0.78% | -0.72% | 102% | 102% | 0.03% | -0.57% | -0.41% | 101% | 100% |  |  |  |  |  |
| Class B | 0.00% | -0.80% | -0.95% | 100% | 101% | -0.04% | -0.41% | -0.42% | 100% | 101% | 0.00% | -0.72% | -0.66% | 99% | 100% |
| Class C | -0.05% | -0.85% | -0.83% | 100% | 103% | -0.01% | -0.64% | -0.49% | 100% | 101% | -0.10% | -0.66% | -0.92% | 101% | 101% |
| Class E | 0.01% | -0.62% | -1.03% | 99% | 102% |  |  |  |  |  | 0.05% | -1.57% | -1.97% | 100% | 99% |
| Overall | -0.01% | -0.73% | -0.93% | 100% | 102% | 0.00% | -0.51% | -0.45% | 100% | 101% | -0.02% | -0.91% | -1.07% | 100% | 100% |
| Class D | -0.10% | -0.66% | -0.75% | 100% | 102% | 0.03% | -0.71% | -0.54% | 99% | 100% | 0.06% | -0.85% | -1.01% | 98% | 100% |

# Conclusions

This contribution presents BIF-Chroma. The proposed method contributes promising coding gains on chroma components while has no harm on Luma component. It is proposed to adopt the BIF-Chroma method into ECM.

# References

- W. Yin, K. Zhang, L. Zhang, “Non-EE2: Bilateral Inloop Filter on Chroma,” JVET-W0098, July, 2021.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).