| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0069-v1 |
| --- | --- |

| Title: | EE2-4.9: A combining test of EE2-4.1 and EE2-4.4/4.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 Nan Hu3, Vadim Seregin3, Marta Karczewicz3 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA 35775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com nanh@qti.qualcomm.com |
| Source: | Bytedance Inc. Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of EE2-4.9, which are combining tests of EE2-4.1 (Chroma Bilateral In-loop Filter) and EE2-4.4 (alternative 2x2 classifier for ALF) /4.5 (alternative sample-based classifier for ALF). On top of ECM-2.0, simulation results (BD-rate changes, encoding time and decoding time) are reported as below:

EE2-4.9a (Combination of 4.1 and 4.4):

AI: -0.17%, -0.69%, -0.89%, 101%, 101%.

RA: -0.37%, -0.37%, -0.33%, 100%, 101%.

LB: -0.56%, -1.02%, -1.01%, 99%, 101%.

EE2-4.9b (Combination of 4.1 and 4.5):

AI: -0.22%, -0.67%, -0.87%, 101%, 100%.

RA: -0.41%, -0.40%, -0.29%, 100%, 100%.

LB: -0.62%, -0.83%, -1.09%, 99%, 99%.

# Introduction

In JVET-W0098 [1], a Bilateral In-loop Filter on Chroma (BIF-Chroma) was proposed. In JVET-W0128 [2], alternative classifiers for ALF were proposed. EE tests on a combination of methods in JVET-W0098 (EE2-4.1) and JVET-W0128 (EE2-4.4/4.5) are reported in this contribution.

# Proposed methods

Two combining tests are investigated in this contribution as below:

| # | Test |
| --- | --- |
| 4.9a | Combination of 4.1 and 4.4 (BIF-Chroma and alternative 2x2 classifier for ALF) |
| 4.9b | Combination of 4.1 and 4.5 (BIF-Chroma and alternative sample-based classifier for ALF) |

# Experimental results

The proposed combinations are tested on top of ECM-2.0 [3], according to the common test conditions defined in [4]. The tool-off test (based on ECM-2.0 with all new features on as the anchor) results of EE2-4.9a and EE2-4.9b are tabulated in Table 1 and Table 2, respectively.

TABLE 1 Performance of the tool-off test in EE2-Test4.9a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.51% | -1.10% | 103% | 101% | -0.29% | -0.21% | -0.39% | 102% | 100% |  |  |  |  |  |
| Class A2 | -0.03% | -0.75% | -0.69% | 104% | 101% | -0.25% | -0.46% | -0.32% | 102% | 100% |  |  |  |  |  |
| Class B | -0.17% | -0.75% | -0.90% | 101% | 100% | -0.60% | -0.31% | -0.26% | 101% | 102% | -0.91% | -0.60% | -0.61% | 101% | 101% |
| Class C | -0.24% | -0.84% | -0.82% | 99% | 101% | -0.25% | -0.48% | -0.37% | 98% | 100% | -0.30% | -1.04% | -1.03% | 96% | 101% |
| Class E | -0.16% | -0.53% | -0.94% | 99% | 100% |  |  |  |  |  | -0.30% | -1.69% | -1.66% | 102% | 100% |
| Overall | -0.17% | -0.69% | -0.89% | 101% | 101% | -0.37% | -0.37% | -0.33% | 100% | 101% | -0.56% | -1.02% | -1.01% | 99% | 101% |
| Class D | -0.14% | -0.66% | -0.75% | 97% | 104% | -0.12% | -0.82% | -0.63% | 92% | 101% | -0.06% | -1.27% | -1.40% | 89% | 101% |

TABLE 2 Performance of the tool-off test in EE2-Test4.9b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.50% | -1.09% | 103% | 101% | -0.34% | -0.32% | -0.34% | 102% | 100% |  |  |  |  |  |
| Class A2 | -0.10% | -0.74% | -0.68% | 103% | 101% | -0.34% | -0.49% | -0.23% | 102% | 100% |  |  |  |  |  |
| Class B | -0.24% | -0.72% | -0.86% | 101% | 99% | -0.62% | -0.30% | -0.17% | 101% | 101% | -0.98% | -0.59% | -0.76% | 101% | 100% |
| Class C | -0.25% | -0.84% | -0.81% | 99% | 101% | -0.25% | -0.51% | -0.45% | 97% | 100% | -0.30% | -0.73% | -0.76% | 96% | 100% |
| Class E | -0.22% | -0.48% | -0.90% | 100% | 100% |  |  |  |  |  | -0.45% | -1.35% | -2.09% | 102% | 99% |
| Overall | -0.22% | -0.67% | -0.87% | 101% | 100% | -0.41% | -0.40% | -0.29% | 100% | 100% | -0.62% | -0.83% | -1.09% | 99% | 99% |
| Class D | -0.17% | -0.63% | -0.73% | 98% | 104% | -0.03% | -0.75% | -0.44% | 92% | 100% | -0.15% | -1.13% | -0.44% | 88% | 100% |

# Conclusions

This contribution presents a combination test of EE2-4.1 and EE2-4.4/4.5. The proposed methods contribute promising coding gains on all luma and chroma components and the complexity increase is limited. It is recommended to adopt one of the combinations into ECM.

# References

- W. Yin, K. Zhang and L. Zhang, “Non-EE2: Bilateral Inloop Filter on Chroma,” JVET-W0098, July, 2021.

- N, Hu, V. Seregin and M. Karczewicz, “AHG12: Alternative classifiers for ALF,” JVET-W0128, July. 2021.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).