| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0070-v1 |
| --- | --- |

| Title: | EE2: Alternative classifiers for ALF (tests 4.4 and 4.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of two-classifier method for adaptive loop filter tested in EE2. For a luma filter set, a flag is added to indicate whether the classifier of ECM-2.0 or the proposed band-based classifier is applied. On top of ECM-2.0 with common test condition, simulation results of the proposed methods are reported as follows.

|  | All intra |  |  | Random access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT |
| EE2.4-4 | -0.15% | 101% | 97% | -0.38% | 101% | 101% | -0.59% | 102% | 102% |
| EE2.4-5 | -0.21% | 100% | 99% | -0.41% | 101% | 100% | -0.64% | 102% | 101% |

# Introduction

In adaptive loop filter (ALF) process of ECM-2.0, a luma coding tree block (CTB) can reference a filter set in an adaptation parameter set (APS). Based on local gradients, classification is applied to each 2x2 luma block in the CTB. For each class, a specific filter is applied among the different filters in the filter set. Based on the classification, geometric transformation, such as 90-degree rotation, diagonal or vertical flip, of the coefficients within a diamond filter shape can also be applied.

# Proposed methods

To extend the classification in ALF, in JVET-W0128, a two-classifier based ALF was proposed. A flag is added to a luma filter set in APS to indicate whether the classifier of ECM-2.0 or the one of the proposed  alternative classifiers is applied. Geometrical transformation is not applied with the proposed band classifier. The number of classes is equal to 25 as in the existed ECM ALF classifier.

## Alternative classifier 1

In the 2x2 block based band-based classifier, the sum of sample values of a 2x2 luma block is calculated at first. Then the class index is calculated as below,

class_index = (sum * 25) >> (sample bit depth + 2).

## Alternative classifier 2

In the sample band-based classifier, the class index of a luma sample is calculated as

class_index = (luma sample * 25) >> (sample bit depth).

# Results

The proposed methods are implemented on top of ECM-2.0 and tested with common test condition described in JVET-W2017.

## EE2-4.4: Alternative classifier 1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.04% | 0.05% | 103% | 100% |
| Class A2 | -0.08% | 0.02% | 0.03% | 101% | 99% |
| Class B | -0.16% | 0.05% | 0.05% | 100% | 100% |
| Class C | -0.19% | 0.00% | 0.00% | 99% | 99% |
| Class E | -0.17% | 0.09% | 0.09% | 100% | 101% |
| Overall | -0.15% | 0.04% | 0.04% | 101% | 100% |
| Class D | -0.04% | 0.00% | 0.00% | 97% | 104% |
| Class F | -0.17% | 0.04% | 0.04% | 99% | 100% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | 0.13% | 0.05% | 103% | 101% |
| Class A2 | -0.31% | 0.07% | 0.23% | 101% | 100% |
| Class B | -0.58% | 0.20% | 0.22% | 101% | 101% |
| Class C | -0.22% | 0.06% | 0.21% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | 0.12% | 0.18% | 101% | 101% |
| Class D | -0.09% | -0.21% | 0.21% | 95% | 102% |
| Class F | -0.15% | 0.06% | 0.08% | 103% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.95% | 0.09% | -0.50% | 105% | 103% |
| Class C | -0.30% | -0.18% | -0.12% | 100% | 102% |
| Class E | -0.39% | -0.45% | -0.39% | 102% | 100% |
| Overall | -0.59% | -0.13% | -0.35% | 102% | 102% |
| Class D | -0.20% | 0.29% | -0.04% | 95% | 102% |
| Class F | -0.51% | -1.03% | -0.50% | 103% | 101% |

## EE2-4.5: Alternative classifier 2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | 0.05% | 0.06% | 104% | 100% |
| Class A2 | -0.14% | 0.04% | 0.04% | 102% | 98% |
| Class B | -0.23% | 0.08% | 0.09% | 100% | 98% |
| Class C | -0.20% | 0.00% | 0.00% | 99% | 100% |
| Class E | -0.22% | 0.13% | 0.13% | 100% | 98% |
| Overall | -0.21% | 0.06% | 0.06% | 100% | 99% |
| Class D | -0.06% | 0.02% | 0.02% | 97% | 105% |
| Class F | -0.59% | 0.24% | 0.24% | 99% | 100% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | 0.09% | -0.04% | 102% | 101% |
| Class A2 | -0.36% | 0.08% | 0.22% | 101% | 99% |
| Class B | -0.61% | 0.26% | 0.32% | 101% | 100% |
| Class C | -0.24% | 0.03% | 0.26% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.41% | 0.13% | 0.21% | 101% | 100% |
| Class D | 0.02% | -0.08% | 0.26% | 95% | 102% |
| Class F | -0.53% | 0.15% | 0.06% | 103% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.03% | 0.04% | -0.17% | 103% | 102% |
| Class C | -0.28% | -0.13% | 0.08% | 100% | 102% |
| Class E | -0.49% | -0.24% | -0.92% | 102% | 100% |
| Overall | -0.64% | -0.09% | -0.27% | 102% | 101% |
| Class D | -0.22% | -0.41% | -0.47% | 93% | 101% |
| Class F | -0.81% | -0.72% | -0.71% | 102% | 101% |

In EE2-4.5, the frequency of filter switch using band classifier in 2x2 luma blocks is summarized in the table below. The ratio is calculated by dividing the number of 2x2 blocks having different filters by the total number of 2x2 blocks using band classifier.

|  | AI | RA | LDB |
| --- | --- | --- | --- |
| Class A1 | 8.96% | 3.15% |  |
| Class A2 | 19.09% | 5.70% |  |
| Class B | 11.35% | 5.72% | 5.47% |
| Class C | 8.47% | 2.24% | 3.07% |
| Class E | 5.33% |  | 3.17% |
| Overall | 10.49% | 4.27% | 3.98% |
| Class D | 4.20% | 4.45% | 5.17% |
| Class F | 19.90% | 11.42% | 6.16% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).