| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0071-v1 |
| --- | --- |

| Title: | EE2-4.8: Joint tests of chroma BIF, ALF and CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA Mohammed Golam Sarwer, Ru-Ling Liao, Jie Chen, Yan Ye, Xinwei Li 525 Almanor Ave, Sunnyvale, CA, USA Wenbin Yin Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | nanh@qti.qualcomm.com m.sarwer@alibaba-inc.com yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Qualcomm Incorporated, Alibaba Group, Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-4.8, which is the combination of all in-loop filter tests in EE2 including chroma bilateral in-loop filters (EE2-4.1), multi-shape cross-component adaptive loop filters (EE2-4.2 and EE2-4.3) and alternative classifiers for adaptive loop filters (EE2-4.4/EE2-4.5). On top of ECM-2.0 with common test condition, simulation results of the joint tests are reported as below.

EE2-4.8a (Combination of 4.1, 4.2a and 4.4):

AI:     -0.13%, -2.56%, -3.31%, 102%, 102%.

RA:    -0.36%, -2.15%, -2.33%, 101%, 100%.

LDB:   -0.54%, -3.65%, -3.89%, 102%, 102%.

EE2-4.8b (Combination of 4.1, 4.2a and 4.5):

AI:    -0.19%, -2.53%, -3.29%, 102%, 101%.

RA:   -0.39%, -2.16%, -2.28%, 101%, 100%.

LDB:  -0.63%, -3.65%, -3.86%, 103%, 101%.

EE2-4.8c (Combination of 4.1, 4.3a and 4.4):

AI:     -0.11%, -2.67%, -3.48%, 101%, 101%.

RA:    -0.36%, -2.04%, -2.20%, 101%, 100%.

LDB: 	 -0.57%, -3.38%, -3.36%, 101%, 101%.

EE2-4.8d (Combination of 4.1, 4.3a and 4.5):

AI:     -0.17%, -2.65%, -3.45%, 101%, 100%.

RA:    -0.40%, -2.02%, -2.19%, 101%, 100%.

LDB: 	 -0.64%, -3.32%, -3.35%, 100%, 99%.

# Introduction

In last JEVT meeting, improved in-loop filters were proposed and studied in EE2. Firstly, chroma bilateral in-loop filters (BIF) were proposed in JVET-W0098 and tested in EE2-4.1. Secondly, to improve cross-component adaptive loop filter (CCALF), multiple filter shapes were proposed in JVET-W0079 and tested in EE2-4.2 and EE2-4.3. At last, two-classifier adaptive loop filters (ALF) were proposed in JVET-W0128 and tested in EE2-4.4 and EE2-4.5. In this contribution, results of combined in-loop filter tests are reported.

# Proposed methods

## Test 4.1: chroma BIF

In EE2-4.1, BIF is extended to chroma components. For each chroma sample, chroma BIF, cross-component sample adaptive offset and chroma sample adaptive offset are applied in parallel by using chroma samples processed by deblocking filters. Three offsets are generated and added to the current chroma sample. On/off chroma BIF flags can be signalled at coding tree block (CTB) level and slice level.

Similar to luma BIF, a 5×5 diamond shape BIF is applied to a chroma sample. The difference between current sample and each surrounding sample is calculated first. The coefficients are extracted from a pre-defined look-up-table based on the difference. The coefficients are retrained for chroma BIF. In luma BIF, a filtering strength parameter is determined based on luma transform size and coding unit mode. While dual-tree partitioing is enabled, the parameter of chroma BIF is determined based on chroma transform size and mode. When dual-tree partitioing is disabled, the parameter is derived based on corresponding luma transform size and mode.

## Test 4.2a: multi-shape CCALF

In EE2-4.2a, two filter shapes are tested. The selection of shape is signalled in the CTB level. In this test, up to 16 filters can be signalled in an APS. For each filter, the shape index (shape_idx in Figure 1) and associated filter coefficients are signalled. For each CTB, a filter index is signalled to specify which filter shape and coefficients are used for that CTB.

Figure 1: CCALF filter shapes used in test 4.2

## Test 4.3a: long tap CCALF

In EE2-4.3a, a 25-tap large filter is used. Up to 16 filters can be signalled in an APS.

Figure 2: A 25-tap long tap filter

## Test 4.4: alternative 2x2 ALF classifier

In test 4.4, two-classifier based ALF is applied. For a signalled luma filter set, a flag is signalled to indicate whether the classifier in ECM-2.0 or the proposed band-based classifier is applied. When the band-based classifier is applied, the sum of sample values of a 2x2 luma block is calculated at first. Then the class index is calculated as below,

class_index = (sum * 25) >> (sample bit depth + 2).

## Test 4.5: alternative sample-based ALF classifier

Similar to test 4.4, in test 4.5, a band-based classifier is applied as a second classifier. When the band-based classifier is applied, the class index of a luma sample is calculated as

class_index = (luma sample * 25) >> (sample bit depth).

## Combined tests

Four combined tests are performed in EE-2.4.8 as below:

| # | Test |
| --- | --- |
| 4.8a | Combination of 4.1, 4.2a and 4.4 (chroma BIF, multi-shape CCALF and alternative 2x2 classifier for ALF) |
| 4.8b | Combination of 4.1, 4.2a and 4.5 (chroma BIF, multi-shape CCALF and alternative sample-based classifier for ALF) |
| 4.8c | Combination of 4.1, 4.3a and 4.4 (chroma BIF, long tap CCALF and alternative 2x2 classifier for ALF) |
| 4.8d | Combination of 4.1, 4.3a and 4.5 (chroma BIF, long tap CCALF and alternative sample-based classifier for ALF) |

# Results

The combined tests are implemented on top of ECM-2.0 and tested with common test condition described in JVET-W2017.

## Test 4.8a: Test 4.1 + Test 4.2a + Test 4.4

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -1.72% | -4.27% | 105% | 105% |
| Class A2 | 0.05% | -3.41% | -4.77% | 103% | 100% |
| Class B | -0.09% | -4.14% | -3.37% | 101% | 100% |
| Class C | -0.25% | -1.43% | -1.46% | 100% | 101% |
| Class E | -0.19% | -1.40% | -3.26% | 102% | 105% |
| Overall | -0.13% | -2.56% | -3.31% | 102% | 102% |
| Class D | -0.12% | -1.55% | -1.90% | 97% | 107% |
| Class F | -0.15% | -2.69% | -1.89% | 101% | 103% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -1.12% | -2.22% | 103% | 101% |
| Class A2 | -0.26% | -2.52% | -3.82% | 102% | 100% |
| Class B | -0.56% | -3.08% | -2.33% | 102% | 102% |
| Class C | -0.24% | -1.50% | -1.28% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -2.15% | -2.33% | 101% | 100% |
| Class D | -0.10% | -1.79% | -1.99% | 93% | 100% |
| Class F | -0.17% | -1.61% | -1.22% | 103% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.84% | -4.42% | -3.66% | 104% | 103% |
| Class C | -0.36% | -3.37% | -2.90% | 99% | 102% |
| Class E | -0.29% | -2.72% | -5.59% | 103% | 102% |
| Overall | -0.54% | -3.65% | -3.89% | 102% | 102% |
| Class D | 0.03% | -3.43% | -3.55% | 94% | 100% |
| Class F | -0.48% | -4.16% | -3.37% | 103% | 101% |

## Test 4.8b: Test 4.1 + Test 4.2a + Test 4.5

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -1.71% | -4.26% | 105% | 101% |
| Class A2 | -0.01% | -3.40% | -4.75% | 103% | 99% |
| Class B | -0.16% | -4.11% | -3.34% | 102% | 100% |
| Class C | -0.25% | -1.43% | -1.45% | 100% | 100% |
| Class E | -0.25% | -1.35% | -3.22% | 101% | 103% |
| Overall | -0.19% | -2.53% | -3.29% | 102% | 101% |
| Class D | -0.14% | -1.52% | -1.87% | 96% | 105% |
| Class F | -0.59% | -2.51% | -1.70% | 100% | 102% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -1.17% | -2.15% | 103% | 99% |
| Class A2 | -0.33% | -2.57% | -3.90% | 102% | 99% |
| Class B | -0.58% | -3.01% | -2.19% | 102% | 101% |
| Class C | -0.24% | -1.55% | -1.29% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | -2.16% | -2.28% | 101% | 100% |
| Class D | -0.01% | -1.74% | -2.13% | 93% | 100% |
| Class F | -0.56% | -1.59% | -1.21% | 103% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.98% | -4.62% | -3.87% | 104% | 101% |
| Class C | -0.39% | -3.34% | -2.65% | 100% | 101% |
| Class E | -0.37% | -2.45% | -5.46% | 104% | 99% |
| Overall | -0.63% | -3.65% | -3.86% | 103% | 101% |
| Class D | -0.07% | -3.71% | -3.00% | 94% | 101% |
| Class F | -0.82% | -3.71% | -2.63% | 103% | 102% |

## Test 4.8c: Test 4.1 + Test 4.3a + Test 4.4

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -1.66% | -4.56% | 104% | 103% |
| Class A2 | 0.08% | -3.73% | -5.45% | 103% | 101% |
| Class B | -0.07% | -4.13% | -3.28% | 101% | 101% |
| Class C | -0.23% | -1.43% | -1.56% | 99% | 100% |
| Class E | -0.17% | -1.81% | -3.29% | 100% | 100% |
| Overall | -0.11% | -2.67% | -3.48% | 101% | 101% |
| Class D | -0.11% | -1.37% | -1.77% | 97% | 102% |
| Class F | -0.14% | -2.45% | -1.70% | 98% | 99% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.79% | -2.19% | 103% | 101% |
| Class A2 | -0.26% | -2.65% | -4.19% | 101% | 99% |
| Class B | -0.56% | -2.89% | -1.93% | 102% | 101% |
| Class C | -0.24% | -1.44% | -1.06% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -2.04% | -2.20% | 101% | 100% |
| Class D | -0.10% | -1.40% | -1.71% | 93% | 100% |
| Class F | -0.15% | -1.34% | -0.80% | 104% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.86% | -3.97% | -3.16% | 104% | 103% |
| Class C | -0.34% | -3.03% | -2.49% | 97% | 101% |
| Class E | -0.39% | -2.85% | -4.86% | 100% | 99% |
| Overall | -0.57% | -3.38% | -3.36% | 101% | 101% |
| Class D | -0.02% | -3.29% | -2.63% | 89% | 98% |
| Class F | -0.44% | -3.61% | -2.74% | 101% | 102% |

## Test 4.8d: Test 4.1 + Test 4.3a + Test 4.5

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -1.65% | -4.55% | 105% | 103% |
| Class A2 | 0.01% | -3.72% | -5.44% | 103% | 98% |
| Class B | -0.14% | -4.10% | -3.25% | 101% | 100% |
| Class C | -0.24% | -1.42% | -1.55% | 99% | 100% |
| Class E | -0.23% | -1.76% | -3.25% | 101% | 101% |
| Overall | -0.17% | -2.65% | -3.45% | 101% | 100% |
| Class D | -0.14% | -1.35% | -1.74% | 96% | 101% |
| Class F | -0.58% | -2.26% | -1.51% | 98% | 98% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.93% | -2.22% | 103% | 100% |
| Class A2 | -0.32% | -2.57% | -4.20% | 101% | 99% |
| Class B | -0.61% | -2.81% | -1.81% | 102% | 101% |
| Class C | -0.25% | -1.45% | -1.15% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.40% | -2.02% | -2.19% | 101% | 100% |
| Class D | -0.01% | -1.28% | -1.45% | 93% | 100% |
| Class F | -0.52% | -1.35% | -0.99% | 102% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.00% | -3.94% | -2.99% | 101% | 100% |
| Class C | -0.30% | -2.96% | -2.28% | 97% | 99% |
| Class E | -0.51% | -2.76% | -5.37% | 102% | 99% |
| Overall | -0.64% | -3.32% | -3.35% | 100% | 99% |
| Class D | -0.06% | -3.57% | -2.91% | 90% | 98% |
| Class F | -0.73% | -3.39% | -2.56% | 101% | 100% |

# Conclusions

This contribution presents four combination tests of all in-loop filter tests in EE2 including chroma bilateral in-loop filters (EE2-4.1), multi-shape cross-component adaptive loop filters (EE2-4.2 and EE2-4.3) and alternative classifiers for adaptive loop filters (EE2-4.4/EE2-4.5). The proposed combination tests contribute promising coding gains on all luma and chroma components and the complexity increase is negligible. It is recommended to adopt one of the combinations into ECM.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).