| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0077 |
| --- | --- |

| Title: | EE2-3.2: GPM with inter and intra prediction (JVET-W0110) |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2-3.2 on geometric partitioning mode (GPM) with inter and intra prediction. In the EE test, the coding performance of GPM is further enhanced by applying the pre-defined intra prediction modes against geometric partitioning line to the GPM in addition to the inter prediction (merge mode). The number of the pre-defined intra prediction modes is different between EE2-3.2a and EE2-3.2b. The parallel and perpendicular angular modes against the geometric partitioning line can be available in EE2-3.2a while the planar mode can be also available besides the two modes in EE2-3.2b. Simulation results of the two tests based on ECM-2.0 (tool-off) are summarized as follow.

- EE2-3.2a based on ECM-2.0 (tool-off)

  - RA: BD-rate YUV: -0.03%, -0.15%, -0.14%; EncT: 101%; DecT: 100%

  - LB: BD-rate YUV: -0.07%, -0.25%, -0.45%; EncT: 101%; DecT: 100%

  - LP: BD-rate YUV: -0.37%, -1.18%, -1.00%; EncT: 105%; DecT: 100%

- EE2-3.2b based on ECM-2.0 (tool-off)

  - RA: BD-rate YUV: -0.02%, -0.22%, -0.10%; EncT: 101%; DecT: 100%

  - LB: BD-rate YUV: -0.08%, -0.54%, -0.40%; EncT: 101%; DecT: 100%

  - LP: BD-rate YUV: -0.45%, -1.16%, -1.18%; EncT: 105%; DecT: 100%

Introduction

In JVET-W0110 [1], geometry partition mode (GPM) with inter and intra prediction was proposed to further improve the coding efficiency of GPM in VVC. With the method, pre-defined intra prediction modes against geometric partitioning line can be selected in addition to merge candidates for each non-rectangular split region in the GPM-applied CU. In the proposed method, whether intra or inter prediction mode is determined for each GPM-separated region with a flag from the encoder. When the inter prediction mode, a uni-prediction signal is generated by MVs from the merge candidate list. On the other hand, when the intra prediction mode, a uni-prediction signal is generated from the neighboring pixels for the intra prediction mode specified by an index from the encoder. Finally, these two uni-prediction signals are blended with the same way of ordinary GPM.

Experimental Results

The proposed method was implemented on the top of ECM-2.0 [2], which are defined as the anchor software in EE2 for tool-off test [3]. Simulation conditions are followed by the common test condition (CTC) [4]. In the simulations, the following two tests are conducted to study the effect from the variation of possible intra prediction modes.

- EE2-3.2a (Method A): Pre-defined intra prediction modes are restricted to the parallel and perpendicular angular mode against geometric partitioning line.

- EE2-3.2b (Method B): Pre-defined intra prediction modes are restricted to the parallel and perpendicular angular mode against geometric partitioning line, and planar mode.

Note that the proposed method includes an implementation omission in ECM-2.0 regarding intra mode storage for GPM-applied blocks, which was tested in EE2-3-10 (the combination of EE2-3.3, EE2-3.4 and EE2-3.5) of the previous meeting [5][6] and was adopted.

Tables 1 and 2 show the EE2-3.2a and EE2-3.2b results over ECM-2.0.

Table 1 : Simulation results of EE2-3.2a over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | -0.03% | -0.19% | -0.08% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | 0.01% | -0.11% | 0.00% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.01% | -0.11% | -0.19% | 101% | 100% | -0.13% | -0.28% | -0.41% | 100% | 100% | -0.35% | -1.13% | -1.03% | 105% | 100% |
| C | -0.07% | -0.19% | -0.24% | 101% | 100% | -0.18% | -0.58% | -0.24% | 101% | 98% | -0.61% | -1.69% | -1.34% | 105% | 99% |
| E |  |  |  |  |  | 0.17% | 0.23% | -0.77% | 101% | 101% | -0.06% | -0.61% | -0.52% | 106% | 101% |
| Overall | -0.03% | -0.15% | -0.14% | 101% | 100% | -0.07% | -0.25% | -0.45% | 101% | 100% | -0.37% | -1.18% | -1.00% | 105% | 100% |
| D | 0.04% | -0.28% | 0.11% | 101% | 100% | -0.04% | -0.04% | -0.43% | 101% | 100% | -0.28% | -1.16% | -0.88% | 105% | 100% |
| F | 0.01% | -0.15% | -0.07% | 100% | 100% | 0.00% | -0.49% | -0.33% | 100% | 100% | -0.07% | -0.48% | 0.06% | 103% | 100% |

Table 2 : Simulation results of EE2-3.2b over ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | -0.02% | -0.28% | -0.06% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | 0.03% | -0.10% | 0.01% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.01% | -0.26% | -0.22% | 101% | 100% | -0.13% | -0.39% | -0.34% | 100% | 100% | -0.38% | -1.30% | -1.08% | 105% | 100% |
| C | -0.06% | -0.20% | -0.07% | 101% | 100% | -0.25% | -0.72% | -0.50% | 100% | 99% | -0.67% | -1.53% | -1.45% | 106% | 99% |
| E |  |  |  |  |  | 0.26% | -0.53% | -0.37% | 102% | 101% | -0.29% | -0.42% | -1.00% | 106% | 101% |
| Overall | -0.02% | -0.22% | -0.10% | 101% | 100% | -0.08% | -0.54% | -0.40% | 101% | 100% | -0.45% | -1.16% | -1.18% | 105% | 100% |
| D | -0.02% | -0.06% | 0.15% | 101% | 100% | -0.01% | -0.51% | -0.37% | 101% | 100% | -0.23% | -1.18% | -2.01% | 105% | 101% |
| F | 0.03% | -0.05% | -0.08% | 100% | 99% | -0.19% | -0.22% | 0.08% | 100% | 100% | -0.34% | 0.11% | 0.23% | 103% | 100% |

Supplemental Results

Tables 3 show the results of ECM-2.0 with the implementation omission described in Sec. 3. over the ECM-2.0. The coding performance is almost not changed in RA and LP configuration while the coding loss of Class E is observed in LD configuration and it affects the overall coding gain.

Table 3 : Simulation results of ECM with the implementation omission based on ECM-2.0 (tool-off).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | _DecT |
| A1 | 0.02% | -0.01% | 0.02% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.01% | -0.05% | 0.03% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.01% | -0.03% | -0.02% | 100% | 100% | -0.01% | 0.24% | 0.02% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| C | 0.00% | -0.08% | 0.08% | 100% | 100% | -0.03% | 0.23% | -0.02% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| E |  |  |  |  |  | 0.13% | -0.19% | -0.64% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | -0.05% | 0.03% | 100% | 100% | 0.02% | 0.13% | -0.16% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| D | -0.02% | -0.15% | 0.00% | 100% | 100% | 0.07% | 0.06% | -0.17% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| F | -0.02% | -0.05% | -0.01% | 100% | 100% | -0.21% | 0.08% | 0.76% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Conclusion

In this contribution, GPM with intra prediction was proposed to improve coding efficiency beyond VVC. Simulation results reportedly show EE2-3.2a and EE2-3.2b methods provide potentially -0.03%/-0.07%/-0.37% and -0.02%/-0.08%/-0.45% BD-rate savings of luma component with almost no increment encoding and decoding runtime over ECM-2.0 in the RA/LB/LP configuration. Therefore, we recommend it will be adopted in the next ECM.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- Y. Kidani, H. Kato, and K. Kawamura, “AHG12: GPM with inter and intra prediction,” JVET-W0110, by teleconference, 7–16 July 2021.

- ECM-2.0: https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/ECM/-/tree/ECM-2.0

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-W2024, by teleconference, 7–16 July 2021.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, by teleconference, 7-16 October. 2020.

- X. Xiu, C.-W. Kuo, X. Wang, R.-L. Liao, Y. Ye, X. Li, J. Chen, Z. Deng, K. Zhang, L. Zhang, N. Zhang, Y. Wang, Y.-J. Chang, H. Huang, V. Seregin, C.-C. Chen, and M. Karczewicz, “EE2-related: Combination of EE2-3.3, EE2-3.4 and EE2-3.5,” JVET-W0097, by teleconference, 7–16 July 2021.

- EE2-3-10 based on ECM-1.0 in JVET-V meeting: https://vcgit.hhi.fraunhofer.de/ecm/jvet-v-ee2/VVCSoftware_VTM/-/tree/TEST_3.3_3.4_3.5

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).